3GPP TSG-SA WG6 Meeting #59	S6-240583

Athens, Greece 26th February – 1st March 2024	(revision of S6-240566)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0051 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD Background data transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Background Data Transfer (BDT) feature enables the SEALDD service with capabilities to negotiate BDT configuration and network resource adaptation with the SEAL NRM server and transfer data at negotiated time. SEALDD can utilize its data storage capability to store the received or fetched application data until requirements are met and the data can be sent. The BDT data can be sent in uplink direction, e.g., uploading drone captured data to the VAL server from the VAL client, or in the downlink direction, e.g., streaming a video clip to the VAL client from the VAL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new SEALDD functionality, SEALDD Background data transfer, is introduced containing the procedures “SEALDD Background data transfer in DL direction”, “SEALDD Background data transfer in UL direction”, “SEALDD Background data transfer update” and “SEALDD Background data transfer delete”. Procedures, APIs and information flow for SEALDD Background data transfer (BDT) is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD service is missing capabilities for Background data transfer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 9.x, 9.x.1, 9.x.2, 9.x.2.1, 9.x.2.2, 9.x.2.3, 9.x.3, 9.x.3.1, 9.x.3.2, 9.x.3.3, 9.x.3.4, 9.x.3.5, 9.x.3.6, 9.x.3.7, 9.x.4, 9.x.4.1, 9.x.4.2, 9.x.4.3, 9.x.4.4, 9.x.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.434 CR 0292 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - Added note to clarify the usage of NRM server by SEALDD server |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

## 9.x	SEALDD Background data transfer

### 9.x.1	General

The following clauses specify procedures and information flow for SEALDD Background data transfer (BDT). The SEALDD supports BDT in downlink (DL) centric direction, e.g., streaming a video clip to the VAL client, and in uplink (UL) centric direction, e.g., uploading drone captured data to the VAL server.

### 9.x.2	Procedures

#### 9.x.2.1	SEALDD Background data transfer

Figure 9.x.2.1-1 illustrates the procedure for SEALDD Background data transfer.

Figure 9.x.2.1-1: SEALDD Background data transfer

1.	The VAL server sends a SEALDD Background data transfer subscription request to the SEALDD server. The request includes the VAL service ID and list of VAL UE IDs or VAL group ID and may also include desired time window, desired area information (e.g., when a UE enters a geographical area), Policy Selection Guidance, data volume.

2.	The SEALDD server authorizes the request from the VAL server.

3.	The SEALDD server starts the BDT configuration negotiation with the SEAL NRM server as described in clause 14.3.13 of TS 23.434 [4]. The SEALDD server performs Network resource adaptation negotiation with the SEAL NRM server as described in clause 14.3.3.3 of TS 23.434 [4]. Policies are set for the BDT data transfer using the previously negotiated BDT Reference ID.

NOTE 1:	The BDT policy can also be set for a future session via NEF_ApplyPolicy by SEALDD server at the end of the BDT negotiation.

NOTE 2:	If there is any change in the network impacting the current BDT policy (i.e. BDT warning notification from CN or trigger from the VAL server), SEALDD server attempts to perform BDT re-selection with the SEAL NRM server using the procedure described in cl 14.3.13.3 of TS 23.434 [4].

NOTE 3:	If the NRM server is a external control plane functionality for the SEALDD server, then the SEALDD server uses step 3 otherwise SEALDD server can use procedure defined in 3GPP TS 23.502 [6] clause 4.16.7.2.

4.	The SEALDD server sends the SEALDD Background data transfer subscription response, containing the result of the operation which includes the result and may include the BDT subscription identifier.

5.	The VAL server uses clause 9.5.2 procedure to sends the BDT data to the SEALDD server. During the transmission time window, the SEALDD server sends the BDT data to SEALDD client(s) and monitors the quality, e.g., latency, jitter, bitrate, packet loss rate, of the transmission. The SEALDD client forwards the BDT data received to the VAL client.

6.	The SEALDD server notifies the VAL server about the delivery result by sending the SEALDD Background data transfer notification including the BDT subscription identifier and optionally the list of VAL UEs which received the BDT data, the executed time window and quality of the BDT data transmission based on the measurements in step 5.

#### 9.x.2.2	SEALDD Background data transfer subscription update

Figure 9.x.2.2-1 illustrates the procedure for SEALDD Background data transfer subscription update from VAL server to the SEALDD server.

Pre-condition:

-	The SEALDD Background data transfer in DL direction procedure has been performed.

Figure 9.x.2.2-1: SEALDD Background data transfer subscription update

1.	The VAL server can trigger a BDT update by sending a SEALDD Background data transfer subscription update request. The request contains BDT subscription identifier and may contain desired time window, desired area information (e.g., when a UE enters a geographical area), Policy Selection Guidance, data volume.

2.	The SEALDD server starts a new BDT configuration negotiation with the SEAL NRM server as described in clause 14.3.13.5 of TS 23.434 [4]. The SEALDD also performs Network resource adaptation negotiation with the SEAL NRM server as described in clause 14.3.3.3 of TS 23.434 [4] for the new BDT transmission.

NOTE:	If the NRM server is a external control plane functionality for the SEALDD server, then the SEALDD server uses step 3 otherwise SEALDD server can directly use procedure defined in 3GPP TS 23.502 [6] clause 4.16.7.2

3.	The SEALDD server sends the SEALDD Background data transfer subscription update response to the VAL server, containing result and optionally the BDT subscription identifier.

#### 9.x.2.3	SEALDD Background data transfer unsubscribe

Figure 9.x.2.3-1 illustrates the procedure for SEALDD Background data transfer unsubscribe from either the SEALDD client or VAL server to the SEALDD server.

Pre-condition:

-	The SEALDD Background data transfer procedure has been performed.

Figure 9.x.2.3-1: SEALDD Background data transfer unsubscribe

1.	The VAL server can trigger a BDT deletion by sending a SEALDD Background data transfer unsubscribe request to the SEALDD server. The request contains the BDT subscription identifier.

2.	The SEALDD server negotiates the BDT configuration with the SEAL NRM server as described in clause 14.3.13.6 of TS 23.434 [4] and deletes any stored application data related to the BDT subscription identifier.

3.	The SEALDD server sends the SEALDD Background data transfer unsubscribe response to the VAL server, containing result of the operation.

### 9.x.3	Information flows

#### 9.x.3.1	SEALDD Background data transfer subscription request

Table 9.x.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD background data transfer request.

Table 9.x.3.1-1: SEALDD Background data transfer subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | Identity of the VAL service for which the background data transfer is requested. |
| List of VAL UE IDs | O (See NOTE) | List of VAL UE IDs for which the transfer policy applies. |
| VAL group ID | O (See NOTE) | VAL group ID for which the transfer policy applies. |
| Desired time window | O | Desired time window for the background data transfer. |
| Desired area information | O | Desired geographical area for the background data transfer. |
| Policy Selection Guidance | O | List that includes guidance in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. If not included, local and ASP-provided policies decide one among multiple transfer policies. |
| Data volume | M | Identify of the data size to be stored. |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.x.3.2	SEALDD Background data transfer subscription response

Table 9.x.3.2-1 describes the information flow from the SEALDD server to the VAL server.

Table 9.x.3.2-1: SEALDD Background data transfer subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure. |
| subscription identifier | O | Indicates the Background data transfer subscription identifier |
| Granted time window | O | Granted time window for the background data transfer when sending in UL direction. |

#### 9.x.3.3	SEALDD Background data transfer notification

Table 9.x.3.2-1 describes the information flow from the SEALDD server to the VAL server.

Table 9.x.3.3-1: SEALDD Background data transfer notification

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the Background data transfer subscription identifier |
| List of VAL UE IDs | O | List of VAL UE IDs or VAL group ID. |
| Executed time windows | O | The actual time window the BDT data was transferred. |
| BDT transmission quality | O | Indicates the transmission quality (e.g., QoE) when transferring the BDT data between SEALDD client and SEALDD server. |

#### 9.x.3.4	SEALDD Background data transfer update subscription request

Table 9.x.3.4-1 describes the information flow from the VAL server to the SEALDD server for sending the SEALDD background data transfer update subscription request.

Table 9.x.3.4-1: SEALDD Background data transfer update subscription request

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the Background data transfer subscription identifier. |
| Desired time window | O (See NOTE) | Desired time window for the background data transfer. |
| Desired area information | O (See NOTE) | Desired geographical area for the background data transfer. |
| Policy Selection Guidance | O (See NOTE) | List that includes guidance in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. |
| Data volume | O (See NOTE) | Identify of the data size to be stored. |
| NOTE: At least one of these IEs shall be present in the message. |  |  |

#### 9.x.3.5	SEALDD Background data transfer update subscription response

Table 9.x.3.5-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD Background data transfer update subscription request.

Table 9.x.3.5-1: SEALDD Background data transfer update subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure. |
| subscription identifier | M | Indicates the Background data transfer subscription identifier. |
| Granted time window | O | Granted time window for the background data transfer when sending in UL direction. |

#### 9.x.3.6	SEALDD Background data transfer unsubscribe request

Table 9.x.3.6-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD background data transfer unsubscribe request.

Table 9.x.3.6-1: SEALDD Background data transfer unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| subscription identifier | M | Indicates the Background data transfer subscription identifier. |

#### 9.x.3.7	SEALDD Background data transfer unsubscribe response

Table 9.x.3.7-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD Background data transfer unsubscribe response.

Table 9.x.3.7-1: SEALDD Background data transfer unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the BDT unsubscribe operation. |

### 9.x.4	APIs

#### 9.x.4.1	General

Table 9.x.4.1-1 illustrates the APIs exposed by SEALDD server for background data transfer.

Table 9.x.4.1-1: List of SEALDD server APIs for background data transfer

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_BDT | Subscribe | Subscribe/Notify | VAL server |
|  | Notify | Subscribe/Notify | VAL server |
|  | Update | Subscribe/Notify | VAL server |
|  | Unsubscribe | Subscribe/Notify | VAL server |

#### 9.x.4.2	Sdd_BDT_Subscribe operation

API operation name: Sdd_BDT_Subscribe

Description: The consumer requests for a Background data transfer service.

Inputs: See clause 9.x.3.1.

Outputs: See clause 9.x.3.2.

See clause 9.x.2.1 for details of usage of this operation.

#### 9.x.4.3	Sdd_BDT_Notify operation

API operation name: Sdd_BDT_Notify

Description: The consumer is notified with Background data transfer result.

Inputs: None

Outputs: See clause 9.x.3.3.

See clause 9.x.2.1 for details of usage of this operation.

#### 9.2.4.4	Sdd_BDT_Update operation

API operation name: Sdd_BDT_Update

Description: The consumer requests to update the SEALDD Background data transfer service.

Inputs: See clause 9.x.3.4.

Outputs: See clause 9.x.3.5.

See clause 9.x.2.2 for details of usage of this operation.

#### 9.2.4.5	Sdd_BDT_Unsubscribe operation

API operation name: Sdd_BDT_Unsubscribe

Description: The consumer requests to unsubscribe the SEALDD Background data transfer service.

Inputs: See clause 9.x.3.6.

Outputs: See clause 9.x.3.7.

See clause 9.x.2.3 for details of usage of this operation.

* * * End of changes * * * *