3GPP TSG-SA Meeting #103	SP-240381

19-22 March 2024, Maastricht, Netherlands

Title:	Summary for Enhanced support of Non-Public Networks Phase 2

Type:	Work Item Summary

Agenda Item:	21.5 (SA)

Source:	Ericsson

Contact:	Peter Hedman (peter.hedman@ericsson.com)

This document is a contribution to TR 21.918 ("Release 18 Description; Summary of Rel-18 Work Items"). It is meant to be endorsed by the plenary.

The summary below excludes content for eNPN_CH that will be submitted separately in SP-240zzz, and for eNPN_Ph2-NGRAN-Core that was submitted in RP-233568.

Summary based on the input provided by Ericsson in SP-240381.

| UID | Name | Acronym | WG | WID | Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 980128 | Non-Public Networks Phase 2 | eNPN_Ph2 |  | SP-220805 | Hedman, Peter, Ericsson |
| 940075 | Study on enhanced support of Non-Public Networks phase 2 | FS_eNPN_Ph2 | S2 | SP-220418 | Hedman, Peter, Ericsson |
| 970015 | Stage 2 of Non-Public Networks Phase 2 | eNPN_Ph2 | S2 | SP-230096 | Hedman, Peter, Ericsson |
| 960033 | Study on security aspects of enhanced support of Non-Public Networks phase 2 | FS_eNPN_Ph2_SEC | S3 | SP-220531 | Jost, Christine, Ericsson, |
| 990043 | Security aspects of support of Non-Public Networks phase 2 | eNPN_Ph2 | S3 | SP-230156 | Jost, Christine, Ericsson |
| 980078 | CT1 aspects of eNPN_Ph2 | eNPN_Ph2 | C1 | CP-231286 | Sedlacek, Ivo, Ericsson |
| 980079 | CT3 aspects of eNPN_Ph2 | eNPN_Ph2 | C3 | CP-231286 | Sedlacek, Ivo, Ericsson |
| 980080 | CT4 aspects of eNPN_Ph2 | eNPN_Ph2 | C4 | CP-231286 | Sedlacek, Ivo, Ericsson |
| 940043 | Study on Charging Aspects for Enhanced Support of Non-Public Networks | FS_eNPN_CH | S5 | SP-211447 | Chen, Ai, China Mobile |
| 990028 | Charging Aspects for Enhanced Support of Non-Public Networks | eNPN_CH | S5 | SP-230177 | Chen, Ai, China Mobile |
| 991138 | Core part: Non-Public Networks Phase 2: NG-RAN aspects | eNPN_Ph2-NGRAN-Core | R3 | RP-231185 | Shengnan Liu, China Telecom |

Introduction

The Rel-17 work item Enhanced support of Non-Public Networks further enhanced the support of non-public networks introduced in Rel-16. The Non-Public Networks provides 5GS coverage within a specific geographic area for non-public use and is a key demand of emerging 5G applications and verticals.

The Rel-18 work item on Non-Public Networks Phase 2 builds on the work in Rel-16 and Rel-17. It enables SNPN support for idle and connected mode mobility between equivalent SNPNs without new network selection, support for non-3GPP access for SNPN, and support for providing access to Localized Services.

Description

Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

a.	UE and AMF support of equivalent SNPN list in NAS;

b.	NG-RAN and AMF support of equivalent SNPNs in NGAP; and

c.	UE/NG-RAN/AMF take equivalent SNPN list into consideration, for supporting relevant functions, e.g., idle/connected mode mobility.

Support for non-3GPP access for SNPN: The Access Network from which the UE can access an SNPN can be a non-3GPP access i.e., Untrusted non-3GPP access network, or Trusted non-3GPP access network. This includes functionality to enable access via wireline access network to 5GCN of an SNPN, Non-Seamless WLAN offload (NSWO) procedure, and Non-5G-Capable over WLAN (N5CW) access to SNPNs. Support for non-3GPP access for SNPN required additional security specifications, since SNPN allows the use of additional EAP methods for mutual authentication between UE and network, e.g., certificate-based authentication, and the usage of anonymous SUCI for identifier privacy.

Support for providing access to Localized Services: Enabling UEs to perform SNPN selection for accessing SNPNs providing access to Localized Services. This was achieved by new lists for automatic SNPN selection i.e., Credentials Holder controlled prioritized list of preferred SNPNs for accessing Localized Services, and Credentials Holder controlled prioritized list of GINs for accessing Localized Services, and by enhancing manual SNPN selection as well.

Informative description how existing 5GS functionality can be used to enable access to Localized Services in SNPN or PNI-NPNs e.g., information that can be included in agreements with the Localized Service Providers, configuration of network, re-use of Session Management functionality, how to enable and restrict the UE's access to Localized Services and support for leaving network that provides access to Localized Services.

References

List of related CRs: select "TSG Status = Approved" in: 
https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=980128,940075,970015,960033,990043,980078,980079,980080,940043,990028,991138

[1]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[2]	3GPP TS 23.502, Procedures for 5G System; Stage 2.

[3]	3GPP TS 23.503, Policy and Charging Control Framework for the 5G System; Stage 2 Service Enabler Architecture Layer for Verticals

[4]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[5]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[6]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[7]	3GPP TS 33.501: "Security architecture and procedures for 5G system".