3GPP TSG-SA Meeting #103	SP-240397

# Maastricht, NL, 19th – 22nd Mar 2024                                               revision of S4-240486

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.117 | CR | 0007 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS3] Correction on Audio encapsulation in ISO BMFF, 3GP file format, CMAF and DASH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Dolby, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, Dolby, Qualcomm, Tencent |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2024-03-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editorial correction ISO BMFF signaling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rephrasing for ISOBMFF encapsulation is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect text, inconsistent interpretation and implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 7.2, 7.3, 7.4, 7.6, 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]	3GPP TS 26.071: "Mandatory Speech Codec speech processing functions; AMR Speech CODEC; General description".

[4]	3GPP TS 26.090: "Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions".

[5]	3GPP TS 26.073: "ANSI-C code for the Adaptive Multi Rate (AMR) speech codec".

[6]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi Rate (AMR) speech codec".

[7]	3GPP TS 26.093: "Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation".

[8]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[9]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[10]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[11]	3GPP TS 26.204: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; ANSI-C code".

[12]	3GPP TS 26.193: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Source controlled rate operation".

[13]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[14]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[15]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)".

[16]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[17]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); Adaptive Multi-Rate - Wideband (AMR-WB) backward compatible functions".

[18]	3GPP TS 26.450: "Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)".

[19]	3GPP TS 26.401: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description".

[20]	3GPP TS 26.402: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Additional decoder tools".

[21]	3GPP TS 26.403: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Encoder specification; Advanced Audio Coding (AAC) part".

[22]	3GPP TS 26.404: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Enhanced aacPlus encoder Spectral Band Replication (SBR) part".

[23]	3GPP TS 26.405: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Encoder specification parametric stereo part".

[24]	3GPP TS 26.410: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Floating-point ANSI-C code".

[25]	3GPP TS 26.411: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Fixed-point ANSI-C code".

[26]	3GPP TS 26.290: "Audio codec processing functions; Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec; Transcoding functions".

[27]	3GPP TS 26.304: "Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec; Floating-point ANSI-C code".

[28]	3GPP TS 26.273: "ANSI-C code for the fixed-point Extended Adaptive Multi-Rate - Wideband (AMR-WB+) speech codec".

[29]	3GPP TS 26.244: "Transparent end-to-end streaming service; 3GPP file format (3GP)".

[30]	ISO/IEC 23000-19 "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[31]	ISO/IEC 23009-1:2019/Amd.1:2020: "Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats."

[32]	CTA-5003: "Web Application Video Ecosystem (WAVE): Device Playback Capabilities Specification" available here https://cdn.cta.tech/cta/media/media/resources/standards/pdfs/cta-5003-final.pdf.

[33]	3GPP TS 26.244: "Transparent end-to-end streaming service; 3GPP file format (3GP)".

[34]	3GPP TS 26.452: "Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators".

[35]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error concealment of lost packets".

[36]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[X]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects - Part 12: ISO base media file format"

[Y]	ISO/IEC 14496-14 "Information technology – Coding of audio-visual objects – Part 14: MP4 file format".

Second Change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAC	Advanced Audio Coding

ABR	Adaptive BitRate

AMR	Adaptive MultiRate

CMAF	Common Media Application Format

DASH	Dynamic Adaptive Streaming over HTTP

DRC	Dynamic Range Control DTX	Discontinuous Transmission

EVS	Enhanced Voice Services

ISO BMFF	ISO Base Media File Format

HTTP	Hyper Text Transfer Protocol

SBR	Spectral Band Replication

URN	Universal Resource Name

WAVE	Web Application Video Ecosystem

Third Change

7.2	AMR Media Profile

7.2.1	Mapping to ISO BMFF

If media is provided following the operation point AMR and is encapsulated in the ISO BMFF [X], then the file format track shall contain the sample entry AMRSampleEntry with box_type 'samr' as defined in TS 26.244 [29] clause 6.5 and conform to the requirements of the sample entry 'samr' as defined in TS 26.244 [29].

7.2.2	Media Profile Definition

7.2.2.1	CMAF Track Definition

If media is provided following the operation point AMR and is encapsulated in a CMAF track, then the CMAF track shall conform to 7.2.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30] as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

7.2.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point AMR and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to 7.2.2.1, and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. CMAF Switching Sets that follow these requirements conform to the CMAF AMR media profile 'camr' defined in this clause.

.

7.2.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point AMR, a CMAF Switching Set conforming to clause 7.2.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set, in that case, the Adaptation Set shall conform to the Adaptation Set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31]. The following parameters shall be present on Adaptation Set level and set:

-	@codecs is set to 'samr'

-	@mimeType is set to be compatible with "audio/mp4 profiles='camr'"

-	@audioSamplingRate is set to '8000'

If the Adaptation Set conforms to the constraints for the AMR Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:amr".

7.2.2.4	Playback Requirements

For a receiver supporting the AMR media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.2.2.2 for any CMAF Track conforming to the CMAF AMR media profile 'camr' as defined in clause 7.2.2.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR media profile 'camr' as defined in clause 7.2.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [29] for any content conforming to a CMAF Switching Set according to CMAF AMR media profile 'camr' as defined in clause 7.2.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

7.2.2.5	Content Generation Requirements

For a transmitter supporting the AMR media profile the following applies:

-	It shall support all media encoding capabilities for AMR as defined in clause 5.3.

-	It shall support the sender requirements for AMR as defined in clause 6.2.2.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.2.2. 1 that conforms to the CMAF Media Profile 'camr' as defined in clause 7.2.2. 2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.2.2.4.

Fourth Change

7.3	AMR-WB Media Profile

7.3.1	Mapping to ISO BMFF

If media is provided following the operation point AMR-WB and is encapsulated in the ISO BMFF, then the file format track shall contain the AMRSampleEntryBox and shall conform to the requirements of the sample entry 'sawb' as defined in TS 26.244 [29] clause 6.5.

7.3.2	Media Profile Definition

7.3.2.1	CMAF Track Definition

If media is provided following the operation point AMR-WB and is encapsulated in a CMAF track, then the CMAF track shall conform to 7.3.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

7.3.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point AMR-WB and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to 7.3.2.1,  and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. CMAF Switching Sets that follow these requirements conform to the CMAF AMR WB media profile 'camw' defined in this clause.

7.3.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point AMR-WB, a switching set conforming to clause 7.3.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set, in that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31] clause 8.12.4.3. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sawb'

-	@mimeType is set to be compatible with "audio/mp4 profiles='camw'"

-	@audioSamplingRate is set to '16000'

If the Adaptation Set conforms to the constraints for the AMR-WB Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:amr-wb".

7.3.2.4	Playback Requirements

For a receiver supporting the AMR-WB media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.2.3.2 for any CMAF Track conforming to the CMAF AMR-WB media profile 'camw' as defined in clause 7.3.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camw' as defined in clause 7.3.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camw' as defined in clause 7.3.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

7.3.2.5	Content Generation Requirements

For a transmitter supporting the AMR-WB media profile the following applies:

-	It shall support all media encoding capabilities for AMR-WB as defined in clause 5.3.

-	It shall support the sender requirements for AMR-WB as defined in clause 6.2.3.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.3.2. 1 that conforms to the CMAF Media Profile 'camw' as defined in clause 7.3.2. 2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.3.2.4.

Fifth Change

7.4	EVS Media Profile

7.4.1	Mapping to ISO BMFF

If media is provided following the operation point EVS and is encapsulated in the ISO BMFF, then the file format track shall contain the EVSSampleEntryBox with box_type and conform to the requirements of the sample entry 'sevs' as defined in TS 26.244 [29].

7.4.2	Media Profile Definition

7.4.2.1	CMAF Track Definition

If media is provided following the operation point EVS and is encapsulated in a CMAF track, t then the CMAF track shall conform to 7.3.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

7.4.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point EVS and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to 7.4.2.1,  and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. A CMAF Switching Set that follows these requirements conforms to the CMAF EVS media profile 'cevs' defined in this clause..

7.4.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point EVS and is provided in a DASH Media Presentation in an Adaptation Set, a switching set conforming to clause 7.4.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31]. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sevs'

-	@mimeType is set to be compatible with "audio/mp4 profiles='cevs'"

-	@audioSamplingRate is set to one of the following: '8000', '16000', '24000', '32000'

If the Adaptation Set conforms to the constraints for the EVS Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:evs.

7.4.2.4	Playback Requirements

For a receiver supporting the EVS media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.2.4.2 for any CMAF Track conforming to the CMAF EVS media profile 'cevs' as defined in clause 7.4.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF EVS media profile 'cevs' as defined in clause 7.4.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF EVS media profile 'cevs' as defined in clause 7.2.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

7.4.2.5	Content Generation Requirements

For a transmitter supporting the EVS media profile the following applies:

-	It shall support all media encoding capabilities for EVS as defined in clause 5.3.

-	It shall support the sender requirements for EVS as defined in clause 6.2.4.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.4.2.1 that conforms to the CMAF Media Profile 'cevs' as defined in clause 7.4.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.4.2.2.

Sixth Change

7.6	eAAC+ stereo Media Profile

7.6.2.1	CMAF Track Definition

If media is provided following the operation point eAAC+ stereo and is encapsulated in a CMAF track, then the CMAF track shall conform toand contain the MP4AudioSampleEntry 'mp4a' as defined in ISO/IEC 14496-14 [Y] ], the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7, the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10 as well as AAC core constraints in clause 10 of ISO/IEC 23000-19 [30].

7.6.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point eAAC+ stereo and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to 7.6.2.1, and the tracks shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7, the general CMAF audio track Switching Set constraints defined in ISO/IEC 23000-19 [30], clause 10 as well as the AAC core Switching Set constraints in clause 10 of ISO/IEC 23000-19 [30]. A CMAF Switching Set following these requirements is defined as the CMAF eAAC+ stereo media profile 'ceac'.

7.6.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point eAAC+ stereo, a switching set conforming to clause 7.6.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31] clause 8.12.4.3. The following parameters shall be present on Adaptation Set level and set:

-	@codecs is set to 'mp4a'

-	@mimeType is set to be compatible with "audio/mp4 profiles='ceac'"

-	@audioSamplingRate is set to '32000','44100', or '48000'

If the Adaptation Set conforms to the constraints for the eAAC+ stereo Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:eAAC+ ".

7.6.2.4	Playback Requirements

For a receiver supporting the eAAC+ stereo media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.3.1.2 for any CMAF Track conforming to the CMAF eAAC+ stereo media profile 'ceac' as defined in clause 7.6.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF eAAC+ media profile 'ceac' as defined in clause 7.6.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'ceac' as defined in clause 7.6.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

7.6.2.5	Content Generation Requirements

For a transmitter supporting the eAAC+ stereo media profile the following applies:

-	It shall support all media encoding capabilities for eAAC+ stereo as defined in clause 5.3.

-	It shall support the sender requirements for eAAC+ stereo as defined in clause 6.6.2.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.6.2.1 that conforms to the CMAF Media Profile 'ceac' as defined in clause 7.6.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.6.2.2.

Seventh Change

7.7	AMR-WB+ Media Profiles

7.7.1	Mapping to ISO BMFF

If media is provided following the operation point AMR-WB+ and is encapsulated in the ISO BMFF, then the file format track shall contain the AMRWPSampleEntry with box_type and conform to the requirements of the sample entry 'sawp' as defined in TS 26.244 [29] clause 6.9.

7.7.2	Media Profile Definition

7.7.2.1	CMAF Track Definition

If media is provided following the operation point AMR-WB+ and is encapsulated in a CMAF track, then the CMAF track shall conform to 7.7.1 and the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19, clause 10.

7.7.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point AMR-WB+ and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to the requirements of clause 7.7.2.1, the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7 . A CMAF Switching Set following these requirements is defined as the CMAF AMR-WB+ media profile 'camp'.

7.7.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point AMR-WB+, a switching set conforming to clause 7.7.2.2 may be provided in a DASH Media Presentation in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31] clause 8.12.4.3. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sawp'

-	@mimeType is set to be compatible with "audio/mp4 profiles='camp'"

-	@audioSamplingRate is set to any of the following values: '8000', '16000', '32000', or '38400'

If the Adaptation Set conforms to the constraints for the AMR-WB+ Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:amr-wb+".

7.7.2.4	Playback Requirements

For a receiver supporting the AMR-WB+ media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.3.2.2 for any CMAF Track conforming to the CMAF AMR-WB+ stereo media profile 'camp' as defined in clause 7.7.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camp' as defined in clause 7.7.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camp' as defined in clause 7.7.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

7.7.2.5	Content Generation Requirements

For a transmitter supporting the AMR-WB+ media profile the following applies:

-	It shall support all media encoding capabilities for AMR-WB+ stereo as defined in clause 5.3.

-	It shall support the sender requirements for AMR-WB+ as defined in clause 6.3.3.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.7.2.1 that conforms to the CMAF Media Profile 'cawp' as defined in clause 7.7.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.7.2.2.