3GPP SA WG2 Meeting #162 	S2-2403975

Changsha, China, 15 – 19 April, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0219 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Configuration Address Information provisioning in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed that Internal Group Identifier cannot be used by AF for HR-SBO case. However, Internal Group Identifier still shown as an input parameter for Nnef_ECSAddress_Subscribe in 6.5.2.6.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing Internal Group Identifier from 6.5.2.6.3. Editorial correction on EACI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.6.1, 6.5.2.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4771 |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.5.2.6	ECS Address Provisioning in Roaming

##### 6.5.2.6.1	General

For both LBO and HR case, the subscription data of the ECS Address Configuration Information in UDM or UDR is stored per PLMN ID.

For the LBO case, an AF in the visited PLMN may provide the EACI via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3] to UDM via H-NEF. This ECS Address Configuration Information is further provided to SMF as part of subscription information.

For the HR case when access to EHE in VPLMN is allowed:

-	The HPLMN has the knowledge of EACI in the VPLMN: For this scenario, the AF is able to interact with NEF in HPLMN. The AF may provide the VPLMN EACI to H-NEF via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3], and further to the UDM. During the HR PDU session establishment procedure, the H-SMF sends the VPLMN EACI to V-SMF and then to UE. The V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

-	HPLMN does not have the knowledge of EACI in VPLMN: For this scenario, the AF can't interact with NEF in HPLMN. As defined in clauses 6.5.2.6.2 and 6.5.2.6.3, V-NEF stores the VPLMN EACI received from AF deployed in the VPLMN in V-UDR, and V-SMF subscribes from V-NEF to retrieve the VPLMN EACI. During the HR PDU Session establishment, the V-SMF sends the VPLMN EACI obtained from V-NEF to the H-SMF, and H-SMF decides the VPLMN ECS Address Configuration Information sent to V-SMF and then to UE according to the PLMN ID additionally.

NOTE:	It depends on the PLMN operation that the HPLMN can decide the EACI if both VPLMN and UDM provides the EACI.

##### 6.5.2.6.3	ECS Address Configuration Information Provision to the SMF in VPLMN

V-SMF supporting HR-SBO may receive the ECS Address Configuration Information from NEF in VPLMN via Subscribe/Notify procedure is defined in this clause.

Figure 6.5.2.6.3-1: ECS Address Configuration Information provisioning to SMF in VPLMN

1-2.	As pre-requisite condition, the SMF subscribes to ECS Address Configuration Information Change Notification from the NEF by sending Nnef_ECSAddress_Subscribe message. The SMF may indicate that the current status of ECS Address Configuration Information shall be notified immediately (if available). The SMF may indicate for which (list of) DNN and/or S-NSSAI it subscribes. NEF may further subscribe to ECS Address Configuration Information Change Notification from the UDR using Nudr_DM_Subscribe.

3-4.	The NEF invokes Nnef_ECSAddress_Notify (ECS Address Configuration Information) to the SMF if the ECS Address Configuration Information is updated. If there is ECS Address Configuration Information available and immediate report is required, the NEF notifies the SMF(s) with such information immediately. NEF may retrieve the ECS Address Configuration Information from UDR using Nudr_DM_Query/Notify.

******************* END of Changes ******************