3GPP TSG-SA3 Meeting #116	S3-242091

Jeju, South Korea,  20th - 24th May 2024

# 00

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.514 | CR | 0027 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of TEST CASE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Editorial corrections of TEST CASE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.1, 4.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******Start of changes*****

#### 4.2.1.1	De-concealment of SUPI from the SUCI based on the protection scheme used to generate the SUCI

Requirement Name: De-concealment of SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Requirement Reference: TS 33.501 [2], clause 5.8.2.

Requirement Description: The SIDF resolves the SUPI from the SUCI based on the protection scheme used to generate the SUCI as specified in TS 33.501 [2], clause 5.8.2.

Threat References: TR 33.926 [4], clause E.2.2.1, Incorrect SUCI de-concealment.

Test Case:

Test Name: TC_DE-CONCEAL_SUPI_from_SUCI_UDM

Purpose:

Verify that the SIDF De-conceals the SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Procedure and execution steps:

Pre-Condition:

-	UDM network product is connected in simulated/real network environment including an AUSF and AMF.

-	Tester shall have access to the subscription data stored in UDR.

-	Tester shall record the SUPI from the UE.

Execution Steps:

Tester shall capture the entire authentication procedure between UE and AMF over N1, N12 and N13 interface using any network analyser.

1.	Tester shall filter the Nudm_UEAuthentication_Get Response message sent from UDM to AUSF over N13 interface containing the SUPI.

2.	Tester shall compare the SUPI gotten from UE and the SUPI retrieved from Nudm_UEAuthentication_Get Response message.

NOTE:	The tester may filter the Nausf_UEAutentication_Authenticate Response message sent from the UDM/AUSF to the AMF over N12 interface containing the SUPI, if the UDM and AUSF network products are collocated without an open N13 interface.

Expected Results:

SIDF resolves the SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

******Start of 2nd changes*****

#### 4.2.2.2	Storing of authentication status of UE by UDM

Requirement Name: Storing of authentication status of UE by UDM.

Requirement Reference: TS 33.501 [2], clause 6.1.4.1a

Requirement Description: The UDM stores the authentication status of the UE (SUPI, authentication result, timestamp, and the serving network name) after authentication as specified in TS 33.501 [2], clause 6.1.4.1a.

Threat References: TR 33.926 [4], clause E.2.2.3, Failure to store of authentication status.

Test Case:

Test Name: TC_AUTH_STATUS_STORE_UDM

Purpose:

Verify that the UDM under test stores the authentication status of UE, which is identical to the UE authentication information sent to/from the AUSF and the AMF.

Procedure and execution steps:

Pre-Condition:

-	UDM network product is connected with an AUSF in simulated/real network environment involving AMF, eNB.

-	The tester shall have access to all the authentication specific data sent over N1 interface, N12 interface and N13 interface.

-	The tester shall have access to the UDM under test.

Execution Steps:

1.	The tester shall capture the entire authentication procedure and authentication confirmation procedure over N12 and N13 interface using any network analyser.

2.	the tester shall filter the Nudm_UEAuthentication_Get Request message sent over the N13 interface to retrieve serving network name.

3.	The ester shall filter the Nudm_UEAuthentication_Get Response message sent over N13 interface to find the SUPI.

4.	The tester shall filter the Nausf_UEAuthentication_Authenticate Response message sent over N12 interface to retrieve the Authentication result (EAP success/failure for EAP-AKA' or Result for 5G AKA).

5.	The tester shall filter the Nudm_UEAuthentication_ResultConfirmation Request message to retrieve the authentication result and time of authentication procedure sent from the AUSF to the UDM over N13 interface.

6.	The tester shall compare the serving network name stored in the UDM against the serving network name retrieved from the Nudm_UEAuthentication_Get Request message and the serving network name retrieved from the Nudm_UEAuthentication_ResultConfirmation Request message.

7.	The tester shall compare the authentication status stored in the UDM against the authentication result retrieved from N12 interface.

8.	The tester shall compare the SUPI stored in the UDM against the SUPI retrieved from the Nudm_UEAuthentication_Get Response message and the SUPI  retrieved from the Nudm_UEAuthentication_ResultConfirmation Request message.

9.	The tester shall compare the timestamp stored in the UDM against the time of authentication procedure retrieved from the Nudm_UEAuthentication_ResultConfirmation Request message.

Expected Results:

The storing of authentication status (SUPI, authentication result, timestamp, and the serving network name) of UE at the UDM is verified.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

NOTE:	this test case does not apply to the deployment scenario where the UDM and AUSF network products are collocated without an open N13 interface.

******End of changes*****