3GPP TSG-SA WG4 Meeting #128	S4-241343

# Korea, Jeju, 20 – 24 May 2024														Revision of S4-241056

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0003 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding ISAR track-a split rendering feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Fraunhofer IIS, Ericsson LM, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ISAR, IVAS_codec |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The split rendering feature enabled with this CR was selected by SA4 according to the ISAR selection procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The feature is introduced in section 4.5 and fully described in a new subsection under section 7 (Functional description of the rendering, rendering control, and pre-rendering). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IVAS codec will not offer a split rendering feature and potentially not be available on lightweight end devices. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Contents, 2, 3.3, 4.5, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.250 TS 26.254 CR 0001 TS 26.255 CR 0001 TS 26.258 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: S4-240709: SA4-endorsed at SA#127-bis-e Rev 1: - Editorial updates, - ISAR pose correction metadata reduction by avoiding coding of redundant bits - Added padding for LC3plus Rev 2: Incorporation of pCR in S4-241004 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

Contents

Foreword	21

1	Scope	22

2	References	22

3	Definitions of terms, symbols and abbreviations	23

3.1	Terms	23

3.2	Symbols	23

3.3	Abbreviations	24

4	General description of the coder	25

4.1	Introduction	25

4.2	IVAS codec overview	25

4.2.1	General	25

4.2.2	Mono (EVS-compatible) Operation	26

4.2.3	Stereo Operation	26

4.2.4	Scene-based Audio (SBA, Ambisonics) Operation	27

4.2.5	Metadata-assisted Spatial Audio (MASA) Operation	27

4.2.6	Objects (Independent Streams with Metadata, ISM) Operation	28

4.2.7	Multi-Channel (MC) Operation	28

4.2.8	Combined Objects and SBA (OSBA) Operation	28

4.2.9	Combined Objects and MASA (OMASA) Operation	28

4.2.10	Discontinuous Transmission (DTX) Operation	29

4.3	Input/output audio configurations	29

4.3.1	Input/output audio sampling rate	29

4.3.2	Input/output audio formats	29

4.4	Algorithmic delay	30

4.5	Rendering overview	31

4.5.1	Rendering introduction	31

4.5.2	Internal IVAS renderer	32

4.5.3	External IVAS renderer	32

4.5.4	Interface for external rendering	32

4.5.5	Split rendering	25

4.6	Organization of the rest of the Technical Standard	33

5	Functional description of the encoder	33

5.1	 Encoder overview	33

5.2	Common processing and coding tools	34

5.2.1	High-pass filtering	34

5.2.2	Core-coder processing	34

5.2.2.1	Overview	34

5.2.2.2	Core-coder front pre-processing	35

5.2.2.2.1	General	35

5.2.2.2.2	Sample rate conversion	35

5.2.2.2.3	Pre-emphasis	36

5.2.2.2.4	Spectral analysis	36

5.2.2.2.5	Signal activity detection	36

5.2.2.2.6	Bandwidth detector	36

5.2.2.2.7	Time domain transient detection	37

5.2.2.2.8	Linear prediction analysis	38

5.2.2.2.9	Open-loop pitch analysis	39

5.2.2.2.10	Coding mode determination	39

5.2.2.2.11	Speech/Music classification	39

5.2.2.2.12	Core-coder technology pre-selection	50

5.2.2.3	Core-coder modules	57

5.2.2.3.1	Core-coder pre-processing	57

5.2.2.3.2	LP based coding	60

5.2.2.3.3	MDCT based coding	88

5.2.2.3.4	Switching coding modes	104

5.2.2.3.5	DTX/CNG operation	107

5.2.3	Common audio coding tools	109

5.2.3.1	General	109

5.2.3.2	Single Channel Element (SCE)	109

5.2.3.3	Channel Pair Element (CPE)	110

5.2.3.4	Multichannel Coding Tool (MCT)	112

5.2.3.4.1	Summary of the system	112

5.2.3.4.2	MCT configurations	113

5.2.3.4.3	Encoder single channel preprocessing up to whitened spectrum	113

5.2.3.4.4	Joint channel Encoding System Description	114

5.2.3.4.5	IGF for multi-channel processing	119

5.2.3.4.6	Bitrate distribution	119

5.2.3.4.7	Quantization and coding of each channel	120

5.2.4	Common spatial metadata coding tools	120

5.2.4.1	General	120

5.2.4.2	Spatial metadata composition	120

5.2.4.3	Direction metadata coding tools	121

5.2.4.3.1	Overview	121

5.2.4.3.2	Direction metadata quantization	121

5.2.4.3.3	Direction metadata raw encoding	124

5.2.4.3.4	Direction metadata entropy encoding tools	124

5.2.4.4	Diffuseness and energy ratio coding methods	125

5.2.4.4.1	Diffuseness and energy ratio definitions	125

5.2.4.4.2	Diffuseness parameter quantization	125

5.2.4.4.3	Diffuseness and energy ratio indices coding	126

5.2.4.4.4	Diffuseness and energy ratio coding with two concurrent directions	127

5.2.4.5	Direction metadata coding methods	129

5.2.4.5.1	Entropy coding 1 (EC1)	129

5.2.4.5.2	Entropy coding 2 (EC2)	134

5.2.4.5.3	Entropy coding 3 (EC3)	135

5.2.4.6	Coherence coding	139

5.2.4.6.1	Spread coherence coding	139

5.2.4.6.2	Surround coherence coding	142

5.2.4.7	DTX coding	142

5.2.5	Modified Discrete Fourier Transform (MDFT) Analysis Filter Bank	143

5.2.5.1	General	143

5.2.5.2	Modified Discrete Fourier Transform (MDFT)	143

5.2.5.3	Filter bank responses	143

5.2.5.3.1	General	143

5.2.5.3.2	Magnitude responses for 5 ms stride operation	144

5.2.5.3.3	Responses for sampling rates below 48 kHz	145

5.3	Stereo audio operation	146

5.3.1	Stereo format overview	146

5.3.2	Unified stereo	147

5.3.2.1	Overview	147

5.3.2.2	Common unified stereo tools	147

5.3.2.2.1	Inter-Channel Bandwidth Extension (IC-BWE)	147

5.3.2.2.2	Front VAD	158

5.3.2.3	TD-based stereo	159

5.3.2.3.1	Inter-channel Alignment (ICA)	159

5.3.2.3.2	TD stereo coder overview	170

5.3.2.3.3	Time-domain stereo downmix	170

5.3.2.3.4	Near out-of-phase operation (NOOP)	173

5.3.2.3.5	LP filter coherence	174

5.3.2.3.6	Open-loop pitch coherence	177

5.3.2.3.7	Excitation coding in the secondary channel using two or four subframes	178

5.3.2.3.8	Bit allocation	180

5.3.2.3.9	Quantization of LP coefficients for the secondary channel	181

5.3.2.4	DFT-based stereo	183

5.3.2.4.1	General	183

5.3.2.4.2	Time Domain ITD compensation	184

5.3.2.4.3	STFT analysis	185

5.3.2.4.4	GCC-PHAT ITD estimation	186

5.3.2.4.5	FD circular time-shift	190

5.3.2.4.6	Stereo parameters estimation	190

5.3.2.4.7	IPD Calculation and stabilization	191

5.3.2.4.8	Calculation of the side and residual prediction gains	194

5.3.2.4.9	Stereo parameter coding	195

5.3.2.4.10	Active Downmix	199

5.3.2.4.11	STFT synthesis	200

5.3.2.4.12	Residual coding	202

5.3.2.5	Stereo classifier	205

5.3.2.5.1	General	205

5.3.2.5.2	Classification of uncorrelated content	205

5.3.2.5.3	Cross-talk detection using logistic regression	208

5.3.2.5.4	Cross-talk detection based on the GCC-PHAT	214

5.3.2.5.5	Stereo mode selection	217

5.3.3	MDCT-based stereo	220

5.3.3.1	General Overview	220

5.3.3.2	ITD compensation	220

5.3.3.3	MDCT core pre-processing	221

5.3.3.3.1	General	221

5.3.3.3.2	TCX-LTP parameter estimation	221

5.3.3.3.3	Time-to-frequency transformations	221

5.3.3.3.4	Mid-high bitrate pre-processing	224

5.3.3.3.5	Temporal noise shaping (TNS)	230

5.3.3.3.6	Spectral Noise Shaping (SNS)	231

5.3.3.4	Stereo processing	241

5.3.3.4.1	General	241

5.3.3.4.2	Stereo spectral bands	241

5.3.3.4.3	Global ILD normalization	243

5.3.3.4.4	Conditional ILD correction	243

5.3.3.4.5	Band-wise M/S decision	244

5.3.3.4.6	M/S decision for IGF	245

5.3.3.4.7	M/S Transformation	246

5.3.3.4.8	Encoding of the stereo parameters	247

5.3.3.5	Power spectrum calculation and spectrum noise measure	247

5.3.3.6	Intelligent gap filling (IGF)	247

5.3.3.6.1	General overview	247

5.3.3.6.2	Stereo IGF encoding	248

5.3.3.7	Bitrate distribution	249

5.3.3.8	Quantization and encoding	249

5.3.3.8.1	Spectrum quantization and coding	249

5.3.3.8.2	Global gain estimator	250

5.3.3.8.3	Rate-loop for constant bit rate and global gain	250

5.3.3.8.4	Stereo noise level estimation	250

5.3.4	Switching between stereo modes	250

5.3.4.1	Overview	250

5.3.4.2	General	251

5.3.4.3	Memory handling	253

5.3.4.4	Switching between DFT and TD stereo modes	254

5.3.4.4.1	Switching from TD stereo to DFT stereo	254

5.3.4.4.2	Switching from DFT stereo to TD stereo	255

5.3.4.5	Switching between DFT and MDCT stereo modes	258

5.3.4.5.1	Switching from DFT stereo to MDCT stereo	258

5.3.4.5.2	Switching from MDCT stereo to DFT stereo	258

5.3.4.6	Switching between TD and MDCT stereo modes	258

5.3.4.6.1	Switching from TD stereo to MDCT stereo	258

5.3.4.6.2	Switching from MDCT stereo to TD stereo	259

5.3.5	DTX operation	259

5.3.5.1	DTX in Unified stereo	259

5.3.5.1.1	Signal activity detection in Unified stereo	259

5.3.5.1.2	General	260

5.3.5.1.3	Stereo CNG side gain encoding	261

5.3.5.1.4	Stereo CNG ITD and IPD encoding	262

5.3.5.1.5	Stereo CNG coherence encoding	262

5.3.5.2	DTX in MDCT-based stereo	265

5.3.5.2.1	Overview	265

5.3.5.2.2	VAD in MDCT-based Stereo	265

5.3.5.2.3	Coherence estimation	265

5.3.5.2.4	Noise parameter estimation and encoding	266

5.4	Scene-based audio (SBA) operation	267

5.4.1	SBA format overview	267

5.4.2	Combined DirAC and SPAR based SBA coding	267

5.4.2.1	Combined DirAC and SPAR based SBA coding overview	267

5.4.2.2	FOA signal coding at all bitrates and HOA2, HOA3 signal coding at bitrates below 256 kbps	269

5.4.2.2.1	Overview	269

5.4.2.2.2	Metadata parameter analysis	269

5.4.2.2.3	Downmixing and core-coding	270

5.4.2.3	HOA2 and HOA3 signal coding overview at 256 kbps	270

5.4.2.3.1	Overview	270

5.4.2.3.2	Metadata parameter analysis	270

5.4.2.3.3	Downmixing and core-coding	271

5.4.2.4	HOA2 and HOA3 signal coding overview at 384 kbps	271

5.4.2.4.1	Overview	271

5.4.2.4.2	Metadata parameter generation	271

5.4.2.4.3	Downmixing and core-coding	271

5.4.2.5	HOA2 and HOA3 signal coding overview at 512 kbps	272

5.4.2.5.1	Overview	272

5.4.2.5.2	Metadata parameter generation	272

5.4.2.5.3	Downmixing and core-coding	272

5.4.3	SBA parameter estimation	273

5.4.3.1	SBA front VAD	273

5.4.3.2	Transient Detection	273

5.4.3.3	Analysis windowing and MDFT	273

5.4.3.4	DirAC parameter estimation	275

5.4.3.4.1	General	275

5.4.3.4.2	High-order operation mode	275

5.4.3.4.3	Low-order operation mode	279

5.4.3.5	Hybrid encoder	280

5.4.3.6	Mono signal detection	280

5.4.3.6.1	Overview	280

5.4.3.6.2	Signalling of Mono through the bitstream	284

5.4.3.7	SPAR Metadata computation	285

5.4.3.7.1	Overview	285

5.4.3.7.2	Banded covariance computation	286

5.4.3.7.3	Active W downmix detector	287

5.4.3.7.4	Banded covariance re-mixing	288

5.4.3.7.5	SPAR parameter estimation	289

5.4.3.7.6	Passive-W channel prediction coefficients computation	289

5.4.3.7.7	Active-W channel prediction coefficients computation	289

5.4.3.7.8	Cross-Prediction coefficients computation	292

5.4.3.7.9	Decorrelation coefficients computation	293

5.4.3.7.10	Overview of quantization, bitrate distribution and coding of SPAR parameters	293

5.4.3.7.11	SPAR bitrate distribution table	294

5.4.3.7.12	Quantization and coding process	297

5.4.3.7.13	Spectral filling in the side channels at 13.2 kbps and 16.4 kbps	300

5.4.3.7.14	Time differential coding	300

5.4.3.7.15	Band interleaving and 40ms MD update rate at 13.2 kbps and 16.4 kbps	301

5.4.3.7.16	Entropy coding with Arithmetic coders	302

5.4.3.7.17	Base2 coding with Huffman coder	304

5.4.3.8	DirAC and SPAR parameter merge	305

5.4.3.8.1	General	305

5.4.3.8.2	DirAC to SPAR parameter conversion	305

5.4.3.8.3	Covariance estimation from DirAC parameters	305

5.4.3.8.4	Directional diffuseness	307

5.4.3.8.5	SPAR parameter estimation from DirAC parameters	307

5.4.3.8.6	Active W prediction from DirAC parameters	307

5.4.3.8.7	Passive W prediction from DirAC parameters	308

5.4.4	Downmix matrix calculation	308

5.4.4.1	SPAR parameters to downmix matrix conversion	308

5.4.4.2	Energy compensation in DirAC bands	309

5.4.4.3	Band Interleaving of downmix matrix at 13.2 and 16.4 kbps	309

5.4.5	Downmix generation	309

5.4.5.1	Overview	309

5.4.5.2	Time domain to MDFT conversion	309

5.4.5.3	Filterbank mixing	310

5.4.5.4	MDFT to Time domain conversion	310

5.4.5.5	Windowing and crossfading	310

5.4.5.6	Time domain mixing	311

5.4.6	Principal Component Analysis (PCA)	311

5.4.7	Automatic Gain Control (AGC)	317

5.4.8	Core-coder encoding	319

5.4.8.1	General	319

5.4.8.2	MCT bitrate distribution in SBA high bitrate mode	319

5.4.9	DTX operation	320

5.4.9.1	Overview	320

5.4.9.2	DirAC parameter estimation	320

5.4.9.3	SPAR parameter estimation	320

5.4.9.3.1	General	320

5.4.9.3.2	Banded covariance smoothing	321

5.4.9.3.3	Quantization and coding in VAD inactive frames	321

5.4.9.4	SPAR and DirAC parameter merge	321

5.4.9.5	CNG parameters encoding	322

5.4.10	SBA bitrate switching	322

5.5	Metadata-assisted spatial audio (MASA) operation	323

5.5.1	MASA format overview	323

5.5.2	MASA format metadata input	324

5.5.2.1	Obtaining the MASA format metadata	324

5.5.2.2	Direction index deindexing	324

5.5.2.3	TF tile based energy calculation	326

5.5.3	Coding of MASA format input data	326

5.5.3.1	Overview	326

5.5.3.2	MASA metadata pre-encoding configuration, processing, and signalling	326

5.5.3.2.1	Overview	326

5.5.3.2.2	Bitrate dependent parameter settings	327

5.5.3.2.3	Metadata composition detection, alignment, and check for validity	328

5.5.3.2.4	Common MASA metadata codec configuration	334

5.5.3.2.5	Energy ratio compensation	336

5.5.3.2.6	Merging of MASA spatial parameter metadata across MASA frequency bands and subframes	337

5.5.3.2.7	Combining of MASA spatial audio metadata across multiple directions	339

5.5.3.2.8	Metadata reductions for low rate	341

5.5.3.2.9	Reordering of directional data with two concurrent directions	342

5.5.3.2.10	Signalling bits	343

5.5.3.2.11	Adjustment of highest transmitted MASA metadata band	343

5.5.3.3	MASA metadata quantization and encoding	344

5.5.3.3.1	Overview	344

5.5.3.3.2	Energy ratio encoding	345

5.5.3.3.3	Direction encoding	345

5.5.3.3.4	Spread coherence encoding	346

5.5.3.3.5	Surround coherence encoding	346

5.5.3.3.6	Coding of the second direction parameters	346

5.5.4	Encoding of MASA audio transport channels	347

5.5.5	DTX operation	347

5.5.6	Bitstream structure	347

5.5.7	MASA bitrate switching	348

5.6	Object-based audio (ISM) operation	348

5.6.1	ISM format overview	348

5.6.2	Discrete ISM coding mode	349

5.6.2.1	Overview	349

5.6.2.2	DiscISM encoding system	349

5.6.2.3	Bitrates distribution between audio streams	350

5.6.2.3.1	Bitrate distribution algorithm	350

5.6.2.3.2	Bitrate adaptation based on ISM importance	351

5.6.3	Parametric ISM coding mode	352

5.6.3.1	General	352

5.6.3.2	Parameters	352

5.6.3.3	Noisy Speech	353

5.6.3.4	Downmix	354

5.6.3.5	Encoded Data	356

5.6.4	ISM metadata coding	356

5.6.4.1	General	356

5.6.4.2	Direction metadata encoding and quantization	357

5.6.4.2.1	General	357

5.6.4.2.2	Intra-object metadata coding logic	358

5.6.4.2.3	Inter-object metadata coding logic	358

5.6.4.3	Extended metadata encoding and quantization	359

5.6.4.4	Panning gain in non-diegetic rendering	359

5.6.5	Bitstream structure	359

5.6.5.1	Overview	359

5.6.5.2	ISM signalling	360

5.6.5.3	Coded metadata payload	360

5.6.5.4	Audio streams payload	360

5.6.6	DTX operation	360

5.6.6.1	Overview	360

5.6.6.2	ISM DTX operation overview	361

5.6.6.3	Classification of Inactive Frames in ISM Encoder	361

5.6.6.3.1	General	361

5.6.6.3.2	Global SID Counter	362

5.6.6.4	SID Metadata Analysis, Quantization and Coding	363

5.6.6.5	CNG parameters encoding	364

5.6.6.6	SID bitstream structure	365

5.6.6.6.1	Overview	365

5.6.6.6.2	ISM Common Signalling in SID Frame	366

5.6.6.6.3	SID Data Payload	366

5.6.6.6.4	SID Audio Streams Payload	366

5.6.7	ISM bitrate switching	367

5.7	Multi-channel audio (MC) operation	367

5.7.1	MC format overview	367

5.7.2	LFE channel encoding	367

5.7.2.1	Low-pass filtering	367

5.7.2.2	MDCT based LFE encoding	368

5.7.2.2.1	Overview	368

5.7.2.2.2	Windowing and MDCT	368

5.7.2.2.3	Quantization	369

5.7.2.2.4	Coding	370

5.7.3	Multi-channel MASA (McMASA) coding mode	371

5.7.3.1	McMASA coding mode overview	371

5.7.3.2	LFE energy computation	373

5.7.3.2.1	LFE energy computation in normal sub-mode	373

5.7.3.2.2	LFE energy computation in separate-channel sub-mode	374

5.7.3.3	McMASA spatial audio parameter estimation	374

5.7.3.4	Even loudspeaker setup determination	377

5.7.3.5	McMASA transport audio signal generation	378

5.7.3.6	 McMASA metadata encoding	379

5.7.3.6.1	McMASA metadata pre-encoding configuration and processing	379

5.7.3.6.2	Spatial metadata encoding	379

5.7.3.6.3	LFE-to-total energy ratio encoding	380

5.7.3.7	McMASA transport audio signal encoding	381

5.7.3.8	Bitstream structure	382

5.7.4	Parametric MC coding mode	382

5.7.4.1	ParamMC coding mode overview	382

5.7.4.2	ParamMC parameter frame index initialization and update	383

5.7.4.3	ParamMC transport channel and parameter band configuration	383

5.7.4.4	ParamMC transport audio signal generation	384

5.7.4.5	ParamMC time domain transient detection	385

5.7.4.6	ParamMC analysis windowing and MDFT	385

5.7.4.7	ParamMC covariance estimation	386

5.7.4.8	ParamMC default settings for the LFE channel	387

5.7.4.9	ParamMC parameter based transient detection	387

5.7.4.10	ParamMC band combining in case of transients	387

5.7.4.11	ParamMC complex valued to real valued covariance conversion	387

5.7.4.12	ParamMC LFE activity detection	388

5.7.4.13	ParamMC parameter quantization	388

5.7.4.13.1	Parameter quantizers	388

5.7.4.13.2	Interchannel Level differences (ICLDs)	388

5.7.4.13.3	Inter-channel coherences (ICCs)	390

5.7.4.14	ParamMC parameter encoding	390

5.7.4.14.1	Common ParamMC parameter encoding	390

5.7.4.14.2	ICC and ICLD Parameter quantization indices encoding	391

5.7.4.15	ParamMC transport audio signal encoding	394

5.7.4.16	ParamMC bit rate switching	394

5.7.5	Parametric upmix MC coding mode	395

5.7.5.1	General	395

5.7.5.2	Sectioning and downmixing	395

5.7.5.3	Metadata parameter calculation	395

5.7.5.4	Quantization	397

5.7.5.5	Entropy Coding	398

5.7.6	Discrete MC coding mode	399

5.7.7	MC bitrate switching	399

5.8	Combined Object-based audio and SBA (OSBA) operation	399

5.8.1	OSBA format overview	399

5.8.2	Low-bitrate pre-rendering OSBA coding mode	399

5.8.3	High-bitrate discrete OSBA coding mode	400

5.8.4	OSBA bitrate switching	400

5.9	Combined Object-based audio and MASA (OMASA) operation	401

5.9.1	OMASA format overview	401

5.9.2	OMASA format configurations	401

5.9.3	OMASA pre-coding processing tools	402

5.9.3.1	OMASA spatial audio parameter analysis	402

5.9.3.2	OMASA MASA metadata combining	403

5.9.3.3	OMASA audio signals downmix	404

5.9.3.4	Determination of an object to be separated	405

5.9.3.5	Separation of an object from other objects	406

5.9.4	Low-bitrate pre-rendering (Rend OMASA) coding mode	407

5.9.4.1	Overview	407

5.9.4.2	Low-bitrate pre-rendering coding method	407

5.9.5	One object with MASA representation (One MASA) coding mode	407

5.9.5.1	Overview	407

5.9.5.2	One object with MASA representation coding method	408

5.9.6	Parametric one object (Param OMASA) coding mode	408

5.9.6.1	Overview	408

5.9.6.2	Determination of OMASA parametric information	408

5.9.6.3	One object with parametric representation coding method	409

5.9.6.3.1	Coding method overview	409

5.9.6.3.2	Encoding of MASA-to-total energy ratios	410

5.9.6.3.3	Encoding of ISM energy ratios	410

5.9.6.3.4	Encoding of ISM metadata	413

5.9.7	Discrete (Disc OMASA) coding mode	414

5.9.8	Inter-format bitrate adaptation	415

5.9.9	OMASA bitrate switching	417

5.9.10	OMASA bitstream structure	418

5.10	EVS-compatible mono audio operation	418

5.11	Stereo downmix operation for EVS mono coding	418

5.11.1	Overview	418

5.11.2	Phase-only correlation (POC) mode	418

5.11.2.1	Overview	418

5.11.2.2	ITD analysis	418

5.11.2.2.1	Cross talk adding procedure in the frequency domain	418

5.11.2.2.2	Approximated phase spectrum	419

5.11.2.2.3	Calculation of phase only correlation	420

5.11.2.2.4	Calculation of ITD and weighting coefficients	420

5.11.2.3	Downmix process	421

5.11.3	Phase compensation (PHA) mode	422

5.11.3.1	Overview	422

5.11.4	Mode selection rule	426

5.11.5	Handling of mode switching	427

6	Functional description of the decoder	427

6.1	Decoder overview	427

6.2	Common processing and decoding tools	428

6.2.1	Common decoder processing overview	428

6.2.1.1	High-pass filtering	428

6.2.1.2	Limiter	428

6.2.1.2.1	Overview	428

6.2.1.2.2	Detailed algorithmic description	429

6.2.1.2.3	Handling of strong saturations	430

6.2.1.3	Time-domain decorrelator	432

6.2.2	Core-decoder processing	433

6.2.2.1	Overview	433

6.2.2.2	LP based decoding	434

6.2.2.2.1	Variable bitrate ACELP decoding	434

6.2.2.2.2	Fast algebraic codebook decoding	434

6.2.2.2.3	AVQ Bit Savings Decoder	434

6.2.2.2.4	Bass Post-Filter	438

6.2.2.2.5	GSC core decoding	438

6.2.2.2.6	Bandwidth extension in time domain	441

6.2.2.2.7	Multimode FD bandwidth extension coding	441

6.2.2.3	MDCT based decoding	441

6.2.2.3.1	General	441

6.2.2.3.2	Variable bitrate MDCT decoding	441

6.2.2.3.3	TCX entropy decoding	441

6.2.2.3.4	PLC method selection	443

6.2.2.3.5	Phase ECU enhancements	444

6.2.2.3.6	Long term prediction processing	445

6.2.2.4	Background noise estimation	447

6.2.2.4.1	General	447

6.2.2.4.2	Power spectrum compression	447

6.2.2.4.3	Compensation for the loss of variance	449

6.2.2.4.4	Spectral smoothing	449

6.2.2.4.5	Initialization period	450

6.2.2.4.6	Power spectrum expansion	452

6.2.2.5	Switching coding modes	452

6.2.2.5.1	Improved handling of envelope stability parameter in HQ MDCT	452

6.2.2.6	DTX/CNG operation	453

6.2.2.6.1	General	453

6.2.2.6.2	FD-CNG VQ global gain dequantization	453

6.2.2.6.3	First stage VQ decoding for FD-CNG	453

6.2.3	Common audio decoding tools	454

6.2.3.1	General	454

6.2.3.2	Single Channel Element (SCE) decoder	454

6.2.3.3	Channel Pair Element (CPE) decoder	455

6.2.3.4	Multichannel coding tool (MCT) decoder	457

6.2.3.4.1	General overview	457

6.2.3.4.2	Core and SNS parameter decoding	457

6.2.3.4.3	MCT parameter decoding	457

6.2.3.4.4	Bitrate distribution	457

6.2.3.4.5	Spectral data decoding and noise filling	458

6.2.3.4.6	Application of IGF and stereo IGF	458

6.2.3.4.7	MCT decoding	458

6.2.3.4.8	Noise shaping	458

6.2.3.4.9	Inverse MDCT transform to time domain	459

6.2.3.4.10	MCT PLC	459

6.2.4	Common spatial metadata decoding tools	459

6.2.4.1	Direction metadata decoding tools	459

6.2.4.1.1	Overview	459

6.2.4.1.2	Direction dequantization tools	459

6.2.4.1.3	Direction metadata raw decoding	460

6.2.4.1.4	Direction metadata entropy decoding tools	460

6.2.4.2	Diffuseness and energy ratio decoding methods	461

6.2.4.2.1	Diffuseness decoding	461

6.2.4.2.2	Diffuseness and energy ratio decoding with two concurrent directions	461

6.2.4.3	Direction metadata decoding methods	462

6.2.4.3.1	Direction metadata decoding overview	462

6.2.4.3.2	Decoding EC1	462

6.2.4.3.3	Decoding EC2	463

6.2.4.3.4	Decoding EC3	463

6.2.4.4	Coherence decoding	465

6.2.4.4.1	Spread coherence decoding	465

6.2.4.4.2	Surround coherence decoding	466

6.2.4.5	DTX decoding	466

6.2.5	Common filter bank operation tools	467

6.2.5.1	Complex Low-delay Filter Bank (CLDFB) analysis	467

6.2.5.2	Complex Low-delay Filter Bank (CLDFB) synthesis	467

6.2.6	CLDFB domain decorrelator	468

6.2.6.1	Overview	468

6.2.6.2	Onset detection and removal	468

6.2.6.3	De-correlation	469

6.2.6.3.1	Overview	469

6.2.6.3.2	Derivation of filter coefficients from lattice coefficients	469

6.2.6.3.3	Decorrelator IIR Filtering	472

6.2.6.4	Energy adjustment	473

6.2.6.5	Final output	473

6.2.7	Common tools for decoding with time scale modification (JBM)	474

6.2.7.1	Overview	474

6.2.7.2	Transport channel buffer management	474

6.2.7.3	Time scale modification	475

6.2.7.3.1	Extension to multi-channel processing	475

6.2.7.3.2	Energy estimation	475

6.2.7.4	Metadata and processing parameter adaption	476

6.2.7.4.1	Overview	476

6.2.7.4.2	Metadata adaption	476

6.2.7.4.3	Processing parameter adaption	478

6.2.7.5	Bitrate switching	480

6.2.7.5.1	Renderer flushing	480

6.2.7.5.2	Transport channel reconfiguration	480

6.2.7.5.3	Renderer discard samples	480

6.3	Stereo audio decoding	481

6.3.1	Common stereo decoding tools	481

6.3.1.1	Common Stereo postprocessing	481

6.3.1.2	ITD compensation	481

6.3.1.2.1	Overview	481

6.3.1.2.2	Inter-channel Alignment (ICA) decoder	481

6.3.2	Unified stereo decoding	482

6.3.2.1	Inter-channel BWE (IC-BWE) decoder	482

6.3.2.1.1	Inter-Channel BWE Decoder overview	482

6.3.2.1.2	Inter-Channel BWE Decoder Block	483

6.3.2.2	TD-based stereo decoding	484

6.3.2.2.1	TD stereo decoder overview	484

6.3.2.2.2	Reusing close-loop pitch and adaptive cobebook of the primary channel	485

6.3.2.2.3	Reusing LP filter coefficients of the primary channel	486

6.3.2.2.4	Estimation of spatial parameters of the background noise	486

6.3.2.3	DFT-based stereo decoding	487

6.3.2.3.1	General	487

6.3.2.3.2	STFT analysis	487

6.3.2.3.3	Stereo parameter decoding	488

6.3.2.3.4	Residual signal decoding	489

6.3.2.3.5	Bass post filter application	490

6.3.2.3.6	Residual predicted signal generation	492

6.3.2.3.7	Stereo upmix	499

6.3.2.3.8	Stereo comfort noise injection	499

6.3.2.3.9	STFT synthesis	501

6.3.2.3.10	DFT-based stereo PLC	502

6.3.3	MDCT-based stereo decoding	505

6.3.3.1	General Overview	505

6.3.3.2	Decoding of parameters	505

6.3.3.2.1	TCX block configuration	505

6.3.3.2.2	Core parameters decoding	505

6.3.3.2.3	SNS parameters decoding	505

6.3.3.2.4	Stereo parameters decoding	507

6.3.3.3	Decoding process	508

6.3.3.3.1	Spectral data decoding and noise filling	508

6.3.3.3.2	Application of stereo IGF	508

6.3.3.4	Stereo decoding process	509

6.3.3.4.1	Inverse mid-side transform	509

6.3.3.4.2	De-normalization to global ILD	511

6.3.3.5	Post-stereo decoding process	511

6.3.3.5.1	General	511

6.3.3.5.2	Spectral noise shaping	511

6.3.3.5.3	Temporal noise shaping	512

6.3.3.6	Frequency-to-time domain transformations	512

6.3.3.6.1	General	512

6.3.3.6.2	Kernel switching	512

6.3.3.6.3	Transition between long and short blocks with double overlap	512

6.3.3.7	PLC in MDCT-based stereo	514

6.3.3.7.1	Frequency-domain concealment	514

6.3.3.7.2	Time-domain concealment	518

6.3.3.7.3	Fading to zero output	519

6.3.4	Switching between stereo modes	519

6.3.4.1	Overview	519

6.3.4.2	General	519

6.3.4.3	Memory handling	522

6.3.4.4	Switching between DFT and TD stereo modes	523

6.3.4.4.1	Switching from TD stereo to DFT stereo	523

6.3.4.4.2	Switching from DFT stereo to TD stereo	526

6.3.4.5	Switching between DFT and MDCT stereo modes	529

6.3.4.5.1	Switching from DFT stereo to MDCT stereo	529

6.3.4.5.2	Switching from MDCT stereo to DFT stereo	529

6.3.4.6	Switching between TD and MDCT stereo modes	529

6.3.4.6.1	Switching from TD stereo to MDCT stereo	529

6.3.4.6.2	Switching from MDCT stereo to TD stereo	530

6.3.5	DTX operation	530

6.3.5.1	DTX in Unified stereo	530

6.3.5.1.1	General	530

6.3.5.1.2	Stereo CNG spectral shape extraction	530

6.3.5.1.3	Stereo CNG side gain, ITD and IPD decoding	533

6.3.5.1.4	Stereo CNG coherence decoding	534

6.3.5.1.5	Stereo CNG synthesis	534

6.3.5.2	DTX in MDCT-based stereo	535

6.3.5.2.1	General overview	535

6.3.5.2.2	Decoding of parametric noise data	535

6.3.5.2.3	Stereo CNG	535

6.3.5.2.4	Smoothing of transitions from DTX frame to active TCX	536

6.3.6	Stereo output format conversion	537

6.3.6.1	Overview	537

6.3.6.2	Stereo-to-Mono output	537

6.3.6.2.1	DFT stereo mono output	537

6.3.6.2.2	TD stereo mono output	538

6.3.6.2.3	MDCT stereo mono output	538

6.3.6.3	Stereo-to-MC output	542

6.3.7	Stereo audio decoding with TSM	542

6.4	Scene-based audio (SBA) decoding	542

6.4.1	Combined DirAC and SPAR based SBA decoding overview	542

6.4.1.1	Overview	542

6.4.1.2	FOA signal decoding at all bitrates and HOA2, HOA3 signal decoding at bitrates below 256 kbps	543

6.4.1.2.1	Overview	543

6.4.1.2.2	Metadata decoding	544

6.4.1.2.3	Core-coder decoding	544

6.4.1.2.4	SBA upmix and rendering	544

6.4.1.3	HOA2 and HOA3 signal decoding at bitrate 256 kbps	545

6.4.1.3.1	Overview	545

6.4.1.3.2	Metadata decoding	545

6.4.1.3.3	Core-coder decoding	545

6.4.1.3.4	SBA upmix and rendering	546

6.4.1.4	HOA2 and HOA3 signal decoding at bitrates 384 kbps	546

6.4.1.4.1	Overview	546

6.4.1.4.2	Metadata decoding	547

6.4.1.4.3	Core-coder decoding	547

6.4.1.4.4	SBA upmix and rendering	547

6.4.1.5	HOA2 and HOA3 signal coding at bitrates 512 kbps	548

6.4.1.5.1	Overview	548

6.4.1.5.2	Metadata decoding	548

6.4.1.5.3	Core-coder decoding	548

6.4.1.5.4	SBA upmix and rendering	548

6.4.2	DirAC parameter decoding	549

6.4.3	SPAR parameter decoding	549

6.4.3.1	General	549

6.4.3.2	Set active W prediction flag	549

6.4.3.3	Decode quantization and coding strategy bits	549

6.4.3.4	Time differential decoding	550

6.4.3.4.1	General	550

6.4.3.4.2	Time differential decoding across same quantization strategies	550

6.4.3.4.3	Time differential decoding across different quantization strategies	551

6.4.3.5	Band interleaved decoding at 13.2 kbps and 16.4 kbps	551

6.4.3.6	Arithmetic decoder	551

6.4.3.7	Huffman decoder	551

6.4.4	SPAR and DirAC parameter merge	551

6.4.4.1	SBA mono handling	551

6.4.4.2	SPAR to DirAC parameter conversion	552

6.4.4.3	DirAC model for the SPAR parameters	553

6.4.4.3.1	Overview	553

6.4.4.3.2	DirAC to SPAR conversion for parameters corresponding to downmix channels	553

6.4.4.3.3	DirAC to SPAR conversion for PR, CP and D coefficients corresponding to parametric channels	553

6.4.5	Upmix matrix calculation	554

6.4.5.1	Overview	554

6.4.5.2	SPAR matrix generation (P and C matrices)	554

6.4.5.3	Upmix matrix generation	555

6.4.6	Decoded audio processing	555

6.4.6.1	Downmix signal decoding (core-decoding)	555

6.4.6.2	AGC	555

6.4.6.3	PCA	556

6.4.6.4	SPAR upmix processing	558

6.4.6.4.1	General	558

6.4.6.4.2	Time domain decorrelator	558

6.4.6.4.3	CLDFB analysis	558

6.4.6.4.4	CLDFB upmixing	558

6.4.6.4.5	MDFT filterbank banded upmix matrix to CLDFB banded upmix matrix conversion	560

6.4.6.4.6	Crossfading in CLDFB	561

6.4.6.4.7	Additional VLBR smoothing	561

6.4.6.5	DirAC synthesis and rendering	561

6.4.6.5.1	Overview	561

6.4.6.5.2	Mode selection	562

6.4.6.5.3	High-order operation mode	562

6.4.6.5.4	Low-order operation mode	565

6.4.6.5.5	Bitrate switching	568

6.4.6.5.6	Binaural rendering	568

6.4.6.5.7	Loudspeaker rendering	569

6.4.6.5.8	Stereo and mono output	571

6.4.6.6	CLDFB synthesis	575

6.4.7	DTX operation	576

6.4.7.1	Overview	576

6.4.7.2	DTX in DirAC	576

6.4.7.3	DTX in SPAR	576

6.4.8	SBA PLC	577

6.4.8.1	PLC in DirAC	577

6.4.8.2	PLC in SPAR	577

6.4.8.2.1	SPAR processing of erroneous frames	577

6.4.8.2.2	Recovery after one or more lost frames	578

6.4.8.2.3	SPAR parameter recovery for 24.4 kbps and above	578

6.4.8.2.4	SPAR parameter recovery for 13.2 kbps and 16.4 kbps	578

6.4.8.2.5	Interpolation of missing SPAR parameters from current existing SPAR parameters	578

6.4.9	SBA bitrate switching	579

6.4.10	SBA output format conversion	579

6.4.11	SBA decoding with TSM	580

6.5	Metadata-assisted spatial audio (MASA) decoding	581

6.5.1	Overview	581

6.5.2	Decoding MASA configuration	581

6.5.3	MASA metadata decoding	582

6.5.3.1	Overview	582

6.5.3.2	Energy ratio decoding	583

6.5.3.2.1	Energy ratio decoding for bitrates up to 256 kbps	583

6.5.3.2.2	Energy ratio decoding for 384 kbps and 512 kbps	583

6.5.3.3	Direction decoding	583

6.5.3.3.1	Direction decoding for bitrates up to 256 kbps	583

6.5.3.3.2	Direction decoding for 384 kbps and 512 kbps	584

6.5.3.4	Spread coherence decoding	584

6.5.3.4.1	Spread coherence decoding for bitrates up to 256 kbps	584

6.5.3.4.2	Spread coherence decoding for 384 kbps and 512 kbps	584

6.5.3.5	Surround coherence decoding	584

6.5.3.5.1	Surround coherence decoding for bitrates up to 256 kbps	584

6.5.3.5.2	Surround coherence decoding for 384 kbps and 512 kbps	584

6.5.3.6	Decoding of the second direction parameters	584

6.5.3.6.1	Joint decoding of parameters associated with first and second sound source directions	584

6.5.3.6.2	Independent decoding of parameters associated with first and second sound source directions	585

6.5.3.7	Reconstruction of MASA metadata from low-rate reduction	585

6.5.3.8	Determining encoding metric based on the quality of the spatial metadata representation in encoding	585

6.5.3.9	Reconstruction of full MASA format metadata	586

6.5.4	MASA DTX operation	587

6.5.5	MASA PLC	587

6.5.6	MASA bitrate switching	587

6.5.7	MASA rendering	587

6.5.7.1	Binaural and stereo rendering	587

6.5.7.2	Multi-channel loudspeaker and Ambisonics rendering	587

6.5.7.2.1	Overview	587

6.5.7.2.2	Determining direct and diffuse power factors and surround coherence ratio	588

6.5.7.2.3	Determining directional responses	589

6.5.7.2.4	Determining diffuse responses	592

6.5.7.2.5	Determining prototype audio signals for multi-channel loudspeaker output	593

6.5.7.2.6	Determining prototype audio signals for Ambisonics output	593

6.5.7.2.7	Decorrelation	596

6.5.7.2.8	Spatial synthesis	597

6.5.7.3	Mono rendering	599

6.5.7.4	Output for external processing	599

6.5.8	MASA decoding with TSM	599

6.6	Object-based audio (ISM) decoding	600

6.6.1	Discrete ISM decoding mode	600

6.6.2	Parametric ISM decoding mode	601

6.6.2.1	General	601

6.6.2.2	Parameters	601

6.6.2.3	Downmix	602

6.6.2.4	ParamISM Decoding and Rendering	602

6.6.2.4.1	Output Formats	602

6.6.2.4.2	Mono, stereo, and binaural output	602

6.6.2.4.3	Loudspeaker and Ambisonics output	603

6.6.2.4.4	EXT output	605

6.6.3	ISM metadata decoding	606

6.6.4	DTX operation decoding	606

6.6.4.1	Overview	606

6.6.4.2	CNG control parameter decoding	606

6.6.4.3	Metadata Dequantization and Decoding	607

6.6.4.4	ISM DTX core decoding and CNG	607

6.6.5	ISM PLC	609

6.6.6	ISM bitrate switching	609

6.6.6.1	Metadata handling in bitrate switching	609

6.6.6.2	Codec reconfiguration in bitrate switching	610

6.6.7	ISM output format conversion	610

6.6.7.1	Overview	610

6.6.7.2	Mono downmix	611

6.6.8	ISM decoding with TSM	612

6.7	Multi-channel audio (MC) decoding	612

6.7.1	LFE channel decoding	612

6.7.1.1	Overview	612

6.7.1.2	Bitstream decoding	612

6.7.1.3	MDCT coefficient reconstruction	613

6.7.1.4	Inverse MDCT	613

6.7.1.5	Windowing and overlap-add	614

6.7.1.6	Output LPF and delay adjustment	614

6.7.1.7	LFE PLC	614

6.7.1.7.1	Overview	614

6.7.1.7.2	Burst loss determination	614

6.7.1.7.3	LFE substitution frame processing	615

6.7.2	Multi-channel MASA (McMASA) decoding mode	617

6.7.2.1	McMASA decoding mode overview	617

6.7.2.2	McMASA metadata decoding	617

6.7.2.2.1	Decoding McMASA configuration	617

6.7.2.2.2	Spatial metadata decoding	617

6.7.2.2.3	LFE-to-total energy ratio decoding	617

6.7.2.3	McMASA transport audio signal decoding	618

6.7.2.4	McMASA rendering	618

6.7.2.4.1	Binaural rendering	618

6.7.2.4.2	Multi-channel loudspeaker rendering	618

6.7.2.4.3	Ambisonics rendering	623

6.7.2.4.4	Stereo rendering	623

6.7.2.4.5	Mono rendering	623

6.7.2.5	McMASA PLC	623

6.7.2.6	McMASA decoding with TSM	623

6.7.3	Parametric MC decoding mode	624

6.7.3.1	ParamMC Overview	624

6.7.3.2	ParamMC configuration	624

6.7.3.3	ParamMC Parameter Decoding	624

6.7.3.3.1	Common ParamMC parameter decoding	624

6.7.3.3.2	ICC and ICLD Parameter decoding	625

6.7.3.4	ParamMC Transport Audio Signal Decoding	627

6.7.3.5	ParamMC Synthesis	627

6.7.3.5.1	ParamMC Synthesis Overview	627

6.7.3.5.2	ParamMC Target Covariance Calculation	627

6.7.3.5.3	ParamMC Mixing Matrix Calculation	629

6.7.3.5.4	Decorrelation	632

6.7.3.5.5	Upmix	632

6.7.3.6	ParamMC output processing	633

6.7.3.6.1	ParamMC output processing selection	633

6.7.3.6.2	ParamMC mono/stereo output processing	633

6.7.3.6.3	ParamMC loudspeaker output processing	634

6.7.3.6.4	ParamMC Ambisonics output processing	635

6.7.3.6.5	ParamMC binaural output processing	635

6.7.3.7	ParamMC PLC	636

6.7.3.8	ParamMC bit rate switching	636

6.7.3.9	ParamMC decoding with TSM	637

6.7.3.9.1	Decoding to mono/stereo	637

6.7.3.9.2	Decoding to all other output formats	637

6.7.4	Parametric upmix MC decoding mode	637

6.7.4.1	General	637

6.7.4.2	Special case of Binaural, Stereo and Mono rendering configurations	638

6.7.4.2.1	Binaural rendering configuration	638

6.7.4.2.2	Stereo and Mono rendering configuration	638

6.7.4.3	Metadata Huffman Decoding	638

6.7.4.4	Dequantization	638

6.7.4.5	Decorrelation	638

6.7.4.6	Upmixing operation	639

6.7.4.7	Output	639

6.7.4.8	Param Upmix PLC	639

6.7.4.9	Param Upmix MC decoding with TSM	640

6.7.5	Discrete MC decoding mode	640

6.7.5.1	Overview	640

6.7.5.2	Discrete MC PLC	640

6.7.6	MC bitrate switching	640

6.7.7	MC output format conversion	641

6.7.7.1	Overview	641

6.7.7.2	Common processing tools	641

6.7.7.2.1	Conversion of transport signals to TCX20 resolution	641

6.7.7.2.2	Computation of Equalization gain	641

6.7.7.3	Equalization of TCX Spectra	641

6.7.7.4	Rendering to a supported IVAS format	642

6.7.7.4.1 	 General	642

6.7.7.4.2	Mono and Stereo output	642

6.7.7.4.3	Remaining formats	642

6.7.7.5	Rendering to a custom loudspeaker layout	647

6.7.7.6	Rendering to binaural output with head tracking	647

6.7.8	MC decoding with TSM	648

6.8	Combined Object-based audio and SBA (OSBA) decoding	648

6.8.1	Low-bitrate pre-rendering OSBA decoding mode	648

6.8.2	High-bitrate discrete OSBA decoding mode	648

6.8.3	OSBA PLC	649

6.8.4	OSBA bitrate switching	649

6.8.5	OSBA output format conversion	649

6.8.6	OSBA decoding with TSM	649

6.9	Combined Object-based audio and MASA (OMASA) decoding	651

6.9.1	OMASA format decoder overview	651

6.9.2	Low-bitrate pre-rendering (Rend OMASA) decoding mode	652

6.9.3	One object with MASA (One OMASA) decoding mode	652

6.9.4	Parametric one object (Param OMASA) decoding mode	652

6.9.4.1	Overview	652

6.9.4.2	MASA-to-total ratios decoding	653

6.9.4.3	ISM ratios decoding	653

6.9.4.4	ISM metadata decoding	656

6.9.5	Discrete (Disc OMASA) decoding mode	657

6.9.6	OMASA rendering metadata generation	657

6.9.7	OMASA rendering	658

6.9.7.1	Binaural rendering	658

6.9.7.1.1	Discrete coding mode	658

6.9.7.1.2	Other coding modes	658

6.9.7.2	Stereo rendering	658

6.9.7.3	Multi-channel loudspeaker and Ambisonics rendering	658

6.9.7.3.1	Overview	658

6.9.7.3.2	Rendering of the MASA part	659

6.9.7.4	Mono rendering	660

6.9.8	OMASA PLC	660

6.9.9	OMASA bitrate switching	660

6.9.10	OMASA decoding with TSM	660

6.10	EVS-compatible mono audio decoding	660

7	Functional description of the rendering, rendering control, and pre-rendering	661

7.1	Rendering overview	661

7.2	Rendering modes	661

7.2.1	Rendering for loudspeaker reproduction	661

7.2.1.1	Overview	661

7.2.1.2	Vector-base amplitude panning (VBAP)	661

7.2.1.2.1	VBAP initialization	661

7.2.1.2.2	VBAP gain determination	666

7.2.1.3	Edge Fading Amplitude Panning (EFAP)	667

7.2.1.3.1	EFAP overview	667

7.2.1.3.2	EFAP initialization	667

7.2.1.3.3	EFAP gain computation	669

7.2.1.4	All-round ambisonic panning and decoding	670

7.2.1.4.1	Overview	670

7.2.1.4.2	Mono and Stereo rendering	670

7.2.1.4.3	Rendering to other loudspeaker outputs	671

7.2.2	Rendering for binaural headphone reproduction	671

7.2.2.1	Binaural rendering overview	671

7.2.2.2	Time Domain binaural renderer	672

7.2.2.2.1	General	672

7.2.2.2.2	HRIR model	673

7.2.2.2.3	Obtain standard spline sample values	677

7.2.2.2.4	Obtain periodic spline sample values	678

7.2.2.2.5	ITD model	679

7.2.2.2.6	ITD synthesis	680

7.2.2.2.7	Distance and Direction Gain	682

7.2.2.2.8	HRIR convolution	684

7.2.2.3	Parametric binauralizer and parametric stereo renderer	685

7.2.2.3.1	Overview	685

7.2.2.3.2	Pre-processing the transport audio signals based on head orientation	686

7.2.2.3.3	Determination of input and target covariance matrices	689

7.2.2.3.4	Determining processing matrices based on input and target covariance matrices	693

7.2.2.3.5	Processing audio signals with the processing matrices	694

7.2.2.3.6	Determining direct part gains	695

7.2.2.3.7	Determining regularization factor	697

7.2.2.3.8	Decorrelation in the parametric binauralizer	698

7.2.2.3.9	SPAR metadata to MASA metadata mapping	698

7.2.2.3.10	Prototype signal generation with SBA format input	701

7.2.2.4	Fast convolution binaural renderer	701

7.2.2.4.1	CLDFB-domain convolution	701

7.2.2.4.2	Filter conversion	702

7.2.2.4.3	Late reverb model	703

7.2.2.4.4	Head tracking	706

7.2.2.5	Crend binaural renderer	706

7.2.2.5.1	Terms and Definitions	706

7.2.2.5.2	General	706

7.2.2.5.3	Convolver	707

7.3	Room acoustics rendering	709

7.3.1	Introduction to room acoustics rendering	709

7.3.2	Room impulse response convolution	709

7.3.3	Sparse frequency-domain reverberator	709

7.3.4	Feedback-delay network reverberator	710

7.3.4.1	Overview	710

7.3.4.2	Reverberator configuration	710

7.3.4.3	Processing	714

7.3.5	Early-reflection synthesis	714

7.3.5.1	General	714

7.3.5.2	Coordinate System	715

7.3.5.3	Source-receiver location correction	715

7.3.5.4	Reflections calculations	716

7.3.5.5	Process Loop	716

7.3.5.6	Low-complexity mode	718

7.4	Rendering control	718

7.4.1	Rendering control overview	718

7.4.2	Scene and listener orientation	719

7.4.2.1	Scene orientation	719

7.4.2.2	Listener orientation	719

7.4.3	Head tracking	719

7.4.3.1	Head tracking via scene displacement	719

7.4.3.2	Conversion from Euler angles to quaternions	720

7.4.3.3	Rotation matrix from quaternions	720

7.4.3.4	Application of rotations	720

7.4.3.4.1	Rotation in the spatial domain	720

7.4.3.4.2	Rotation in the spherical harmonic domain	721

7.4.4	Orientation tracking	721

7.4.4.1	Orientation tracking introduction	721

7.4.4.2	External reference orientation	722

7.4.4.3	External reference vector orientation	722

7.4.4.4	External reference levelled vector orientation	723

7.4.4.5	Adaptive long-term average reference orientation	723

7.4.5	External orientation input handling	725

7.4.5.1	Overview	725

7.4.5.2	Processing of the external orientation data	727

7.4.6	Combined rotations for rendering	727

7.4.6.1	Combining head and external rotations	727

7.4.6.2	External rotation interpolation	729

7.4.6.3	Initial values for combined rotation variables	730

7.4.7	HRTF and BRIR sets	731

7.4.7.1	HRTF and BRIR latency	731

7.4.7.2	Parametrization of Binaural renderers using binary file	731

7.4.7.3	HRTFs and BRIR conversion methods	732

7.4.7.3.1	Conversion from spatial domain to spherical harmonics domain	732

7.4.7.3.2	Conversion from Time domain to CLDFB domain for HRIRs (Fast convolution binaural renderer)	735

7.4.7.3.3	Conversion from Time domain to CLDFB domain for BRIRs (Fast convolution binaural renderer)	736

7.4.7.3.4	Conversion from Time domain to HRIR/ITD model (Time Domain binaural renderer)	736

7.4.7.3.5	Conversion from HRTF/BRIR to Crend binaural renderer convolver parameters	741

7.4.7.3.6	Conversion from Time domain to SH CLDFB domain for parametric binauralizer	745

7.4.8	Room acoustics parameters	745

7.4.8.1	Overview	745

7.4.8.2	Late reverb parameters	746

7.4.8.3	Early reflections parameters	746

7.4.9	Use of custom loudspeaker layouts	747

7.4.9.1	General	747

7.4.9.2	MASA, OMASA, and McMASA	748

7.4.9.3	ParamMC and Parametric Upmix MC	748

7.4.9.4	Remaining formats	748

7.5	Pre-rendering	748

7.5.1	Overview	748

7.5.2	Pre-rendering into SBA format	749

7.5.2.1	ISM to SBA rendering	749

7.5.2.2	Channel-based audio to SBA rendering	749

7.5.2.3	MASA to SBA rendering	749

7.5.3	Pre-rendering into MASA format	749

7.5.3.1	Overview	749

7.5.3.2	ISM to MASA rendering	749

7.5.3.3	Multi-channel to MASA rendering	749

7.5.3.4	SBA to MASA rendering	750

7.5.3.5	Merging audio signals rendered into MASA format	750

7.5.3.5.1	MASA input to MASA output rendering	750

7.5.3.5.2	MASA signal merge	750

7.5.3.5.3	Merging MASA metadata from ISM format input with inputs of other formats	751

7.5.3.5.4	Merging MASA metadata from other input formats	751

7.5.4	Pre-rendering into Binaural format	751

7.5.4.1	SBA and MC to Binaural rendering	751

7.5.4.2	ISM to Binaural rendering	752

7.5.4.3	MASA to Binaural rendering	752

7.5.5	Pre-rendering into MC format	752

7.5.5.1	ISM to MC rendering	752

7.5.5.2	SBA to MC rendering	752

7.5.5.3	MASA to MC rendering	752

7.5.5.4	MC to MC rendering	752

7.6	Split rendering	26

7.6.1	Overview	26

7.6.2	Split pre-rendering	27

7.6.2.1	Overview	27

7.6.2.2	Supported Split Rendering bitrates with LCLD or LC3plus codec	28

7.6.2.3	Supported Split Rendering bitrates with PCM output	28

7.6.2.4	Split pre-rendering of SBA	28

7.6.2.5	Split pre-rendering of MASA	28

7.6.2.6	Split pre-rendering of CBA	29

7.6.2.7	Split pre-rendering of ISM	29

7.6.2.8	Split pre-rendering of OSBA	29

7.6.2.9	Split pre-rendering of OMASA	29

7.6.3	Intermediate split renderer metadata format	29

7.6.3.1	Overview	29

7.6.3.2	Metadata computation, quantization and coding	30

7.6.3.2.1	Metadata computation for deviations about Yaw axis	30

7.6.3.2.2	Quantization and coding of Yaw metadata	31

7.6.3.2.3	Metadata computation for deviations about Pitch axis	32

7.6.3.2.4	Quantization and coding of Pitch metadata	32

7.6.3.2.5	Metadata computation for deviations about Roll axis	32

7.6.3.2.6	Quantization and coding of Roll metadata	32

7.6.3.3	Common split rendering metadata quantization and coding strategies	33

7.6.3.4	Intermediate split renderer metadata decoder	33

7.6.3.5	Intermediate split renderer metadata loss concealment	34

7.6.4	LCLD coded intermediate split renderer binaural audio format	34

7.6.4.1	LCLD codec overview	34

7.6.4.2	LCLD encoder	34

7.6.4.2.1	Overview	34

7.6.4.2.2	Perceptual Banding	35

7.6.4.2.3	Joint Channel Coding	36

7.6.4.2.3.1	Overview	36

7.6.4.2.3.2	Bitstream Syntax	37

7.6.4.2.3.3	Parameter Computation and Quantization	37

7.6.4.2.3.4	Joint Coding Type Decision	38

7.6.4.2.3.5	Entropy Coding of Parameters	39

7.6.4.2.4	Temporal Grouping	39

7.6.4.2.5	RMS Envelope	43

7.6.4.2.5.1	RMS Envelope Calculation	43

7.6.4.2.5.2	Normalizing the CLDFB Coefficients with the RMS Envelope	44

7.6.4.2.5.3	RMS Envelope Transmission	44

7.6.4.2.6	Perceptual Model	44

7.6.4.2.7	Linear Prediction	51

7.6.4.2.7.1	Overview	51

7.6.4.2.7.2	Bitstream Syntax	51

7.6.4.2.7.3	Prediction for 20ms frames	51

7.6.4.2.7.4	Prediction for frames shorter than 20ms	52

7.6.4.2.7.5	Prediction Signalling	52

7.6.4.2.7.6	Quantization of Prediction Parameters	53

7.6.4.2.7.7	Estimation of Prediction Parameters	53

7.6.4.2.8	Bit Allocation	54

7.6.4.2.9	Quantization of the Normalized CLDFB Coefficients	55

7.6.4.2.9.1	Overview	55

7.6.4.2.9.2	Differential Coding of the Normalized CLDFB Coefficients	55

7.6.4.2.9.3	Quantization of Normalized CLDFB coefficients and Prediction Residuals	56

7.6.4.2.9.4	Huffman Coding of Quantized Normalized CLDFB coefficients and Quantized Prediction Residuals	57

7.6.4.3	LCLD decoder	59

7.6.4.3.1	Overview	59

7.6.4.3.2	Decoding Group Information	59

7.6.4.3.3	Decoding RMS Envelope Information	59

7.6.4.3.4	Perceptual Model	60

7.6.4.3.5	Bit Allocation	60

7.6.4.3.6	Normalized CLDFB Coefficient and Prediction Residual Huffman Decoding and Inverse Quantization	60

7.6.4.3.7	Inverse Prediction	61

7.6.4.3.7.1	Overview	61

7.6.4.3.7.2	Status Tracking after Frame Loss	61

7.6.4.3.8	Inverse RMS Envelope Normalization	62

7.6.4.3.9	Inverse Joint Stereo Processing	62

7.6.4.4	LCLD packet loss concealment	63

7.6.4.4.1	General	63

7.6.4.4.2	Synthesis model	63

7.6.4.4.3	Analysis and parameter estimation	64

7.6.4.4.4	Tonality determination	65

7.6.4.4.5	Sinusoidal extension	65

7.6.4.4.6	Predictive extension	65

7.6.4.4.7	Cross-fade	65

7.6.4.4.8	Burst-loss handling	66

7.6.5	LC3plus coded intermediate split renderer binaural audio format	66

7.6.5.1	Introduction (Informative)	66

7.6.5.2	Overview	66

7.6.5.3	Encoder	66

7.6.5.4	Decoder	66

7.6.5.5	Frame Structure	66

7.6.5.6	Packet Loss Concealment	66

7.6.5.7	LC3 interoperable mode	67

7.6.6	Split post-rendering	67

7.6.6.1	Overview	67

7.6.6.2	Post rendering with pose correction	67

7.6.6.2.1	Metadata decoding	67

7.6.6.2.2	Metadata interpolation or extrapolation	67

7.6.6.2.3	Matrix mixing	68

7.6.6.3	Post rendering in 0 DOF mode	68

7.6.7	Bit allocation for Split rendering	68

7.6.8	Interface for Split rendering	70

8	 Description of the transmitted parameter indices	752

8.1	Bit allocation overview	752

8.2	Bit allocation for stereo audio	752

8.2.1	Bit allocation for stereo in active frames	752

8.2.2	Bit allocation for stereo in SID frames	753

8.3	Bit allocation for scene-based audio (SBA)	753

8.3.1	Bit allocation for SBA in active frames	753

8.3.2	Bit allocation for SBA in SID frames	755

8.4	 Bit allocation for metadata-assisted spatial audio (MASA)	756

8.4.1	Bit allocation for MASA in active frames	756

8.4.2	Bit allocation for MASA in SID frames	756

8.5	Bit allocation for object-based audio (ISM)	756

8.5.1	Bit allocation for ISM in active frames	756

8.5.2	Bit allocation for ISM in SID frames	756

8.6	Bit allocation for multi-channel audio (MC)	756

8.7	Bit allocation for combined Object-based audio and SBA (OSBA)	757

8.8	Bit allocation for combined Object-based audio and MASA (OMASA)	757

8.9	Bit allocation for EVS-compatible mono audio	757

Annex A (normative): RTP Payload Format and SDP Parameters	758

A.1	Introduction	758

A.2	Conventions, Definitions and Acronyms	758

A.2.1	Byte Order	758

A.2.2	List of Acronyms	758

A.3	Payload Format	758

A.3.1	Format Overview	758

A.3.2	RTP Header Usage	759

A.3.3	Packet Payload Structure	759

A.3.3.1	General	759

A.3.3.2	Format Description	760

A.3.3.3	Payload Header	760

A.3.3.3.1	General	760

A.3.3.3.2	ToC byte	760

A.3.3.3.3	E (Extra) byte	761

A.3.4	Frame Data	761

A.4	Payload Format Parameters	762

A.4.1	IVAS Media Type Registration	762

Annex B (informative): Change history	764

* * * Next Change * * * *

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[3]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[4]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error concealment of lost packets".

[5]	3GPP TS 26.448: "Codec for Enhanced Voice Services (EVS); Jitter Buffer Management"

[6]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[7]	3GPP TS 26.251: "Codec for Immersive Voice and Audio Services (IVAS); C code (fixed-point)".

[8]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services (IVAS); Test Sequences".

[9]	3GPP TS 26.254: "Codec for Immersive Voice and Audio Services (IVAS); Rendering".

[10]	3GPP TS 26.255: "Codec for Immersive Voice and Audio Services (IVAS); Error concealment of lost packets".

[11]	3GPP TS 26.256: "Codec for Immersive Voice and Audio Services (IVAS); Jitter Buffer Management".

[12]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating point)".

[13]	C. de Boor and K. Höllig (1987), B-splines without divided differences, in Geometric Modeling, G. Farin ed., SIAM, 21–27.

[14]	Borß, C. A Polygon-Based Panning Method for 3D Loudspeaker Setups. In Audio Engineering Society Convention 137, Los Angeles, USA, Oct. 2014.

[15]	Zotter, F. and Frank, M., All-Round Ambisonic Panning and Decoding, J. Audio Eng. Soc., vol. 60, no. 10, pp. 807-820 (Oct. 2012)

[16]	Allen, J. B., & Berkley, D. A., Image method for efficiently simulating small room acoustics. Journal of the Acoustical Society of America, 65(4), 943-950, (1979).

[17]	Box, G. E. P. and Cox, D. R. (1964). An analysis of transformations, Journal of the Royal Statistical Society, Series B, 26, 211-252.

[18]	3GPP TS 26.118: “Virtual Reality (VR) profiles for streaming applications”.

[19]	Ivanic, J., & Ruedenberg, K., Rotation matrices for real spherical harmonics. Direct determination by recursion. The Journal of Physical Chemistry, 100(15), 6342-6347, 1996.

[20]	C. R. Helmrich and B. Edler, “Signal-Adaptive Transform Kernel Switching for Stereo Audio Coding,” in Proc. IEEE WASPAA, New Paltz, NY, USA, Oct. 2015.

[21]	AES, “AES69-2022: AES standard for file exchange - Spatial acoustic data”, Audio Engineering Society, 2022.

[22]	F. Thomas, "Approaching Dual Quaternions From Matrix Algebra," in IEEE Transactions on Robotics, vol. 30, no. 5, pp. 1037-1048, Oct. 2014.

[23]	J, Daniel “R,eprésentation de champs acoustiques, application à la transmission et à la reproduction de scènes sonores complexes dans un contexte multimedia”, Thèse de doctorat de l’Université Paris 6, 2001

[24]	M. Chapman, “A Standard for Interchange of Ambisonic Signal Sets. Including a file standard with metadata”, Ambisonics Symposium 2009, Graz, June 25-27

[25] 	ISO/IEC 23091-3:2018 - Information technology Coding-independent code points Part 3: Audio

[26] 	ISO/IEC 23008-3:2015 - Information technology High efficiency coding and media delivery in heterogeneous environments Part 3: 3D audio

[27] 	3GPP TS 26.249: " Immersive Audio for Split Rendering Scenarios; Detailed Algorithmic Description of Split Rendering Functions".

[X1]	ETSI TS 103 634: " Digital Enhanced Cordless Telecommunications (DECT); Low Complexity Communication Codec plus (LC3plus)"

[X2]	ETSI TR 103 633: " Digital Enhanced Cordless Telecommunications (DECT); Low Complexity Communication Codec plus (LC3plus); Performance characterization"

[X3]	ETSI TS 103 624: “Characterization Methodology and Requirement Specifications for the ETSI LC3plus speech codec”

[X4]	Bluetooth Special Interest Group, ‘Basic Audio Profile’, version 1.0.1

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACELP	Algebraic Code-Excited Linear Prediction

AGC	Adaptive Gain Control

AllRAD	All-Round Ambisonics Decoding

BPF	Bass PostFilter

BRIR	Binaural Room Impulse Response

DirAC	Directional Audio Coding

CBR	Constant Bit Rate

CLDFB	Complex Low-Delay FilterBank

CNA	Comfort Noise Addition

CNG	Comfort Noise Generation

CPE	Channel Pair Element

DFT	Discrete Fourier Transform

DoA	Direction of Arrival

DoF	Degree of Freedom

DTX	Discontinuous Transmission

EC	Entropy Coding

EFAP	Edge Fading Amplitude Panning

FEC	Frame Erasure Concealment

FOA	First-Order Ambisonics

GCC-PHAT	Generalized Cross-Correlation PHAse Transform

HQ	High-Quality

HOA	Higher-Order Ambisonics

HRIR	Head-Related Room Impulse Response

HRTF	Head-Related Transfer Function

IC-BWE	Inter-Channel BandWidth Extension

ICA	Inter-Channel Alignment

ICC	Inter-Channel Coherence

IGF	Intelligent Gap Filling

ILD	Inter-Channel Level Difference

ITD	Inter-Channel Time Difference

ISM	Independent Streams with Metadata

JBM	Jitter Buffer Management

LCLD (codec)	Low-Complexity Low-Delay (codec)

LC3plus	Low Complexity Communication Codec plus

LFE	Low-Frequency Effects

LP	Linear Prediction

MASA	Metadata-Assisted Spatial Audio

McMASA		Multi-Channel MASA

MC	Multi-Channel

MCT	Multi-channel Coding Tool

MD	MetaData

MDCT	Modified Discrete Cosine Transform

MDFT	Modified Discrete Fourier Transform

MDST	Modified Discrete Sine Transform

OLA	OverLap-Add

OMASA	Objects with MASA

OSBA	Objects with SBA

PCA	Principal Component Analysis

PLC	Packet Loss Concealment

SAD	Sound Activity Detection

SBA	Scene-Based Audio

SCE	Single Channel Element

SH	Spherical Harmonics

SNS	Spectral Noise Shaping

SPAR	Spatial Reconstruction

STFT	Short-Term Fourier Transform

TCX	Transform-Coded eXcitation

TD	Time-Domain

TF	Time-Frequency

TNS	Temporal Noise Shaping

TSM	Time Scale Modification

VAD	Voice Activity Detection

VBAP	Vector Base Amplitude Panning

VBR	Variable Bit Rate

* * * Next Change * * * *

## 4.5	Rendering overview

### 4.5.1	Rendering introduction

The codec for Immersive Voice and Audio Services is part of a framework comprising of an encoder, decoder, and renderer. An overview of the audio processing functions of the receive side of the codec is shown in Figure 4.5-1. This diagram is based on [6], with rendering features highlighted.

Figure 4.5-1: Overview of IVAS audio processing functions – receiver side

The interfaces are marked consistently with [6] using the following numbers:

3: Encoded audio frames (50 frames/s), number of bits depending on IVAS codec mode

4: Encoded Silence Insertion Descriptor (SID) frames

5: RTP Payload packets

6: Lost Frame Indicator (BFI)

7: Renderer config data

8: Head-tracker pose information and scene orientation control data

9: Audio output channels (16-bit linear PCM, sampled at 8 (only EVS), 16, 32, or 48 kHz)

10: Metadata associated with output audio

Rendering is the process of generating digital audio output from the decoded digital audio signal. Rendering is used when output format is different than input format. In case output format is the same as input format, the decoded audio channels are simply passed through to the output channels. Binaural rendering is a special case, where binaural output channels are prepared for headphone reproduction. This process includes head-tracking and scene orientation control, head-related transfer function processing, and room acoustic synthesis. IVAS rendering is integrated with IVAS decoder but can also be operated standalone as external rendering while bypassing the internal renderer. The external renderer can be applied e.g., in the case of rendering outputs originating from multiple sources, such as decoders or audio streams.

A special feature of the renderer is that it supports split operation with pre-rendering and transcoding to a head-trackable intermediate representation that can be transmitted to a post-rendering end-device. This enables moving a large part of the processing load and memory requirements for IVAS decoding and rendering to a (more) capable node/UE while offloading the final rendering end-device. The IVAS specific split rendering functionality is mostly described in the present document whereas more generic split rendering functionality is specified in TS 26.249 [27].

### 4.5.2	Internal IVAS renderer

The internal IVAS renderer is integrated into the IVAS decoder. In case of specific operating points, this integration allows for combining decoding and rendering processes, resulting in efficient processing. Therefore, rendering algorithm descriptions associated with specific audio formats are discussed in clause 6. The rendering modes and rendering control are discussed separately in clause 7.

### 4.5.3	External IVAS renderer

The external IVAS renderer supports all the functionality of the internal renderer. However, since the external renderer operates stand-alone, combined decoding and rendering processing is not available.

### 4.5.4	Interface for external rendering

IVAS renderer and its interface provide support to IVAS codec design constraints. The details of the rendering library API are provided in [7] for the fixed-point code and [12] for the floating-point code. The details of the rendering library API are provided in [9].

### 4.5.5	Split rendering

The IVAS renderer supports splitting the rendering process between a capable device or network node (where the IVAS decoding and rendering happen) and a less capable device with limited computing and memory resources and motion-sensing for head-tracked binaural output. Split Rendering support consists of three core components, a metadata-based pose correction scheme (in CLDFB domain), a coding scheme in CLDFB domain for low-complexity low-delay stereo coding (LCLD), see clause 7.6.4, and the pre-existing coding scheme LC3plus, see clause 7.6.5. IVAS split rendering functionality is illustrated in Fig. 4.5-2.

The metadata-based pose correction scheme allows adjusting in a lightweight process a binaural audio signal originally rendered for a first pose according to a second pose. In many split rendering scenarios, the first pose is the potentially outdated lightweight-device pose available at the pre-renderer while the second pose is the current and accurate pose available at the lightweight-device. The metadata is calculated at the capable device or network node based on additional binaural renditions at probing poses different from the first pose. For increasing degrees-of-freedom (DOF) an increasing number of additional binaural renditions at different probing poses is required. The metadata is transmitted to the lightweight device along with the coded binaural audio signal rendered for the first pose.

The binaural audio signal rendered to the first pose is encoded using one of the two codecs, LCLD or LC3plus. The two codecs have complementary properties giving implementors the freedom to make individual trade-offs between complexity, memory, latency, and rate-distortion performance and to implement a design that is optimized for a given IVAS service and hardware configuration. The coding scheme for the binaural audio signal defaults based on the domain the binaural renderer operates in, with LCLD being the default for CLDFB-based rendering and LC3plus being the default where rendering is performed in time-domain. The default can also be overwritten, meaning that irrespective of the rendering domain, either the LCLD or the LC3plus codec or even transmission over a PCM interface may be selected. The latter allows using further coding solutions for the binaural audio signal.

The split rendering can operate at various DOFs, ranging from 0-DOF (no pose correction) to 3-DOF (pose correction on the three rotational axes yaw, pitch, roll) at bit rates from 256 kbps (0-DOF) to 384 - 768 kbps (1 to 3-DOF). The split rendering operates on and delivers audio sampled at a rate of 48 kHz.

Figure 4.5-2: IVAS Split Rendering functions

* * * Next Change * * * *

## 7.6	Split rendering

### 7.6.1	Overview

IVAS supports split rendering wherein the process of binaural rendering and headtracking is split between a main device (pre-renderer) and a light-weight head-worn device (post-renderer). The split-rendering architecture in IVAS is such that the complexity at the post-renderer is substantially less than the complexity of the IVAS decoder and renderer.

There are two architectures of split rendering supported in IVAS. The first architecture, described in Figure 7.6-1, extends IVAS decoder and internal renderer to perform the pre-rendering part of split renderer, whereas the post rendering is done using a separate post-renderer. In the second architecture, described in Figure 7.6-2, the IVAS decoder runs in pass-through mode and the IVAS external renderer is extended to perform the pre-rendering part of split renderer.

Split rendering in IVAS is supported with all immersive input formats and 48kHz sampling rate. For configuration of split rendering, a degree of freedom (DOF) may be specified at the main device (pre-renderer), which determines the rotation axes for which pose correction can be performed at the light-weight device (post-renderer). The DOF value specifies the number of axes compensated, and a special value of 0 DOF means no pose correction capability is available at the post-renderer. The pre-rendering configuration is further described in 7.6.2.

Figure 7.6-1: Split rendering architecture with IVAS decoder

Figure 7.6-2: Split rendering architecture with IVAS external

### 7.6.2	Split pre-rendering

#### 7.6.2.1	Overview

As shown in Figure 7.6-1, the pre-renderer takes IVAS bitstream b1 as input and runs IVAS decoder to generate the immersive audio A. The pre-renderer also obtains a reference head Pose P', associated with the user of the light-weight device or post-renderer, via a backchannel from post-renderer to pre-renderer. The post-renderer may encode the reference head pose into a pose bitstream bp, in which case P' shall be obtained by decoding bp. Alternatively, the reference pose can also be a static value (the default is forward-looking pose, with rotations along yaw, pitch and roll axis set to 0 degrees). Once the reference head pose is obtained, a reference binaural signal is generated with the reference head pose P' and the default binaural renderer for a given format. Next if pose correction is enabled, one or more binaural pre-renditions are computed with the same default binaural renderer, corresponding to one or more probing head poses Pn , where n > 1 and Pn is computed by adding fixed offsets to the reference pose P' such that each probing pose differs from the reference pose along one rotation axis only, as described in Table 7.6-1. If pose correction is not enabled, then the additional pre-renditions are not generated.

If the rendering happens in time domain, then all binaural signals are converted to CLDFB domain and the CLDFB domain binaural signals are fed to the metadata computation block, which computes metadata M such that all binaural pre-renditions can be reconstructed using the reference binaural signal and metadata M.

The reference binaural signal is either coded with the Low Complexity Low Delay (LCLD) codec or the Low Complexity Communication Codec plus (LC3plus) depending on the default transport codec settings, as described in Table 7.6-2. Alternatively, the codec can be set explicitly via the split rendering interface and either LCLD, LC3plus or PCM can be used with any IVAS immersive input format. The reconstruction metadata M is quantized and coded by the metadata encoder block using the symmetries in probing poses and metadata. The encoded reference binaural signal and encoded metadata are multiplexed to generate the split rendering bitstream b2 that is transmitted to the post-renderer.

The reference head pose P' is coded in the bitstream b2 as part of metadata M, whereas the offsets to compute probing poses from the reference pose are static and known a priori to both pre-renderer and post-renderer.

Table 7.6-1: Probing poses configuration

| DOF | Default Axes | Total number of probing poses | Offsets from reference pose {yaw_offset, pitch_offset, roll_offset} in degrees |
| --- | --- | --- | --- |
| 0 DOF | N/A | 0 | N/A |
| 1 DOF | Yaw | 2 | {15, 0, 0}, {-15, 0, 0} |
| 2 DOF | Yaw, Pitch | 4 | {15, 0, 0}, {-15, 0, 0}, {0, 15, 0}, {0, -15, 0} |
| 3 DOF, HQMODE OFF | Yaw, Pitch, Roll | 4 | {15, 0, 0}, {-15, 0, 0}, {0, 10, 0}, {0, 0, 10} |
| 3 DOF, HQMODE ON | Yaw, Pitch, Roll | 6 | {15, 0, 0}, {-15, 0, 0}, {0, 15, 0}, {0, -15, 0}, {0, 0, 15}, {0, 0, -15} |

Table 7.6-1: Probing poses configuration

Table 7.6-2: Default split rendering transport codec configuration

| IVAS Renderer domain (CLDFB domain OR Time (TD) domain) | Default split rendering transport codec |
| --- | --- |
| CLDFB | LCLD |
| TD | LC3plus |
| CLDFB + TD (for combined formats like OSBA and OMASA) | LCLD |

#### -

#### 7.6.2.2	Supported Split Rendering bitrates with LCLD or LC3plus codec

#### Table 7.6-3: Supported Split Rendering bitrates with the LC3plus codec

| LC3plus Configuration | DoF | Split rendering total bitrate (kbps) | Split rendering frame size (ms) |
| --- | --- | --- | --- |
| 5ms or 10ms frame duration | 0 | 256, 384, 512 | 5 or 10 |
| 5ms or 10ms frame duration | 1-3 | 384, 512, 768 | 20 |

Table 7.6-4: Supported Split Rendering bitrates with the LCLD codecTable 7.6-3: Supported Split Rendering bitrates with the LC3plus cod

| LCLD configuration | DoF | Split rendering total bitrate (kbps) | Split rendering frame size (ms) |
| --- | --- | --- | --- |
| 5ms or 10ms or 20ms frame duration | 0 | 256, 384, 512 | 5 or 10 or 20 |
| 20ms frame duration | 1-3 | 384, 512, 768 | 20 |

#### 7.6.2.3	Supported Split Rendering bitrates with PCM output

In this mode the pre-renderers output PCM binaural data and a separate metadata bitstream. The split rendering bitrate provided via API is the bitrate of pose correction metadata. This mode is only supported with 1 to 3 DOF configurations. 0 DOF is the equivalent of the IVAS binaural output.

Table 7.6-5: Supported Split Rendering bitrates with the PCM binaural output

| DoF | Supported bitrate | Split rendering frame size (ms) |
| --- | --- | --- |
| 1 | Greater than or equal to 50kbps | 20 |
| 2 | Greater than or equal to 66kbps | 20 |
| 3 | Greater than or equal to 128kbps | 20 |

Table 7.6-5: Supported Split Rendering bitrates with the PCM binaural output

#### 7.6.2.4	Split pre-rendering of SBA

When the IVAS input format is SBA, the IVAS decoder runs the default ambisonics to binaural internal renderer  n+1  times including one binaural rendition with reference head pose P’ and n binaural renditions with n probing head poses, respectively, as described in Figure 7.6-1.  SBA rendering is always in CLDFB domain and the n+1 binaural signals in CLDFB domain are then fed to the metadata generator, as described in clause 7.6.3. The reference binaural signal is coded with the split rendering transport codec and the coded bits are combined with coded metadata bits to form split rendering bitstream.

When the external renderer is running in split pre-rendering mode then the IVAS decoder decodes the IVAS bitstream and outputs SBA-format audio. The external renderer then performs n+1 ambisonics to binaural renditions, as described in Figure 7.6-2.

#### 7.6.2.5	Split pre-rendering of MASA

When the IVAS input format is MASA, n+1 MASA to binaural renditions are done by the default IVAS MASA renderer.

When the external renderer is running in split pre-rendering mode then the IVAS decoder decodes the IVAS bitstream and outputs MASA format audio. The external renderer then performs n+1 MASA to binaural renditions, as described in Figure 7.6-2.

#### 7.6.2.6	Split pre-rendering of CBA

When the IVAS input format is CBA, n+1 multi-channel to binaural renditions are done by the default IVAS multi-channel renderer.

When the external renderer is running in split pre-rendering mode then the IVAS decoder decodes the IVAS bitstream and outputs channel-based format audio. The external renderer then performs n+1 multi-channel to binaural renditions, as described in Figure 7.6-2.

#### 7.6.2.7	Split pre-rendering of ISM

When the IVAS input format is ISM, n+1 ISM to binaural renditions are done by the default IVAS ISM renderer. In this format, an ILD flag is set and used in split-rendering metadata computation as described in clause 7.6.3.

When the external renderer is running in split pre-rendering mode then the IVAS decoder decodes the IVAS bitstream and outputs ISM format audio. The external renderer then performs n+1 ISM to binaural renditions, as described in Figure 7.6-2.

#### 7.6.2.8	Split pre-rendering of OSBA

When the IVAS input format is OSMA, the objects and SBA signals are rendered to binaural separately and then added together to form the final binaural signal. The n+1 ISM to binaural renditions are done using the default ISM renderer and the n+1 SBA to binaural renditions are done using the default SBA renderer. In this format, an ILD flag is set and used in split-rendering metadata computation as described in clause 7.6.3.

#### 7.6.2.9	Split pre-rendering of OMASA

When the IVAS input format is OMASA, objects and MASA signals are rendered to binaural separately and then added together to form the final binaural signal. The n+1 ISM to binaural renditions are done by the default ISM renderer and the n+1 MASA to binaural renditions are done by the default MASA renderer. In this format, an ILD flag is set and used in split-rendering metadata computation as described in clause 7.6.3.

### 7.6.3	Intermediate split renderer metadata format

#### 7.6.3.1	Overview

Split rendering metadata is computed to perform pose correction on the reference binaural signal from head pose P' at the pre-renderer to the latest head pose P at the post-renderer. The metadata is computed in CLDFB domain, wherein the 60 CLDFB bands are grouped into 20 metadata bands with the mapping shown in Table 7.6-6.

Table 7.6-6: Metadata banding

| Metadata bands | CLDFB bands |
| --- | --- |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 6 |
| 7 | 7 |
| 8 | 8 |
| 9 | 9 |
| 10 | 10 |
| 11 | 11 |
| 12 | 12, 13 |
| 13 | 14,15 |
| 14 | 16, 17, 18, 19, 20 |
| 15 | 21, 22, 23, 24, 25 |
| 16 | 26, 27, 28, 29, 30 |
| 17 | 31, 32, 33, 34, 35 |
| 18 | 36, 37, 38, 39, 40 |
| 19 | 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50 |
| 20 | 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60 |

Table 7.6-6: Metadata banding

The banded covariance computation between a binaural signal corresponding to head pose P1 and a binaural signal corresponding to head pose P2 , where P1 and P2 are any two poses from the  n+1 head poses, is done as follows:

R y, p1, p2 (b)= m=1m=16k=k0bk=k1bSP1m, kSP2Hm, k,

where, SP1m, k is the [2x1] matrix of the binaural signal corresponding to head pose P1 at time slot m and CLDFB band index k, SP2Hm, k is  the complex conjugate matrix of the binaural signal corresponding to head pose P2 at time slot m and band index k, b is the metadata band index with range 1≤b≤20, k0b and k1b are the start and end CLDFB band indices as per Table 7.6-6.

#### 7.6.3.2	Metadata computation, quantization and coding

##### 7.6.3.2.1	Metadata computation for deviations about Yaw axis

The metadata corresponding to probing poses that deviate only along Yaw axis is computed as follows. First, a transformation matrix Mps in each metadata band is computed as follows:

Mps=R y, ps, p' Ry, p', p' +Iϵ-1,

(7.6-1)

where, Ry, p', p'  is the 2x2 covariance matrix of the reference binaural signal Binp' corresponding to reference pose  P', R y, ps, p'  is the 2x2 covariance matrix of the reference binaural signal and the binaural signal corresponding to probing pose  Ps .

- With the above transformation matrix, a post prediction matrix is computed as

R y,ps, ps ,= MpsRy, p', p' Mps*  .

(7.6-2)

The enhanced metadata prediction matrix Mps is computed by multiplying Mps with a gain matrix Gps to further match the energy in the post-prediction matrix. The gain matrix Gps is computed as follows.

Gps=gl,ps00gr,ps,

(7.6-3)

where

gl,ps=sqrt(Ry,ps,ps1,1Ry,ps,ps1,1)   , gr,ps=sqrt(Ry,ps,ps2,2Ry,ps,ps(2,2)).

(7.6-4)

Ry,ps,ps is the 2x2 covariance matrix of the binaural pre-rendition corresponding to probing pose Ps, and Ry,ps,ps is the 2x2 covariance matrix of the reconstructed binaural pre-rendition corresponding to probing pose Ps as given in equation (7.6-2).

Mps = Gps  Mps

(7.6-5)

The Mpsmatrix is quantized and coded in the metadata bitstream.

The yaw metadata in high frequency range is computed differently when ILD flag (as mentioned in clause 7.6.2.5) is set OR when the quantization strategy is Q2 or Q3 as described in Table 7.6-7. If the ILD flag is set or if the quantization strategy is Q3, the first 4 metadata bands, i.e., frequency range 0 to 1600 Hz, Mps matrix is computed as shown in equation (7.6-5) to enable the reconstruction of both magnitude and phase of the binaural signal corresponding to probing pose Ps from the reference binaural signal. In the remaining metadata bands, i.e., frequencies above 1600 Hz, a real only 2x2 diagonal matrix D is computed to enable the reconstruction of just magnitude of the binaural signal corresponding to probing pose Ps from the reference binaural signal. The computation of D is as follows:

Mps

DPs=dl00dr,

(7.6-6)

where

dl=sqrt(Ry,ps,ps1,1Ry,p',p'1,1)   , dr=sqrt(Ry,ps,ps2,2Ry,p',p'2,2).

(7.6-7)

Mps and D matrix is quantized and coded in the metadata bitstream which is then combined with coded reference binaural signal bitstream to form split rendering output bitstream.If the quantization strategy is Q2, the first 10 metadata bands, i.e., frequency range 0 to 4000 Hz, Mps matrix is computed as shown in equation (7.6-5). In the remaining metadata bands, i.e., frequencies above 4000 Hz, the real only 2x2 diagonal matrix D is computed as per equation (7.6-7).

Mps and D matrix is quantized and coded in the metadata bitstream which is then combined with coded reference binaural signal bitstream to form split rendering output bitstream.

##### 7.6.3.2.2	Quantization and coding of Yaw metadata

The high frequency elements of matrix Mps may be converted to absolute values depending on the quantization strategy as described in clause 7.6.3.3. Then, a fix QPs matrix is subtracted from the matrix Mps (and DPs when ILD flag is enabled) before quantization.

Mq,ps= Mps-QPs, Dq,ps= Dps-QPs,

(7.6-8)

Where, QPs is a 2x2 identity matrix for the bands with real only diagonal matrix D coefficients. For the complex Mps coefficients, QPs is computed as follows.

QPs=cos⁡(dy)00cos⁡(dy)

(7.6-9)

and dy is the deviation between probing pose Ps and reference pose P' about the Yaw axis.

Each element is uniformly quantized and the quantized index is computed as shown below:

Yindex= roundqlvlY-1maxYmin,minYmax, ValYmax- Ymin,

(7.6-10)

where Ymin= -1.4 , Ymax=1.4 , qlvlY is the number of quantization points

, Val is the unquantized element of  Mq,ps and Dq,ps matrix.

The value of qlvlY depends on the quantization strategy as described in clause 7.6.3.3. The quantized index Yindex  is then coded using either a base2 coding scheme or Huffman coding scheme as described in clause 7.6.3.3. If Huffman coding is used then the quantized indices are first differentially coded using the symmetries in the metadata of various probing poses as given below.

Mindex,  Ps=Yindex(0,0)Yindex(0,1)Yindex(1,0)Yindex(1,1)

Mdifferential, index,  Ps=Yindex(0,0)Yindex(0,1)Yindex1,0-sYindex(0,1)Yindex1,1-sYindex(0,0)

Where, s=1 for real values and s=-1 for imaginary values. For the real only diagonal matrix D indices, s=-1.

Then modulo qlvly is performed to keep the index range as {Yminindex,Ymaxindex }

YTDindex=  YTDindex+ qlvlY   if YTDindex< Yminindex YTDindex- qlvlY  if YTDindex> YmaxindexYTDindex                                 otherwise

(7.6-11)

where Yminindex= roundqlvlY-1*YminYmax- Ymin, Ymaxindex= roundqlvlY-1*YmaxYmax- Ymin and YTDindex is the differentially coded index.

##### 7.6.3.2.3	Metadata computation for deviations about Pitch axis

Metadata computation for pose deviation along pitch axis is done by using the set of probing poses that deviate from reference pose only by rotation around the pitch axis. The metadata is computed to enable the reconstruction of binaural pre-renditions (BINn) corresponding to the pitch deviation probing poses from the reference binaural signal Binp' corresponding to reference pose  P'. The pitch reconstruction metadata, as shown below, includes a diagonal real 2x2 pitch correction matrix (H) for each time-frequency tile as it is assumed that ITD cues do not change with pitch deviation.

HPs=hl00hr

(7.6-12)

hl=sqrt(Ry,ps,ps1,1Ry,p',p'1,1)   , hr=sqrt(Ry,ps,ps2,2Ry,p',p'2,2)

(7.6-13)

where Ry, p', p'  is the covariance matrix of reference binaural presentation Binp', and Ry,ps,ps is the covariance matrix of a binaural pre-rendition corresponding to pose Ps.

##### 7.6.3.2.4	Quantization and coding of Pitch metadata

The diagonal elements of HPs matrix are uniformly quantized with 15 quantization points and the quantization range is limited between the values 0.5 and 1.5. The quantized indices are then either coded using a base2 coding scheme or Huffman coding scheme as described in clause 7.6.3.3.

##### 7.6.3.2.5	Metadata computation for deviations about Roll axis

Metadata computation for pose deviation along roll axis is done by using the set of probing poses that deviate from reference pose only by rotation around the roll axis. In the first 4 metadata bands, i.e., frequency range 0 to 1600 Hz, a complex valued Mps matrix is computed to enable the reconstruction of both magnitude and phase of the binaural signal corresponding to probing pose Ps from the reference binaural signal. In the remaining metadata bands, i.e., frequencies above 1600 Hz, a real only 2x2 diagonal matrix Dps is computed to enable the reconstruction of just magnitude of the binaural signal corresponding to probing pose Ps from the reference binaural signal. The computation of Mps and DPs is similar to the metadata computation for deviation about Yaw axis when ILD flag is enabled.

##### 7.6.3.2.6	Quantization and coding of Roll metadata

A 2x2 identity matrix IPs is subtracted from the matrix Mps and DPs before quantization.

Mq,ps= Mps-IPs

(7.6-14)

Dq,ps= Dps-IPs

(7.6-15)

Then each element is uniformly quantized with 31 quantization points and the quantization range is limited between the values -1.4 and 1.4. The quantized index is then coded using either a base2 coding scheme or Huffman coding scheme as described in clause 7.6.3.3. If Huffman coding is used, then the quantized indices are first differentially coded using the symmetries in the metadata of various probing poses as described in clause 7.6.3.2.2.

#### 7.6.3.3	Common split rendering metadata quantization and coding strategies

Table 7.6-7: Quantization strategies

| Quantization strategies | Yaw |  | Pitch |  | Roll |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Number of quantization points | Complex valued MD band threshold (bth) | Number of quantization points | Complex valued MD band threshold (bth) | Number of quantization points | Complex valued MD band threshold (bth) |
| Q0 | 63 | 20 | 15 | 0 | 31 | 4 |
| Q1 | 31 | 20 | 15 | 0 | 31 | 4 |
| Q2 | 31 | 10 | 15 | 0 | 31 | 4 |
| Q3 | 31 | 4 | 15 | 0 | 31 | 4 |

The metadata is quantized and coded in a quantization and coding loop which includes iterating over 4 quantization strategies in the order Q0, Q1, Q2, Q3. For each quantization strategy, metadata is quantized with the number of quantization points mentioned in Table 7.6-7. The complex valued MD band threshold (bth) indicates the frequency threshold for complex valued metadata matrix. For pitch axis deviation metadata, the elements are already real valued and hence  bth is set to 0.

bthbthbth

The quantized metadata indices are then coded with Huffman coder. The indices for Yaw and Roll axis metadata are differentially coded with Huffman coder as per equation (7.6-11). If the total Huffman bits are greater than base2 coded bits than metadata indices are coded with base2 coding. If the coded bits are more than the available metadata bit budget, as given in Table 7.6-8, then the metadata is quantized with next quantization strategy and coded with either Huffman coder or base2 coder depending on whichever coding method results in lesser number of coded bits. The quantization strategy is coded in the bitstream with 2 bits and the coding strategy is coded in the bitstream with 1 bit.

Table 7.6-8: Metadata bit budget

| Split rendering bitrate (bps) | Available metadata bitrate (bps) |
| --- | --- |
| 768000 | 256000 |
| 512000 | 128000 |
| 384000 | 128000 |
| 256000 | 0 (only 0 DOF mode supported) |

In addition to rotation axis metadata, reference head pose, DOF, HQ mode, ILD flag and rotation axes are also coded in the bitstream. To code reference head pose, yaw, pitch and roll Euler angles are rounded to nearest integer and coded with 9 bits each. The DOF parameter can take values 0 to 3 and is coded with 2 bits. The ILD flag is coded with 1 bit. The rotation axes indicate the axes for which the metadata is being coded in the bitstream when DOF is set to 1 or 2. For 1DOF, it can take following values {DEFAULT_AXIS, YAW, PITCH, ROLL } and for 2DOF it can take {DEFAULT_AXIS, YAW_PITCH, YAW_ROLL, PITCH_ROLL}. The rotation axes are coded with 2 bits for 1DOF and 2DOF modes. The default rotation axis for 1DOF is YAW and the default for 2DOF is YAW_PITCH.

The remaining bits are then used by either LCLD codec or LC3plus codec to encode the reference binaural signal.

#### 7.6.3.4	Intermediate split renderer metadata decoder

The split renderer metadata decoder decodes the reference head pose P', DOF, HQ mode, ILD flag, rotation axis and the metadata corresponding to the set of probing poses (Pn) from the bitstream. The metadata decoder decodes coding strategy and quantization strategy from the bitstream. Based on the coding strategy, bits are decoded with either base2 decoder or Huffman decoder to obtain the quantized indices. If Huffman decoder is used then for yaw or roll axis related metadata, differential indices are converted to absolute values as follows.

Mdifferential, index,  Ps=Yindex(0,0)Yindex(0,1)Yindex(1,0)Yindex(1,1)

(7.6-16)

Mindex,  Ps=Yindex(0,0)Yindex(0,1)Yindex1,0-sYindex(0,1)Yindex1,1-sYindex(0,0)

(7.6-17)

Where, s=-1 for real values and s=1 for imaginary values, Mdifferential, index,  Ps are the Huffman decoded indices for each time-frequency tile and Mindex,  Ps are the absolute indices. For real only diagonal matrix D indices, s=1.

Then modulo qlvly is performed, as described in equation (7.6-11), to keep the index range as {Yminindex,Ymaxindex }.

The quantized indices are then converted to quantized values as shown below.

Yquantized=  YindexYmax- YminqlvlY-1

(7.6-18)

Where, Ymax and Ymin are the maximum and minimum values of the quantization range for a given type of metadata parameter, quantization range for metadata parameters along yaw, pitch and roll axis are given in clause 7.6.3.2.2, 7.6.3.2.4 and 7.6.3.2.6, qlvlY is the number of quantization points as per the quantization strategy given in Table 7.6-4.

The metadata parameters corresponding to yaw and roll axis are stored in 2x2 matrix form Myaw, q,ps,Mroll, q,ps  for each time-frequency tile wherein the matrix Mq,ps is either real valued or complex valued. The metadata parameters corresponding to pitch axis are stored in real valued 2x2 diagonal matrix form Hpitch ,ps.

A fix QPs matrix is added to the Myaw, q,ps as shown below.

Myaw,ps= Myaw, q,ps+QPs

(7.6-19)

Where QPs is given in clause 7.6.3.2.2.

QPs

A 2x2 identity matrix IPs matrix is added to the Mroll, q,ps as shown below.

Mroll,ps= Mroll, q,ps+IPs

(7.6-20)

The metadata matrices Myaw,ps, Mroll,ps and Hpitch,ps are interpolated and extrapolated as shown in clause 7.6.6.2 to obtain final 2x2 pose correction matrix.

#### 7.6.3.5	Intermediate split renderer metadata loss concealment

In case of packet loss identified by raised BFI flag, the most recent set of successfully decoded split rendering metadata is kept and reapplied for the current (lost) frame.

### 7.6.4	LCLD coded intermediate split renderer binaural audio format

#### 7.6.4.1	LCLD codec overview

The LCLD codec is designed to be a low complexity and low latency codec to transport the oversampled CLDFB (Complex Low Delay Filter Bank) coefficients between devices. Transporting the oversampled CLDFB coefficients allows for post-processing operations to be split between multiple devices, while ensuring no additional latency is incurred due to the conversion between time and filter-bank domains.

The LCLD codec provides support for 1 and 2 channel content sampled at a rate of 48kHz. The LCLD codec also supports multiple frame rates of 5ms, 10ms, and 20ms.

#### 7.6.4.2	LCLD encoder

#### 7.6.4.2.1	Overview

The structure of the LCLD codec is shown in figure 7.6-1. The input to the LCLD codec is blocks of oversampled CLDFB coefficients, where the number of blocks is dependent on the desired frame rate. For a frame length of 20ms there will be 16 blocks of CLDFB coefficients, for a frame length of 10ms there will be 8 blocks of coefficients, and for a frame length of 5ms there will be 4 blocks of coefficients. For signals with 2 channels the first operation of the encoder is joint channel coding, which is detailed in section 7.6.4.2.3. After the joint channel coding process, the blocks of CDLFB coefficients are partitioned into groups of slots that will share RMS envelope information. The merging of the blocks into groups is detailed in section 7.6.4.2.4. The RMS envelope for each group of slots is computed in approximate critical bands. The RMS envelope is then used to normalize the CLDFB coefficients. The specific banding of the CLDFB coefficients into perceptual bands is provided in section 7.6.4.2.2, while the RMS envelope calculation, envelope coding, and the normalization of the CLDFB coefficient are detailed in section 7.6.4.2.5. The RMS envelope is further used to calculate a backward adaptive perceptual model, which is designed for integer bit-exact calculation, such that the perceptual model can be computed in the decoder and get the same result. The calculation of the bit-exact perceptual model is covered in section 7.6.4.2.6. The output of the perceptual model is then used to calculate the bit allocation, which is detailed in section 7.6.4.2.8. The actual quantization of the normalized CLDFB coefficients can be achieved in two possible ways; direct scalar quantization followed by Huffman coding or scalar quantization of prediction residual (DPCM) followed by Huffman coding. The DPCM based quantization uses a forward adaptive complex first order predictive structure. The calculation and coding of the prediction coefficients for the DPCM based quantization are covered in section 7.6.4.2.7, while the details of the quantization procedure are provided in section 7.6.4.2.9.

Figure 7.6-1. Schematic of the LCLD Encoder Process

#### 7.6.4.2.2	Perceptual Banding

While the LCLD codec transports 60 CLDFB bands many of the encoder and decoder operations are performed on a courser banding structure that approximates critical bands. The number of CLDFB bands contained in the coarser perceptual bands are provided in table 7.6-9. Where bm is the number of CLDFB bands in a perceptual band, 
BmLow is the CLDFB start band, BmHigh is the CDLFB stop band, FmLow is the start frequency of the band in Hz, and FmHighis the stop frequency of the band in Hz.

Table 7.6-9: Perceptual Banding Structure

| Band (m) | bm | BmLow | BmHigh | FmLow (Hz) | FmHigh (Hz) |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 400 |
| 1 | 1 | 1 | 2 | 400 | 800 |
| 2 | 1 | 2 | 3 | 800 | 1200 |
| 3 | 1 | 3 | 4 | 1200 | 1600 |
| 4 | 1 | 4 | 5 | 1600 | 2000 |
| 5 | 1 | 5 | 6 | 2000 | 2400 |
| 6 | 1 | 6 | 7 | 2400 | 2800 |
| 7 | 1 | 7 | 8 | 2800 | 3200 |
| 8 | 1 | 8 | 9 | 3200 | 3600 |
| 9 | 1 | 9 | 10 | 3600 | 4000 |
| 10 | 1 | 10 | 11 | 4000 | 4400 |
| 11 | 2 | 11 | 13 | 4400 | 5200 |
| 12 | 2 | 13 | 15 | 5200 | 6000 |
| 13 | 2 | 15 | 17 | 6000 | 6800 |
| 14 | 2 | 17 | 19 | 6800 | 7600 |
| 15 | 2 | 19 | 21 | 7600 | 8400 |
| 16 | 3 | 21 | 24 | 8400 | 9600 |
| 17 | 3 | 24 | 27 | 9600 | 10800 |
| 18 | 4 | 27 | 31 | 10800 | 12400 |
| 19 | 6 | 31 | 37 | 12400 | 14800 |
| 20 | 6 | 37 | 43 | 14800 | 17200 |
| 21 | 7 | 43 | 50 | 17200 | 20000 |
| 22 | 10 | 50 | 60 | 20000 | 24000 |

#### 7.6.4.2.3	Joint Channel Coding

###### 7.6.4.2.3.1	Overview

The Joint Channel Coding module consists of 3 sub-modules:

- Phase Alignment

- Mid/Side Transform

- Real-valued Side Signal Prediction

The usage of each of the sub modules can be signalled efficiently and the associated parameters are efficiently entropy coded. For example, for an amplitude panned stereo signal, Side signal prediction parameters may be used and transmitted to the decoder but no phase alignment parameters. The parameter frequency resolution corresponds to the perceptual frequency bands as shown in table Perceptual Banding Structure and the time resolution corresponds to the frame length. A carefully designed detector decides on the best coding mode in each frame considering the side rate and perceptual benefits using joint channel coding based on the level-dependent perceptual model as described in section 7.6.4.2.6 Perceptual Model.

The encoder joint channel coding can be described in matrix notation as:

Xb,nMXb,nS=10-pkQ10.50.50.5-0.5100ejφkQXb,nLXb,nR ,

where Xb,nL and Xb,nRare the left and right complex input samples in LCLD band b, k is the perceptual band to which the LCLD band b belongs, n is the time sample index within the coded frame, and φkQ is the quantized phase alignment parameter and pkQ the quantized prediction parameter.pkQejφQ

###### 7.6.4.2.3.2	Bitstream Syntax

The transmission of joint coding parameters depends on the bitstream element MSMode and 2 additional flags AnyNonZeroPhase and AnyNonZeroPred. Based on the values of these bitstream elements different stereo coding types can be identified as shown in the following table:

Table 7.6-10: Joint Stereo coding types

| Stereo Coding Type | Description | MSMode | AnyNonZeroPhase | AnyNonZeroPred |
| --- | --- | --- | --- | --- |
| 0 | No joint coding | 0 | NA | NA |
| 1 | Basic Mid/Side | 1 or 2 | NA | NA |
| 2 | Mid/Side with phase alignment only | 3 | 1 | 0 |
| 3 | Mid/Side with side prediction only | 3 | 0 | 1 |
| 4 | Mid/Side with phase alignment and side prediction | 3 | 1 | 1 |

###### 7.6.4.2.3.3	Parameter Computation and Quantization

The phase alignment and prediction parameters are computed based on signal energies and (complex-valued) left/right covariance. Phase alignment is only considered if the left/right cross-correlation coefficient exceeds a threshold of 0.5, otherwise the phase parameter it is set to 0.

The left and right signal energies are computed as

EkL=n=0NFrame-1b∈k Xb,nL2 ,

EkR=n=0NFrame-1b∈k Xb,nR2 ,

Where Xb,nL and Xb,nRis the complex left and right input sample respectively and time index n, b is the LCLD band belonging to the perceptual band k and NFrame is the number of samples per frame and LCLD band. The left/right covariance is computed as

CkLR=n=0NFrame-1b∈k Xb,nLXb,n*R ,

where the * indicates complex conjugate. The inter-channel phase difference is computed from the covariance as

φk=arctan2realCkLRimagCkLR if CkLR2>0.5⋅EkLEkR .

Otherwise

φk=0.

The phase quantization index is given as φkidx=roundφk⋅12π. If φkidxequals 12 then φkidxis set to 0.

And the quantized phase is

φkQ=φkidx⋅π12 .

In the case of left/right phase alignment using the quantized phase, the modified covariance is given as

Ck,φLR=CkLR⋅ejφkQ* .

The mid and side signal energies are computed from original or phase-modified covariance as

Ek,φM=0.25⋅EkL+EkR+2⋅realCk,φLR ,

Ek,φS=0.25⋅EkL+EkR-2⋅realCk,φLR .

The real-valued side prediction parameter which minimizes the side signal energy is computed as

pk=0.25⋅EkL-EkREk,φM  .

The quantized prediction coefficient is computed as

pkQ=maxminroundpk⋅12,12,-1212 .

Finally, the side signal (residual) energy, dependent on application of phase alignment and/or side prediction is computed as

Ek,φ,pS=Ek,φS+pkQ2Ek,φM-0.5⋅pkQ⋅EkL-EkR .

The mid and side signal energies can also be expressed in terms of the stereo coding type t as shown in table Joint Stereo Coding Types and below where k is the perceptual band index:

| Coding Type t | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Parameters | φkQ=0, pkQ=0 | φkQ, pkQ=0 | φkQ=0, pkQ | φkQ, pkQ |

###### 7.6.4.2.3.4	Joint Coding Type Decision

For each of the 4 joint stereo coding types (1,2,3,4) listed in above table, a coding gain compared to independent left/right coding (type 0) is estimated based on the level-dependent perceptual model. For each joint coding type the inter-channel level difference of the jointly coded perceptual band is compared to the inter-channel level difference of the independently coded band. If the magnitude of the inter-channel level difference of the jointly coded band exceeds this of the independently (left/right) coded band, then the band is flagged as candidate for the joint coding type in an array called MSFlags. The coding gain in units of bits for the band is calculated as follows:

BitsGaink,t=Sk⋅maxDk,t-Dk,0,02⋅BWidth,k⋅Nframe⋅FFrame ,

where k refers to the perceptual band as given in Table 7.6-9 (Perceptual Banding Structure), Skis the perceptual model slope parameter as given in Table 7.6-11 (Perceptual Model SMR Slope) for a given band, BWidth,kis the number of complex-valued LCLD bands belonging to the perceptual band, Nframeis the number of complex-valued samples per frame and Fframe is an estimate of the number of Bits needed for an SNR increase by 1dB. The estimate depends on the frame length as follows:

Fframe=0.1log102⋅0.7+Nframe-416-4 ⋅1.0-0.7 .

Table 7.6-11: Perceptual Model SMR Slope Parameter S dependent on the perceptual Band

| Band | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.4375 | 0.4375 | 0.375 | 0.3125 | 0.3125 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 |
| Band | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 |  |
|  | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 |  |

The inter-channel level differences D are computed as follows:

Dk,t=10⋅log10Ek,tMEk,tS, t=1,2,3,4 ,

Dk,0=10⋅log10EkLEkR .

The joint stereo coding candidate bands are given for each stereo coding type t for all k as

MSFlagsk,t=1, if Dk,t>Dk,0 ,for all k

MSFlagsk,t=0, if Dk,t≤Dk,0 ,

where Ek,tMis the energy of Mid signal in band k for joint coding type t, Ek,tS'is the energy of the Side (residual) signal in band k for coding type t, and EkLand EkRare the energies of the left and right signals respectively.

For each of the joint coding types, a total estimated Bit Saving compared to independent coding is computed as:

BitsTotalGaint=max-BitsCostt+k=0K-1BitsGaink,t,0 ,

where BitsCostt is the number of signalling and parameter bits needed for a joint coding type t and K is the total number of coded perceptual bands. The joint coding type with the highest estimated bit saving is identified as tBest.

The final joint coding type decision is as follows in pseudo code:

If ( tBest== 2 OR tBest == 3 OR tBest== 4) AND BitsTotalGaintBest>1.1BitsTotalGain1

tuse=tBest

Else if (BitsTotalGain1> 0 )

tuse=1

Else

tuse=0

###### 7.6.4.2.3.5	Entropy Coding of Parameters

Phase alignment parameters are entropy coded by computing second order frequency differentials from the phase quantization indices with phase wrapping and Huffman coding using the same code book which is used to code the RMS envelopes. Specifically, the second order differences are computed by first computing the index differences over the perceptual bands and then computing the differences over the first differences. All index differences are wrapped into the quantization range of [-12,12] which exactly corresponds to [-π,+π], This means that the value for the first frequency band is unmodified, the value for the second frequency band corresponds to a difference and the values in the third frequency band and all other bands correspond to a difference of the first difference. This scheme is efficient for stereo signals with constant inter-channel phase shift and with inter-channel time delay which can be observed for example with HRTF processed signals with a dominant sound source.

Side prediction parameters are entropy coded by quantization and taking differences of the quantization indices over perceptual bands and Huffman coding, again using the same code book as is used for the RMS envelopes. Given the quantization ranges of the joint stereo coding parameters and that of the RMS envelopes index differences never exceed the RMS envelope quantization table range.

##### 7.6.4.2.4	Temporal Grouping

A LCLD frame either contains 4,8, or 16 temporal slots of the CLDFB coefficients depending on the specific frame length in use. To reduce side-information cost of transmitting the RMS Envelope, the individual temporal slots of the CLDFB coefficients can be grouped together into groups, where a group is defined to be 1 or more temporal slots of CLDFB coefficients. The LCLD codec transmits a new set of RMS envelope data for each group in a frame. Every frame starts with a new group (i.e. groups cannot cross frame boundaries).

Grouping control is transmitted to the decoder as a set of single-bit flags for each slot in the frame except the first slot (the first slot is guaranteed to be the start of a new group). A ‘1’ is transmitted to indicate the start of new group, while a ‘0’ is transmitted to indicate the continuation of a group. For example, the group information for a 16-slot frame will contain 15 single-bit flags as the first slot is known to be the start of new group.

Furthermore, for 2-channel content, the encoder can either send grouping information for each individual channel or transmit a single set of grouping information that is applicable to both channels. When 2 channels are present, a single-bit flag is transmitted prior to the grouping information to indicate if the grouping is common for both channels. A ‘1’ means grouping is common and there will only be 1 set of grouping information shared across both channels, while a ‘0’ means the grouping is not common for each channel and 2 sets of grouping information will exist.

The grouping decision is computed using an iterative greedy-merge algorithm. The greedy-merge algorithm attempts to trade-off the cost of transmitting the RMS envelope versus the accuracy of the merged RMS envelope for each individual slot. The greedy-merge algorithm works as follows:

- Initialize groups to have at least 1 temporal slot each group. The groups can be enumerated g = 0, 1, ... G-1, where G is the total number of groups.

- For each group compute a cost function associated with merging the group to the next adjacent group. Starting from group 0, compute the merge cost to merge group 0 and group 1. Then compute the cost of merging group 1 and group 2, proceed until the merge cost of group G-2 and G-1 has been evaluated.

- After all possible group merges have been evaluated, find the minimum merge cost computed in step 2. If the merge cost is below a threshold merge the two groups associated with minimum merge cost, reduce the total number of groups (G = G -1), and return to step 2. If the minimum merge cost is greater than a threshold, the greedy-merge algorithm terminates. If the number of groups is 1 the algorithm terminates.

Figure 7.6-2 shows the flow diagram for the greedy-merge algorithm.

Figure 7.6-2. Flow diagram of the greedy-merge algorithm

The merge cost used in the greedy-merge algorithm is computed using the following steps:

Step 1: Compute the weighted cost of the first group in the possible merge as shown in equation 7.6-21:

C1=k=N1N2-1m=0M-12bmwmRm1-EnmdB ,						(7.6-21)

where:

C1 is the weighted cost associated with the first group in a possible merge,

n is the temporal slot index,

N1 is the temporal slot index of the first group in the possible merge,

N2 is the temporal slot index of the second group in the possible merge,

m is the perceptual sub-band index,

M is the total number of perceptual sub-bands,

bm is the number of CLDFB coefficients in the mth perceptual band,

wm is the weight for the mth perceptual band (the weight values are provided in table 7.6-9,

EnmdB is the energy in dB for the mth perceptual band for the nth slot, and

Rm1 is the estimated RMS envelope for the first group in the possible merge in dB

The energy in dB in each perceptual band (Snk)for the nth temporal slot can computed using equations 7.6-22 and 7.6-23.

Enm=1bmk=BmLowBmHigh-1SnkCLDFB2 ,							(7.6-22)

EnmdB=10log10Enm ,								(7.6.23)

where:

SnkCLDFB is the CLDFB domain signal,

n is the temporal slot index,

k is the CLDFB band index,

BmLow is the CLDFB band index at the start of the mth perceptual band,

BmHigh is the CLDFB band index at the end of the mth perceptual band, and

Xnb is the complex CLDFB coefficient for the bth CLDFB band and nth temporal block in the frame.

The estimated RMS envelope values in dB for the first group in the possible merge is given by equation 7.6-24:

Rm1=3.01*ROUNDlog21N2-N1n=N1N2-1Enm .					(7.6-24)

Step 2: Compute the weighted cost of the second group in the possible merge as shown in equation 7.6-25:

C2=n=N2N3-1m=0M-12bmwmRm2-EnmdB , 						(7.6-25)

where:

C2 is the weighted cost associated with the second group in a possible merge, and

N3 is the temporal slot index of the end of the second group in the possible merge.

The estimated RMS envelope values in dB for the second group in the possible merge is given by equation 7.6-26.

Rm2=3.01*ROUNDlog21N3-N2n=N2N3-1Enm .					(7.6-26)

Step 3: Compute the weighted cost of the group resulting from merging the group 1 and group 2 as shown in equation 7.6-27:

CM=n=N1N3-1m=0M-12bmwmRmM-EnmdB 				,		(7.6-27)

where:

CM is the weighted cost associated with the merge of groups 1 and 2, and

RmM is the estimated RMS envelope in dB for the merge of groups 1 and 2.

The estimated RMS envelope values in dB for the merge of group 1 and group 2 is given by equation 7.6-28:

RmM=3.01*ROUNDlog21N3-N1n=N1N3-1Enm .					(7.6-28)

Step 4: Compute the total merge cost for the 2 groups in the possible merge as shown in equation 7.6-29:

Tg=CM-C1+C26.02-B1+B2-Bm ,							(7.6.-29)

where:

Tg is the merge cost for merging the group g and g + 1,

CM is the weighted cost associated with merging group 1 and group 2,

B1 is the estimated bit-cost of transmitting the RMS envelope for group 1,

B2 is the estimated bit-cost of transmitting the RMS envelope for group 2, and

Bm is the estimated bit-cost of transmitting the RMS envelope for the merged group.

The estimated bit-cost (B1, B2, and Bm) is based on the delta coding of the RMS envelope, which is detailed in section 7.6.4.2.5.

For the case of 2-channel content when common grouping is used across both channels the cost functions are modified in each step of the greedy-merge algorithm to include both channels in the calculations.

The greedy-merge algorithm terminates under 2 conditions; if the number of groups (G) is 1, meaning all blocks in a frame have merged into a single group, or the minimum merge cost function is greater than a threshold. The optimal threshold used for the cost function in equation 7.6-29 is zero. The result of the greedy-merge grouping algorithm is the number of groups G and a set of group lengths (number of temporal slots in a group) Lg for each of the G groups.

Table 7.6-12: Weight values for computing the weighted group costs.

| Perceptual Band (m) | Weight (wm) | Perceptual Band (m) | Weight (wm) |
| --- | --- | --- | --- |
| 0 | 0.5625 | 12 | 0.75 |
| 1 | 0.5625 | 13 | 0.75 |
| 2 | 0.625 | 14 | 0.75 |
| 3 | 0.6875 | 15 | 0.75 |
| 4 | 0.6875 | 16 | 0.75 |
| 5 | 0.75 | 17 | 0.75 |
| 6 | 0.75 | 18 | 0.75 |
| 7 | 0.75 | 19 | 0.75 |
| 8 | 0.75 | 20 | 0.75 |
| 9 | 0.75 | 21 | 0.75 |
| 10 | 0.75 | 22 | 0.75 |
| 11 | 0.75 |  |  |

##### 7.6.4.2.5	RMS Envelope

##### 7.6.4.2.5.1	RMS Envelope Calculation

The RMS envelope values are computed for each group in a frame. The output of the grouping algorithm (see section 7.6.4.2.4) is the number of groups G and the length or number Lg. We can also define the temporal block index of the start of a group (Ng) as the sum of the group lengths of the prior groups as shown in equation 7.6-30. Group 0 starts at 0th temporal block index (i.e., N0 = 0).

Ng=n=0g-1Ln .									(7.6-30)

The RMS envelope for each group and each perceptual band can then be calculated as shown in equation 7.6-31.

Rgm=ROUNDlog21bmLgn=NgNg+Lg-1m=BmLowBmHigh-1SnkCLDFB2 ,				(7.6-31)

where:

Rgm is the RMS envelope for the gth group and the mth perceptual band index,

SnkCLDFB is the complex CLDFB coefficient for the kth CLDFB band and nth temporal slot in the frame,

bm is the number of CLDFB coefficients in the mth perceptual band,

BmLow is the CLDFB band index at the start of the mth perceptual band, and

BmHigh is the CLDFB band index at the end of the mth perceptual band.

The RMS envelope values are limited to the range -64 to 63 as shown in equation 7.6-32:

Rgm=-64Rgm≤-64Rgm-64<Rgm<6363Rgm≥63-64Rgm≤-64Rgm-64<Rgm<6363Rgm≥63					   .		(7.6-32)

##### 7.6.4.2.5.2	Normalizing the CLDFB Coefficients with the RMS Envelope

The raw CLDFB coefficients are normalized by the RMS envelope in each group and each perceptual band prior to the DPCM and quantization stage discussed in section 7.6.4.2.9. The normalization of the CLDFB coefficients is shown in equation 7.6-33:

SnkCLDFB=SnkCLDFB2-0.5Rgm ,								(7.6-33)

where:

SnkCLDFB is the normalized CLDFB coefficient at the nth temporal block index and kth CLDFB band index,

SnkCLDFB is the original CLDFB coefficient at the nth temporal block index and kth CLDFB band index,

n is the temporal slot index that belongs in gth group, n∈Ng,⋯,Ng+Lg-1, and

k is the CLDFB band index that belongs in the mth perceptual band, k∈BmLow,⋯,BmHigh-1

.

##### 7.6.4.2.5.3	RMS Envelope Transmission

The RMS envelope values in each group and each perceptual band excluding the 0th band are transmitted as the Huffman coded difference to the preceding perceptual bands RMS envelope value. The differential RMS envelope values are calculated and limited to the range -32 to 31 as shown in equation 7.6-34.

∆Rgm=-32Rgm-Rgm-1≤-32Rgm-Rgm-1-32<Rgm-Rgm-1<3131Rgm-Rgm-1≥31-32Rgm-Rgm-1≤-32Rgm-Rgm-1-32<Rgm-Rgm-1<3131Rgm-Rgm-1≥31∀m≥1 .		(7.6-34)

The RMS envelope values for the 0th perceptual band for all groups in a frame are transmitted as 7-bit unsigned integer by adding 64 to the value (Rg0+64).

The differential RMS envelope for all bands above the 0th band are coded using a fixed Huffman codebook.

#### 7.6.4.2.6	Perceptual Model

The perceptual model is designed to operate in both the encoder and decoder based only on transmitted information. Specifically, only the RMS envelope (see section 7.6.4.2.5) is used to compute the target SNR requirements, for each channel, for each perceptual band, and each group segment of a frame. As the target SNR calculated by the perceptual model in both the encoder and decoder must match exactly, all arithmetic calculations described here are integer calculations with a maximum precision of 24bits.

The perceptual model computes the target SNR for just noticeable distortion JND (the signal to mask ratio) for each group in a frame and for each perceptual band. As the RMS envelope is transmitted to the decoder in a log space the perceptual model is also computed in a log space. The following steps are used to compute the signal to mask ratio (SMRgm) for each group and perceptual band:

Step 1: Scale the RMS envelope and add the log domain width of the perceptual band as shown in equation 7.6-35:

Egm=64∙Rgm+64∙log2bm ,							(7.6-35)

where:

Egm is the scaled energy for the gth group and mth perceptual band,

Rgm is the RMS envelope for the gth group and mth perceptual band,

64∙log2bm is the scaled log domain bandwidth of the mth perceptual band, and

∙ is a floor operation.

The scaled log domain bandwidth (64∙log2bm ) values are constant so can be pre-calculated as shown in Table 7.6-13. In Table 7.6-13, 64∙log2bm is expressed as BWAm.

Step 2: Compute the sensation level for each perceptual band as the energy level of the band relative to the absolute threshold of hearing as shown in equation 7.6-36:

SVgm=Egm-TQm ,								(7.6-36)

where:

SVgm is the sensation level for the gth group in a frame and mth perceptual band, and

TQm is the hearing threshold in quiet. The values for  TQm are provided in Table 7.6-14.

Step 3: Compute the mask offset due to the sensation level as shown in equation 7.6-37:

SVmOffset= DmSVm256 ,								(7.6-37)

where:

SVmOffset is the mask offset due to sensation level, and

Dm is the sensation level offset gradient for each perceptual band. The values for Dm are provided in Table 7.6-15.

Step 4: Offset the energy in each perceptual band by the sensation level offset as shown in equation 7.6-38:

EgmOffset=Egm-SVmOffset ,							(7.6-38)

where:

EgmOffset is the energy for the gth group and mth perceptual band offset by the sensation level offset.

Step 5: Convert the sensation offset energy in each perceptual band to the loudness domain as shown in 
equation 7.6-39:

EgmLoudnes=614∙EgmOffset2048 .								(7.6-39)

Step 6: Spread the loudness energy in each perceptual band with a spreading function as shown in equation 7.6-40:

MgmLoudness=LogAddEgmLoudnes,EglLoudnes+SFml  ∀l= 0,1,…M-1 ,	(7.6-40)

where:

MgmLoudness is the loudness domain masking level,

SFml is the spreading function applied to the mth perceptual band due to the energy in the lth perceptual band. The values for SFml are provided in table 7.6-16.

LogAdda,b is a table look-up approximation of a multiply and add in the linear domain as shown in equation 
7.6-41:

LogAdda,b=a+La-ba>bb+Lb-ab≥a  ,						(7.6-41)

where:

La is a table look up, the values of La are provided in Table 7.6-17.

Step 7: Convert mask back from the loudness domain as shown in equation 7.6-42:

Mgm=6827∙MgmLoudness2048	  .							(7.6-42)

Step 8: Compute the signal to mask ratio (target SNR at JND) for the gth group and mth perceptual band as shown in equation 7.6-43:

SMRgm=Egm-Mgm .								(7.6-43)

The above steps are for mono or independent channel coding, however, if 2 channels are present in the signal and joint channel coding is in use, then there is an additional step after step 7. The additional step sets the masking level in both channels to be the greater of the two masking levels as shown in equation 7.6-44. This additional step is only used if there are 2 channels present, the mth band is jointly coded, and the band m is greater than 0.

Mgm1=Mgm1Mgm1≥Mgm2Mgm2Mgm2>Mgm1  ,							(7.6-44)

Mgm2=Mgm1  .

Table 7.6-13: Bandwidth adjust values (BWAm) for each perceptual band

| Band (m) | BWAm | Band (m) | BWAm |
| --- | --- | --- | --- |
| 0 | 0 | 12 | 64 |
| 1 | 0 | 13 | 64 |
| 2 | 0 | 14 | 64 |
| 3 | 0 | 15 | 64 |
| 4 | 0 | 16 | 101 |
| 5 | 0 | 17 | 101 |
| 6 | 0 | 18 | 128 |
| 7 | 0 | 19 | 165 |
| 8 | 0 | 20 | 165 |
| 9 | 0 | 21 | 180 |
| 10 | 0 | 22 | 213 |
| 11 | 64 |  |  |

Table 7.6-14: Threshold in quiet (TQm)for each perceptual band

| Band (m) | TQm | Band (m) | TQm |
| --- | --- | --- | --- |
| 0 | 133 | 12 | 282 |
| 1 | 133 | 13 | 307 |
| 2 | 133 | 14 | 330 |
| 3 | 133 | 15 | 352 |
| 4 | 133 | 16 | 404 |
| 5 | 133 | 17 | 461 |
| 6 | 133 | 18 | 525 |
| 7 | 133 | 19 | 631 |
| 8 | 138 | 20 | 1249 |
| 9 | 159 | 21 | 1511 |
| 10 | 183 | 22 | 1519 |
| 11 | 241 |  |  |

Table 7.6-15. Sensation level offset gradient (Dm) for each perceptual band

| Band (m) | Dm | Band (m) | Dm |
| --- | --- | --- | --- |
| 0 | 112 | 12 | 64 |
| 1 | 112 | 13 | 64 |
| 2 | 96 | 14 | 64 |
| 3 | 80 | 15 | 64 |
| 4 | 80 | 16 | 64 |
| 5 | 64 | 17 | 64 |
| 6 | 64 | 18 | 64 |
| 7 | 64 | 19 | 64 |
| 8 | 64 | 20 | 64 |
| 9 | 64 | 21 | 64 |
| 10 | 64 | 22 | 64 |
| 11 | 64 |  |  |

Table 7.6-16. Spreading function (SFml)

| m | SFml |
| --- | --- |
| 0 | 0, -1561, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 1 | -289, -4, -1234, -2295, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 2 | -569, -229, -8, -905, -1705, -2324, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 3 | -789, -445, -173, -16, -656, -1271, -1765, -2172, -2520, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 4 | -961, -616, -340, -136, -28, -488, -976, -1382, -1729, -2032, -2305, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 5 | -1088, -743, -465, -257, -148, -31, -371, -769, -1114, -1417, -1689, -2054, -2483, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 6 | -1198, -852, -574, -364, -209, -148, -42, -300, -635, -936, -1207, -1572, -2000, -2376, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 7 | -1293, -948, -669, -458, -301, -183, -145, -56, -258, -547, -816, -1179, -1606, -1982, -2311, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 8 | -1375, -1029, -750, -539, -381, -260, -180, -142, -68, -231, -487, -846, -1272, -1647, -1976, -2261, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 9 | -1444, -1099, -820, -608, -449, -328, -233, -194, -138, -77, -213, -555, -978, -1352, -1681, -1966, -2268, -2552, -2552, -2552, -2552, -2552, -2552 |
| 10 | -1501, -1155, -876, -665, -505, -383, -287, -210, -193, -130, -79, -298, -711, -1083, -1411, -1696, -1997, -2288, -2550, -2552, -2552, -2552, -2552 |
| 11 | -1567, -1221, -942, -730, -570, -448, -351, -272, -206, -189, -151, -72, -349, -713, -1039, -1324, -1625, -1915, -2177, -2448, -2552, -2552, -2552 |
| 12 | -1650, -1304, -1025, -813, -653, -530, -432, -352, -285, -227, -177, -163, -69, -297, -613, -895, -1195, -1485, -1746, -2017, -2238, -2401, -2545 |
| 13 | -1727, -1381, -1102, -890, -730, -607, -509, -428, -360, -301, -249, -180, -153, -72, -257, -527, -824, -1112, -1373, -1643, -1865, -2028, -2171 |
| 14 | -1798, -1452, -1173, -960, -800, -677, -579, -498, -430, -370, -317, -246, -192, -145, -76, -224, -505, -790, -1050, -1320, -1540, -1703, -1847 |
| 15 | -1860, -1514, -1234, -1022, -862, -738, -640, -559, -490, -430, -377, -306, -224, -197, -136, -81, -242, -515, -771, -1040, -1260, -1422, -1566 |
| 16 | -1923, -1577, -1297, -1085, -925, -801, -703, -621, -553, -492, -439, -367, -284, -213, -198, -144, -83, -235, -479, -744, -963, -1125, -1268 |
| 17 | -1986, -1640, -1360, -1148, -988, -864, -766, -684, -615, -555, -501, -429, -345, -273, -211, -204, -146, -89, -216, -465, -680, -841, -984 |
| 18 | -2043, -1697, -1417, -1205, -1044, -921, -822, -741, -672, -611, -557, -485, -401, -328, -264, -211, -205, -140, -93, -227, -430, -588, -729 |
| 19 | -2104, -1758, -1479, -1266, -1106, -982, -884, -802, -733, -673, -619, -546, -461, -388, -324, -269, -212, -211, -151, -100, -195, -336, -472 |
| 20 | -2163, -1817, -1537, -1324, -1164, -1040, -942, -860, -791, -731, -676, -604, -519, -445, -380, -325, -268, -226, -219, -147, -114, -167, -280 |
| 21 | -2203, -1857, -1577, -1365, -1205, -1081, -982, -901, -831, -771, -717, -644, -559, -485, -420, -364, -306, -252, -239, -206, -132, -122, -163 |
| 22 | -2224, -1878, -1598, -1386, -1225, -1102, -1003, -921, -852, -792, -737, -665, -580, -505, -441, -385, -326, -271, -222, -224, -176, -121, -114 |

Table 7.6-17. Log Add look up table values (La)

| a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 64 | 64 | 37 | 128 | 21 | 192 | 11 | 256 | 6 | 320 | 3 | 384 | 1 | 448 | 1 |
| 1 | 64 | 65 | 37 | 129 | 20 | 193 | 11 | 257 | 6 | 321 | 3 | 385 | 1 | 449 | 1 |
| 2 | 63 | 66 | 37 | 130 | 20 | 194 | 11 | 258 | 5 | 322 | 3 | 386 | 1 | 450 | 1 |
| 3 | 63 | 67 | 36 | 131 | 20 | 195 | 11 | 259 | 5 | 323 | 3 | 387 | 1 | 451 | 1 |
| 4 | 62 | 68 | 36 | 132 | 20 | 196 | 10 | 260 | 5 | 324 | 3 | 388 | 1 | 452 | 1 |
| 5 | 62 | 69 | 36 | 133 | 20 | 197 | 10 | 261 | 5 | 325 | 3 | 389 | 1 | 453 | 1 |
| 6 | 61 | 70 | 35 | 134 | 19 | 198 | 10 | 262 | 5 | 326 | 3 | 390 | 1 | 454 | 1 |
| 7 | 61 | 71 | 35 | 135 | 19 | 199 | 10 | 263 | 5 | 327 | 3 | 391 | 1 | 455 | 1 |
| 8 | 60 | 72 | 35 | 136 | 19 | 200 | 10 | 264 | 5 | 328 | 3 | 392 | 1 | 456 | 1 |
| 9 | 60 | 73 | 35 | 137 | 19 | 201 | 10 | 265 | 5 | 329 | 3 | 393 | 1 | 457 | 1 |
| 10 | 59 | 74 | 34 | 138 | 19 | 202 | 10 | 266 | 5 | 330 | 3 | 394 | 1 | 458 | 1 |
| 11 | 59 | 75 | 34 | 139 | 19 | 203 | 10 | 267 | 5 | 331 | 3 | 395 | 1 | 459 | 1 |
| 12 | 58 | 76 | 34 | 140 | 18 | 204 | 10 | 268 | 5 | 332 | 2 | 396 | 1 | 460 | 1 |
| 13 | 58 | 77 | 33 | 141 | 18 | 205 | 10 | 269 | 5 | 333 | 2 | 397 | 1 | 461 | 1 |
| 14 | 57 | 78 | 33 | 142 | 18 | 206 | 9 | 270 | 5 | 334 | 2 | 398 | 1 | 462 | 1 |
| 15 | 57 | 79 | 33 | 143 | 18 | 207 | 9 | 271 | 5 | 335 | 2 | 399 | 1 | 463 | 1 |
| 16 | 56 | 80 | 32 | 144 | 18 | 208 | 9 | 272 | 5 | 336 | 2 | 400 | 1 | 464 | 1 |
| 17 | 56 | 81 | 32 | 145 | 17 | 209 | 9 | 273 | 5 | 337 | 2 | 401 | 1 | 465 | 1 |
| 18 | 55 | 82 | 32 | 146 | 17 | 210 | 9 | 274 | 5 | 338 | 2 | 402 | 1 | 466 | 1 |
| 19 | 55 | 83 | 32 | 147 | 17 | 211 | 9 | 275 | 5 | 339 | 2 | 403 | 1 | 467 | 1 |
| 20 | 55 | 84 | 31 | 148 | 17 | 212 | 9 | 276 | 5 | 340 | 2 | 404 | 1 | 468 | 1 |
| 21 | 54 | 85 | 31 | 149 | 17 | 213 | 9 | 277 | 4 | 341 | 2 | 405 | 1 | 469 | 1 |
| 22 | 54 | 86 | 31 | 150 | 17 | 214 | 9 | 278 | 4 | 342 | 2 | 406 | 1 | 470 | 1 |
| 23 | 53 | 87 | 30 | 151 | 16 | 215 | 9 | 279 | 4 | 343 | 2 | 407 | 1 | 471 | 1 |
| 24 | 53 | 88 | 30 | 152 | 16 | 216 | 8 | 280 | 4 | 344 | 2 | 408 | 1 | 472 | 1 |
| 25 | 52 | 89 | 30 | 153 | 16 | 217 | 8 | 281 | 4 | 345 | 2 | 409 | 1 | 473 | 1 |
| 26 | 52 | 90 | 30 | 154 | 16 | 218 | 8 | 282 | 4 | 346 | 2 | 410 | 1 | 474 | 1 |
| 27 | 51 | 91 | 29 | 155 | 16 | 219 | 8 | 283 | 4 | 347 | 2 | 411 | 1 | 475 | 1 |
| 28 | 51 | 92 | 29 | 156 | 16 | 220 | 8 | 284 | 4 | 348 | 2 | 412 | 1 | 476 | 1 |
| 29 | 51 | 93 | 29 | 157 | 15 | 221 | 8 | 285 | 4 | 349 | 2 | 413 | 1 | 477 | 1 |
| 30 | 50 | 94 | 28 | 158 | 15 | 222 | 8 | 286 | 4 | 350 | 2 | 414 | 1 | 478 | 1 |
| 31 | 50 | 95 | 28 | 159 | 15 | 223 | 8 | 287 | 4 | 351 | 2 | 415 | 1 | 479 | 1 |
| 32 | 49 | 96 | 28 | 160 | 15 | 224 | 8 | 288 | 4 | 352 | 2 | 416 | 1 | 480 | 1 |
| 33 | 49 | 97 | 28 | 161 | 15 | 225 | 8 | 289 | 4 | 353 | 2 | 417 | 1 | 481 | 1 |
| 34 | 49 | 98 | 27 | 162 | 15 | 226 | 8 | 290 | 4 | 354 | 2 | 418 | 1 | 482 | 0 |
| 35 | 48 | 99 | 27 | 163 | 15 | 227 | 8 | 291 | 4 | 355 | 2 | 419 | 1 | 483 | 0 |
| 36 | 48 | 100 | 27 | 164 | 14 | 228 | 8 | 292 | 4 | 356 | 2 | 420 | 1 | 484 | 0 |
| 37 | 47 | 101 | 27 | 165 | 14 | 229 | 7 | 293 | 4 | 357 | 2 | 421 | 1 | 485 | 0 |
| 38 | 47 | 102 | 26 | 166 | 14 | 230 | 7 | 294 | 4 | 358 | 2 | 422 | 1 | 486 | 0 |
| 39 | 47 | 103 | 26 | 167 | 14 | 231 | 7 | 295 | 4 | 359 | 2 | 423 | 1 | 487 | 0 |
| 40 | 46 | 104 | 26 | 168 | 14 | 232 | 7 | 296 | 4 | 360 | 2 | 424 | 1 | 488 | 0 |
| 41 | 46 | 105 | 26 | 169 | 14 | 233 | 7 | 297 | 4 | 361 | 2 | 425 | 1 | 489 | 0 |
| 42 | 45 | 106 | 25 | 170 | 14 | 234 | 7 | 298 | 4 | 362 | 2 | 426 | 1 | 490 | 0 |
| 43 | 45 | 107 | 25 | 171 | 13 | 235 | 7 | 299 | 4 | 363 | 2 | 427 | 1 | 491 | 0 |
| 44 | 45 | 108 | 25 | 172 | 13 | 236 | 7 | 300 | 4 | 364 | 2 | 428 | 1 | 492 | 0 |
| 45 | 44 | 109 | 25 | 173 | 13 | 237 | 7 | 301 | 3 | 365 | 2 | 429 | 1 | 493 | 0 |
| 46 | 44 | 110 | 24 | 174 | 13 | 238 | 7 | 302 | 3 | 366 | 2 | 430 | 1 | 494 | 0 |
| 47 | 43 | 111 | 24 | 175 | 13 | 239 | 7 | 303 | 3 | 367 | 2 | 431 | 1 | 495 | 0 |
| 48 | 43 | 112 | 24 | 176 | 13 | 240 | 7 | 304 | 3 | 368 | 2 | 432 | 1 | 496 | 0 |
| 49 | 43 | 113 | 24 | 177 | 13 | 241 | 7 | 305 | 3 | 369 | 2 | 433 | 1 | 497 | 0 |
| 50 | 42 | 114 | 24 | 178 | 13 | 242 | 6 | 306 | 3 | 370 | 2 | 434 | 1 | 498 | 0 |
| 51 | 42 | 115 | 23 | 179 | 12 | 243 | 6 | 307 | 3 | 371 | 2 | 435 | 1 | 499 | 0 |
| 52 | 42 | 116 | 23 | 180 | 12 | 244 | 6 | 308 | 3 | 372 | 2 | 436 | 1 | 500 | 0 |
| 53 | 41 | 117 | 23 | 181 | 12 | 245 | 6 | 309 | 3 | 373 | 2 | 437 | 1 | 501 | 0 |
| 54 | 41 | 118 | 23 | 182 | 12 | 246 | 6 | 310 | 3 | 374 | 2 | 438 | 1 | 502 | 0 |
| 55 | 41 | 119 | 22 | 183 | 12 | 247 | 6 | 311 | 3 | 375 | 2 | 439 | 1 | 503 | 0 |
| 56 | 40 | 120 | 22 | 184 | 12 | 248 | 6 | 312 | 3 | 376 | 2 | 440 | 1 | 504 | 0 |
| 57 | 40 | 121 | 22 | 185 | 12 | 249 | 6 | 313 | 3 | 377 | 2 | 441 | 1 | 505 | 0 |
| 58 | 39 | 122 | 22 | 186 | 12 | 250 | 6 | 314 | 3 | 378 | 2 | 442 | 1 | 506 | 0 |
| 59 | 39 | 123 | 22 | 187 | 11 | 251 | 6 | 315 | 3 | 379 | 2 | 443 | 1 | 507 | 0 |
| 60 | 39 | 124 | 21 | 188 | 11 | 252 | 6 | 316 | 3 | 380 | 1 | 444 | 1 | 508 | 0 |
| 61 | 38 | 125 | 21 | 189 | 11 | 253 | 6 | 317 | 3 | 381 | 1 | 445 | 1 | 509 | 0 |
| 62 | 38 | 126 | 21 | 190 | 11 | 254 | 6 | 318 | 3 | 382 | 1 | 446 | 1 | 510 | 0 |
| 63 | 38 | 127 | 21 | 191 | 11 | 255 | 6 | 319 | 3 | 383 | 1 | 447 | 1 | 511 | 0 |

#### 7.6.4.2.7	Linear Prediction

###### 7.6.4.2.7.1	Overview

Since the frequency resolution of the LCLD frequency bands is relatively coarse (400 Hz at 48kHz sample rate) there is no significant coding gain for signals with strong harmonic content. However, the signal redundancy within LCLD bands for this type of signal can be greatly reduced by linear prediction over time and the SNR increased. .

###### 7.6.4.2.7.2	Bitstream Syntax

For each frame flags for every LCLD band in a certain LCLD band range are transmitted in the bitstream which enable or disable linear predictive coding (LPC). If LPC is enabled for a certain band the magnitude and the phase of the first order prediction parameter are transmitted at the total cost of 8 bit per band and frame. Additional bitstream elements indicate if any LPC is active for a channel and maximum band using LPC. For short frames like 10ms or 5ms a dedicated signalling scheme is used to reduce the prediction side rate.

The prediction processing at the encoder is done in a closed loop fashion, and a dedicated Huffman code book is used to encode the prediction residual. Due to the closed loop encoding, the prediction is part of the quantization and bit allocation loop which determines the allocation offset and associated quantizers for each perceptual band.

###### 7.6.4.2.7.3	Prediction for 20ms frames

For every LCLD band b with active prediction, the quantized prediction residual is computed as

Xn,bQ=QXn,b-Sn-1,bQ⋅PbQ, n=0,1,…,Nframe-1 ,

where Xn,b is the complex LCLD input sample at index n and LCLD band b.

Note that the Superscript Q is used here to indicate quantized and inverse quantized numbers,

Sn-1,bQ is the prediction state which is computed as:

Sn,bQ=Xn,bQ+Sn-1,bQPbQ ,

where Pn,bQ is the quantized Prediction coefficient.

For 20ms frame length, the first sample in each LCLD band is not predicted, thus the prediction state is initialized as  S-1,bQ=0.

Q.. indicates quantization and inverse quantization of a complex number, where the scale factor and the quantization range are determined by the perceptual model and the allocation offset.

###### 7.6.4.2.7.4	Prediction for frames shorter than 20ms

The prediction operation depends on a certain subset of LCLD bands (subset in the following text) and the number of possible subsets. Typically, the number K of possible subsets is

K=16Nframe  .

For example, Nframe=4 in the case of 5ms frame length.

Then bands bs belonging to the subset s are defined as

bs =b ∈ modb,K=s, b=0,1,…,NLCLD-1, s=0,1,..,K-1 ,

where NLCLD is the total number LCLD bands (60).

For each frame the encoder determines a current subset scur which is, together with the number K of possible subsets, transmitted in the bitstream. The current subset shall cycle through the possible subsets in ascending order as 0,1,…,K-1, 0, 1,… and so on for each new frame.

The prediction state for the first sample in the current frame for all bands belonging to the current subset is initialized to zero just as in the 20ms frame length case:

S-1,bscur Q=0 .

Otherwise, the state is according to the last sample of the previous frame. In other words, the prediction is only reset at the beginning of a frame if the LCLD band belongs to the current subset of LCLD bands.

###### 7.6.4.2.7.5	Prediction Signalling

Prediction processing is determined by the following bitstream elements:

Table 7.6.-18: Prediction Syntax Elements

| Syntax Element | Comment | Number of Bits |
| --- | --- | --- |
| NumSubSets | Possible number of Subsets K | 3 |
| SubSetId | Current subset scur | if K > 4 3 Elseif K > 1 2 Else 0 |
| PredChanEnable | Prediction active/passive flags, one bit for each possible subset | K (per channel) |
| NumPredBands | The maximum LCLD band with active prediction for the current subset | if K >= 4 4 elseif K > 1 5 Else 6 (per channel) |
| PredBandEnable | Array of active/passive flags for all LCLD bands up to NumPredBands for each audio channel. | Int(NumPredBands / K)+1 (per channel) |
| PredictionMagnitude | For every active LCLD band belonging to the current subset for each audio channel | 3 bits per active LCLD band |
| Prediction Phase | For every active LCLD band belonging to the current subset for each audio channel | 5 bits per active LCLD band |

###### 7.6.4.2.7.6	Quantization of Prediction Parameters

The prediction magnitude quantization indices are computed as follows:

PMidq=int scaleM⋅arcsinPM+0.5  ,

where PM is the optimal prediction magnitude and the scale is given as

scaleM = 17π .

The “quantized” (quantized and inverse quantized) prediction magnitude is given as

PMQ = sinPMidq⋅1scale .

The prediction phase quantization indices are computed as

PPhidq=roundscalePhPPh  ,

where round() indicates rounding to the nearest integer and the quantized prediction phase is given by

PPhQ=PPhidq⋅1scalePh  .

The scale is given as scalePh = 16π .

###### 7.6.4.2.7.7	Estimation of Prediction Parameters

Regardless of the frame length prediction parameters are computed based on 20ms (16 samples at 48kHz) of audio data per LCLD band. Prediction parameters may be updated only for the current subset of LCLD bands (all LCLD bands if the number of possible subsets is 1). For LCLD bands belonging to other subsets which are not the current subset may either be deactivated or can be continuously used without any modification. This information is available in the bitstream element PredChanEnable as shown in table Prediction Syntax Elements.

The optimal, complex first order prediction coefficient per band b is computed as

Pb=Rxx1Rxx0 ,

where Rxx0and Rxx1 is the auto-correlation function at lag 0 and lag 1 respectively. The auto-correlation function at lag 0 (Energy) is computes as follows:

Rxx,b0=n=0Nframe20ms-1Xn,b2  .

The auto-correlation function at lag 1 is computed as

Rxx,b1=n=1Nframe20ms-1Xn-1,b⋅Xn,b*  .

The prediction magnitude is given as

PM,b=Pb  .

And the prediction phase is given as

PPh,b=arctan2imag PbrealPb  .

Based on the optimal prediction coefficient a prediction gain (ratio of signal energy to residual signal energy) is computed for every LCLD band as

GbP=11-minPb,M,Pb,MQ  .

Based on the prediction gain a bitrate saving is estimated as follows:

BitsGainbP=0.65⋅log2GbPNframe20ms-8  ,

where 8 corresponds to the fixed number of bits needed to transmit the prediction parameters.

Now, LCLD bands b with BitsGainbbP> 0 are candidates for prediction.

Finally, the optimal maximum prediction band B-1 B-1 is determined by maximizing the accumulated bitrate saving, subject to the cost of signalling the active prediction bands.

B = argmaxb∈bscurB-1BitsGainbP-NB-7  ,

where NB is the number of prediction band candidates for b<B  and 7 corresponds to a fixed signalling cost.

To improve the recovery after frame loss, the audio data is written in chunks where each chunk corresponds to one subset of LCLD bands. The data associated with the current subset is always written first followed by the chunks containing data for subsets with decremented IDs. If, after decrementing, the subset ID becomes –1 then the subset ID of the written chunk is set to K-1 (K being the number possible subsets).

#### 7.6.4.2.8	Bit Allocation

The bit allocation routine distributes the available bits in a frame proportion to the target SNR (SMR) in each channel, group, and perceptual band. The allocation can be viewed as two components; a core allocation that takes the SMR generated by the perceptual model and a single control parameter (AllocOffset) to generate an allocation to channels, groups, and perceptual bands. The second component of the allocation in the encoder only is the search for the control parameter (AllocOffset). As the control parameter is transmitted to the decoder, the decoder can exactly mirror the core allocation routine, thus, mitigating the need to transmit explicit allocations for channels, groups, and bands.

The core allocation is shown in equation 7.6-45, similar to the perceptual model described in section 7.6.4.2.6, the core allocation uses integer arithmetic such that it can be exactly replicated in the decoder.

Ac,g,m=SMRc,g,m+8∙AllocOffset32  ,							(7.6-45)

where:

Ac,g,m is the allocation level for channel c, group g, and perceptual band m,

SMRc,g,m is the target SNR for channel c, group g, and perceptual band m computed by the perceptual model, and

AllocOffset is the control parameter that will be transmitted to the decoder.

The bit allocation control parameter (AllocOffset) is limited to the range between -128 and 127 and transmitted to the decoder as 8-bit unsigned value by adding 128 to the derived value.

The search for the control parameter requires the encoder to quantize the normalized CLDFB coefficients and count the number of bits needed to Huffman code the quantized CLDFB coefficients. The quantization and Huffman coding of the normalized CLDFB coefficients is described in section 7.6.4.2.9. Furthermore, if the prediction is enabled (see section 7.6.4.2.7), then the prediction stage must also be computed during the quantization and bit counting. The search for the control parameter could be brute force, that is, try every possible value of the AllocOffset parameter and choose the value that would lead to the number of bits needed to code the quantized CLDFB coefficients that is closest to the available bits without exceeding the available bits. However, a binary search algorithm will arrive at the same solution while mitigating some complexity.

The iterative binary search to derive the allocation offset parameter can be summarised in the following steps:

- Initialize AllocOffset to 0 and delta to 128

- Compute the core allocation shown in equation 7.6-45

- Quantize the normalized CLDFB coefficients and count the number of bits needed to Huffman code the coefficients.

- Compare the number of bits used to code the CLDFB coefficients and the available bits, return to step 2 or terminate based on the following conditions:

  - If the number of bits needed to the CLDFB coefficients is greater than the available bits and delta is greater than 0, set AllocOffset = AllocOffset - delta, set delta = delta≫1, return to step 2.

  - If the number of bits needed to the CLDFB coefficients is greater than the available bits and delta is equal to  0, set AllocOffset = AllocOffset - 1, set delta = 1, return to step 2.

  - If the number of bits needed to the CLDFB coefficients is less than the available bits and delta is greater than 0, set AllocOffset = AllocOffset + delta, set delta = delta≫1, return to step 2.

  - If the number of bits needed to the CLDFB coefficients is less than the available bits and delta is equal to 0, terminate search.

  - If the number of bits needed to the CLDFB coefficients is less than the available bits and AllocOffset is equal to 127, terminate search.

  - If the number of bits needed to the CLDFB coefficients is equal to the available bits, terminate the search.

#### 7.6.4.2.9	Quantization of the Normalized CLDFB Coefficients

###### 7.6.4.2.9.1	Overview

LCLD supports both direct quantization of the normalized CLDFB coefficients and quantization of differentially coded coefficients (DPCM), where the differential is relative to the previous slot value within the same CLDFB band. In both cases the quantization technique is the same. This section is split into three parts, the differential coding of the normalized CLDFB coefficients, the quantization of either the differential residual or the direct normalized CLDFB coefficients, and the Huffman coding and transmission of the quantized CLDFB coefficients.

#### 7.6.4.2.9.2	Differential Coding of the Normalized CLDFB Coefficients

The differential coding of the normalized CLDFB coefficients uses a complex first-order predictor stage that is running along time (slots) within the same CLDFB band. The calculation of the first-order predictor is described in section 7.6.4.2.7. The differential coding can be disabled based on the prediction gain achieved as described in section 7.6.4.2.7. For the longer LCLD frames, the first slot in a frame is not predicted, that is a predictor reset occurs at the start of the frame.

The prediction process is described in equation 7.6-46:

enk= SnkCLDFB+PkSn-1,kCLDFB  ,							(7.6-46)

where:

enk is the prediction residual for the nth slot and kth CLDFB band,

SnkCLDFB is the normalized CLDFB coefficient for the nth slot and kth CLDFB band,

Sn-1,kCLDFB is the quantized normalized CLDFB coefficient for the previous slot and kth CLDFB band, and

Pk is the prediction coefficient for the kth CLDFB band.

If the nth slot is directly after the predictor reset, then Sn-1,kCLDFB is the quantized value from the previous slot. However, if the nth slot follows a previously predicted slot, then the Sn-1,kCLDFB is reconstructed from the quantized residual as shown in equation 7.6-47:

Sn-1,kCLDFB=en-1,k-PkSn-2,kCLDFB  ,							(7.6-47)

where en-1,k is the quantized residual signal from the nth -1 slot and kth CLDFB band.

#### 7.6.4.2.9.3	Quantization of Normalized CLDFB coefficients and Prediction Residuals

The quantization of both normalized CLDFB coefficients and predictor residuals are the same and controlled by an allocation level generated by the bit allocation algorithm (see section 7.6.4.2.8). The allocation level (Agm) is calculated for groups and perceptual bands, so the following equations assume the nth slot is a member of the gth group and the kth CLDFB band is a member of the mth perceptual band. The quantization of either the normalized CLDFB coefficients or the prediction residual is shown in equation 7.6-48:

Ink=ROUNDgAgm∙xnk  ,							(7.6-48)

where:

Qnk is a complex signed integer that will be Huffman coded and transmitted to the decoder,

Agm is the allocation level for the gth group and mth perceptual band,

gAgm is the allocation level scaling factor provided in table 7.6-19, and

xnk is either the normalized CLDFB coefficient (SnkCLDFB) or the prediction residual (enk) for the nth slot and kth CLDFB band. The nth slot is assumed to be an element of the gth group and the kth CLDFB band is assumed to be an element of the mth perceptual band.

As the quantized values for both the normalized CLDFB and the residual are needed when the prediction is enabled, equation 7.6-49 shows the inverse quantization from the quantization index (Ink):

xnk= g-1Agm∙Qnk  ,								(7.6-49)

where:

xnk is either the quantized CLDFB coefficient or the quantized residual  and

g-1Agm is the inverse quantization scale factor provided in Table 7.6-19.

Table 7.6-19. Quantizer scaling factors () and inverse quantizer scaling factors () for each allocation level (Agm)

| Agm | gAgm | g-1Agm | Agm | gAgm | g-1Agm |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 16 | 4.7568 | 0.2098 |
| 1 | 0.3536 | 2.3675 | 17 | 5.6569 | 0.1765 |
| 2 | 0.4204 | 2.0464 | 18 | 6.7272 | 0.1485 |
| 3 | 0.5 | 1.7759 | 19 | 8 | 0.1249 |
| 4 | 0.5946 | 1.5364 | 20 | 9.5137 | 0.1051 |
| 5 | 0.7071 | 1.3231 | 21 | 11.3137 | 0.0884 |
| 6 | 0.8409 | 1.1329 | 22 | 13.4543 | 0.0743 |
| 7 | 1 | 0.9658 | 23 | 16 | 0.0625 |
| 8 | 1.1892 | 0.8213 | 24 | 19.0273 | 0.0526 |
| 9 | 1.4142 | 0.6951 | 25 | 22.6274 | 0.0442 |
| 10 | 1.6818 | 0.5878 | 26 | 26.9087 | 0.0372 |
| 11 | 2 | 0.4958 | 27 | 32 | 0.0312 |
| 12 | 2.3784 | 0.4181 | 28 | 38.0546 | 0.0263 |
| 13 | 2.8284 | 0.3522 | 29 | 45.2548 | 0.0221 |
| 14 | 3.3636 | 0.2962 | 30 | 53.8174 | 0.0186 |
| 15 | 4 | 0.2494 | 31 | 64 | 0.0156 |

#### 7.6.4.2.9.4	Huffman Coding of Quantized Normalized CLDFB coefficients and Quantized Prediction Residuals

As discussed in the previous section (7.6.4.2.9.3), the output of the quantizer is a complex signed integer (Qnk) that will Huffman coded and transmitted to the decoder. Prior to Huffman coding, the complex signed integer is first separated into real and imaginary as shown in equation 7.6-50 and then further separated into a magnitude (Qnkrand Qnki) and a sign bit as shown in equation 7.6-51 for both real and imaginary parts:

Qnkr= RQnkQnki= IQnk									(7.6-50)

signnkr=1,  &Qnkr<00,  &Qnkr≥0signnki=1,  &Qnki<00,  &Qnki≥0  .								(7.6-51)

The magnitude component is then limited to a max value that is dependent on the allocation level (Agm), as shown in equation 7.6-52:

Qnkr=Qnkr,  &Qnkr<maxAgmmaxAgm,  &Qnkr≥maxAgmQnki=Qnki,  &Qnki<maxAgmmaxAgm,  &Qnki≥maxAgm   , 						(7.6-52)

where maxAgm is the maximum supported value depending on the allocation level Agm, the maximum values are provided in Table 7.6-20.

The Huffman codebook used to code the real and imaginary components is dependent on the allocation level (Agm,) and dependent on whether the underlying quantized values are prediction residuals or normalized CLDFB coefficients. Furthermore, for low values of the allocation level, the real and imaginary quantized magnitudes are packed into a single codeword. The Huffman packaging depends on the following logic:

- If Agm is equal to 0, no information is transmitted, and the decoder generates 0’s for all CLDFB coefficients in the gth group and mth perceptual band.

- If Agm is greater than 0 and less than 12, then the real and imaginary parts of the quantized magnitude are packed into a single code word. After the Huffman code the sign bit for Qnkr if Qnkr is greater than 0 then the sign bit for Qnki if Qnki is greater than 0. The code and number of bits written can be expressed as a table look up as follows:

code,length=HuffLookupresnkAgmQnkr∙max⁡[Agm+1+Qnki ,

where:

code is the codeword written to stream,

length is the number bits to write the code, and

resnk is a binary value that is 1 if the underlying quantized values are prediction residuals or 0 if the underlying quantized values are normalized CLDFB coefficients.

- If Agm is greater than or equal to 12, then the real and imaginary parts of the quantized magnitude are packed into separate codewords. After the Huffman code the sign bit for Qnkr if Qnkr is greater than 0 then the sign bit for Qnki if Qnki is greater than 0. The code and number of bits written can be expressed as a table look up as follows:

coder,lengthr=HuffLookupresnkAgmQnkr  ,

codei,lengthi=HuffLookupresnkAgmQnki  .

Due to the size of the Huffman lookup tables, the tables can be found in the reference code.

Table 7.6-20. Maximum values maxAgm per allocation level Agm

| Agm | maxAgm | Agm | maxAgm |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 26 |
| 1 | 3 | 17 | 26 |
| 2 | 3 | 18 | 27 |
| 3 | 4 | 19 | 28 |
| 4 | 5 | 20 | 31 |
| 5 | 5 | 21 | 36 |
| 6 | 6 | 22 | 38 |
| 7 | 7 | 23 | 45 |
| 8 | 8 | 24 | 54 |
| 9 | 9 | 25 | 64 |
| 10 | 12 | 26 | 76 |
| 11 | 13 | 27 | 90 |
| 12 | 16 | 28 | 108 |
| 13 | 17 | 29 | 128 |
| 14 | 19 | 30 | 152 |
| 15 | 23 | 31 | 180 |

#### 7.6.4.3	LCLD decoder

##### 7.6.4.3.1	Overview

Figure 7.6-3 shows a high-level schematic of the LCLD decoder structure. As shown in figure 7.6-3, the grouping information that controls the number and update of the RMS envelope is first decoded as described in section 7.6.4.3.2. Following the grouping information, the RMS envelope is Huffman decoded and reconstructed as described in section7.6.4.3.3.  The RMS envelope values are then used to compute a backward adaptive perceptual model and bit allocation as decreed in sections 7.6.4.3.4 and 7.6.4.3.5 respectively. The bit allocation information derived in the decoder is then used to Huffman decode and inverse quantize the CLDFB coefficients as described in section 7.6.4.3.6. Following inverse quantization, a prediction filter is applied as described in section 7.6.4.3.7. The CLDFB coefficients are then inverse normalized to reinstate the original signal’s energy levels as described in section 7.6.4.3.8. The final stage of the decoder is the application of joint channel decoding, which is described in section 7.6.4.3.9.

Figure 7.6-3: The structure of the LCLD decoder

#### 7.6.4.3.2	Decoding Group Information

The group information which controls the number of RMS envelopes and therefore bit-allocation is coded as a set of single-bit flags. A value 1 indicates the start of a new group, while a value of 0 indicates the continuation of a group. The first slot in a frame is always the start of a new group and no flag is transmitted for the first slot in a frame. If a flame contains 16 slots there will be 15 flags containing group information. For 2 channel content, a single-bit flag is transmitted prior to the grouping information to specify common grouping. If the common grouping flag is 1, then a single set of grouping information follows and both channels share grouping information. If the common grouping flag is 0, then the channels have independent groups and 2 sets of grouping information follow.

#### 7.6.4.3.3	Decoding RMS Envelope Information

RMS envelope information is present in the bitstream for each channel and each group for that channel. The ordering of RMS envelope information is by channel and then group. The RMS envelope in each channel and each group are differentially and Huffman coded except for the envelope values for the first perceptual band (0th index). The envelope value for first perceptual band for all groups in a channel is transmitted as an absolute number in an 7-bit unsigned integer. To obtain the signed RMS envelope for the first perceptual band 64 is subtracted from the value read from the bitstream. Equation 7.6-53 shows the process to obtain the RMS envelope values for remaining perceptual bands in a group:

Rgm=Rg,m-1+∆Rgm ∀m>0  ,							(7.6-53)

where:

Rgm is the RMS envelope values for the gth group and mth perceptual band, and

∆Rgm is the RMS envelope delta that is Huffman decode from the bitstream.

The Huffman decode tables can be obtained from the reference code.

#### 7.6.4.3.4	Perceptual Model

The perceptual model used in the decoder is the same as the perceptual model calculation in the encoder, see section 7.6.4.2.6.

#### 7.6.4.3.5	Bit Allocation

The bit allocation in the LCLD is the same as the core allocation in the encoder (see section 7.6.4.2.8). The firsts step of the bit allocation is to read the AllocOffset from the bitstream, which is transmitted as an 8-bit unsigned integer. The AllocOffset parameter is then converted back to a signed integer by subtracting 128. The bit allocation for each channel, group, and perceptual band is then calculated as shown in equation 7.6-54:

Ac,g,m=SMRc,g,m+8∙AllocOffset32  ,							(7.6-54)

where:

Ac,g,m is the allocation level for channel c, group g, and perceptual band m,

SMRc,g,m is the target SNR for channel c, group g, and perceptual band m computed by the perceptual model, and

AllocOffset is the control parameter that is read from the bitstream.

#### 7.6.4.3.6	Normalized CLDFB Coefficient and Prediction Residual Huffman Decoding and Inverse Quantization

The normalized CLDFB coefficients or the prediction residual in the bitstream are Huffman coded. The following description assumes the nth slot is an element of the gth group and the kth CLDFB band is element of the mth perceptual band. The specific Huffman codebook used to code the coefficients or residual is dependent on the allocation level (Agm) and whether the values are normalized CLDFB coefficients or prediction residuals. The quantized values that are Huffman coded in the bitstream represent complex integers. The complex integers are expressed as separate magnitudes and sign bits. The packaging of the complex integers is dependent on the allocation level (Agm) as follows:

- If Agm is equal to 0, no information is in the bitstream and all values in the gth group and mth perceptual band are set to 0.

- If Agm is greater than 0 and less than 12, the real and imaginary parts of the complex integer (Qnk) are packed into a single Huffman codeword. After Huffman decoding, the magnitude of the real and imaginary parts of the complex integer can be obtained with the following equations 7.6-55 and 7.6-56. If Qnkr is greater than 0, then a sign bit (signnkr) for real part of the complex integer follows in the bitstream. If Qnki is greater than 0, then a sign bit (signnki) for imaginary part of the complex integer follows in the bitstream.

Qnkr=valuemaxAgm+1  ,								(7.6-55)

Qnki=modvalue,maxAgm+1  ,						(7.6-56)

where:

Qnkr is the magnitude of the real part of the complex integer,

Qnkiis the magnitude of the imaginary pert of the complex integer,

value is the Huffman decoded value from the bitstream, and

maxAgm is the max values which are provided in Table 7.6-20 in section 7.6.4.7.3.

- If Agm is greater than or equal to 12, then the magnitude of the real part of the complex integer (Qnk) and the magnitude of the imaginary part of the complex integer (Qnk) are coded in separate Huffman codes. If Qnkr is greater than 0, then a sign bit for real part of the complex integer follows in the bitstream. If Qnkr is greater than 0, then a sign bit (signnkr) for real part of the complex integer follows in the bitstream. If Qnki is greater than 0, then a sign bit (signnki) for imaginary part of the complex integer follows in the bitstream.

- Once the magnitudes of the real and imaginary parts are Huffman decode and the sign bits are read, the complex integer (Qnk) can be reconstructed as shown in equation 7.6-57:

Qnk=signnkr∙Qnkr+i∙signnki∙Qnki  .						(7.6-57)

While the Huffman decoding is dependent on the type of values in the bitstream, the inverse quantization is only dependent on the allocation level as shown in equation 7.6-58.

Xnk= g-1Agm∙Qnk  ,								(7.6-58)

where:

Xnk is either the quantized CLDFB coefficient or the quantized residual,

Qnk is the complex signed integer decode from the bit stream, and

g-1Agm is the inverse quantization scale factor provided in table 7.6-19 in section 7.6.4.7.2.

#### 7.6.4.3.7	Inverse Prediction

###### 7.6.4.3.7.1	Overview

The reconstruction of transmitted prediction residuals is identical to the processing at the encoder side where the prediction state corresponds to the reconstructed sample as shown below:

Sn,bQ=Xn,bQ+Sn-1,bQPbQ  ,

where Sn,bQis the reconstructed complex sample at time sample index n and LCLD band b and Xn,bQ are the transmitted residual samples.

#### 7.6.4.3.7.2	Status Tracking after Frame Loss

In case of short frames, part of the prediction parameters may be held constant for one or more frames as indicated in the syntax element PredChanEnable. After frame loss, for this this type of prediction metadata the data in the decoder memory might be incorrect which is indicated as unresolved in the following text. Since the selection of Huffman code books to decode the audio data depends on the prediction metadata, audio data cannot be safely decoded. To handle this situation, the decoder keeps track on the Unresolved status of prediction metadata in a persistent array named DecodingUnresolved with of size NumChannels x NumPossibleSubsets.

During frame loss all entries of DecodingUnresolved are set to true (no prediction data present), When reading a good frame after frame loss the variable is updated as follows:

For the current subset prediction metadata is always present in the bitstream and the status for the current subset will be set to Resolved (false in the variable). If prediction is inactive for a subset, then the status is also resolved. For other subsets the status will remain unresolved in the first good frame after frame loss.

Based on the DecodingResolved status, audio data is decoded in chunks corresponding to subsets until an Unresolved status occurs. The information of correctly decoded subsets is stored in the additional, persistent arrays DecodingFailed and DecodingFailedPrev for the current and the previous frame respectively which are used to guide packet loss concealment for the first few good frames after packet loss as shown in Figure 7.6-4.

During frame loss all entries of DecodingFailed and DecodingFailedPrev are set to true (no data decoded). During frame loss and shortly after frame loss, data for subsets for which decoding failed are concealed (all data during frame loss). Correctly decoded subsets are combined with concealed subsets until all subsets can be decoded correctly. If the status changes from concealed to decoded (as indicated by variables DecodingFailed and DecodingFailedPrev), then the first few samples are crossfaded from concealed to decoded data.

More details on LCLD packet loss concealment are provided in clause 7.6.4.4.

Figure

7.6-4

: High-level flowchart for concealment operations during packet loss and following packet loss

#### 7.6.4.3.8	Inverse RMS Envelope Normalization

The inverse normalization of the CLDFB coefficients by the RMS envelope returns the normalized spectrum to the original energy levels. In this description of the inverse normalization process, the nth slot is assumed to be an element of the gth group and the kth CLDFB band is assumed to be an element the mth perceptual band. The inverse normalization of the CLDFB coefficients are shown in equation 7.6-59:

SnkCLDFB=SnkCLDFB20.5∙Rgm  ,								(7.6-59)

where:

SnkCLDFB is the inverse normalized CLDFB coefficients for the nth slot and kth CLDFB band,

SnkCLDFB is the normalized CLDFB coefficients for the nth slot and kth CLDFB band, and

Rgm is the RMS envelope for the gth group that contains the nth slot and mth perceptual band that contains the kth CLDFB band.

#### 7.6.4.3.9	Inverse Joint Stereo Processing

Depending on the read bitstream element MSMode and other flags, inverse joint stereo processing is applied for all LCLD bands b belonging to perceptual band k as described in the below matrix notation:

Xb,nLXb,nR=100e-jφkQ111-110pkQ1Xb,nMXb,nS  ,

where Xb,nLand Xb,nRare the decoded left and right complex input samples, Xb,nM and Xb,nS are the transmitted jointly coded samples and n is the time sample index within the coded frame.

The joint stereo coding parameters pkQand φkQare the transmitted prediction and phase alignment parameters which are found as follows:

- Prediction Parameters

  - Undo frequency differential coding of prediction parameters

  - De-Quantization of prediction parameters

- Phase Parameters

  - Undo second order frequency differential coding

  - Wrap indices into the quantization range of [-12,12]

  - Derive phase rotation factors e-jφkQby table-lookup

#### 7.6.4.4	LCLD packet loss concealment

##### 7.6.4.4.1	General

LCLD packet loss concealment is a low-complexity method operating in CLDFB domain. Whenever a frame is marked as lost or unavailable by the BFI flag raised, frame loss concealment operations produce substitution CLDFB coefficients for each CLDFB band. The band-wise operation follows either a sinusoidal extension strategy or a strategy based on linear prediction combined with repetition of CLDFB coefficients of the same CLDFB band of the preceding frame. The selection of strategy is made based on a tonality determination of the CLDFB band signal. After band-wise substitution frame generation in CLDFB domain, the substitution frame is converted to time domain using CLDFB synthesis like a regularly decoded good frame.

To ensure smooth signal evolution at frame boundaries, a crossfade mechanism is active at transitions from a substitution frame to a succeeding good frame.

Also provided is a mechanism for burst loss handling including muting.

Figure 7.6-4 above is a high-level flowchart of the LCLD frame loss concealment operations. It illustrates how and under what conditions decoded (good) frames and concealed, i.e., substituted frames, are used to generate an output frame to be converted back to time domain using CLDFB synthesis. It is further shown how state information about successfully decoding of a previous frame and a current frame is used to decide if crossfading is used when transitioning from a substituted bad frame to a decoded good frame.

##### 7.6.4.4.2	Synthesis model

Two alternative synthesis models are available for generation of the CLDFB TF tiles of a given CLDFB band k of the substitution frame.

Sinusoidal model

The one is a sinusoidal model, capable of extending a damped sinusoid in the given CLDFB band of the preceding frame into the substitution frame. This model is especially useful for relatively tonal signals. According to this model, a complex sinusoid in the given band k of the preceding frame xn, n=0…L-1 can be expressed with the following equation:

xkn=a0,k κkn ejnΔϕk ,	(7.6-60)

wherein a0,k is a complex valued start parameter equal to the value of the first CLDFB sample (i.e., n=0) in that frame, κk is a by-sample magnitude evolution parameter, and Δϕk is a per-sample phase evolution parameter. This leads to the following recursive formulation:

xkn=xkn-1 κk ejΔϕk, n>1	(7.6-61)

with x0=a0.

To account for the real-world case that that the assumed sinusoid is not a clean sinusoid with strict by-sample phase evolution by Δϕk, additive phase perturbation term is δϕk(n) is used to model random phase deviations.

Based on these model assumptions, the CLDFB samples of the given band k of the substitution frame to be generated ykn, n=0… L is obtained as

ykn=bknyk(n-1) ,	(7.6-62)

where y-1,k is a complex start value and

bkn=κkejΔϕk+δϕk(n)  	(7.6-63)

In the sections below, it is described how y-1,k, Δϕk, δϕk(n) and κk are calculated from the frame preceding the frame to be substituted.

Linear predictive model

The other model available for substitution of the CLDFB samples of the given CLDFB band k is a first-order linear predictor combined with repetition of the same-band CLDFB samples of the preceding frame. It is especially useful for noise-like signals potentially with weak tonal components.

The first-order predictor is expressed by the following equation:

xkn=γ1,kxkn-1  ,	(7.6-64)

wherein γ1,k is a complex coefficient.

Together with the CLDFB samples of the preceding frame xkn, n=0…L-1, this leads to the following model for the generation of a given CLDFB band k of the substitution frame:

ykn= y-1,k γ1,kn+1+xkn(1-γ1,kn+1).	(7.6-65)

The factor (1-γ1,kn+1) is a complementary weighting factor for the decreasing contribution of the linear predictor, whereby it is assumed that the magnitude of the predictor is less than 1. This is a cross-fade approach facilitating a smooth transition from the preceding frame to the substituted frame. Signal discontinuities often observed in frame-repetitive methods are effectively avoided with this technique.

In the sections below, it is described how y-1,k and γ1,k are calculated from the frame preceding the frame to be substituted.

##### 7.6.4.4.3	Analysis and parameter estimation

To calculate the phase parameter Δϕk, the phase perturbation parameter sequence δϕk(n), the magnitude evolution parameter κ and input parameters for the tonality estimator, the CLDFB samples of the given band k of the preceding frame are first converted to magnitude/phase representation:

xkn→ρkn,θkn, n=0…L-1 . 	(7.6-66)

The phases are obtained using the atan2 function, which returns wrapped phase values. These phase values are subsequently unwrapped, yielding a sequence of unwrapped phases θkn.  After that, per-sample phase differences are calculated according to

Δθkn=θkn+1-θkn,  n=0…L-2 . 	(7.6-67)

After that the mean value Δϕk and standard deviation σδϕ,k of the by-sample phase differences is calculated for the sequence Δθkn. The latter will be used as an input parameter of the tonality estimator. A sequence of phase perturbation parameters is calculated by subtracting the mean value Δϕk from the sequence of per-sample phase differences:

δϕn=    Δθn-Δϕk ,    n=0…L-2  Δθn-L+1-Δϕk ,    L-1≤n≤ L+1     . 	(7.6-68)

Note: Phase perturbation parameters for L-1<n≤ L+1 are calculated for application of the sinusoidal model in a cross-fade region spanning 2 time slots length following the current substitution frame.

The magnitude evolution parameter κk is calculated in a similar manner. This parameter is calculated as mean over the sequence of L-1 per-sample magnitude ratios ρk(n+1)ρk(n), n=0…L-2, of the preceding frame. To avoid modelling exponentially increasing magnitudes, κk is limited to 1 as maximum value.

In addition, the standard deviation σκ,k of that sequence is calculated to be used as a further input parameter of the tonality estimator.

The start value for the sinusoidal model y-1,k for a given CLDFB band k is subsequently calculated as the average of the last CLDFB sample of the preceding frame and the second last CLDFB sample with applied sinusoidal model, i.e.,

y-1,k=12κkejΔϕkxkL-2+xk(L-1) . 	(7.6-69)

The averaging yields a smoothed start value where potential statistical variations are reduced.

The start value for the substitution with the linear predictive model is the last CLDFB sample of the preceding frame, i.e.,

y-1,k=xk(L-1) . 	(7.6-70)

The complex predictor coefficient of the first-order linear predictive model is calculated following the Levinson-Durbin method based on the autocorrelation sequence rxx,kn , n=0,1 of the (rectangularly windowed) CLDFB band samples xk(n) of the preceding frame. The first-order predictor coefficient is obtained as:

γ1,k=rxx,k(1)rxx,k(0)    .	(7.6-71)

##### 7.6.4.4.4	Tonality determination

Tonality determination is made separately for each CLDFB band k based on the corresponding standard deviation values σδϕ,k and σκ,k. A CLDFB band of the preceding frame is declared tonal in case  σδϕ,k<π/6 or σκ,k<0.1. Otherwise, it is declared noise-like.

##### 7.6.4.4.5	Sinusoidal extension

In case a CLDFB band k of the preceding frame is tonal, the sinusoidal model is used to generate the corresponding band of the substitution frame.

Using the start value y-1,k according to equation 7.6-69 and model parameters Δϕk, δϕk(n) and κk for the given CLDFB band k, the substitution frame band is generated according to equations 7.6-62 and 7.6-63.

##### 7.6.4.4.6	Predictive extension

In case a CLDFB band of the preceding frame is noise-like, the linear predictive model is used to generate the corresponding band k of the substitution frame.

Using the start value y-1,k according to equation 7.6-70, the complex first-order coefficient γ1,k for the given CLDFB band k and the CLDFB band samples of the previous frame xkn , n=0...L-1 , the substitution frame band is generated according to equation 7.6-65.

##### 7.6.4.4.7	Cross-fade

Cross-fade is used to avoid signal discontinuities at the boundary from a substituted frame to the next frame. The model-based technique described above ensures that such discontinuities to a subsequent substitution frame do not occur. However, when a good frame (marked with BFI flag=0) follows a bad frame, a special cross-fading technique is applied. For that case, during substitution processing of the bad frame, cross-fade to a potential good frame following is prepared as follows:

In case of band k determined tonal, two extra CLDFB band samples are calculated beyond the end of the current frame according to the method described in clause 7.6.4.4.5 and equations 7.6-62 and 7.6-63 for sample indexes n=L and L+1. These samples are multiplied with fading weights of 2/3 for ykL and, respectively, 1/3 for ykL+1 and stored instead of the original samples. The respective complementary weights of 1/3 and 2/3 are stored in a buffer for frame onset weights βkn, n=0, 1 for each CLDFB band k.

In case of band k determined noise-like, two extra CLDFB band samples are calculated beyond the end of the current frame according to the method described in clause 7.6.4.4.6 and equation 7.6-65 for sample indexes n=L and L+1. These samples are fading out exponentially with the magnitude of the predictor coefficient γ1,k. Accordingly, no specific additional fading weight is applied. The respective complementary weights of 1-γ1,k and, 1-γ1,k2 are stored in the buffer for frame onset weights βn, n=0, 1 for each CLDFB band.

Upon the reception of a good frame following a bad frame, the first two samples (in each CLDFB band) are replaced by cross-faded samples while the other (later) samples are retained. This is illustrated by the following equation that is carried out for each CLDFB band k:

xkn=xknβkn+ykL+n , n=0, 1            xkn                    ,n≥2 .	(7.6-72)

##### 7.6.4.4.8	Burst-loss handling

In case of burst frame loss with more than 10 frames in a row, the substitution frames generated according to the above-described technique are increasingly attenuated with 3 dB per additional bad frame. After 30 bad frames in a row, the substitution frames are completely zeroed.

### 7.6.5	LC3plus coded intermediate split renderer binaural audio format

#### 7.6.5.1	Introduction (Informative)

The Low Complexity Communication Codec plus (LC3plus) [X1] has been designed as the low complexity counterpart of EVS [REF] in order to make SWB also available on computationally constrained terminals such as VoIP, DECT [X3] or wireless headsets. The codec allows perfect interoperability between mobile and other networks by means of transcoding [X2][X3] and fits complexity wise very well to the requirements of low power embedded devices. Due to the codec's flexible design the applications are not limited to voice services but can be extended to high quality music streaming as well [X2]. The LC3plus codec can also be configured to be interoperable with LC3 coding as used in the Basic Audio Profile for Bluetooth Low Energy audio [X4].

The application of LC3plus for coding the intermediate split renderer binaural audio format is described in the following clauses.

#### 7.6.5.2	Overview

The LC3plus specification includes a full algorithmic description of both the encoder and decoder. It includes reference fixed-point and floating-point ANSI C source code (clauses 4.3 and 6 of [X1]) and conformance test procedures (clause 4.4 of [X1]). The entire codec as specified in [X1] is included in the present document.

The codec can operate at highly flexible modes, including operation at multiple bandwidths, multiple frame durations and at any byte aligned bit rate (clause 5.1 of [X1]). It includes the support of a high-performance Packet Loss Concealment (PLC) (see clause 5.6 of [X1]) for all kinds of audio signals. LC3plus comes with an RTP Payload Format for transmission over IP/UDP, defined in Annex B of [X1].

The following clauses provide additional details and information specific to the application as split renderer binaural audio format.

#### 7.6.5.3	Encoder

The encoding process for LC3plus follows the procedures of clause 5.3 of [X1]. To make full use of the available bits remaining after metadata encoding (clause 7.6.3), the LC3plus encoder may change the size of a compressed audio frame in a seamless manner, following the procedures of clause 5.7 of [X1].

#### 7.6.5.4	Decoder

The decoding process for LC3plus follows the procedures of clause 5.4 of [X1].

#### 7.6.5.5	Frame Structure

The frame structure of LC3plus follows the procedures of clause 5.5 of [X1]. Multiple LC3plus 5ms or 10ms frame data blocks are used for the split renderer binaural audio format, following the procedures of Annex B using the payload structure as defined in B.2.4 of [X1].

#### 7.6.5.6	Packet Loss Concealment

The packet loss concealment for LC3plus follows the procedures of clause 5.6 of [X1].

#### 7.6.5.7	LC3 interoperable mode

The LC3plus codec can also be configured to be interoperable with LC3 coding as used in the Basic Audio Profile for Bluetooth Low Energy audio using the configurations 48_2, 48_4 and 48_6 as described in Table 3.5 of [X4]. In these configurations the bandwidth is FB, high resolution mode is disabled, error protection is disabled, and the frame duration is 10 ms.

To generate 48_6 compatible LC3plus frames for the  256000 bps split rendering configuration the bit allocation as as defined in Table 7.6-24 shall be followed. The Frame Data Length Request (FDLR) (see clause B.2.5 of [X1]) inside the LC3plus Payload Header shall use FDLR1 (1 octet) only and indicate NO_REQ.

NOTE:	For this configuration, the overhead of the LC3plus payload format amounts to 40 bits per split rendering frame. The LC3plus frame data block is followed by 40 bits of zero padding, to fill up the 2560 bits of the split rendering frame. This leaves 2480 bits for the LC3plus frame data block, which results in 1240 bits per LC3plus frame data. This amounts to an LC3plus per channel bit rate of 124000 bps, which matches the 48_6 configuration defined in BAP Table 3.5.

### 7.6.6	Split post-rendering

#### 7.6.6.1	Overview

The split rendering post-renderer performs lightweight audio processing with either pose correction (1-3 DOF) around multiple rotation axes or direct decoding (0 DOF). The post-renderer receives the coded bitstream b2 from the pre-renderer as shown in Figure 7.6-1 and 7.6-2. The bitstream includes LCLD, LC3plus or PCM audio coded bits and coded metadata bits for pose correction (1-3 DOF). The audio coded bits are decoded to obtain reference binaural signal associated with reference head pose P'. The LCLD decoder outputs a reference binaural signal in CLDFB domain, whereas LC3plus or PCM outputs are in the time domain.

If pose correction metadata was transmitted, time domain outputs are converted to CLDFB domain by applying CLDFB analysis. Subsequently the metadata is decoded as in clause 7.6.6.2, and then post rendering is performed, depending on DOF either as described in clause 7.6.6.2 or 7.6.6.3.

If a backchannel exists from post-renderer to pre-renderer, the latest head pose P is encoded and transmitted to the pre-renderer.

#### 7.6.6.2	Post rendering with pose correction

##### 7.6.6.2.1	Metadata decoding

The metadata bits are decoded (as described in clause 7.6.3.4) to obtain the CLDFB banded metadata that enables the reconstruction of the set of binaural signals (BINn) associated with the set of probing poses (Pn) from the reference binaural signal. The metadata associated with the set of probing poses (Pn) represents deviation from the reference head pose (P') by rotation around the multiple rotational axes.

The reference head pose P' is also decoded as part of metadata decoding, as described in clause 7.6.3.4. The probing poses (Pn) are then computed by adding the offsets given in Table 7.6-1 to P'. The post-render obtains the latest head pose (P) associated with the user. Then axis-specific metadata for each rotation axis is computed by selecting the probing pose that is closest to pose P along the given rotation axis and performing interpolation (or extrapolation) on the metadata associated with the selected probing pose.  The interpolation is done based on the difference between the latest head pose P and reference head pose P' along the given rotation axis.

The interpolated axis-specific metadata for each rotation axis is combined into one 2x2 pose correction matrix per frequency band. The combined matrix is applied to the reference binaural signal to obtain the output binaural signal corresponding to the latest head pose P. The time domain output is generated by performing CLDFB synthesis on the CLDFB domain output binaural signal.

The metadata bits in bitstream b2 that correspond to roll axis or yaw axis (if the ILD flag was enabled at the pre-renderer) are decoded to obtain a complex-valued matrix MPs in low frequency bands (0 to 1600 Hz) that enables reconstruction of both the magnitude and phase of the set of binaural signals (BINn) from the reference binaural signal, and a real valued diagonal matrix DPs in high frequency bands that enables reconstruction of only the magnitude of the set of binaural signals (BINn) from the reference binaural signal. The output binaural signal in the low frequency bands is generated by applying the interpolated or extrapolated matrix MPs to the corresponding lowfrequency bands of the reference binaural signal. Similarly, the output binaural signal in the high frequency bands is generated by applying the interpolated or extrapolated real-valued diagonal matrix DPs to the corresponding high frequency bands of the reference binaural signal. Here, the interpolation or extrapolation is done based on the difference between the latest head pose P and reference head pose P' along the given rotation axis, as described in clause 7.6.6.2.2. The output binaural signal is generated by combining the output binaural signal in low frequency bands and high frequency bands.

#### 7.6.6.2.2	Metadata interpolation or extrapolation

The metadata interpolation is done using linear interpolation method and the same approach is used for every rotation axis. First, the probing pose that is closest to the latest head pose P along a given rotation axis is selected. Then metadata parameters are interpolated as follows:

Mp= j=0j=1ajMpj ,

(7.6-21)

where Mp0 is an identity matrix, Mp1 is the metadata matrix corresponding to the closest probing pose P1 and aj are interpolation coefficients as given below:

a0=1-doffset, a1=doffset  ,

where offset is the deviation between the closest probing pose P1 and reference pose P’ along the said rotation axis such that offset=P1-P', d is the deviation between the latest head pose P and reference pose P’ along the said rotation axis such that d=P-P'.

When absolute value of d is greater than absolute value of offset, then extrapolation is performed. In this case, Mp1 is an identity matrix, Mp0 is the metadata matrix corresponding to the closest probing pose P0 and aj are extrapolation coefficients as given below:

a0=1-sin⁡(d)sin⁡(offset), a1=sin⁡(d)sin⁡(offset)  ,

where offset is the deviation between the closest probing pose P0 and reference pose P’ along the said rotation axis such that offset=P'- P0, d is the deviation between the latest head pose P and closest probing pose P0 along the said rotation axis such that d=P0-P.

#### 7.6.6.2.3	Matrix mixing

The interpolated or extrapolated matrices Myaw, p, Mroll,p and Hpitch,p are then combined into one 2x2 pose correction matrix per frequency band as shown below:

M[2x2]= Mroll,p Hpitch,p Myaw,p .

(7.6-22)

The output binaural signal in each CLDFB band is computed as

BIN[2x1]out= M[2x2] BIN[2x1]ref  ,

(7.6-23)

where, BIN[2x1]ref is the reference binaural signal corresponding to reference head pose P’.

#### 7.6.6.3	Post rendering in 0 DOF mode

In 0 DOF mode or no pose correction mode, the split rendering bitstream received by the post-renderer includes LCLD or LC3plus coded bits which are decoded to obtain the reference binaural signal associated with the reference head pose P'. The LC3plus decoder outputs in time domain, whereas the LCLD decoder outputs reference binaural signal in CLDFB domain which is then converted to time domain by applying CLDFB synthesis. The time domain reference binaural signal is the output of the post-renderer.

### 7.6.7	Bit allocation for Split rendering

Detailed bit allocation principles at different bitrates of the Split rendering operation are provided in Table 7.6-21, Table 7.6-22, Table 7.6-23, Table 7.6-24, Table 7.6-25 and Table 7.6-26.

Table 7.6-21: Bit allocation for split rendering with 1, 2 and 3 DOF pose correction with LCLD or LC3plus codecs

| Description | 384 kbps | 512 kbps | 768 kbps |
| --- | --- | --- | --- |
| Total number of bits per 20ms frame | 7680 | 10240 | 15360 |
| DOF (degree of freedom) | 2 | 2 | 2 |
| HQ mode | 1 | 1 | 1 |
| Rotation axes | 2 (if 1, 2 DoF), 0 otherwise | 2 (if 1, 2 DoF), 0 otherwise | 2 (if 1, 2 DoF), 0 otherwise |
| HF ILD flag | 1 | 1 | 1 |
| Reference Pose P’ (yaw, pitch and roll angle) | 27 | 27 | 27 |
| Coding strategy | 1 | 1 | 1 |
| Quantization strategy | 2 | 2 | 2 |
| Metadata bits | Variable | variable | variable |
| LCLD or LC3plus bits | Variable | variable | variable |

In case of LC3plus, LC3plus bits are prepended with byte-alignment 0-bits to the next closest octet boundary. If the number of bits available for audio coding exceeds the maximum frame size of LC3plus, the leftover bytes in the LC3plus section are filled up with padding 0-bytes.

Table 7.6-22: Bit allocation for split rendering with 0 DOF (no pose correction) with 5ms split rendering frame size

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 5ms frame | 1280 | 1920 | 2560 |
| LCLD or LC3plus bits | 1280 | 1920 | 2560 |

Table 7.6-23: Bit allocation for split rendering with 0 DOF (no pose correction) with 10ms split rendering frame size using LCLD

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 10ms frame | 2560 | 3840 | 5120 |
| LCLD | 2560 | 3840 | 5120 |

Table 7.6-24: Bit allocation for split rendering with 0 DOF (no pose correction) with 10ms split rendering frame size using LC3plus

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 10ms frame | 2560 | 3840 | 5120 |
| LC3plus bits | 2520 | 3840 | 5120 |
| Zero bits | 40 | 0 | 0 |

Table 7.6-25: Bit allocation for split rendering with 0 DOF (no pose correction) with 20ms split rendering frame size

| Description | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- |
| Total number of bits per 20ms frame | 5120 | 7680 | 10240 |
| LCLD or LC3plus bits | 5120 | 7680 | 10240 |

Table 7.6-26: Bit allocation for split rendering with 1, 2 and 3 DOF pose correction with PCM binaural output

| Description | Bitrate >= 34 kbps |
| --- | --- |
| Total number of bits per 20ms frame | >= 680 |
| DOF (degree of freedom) | 2 |
| HQ mode | 1 |
| Rotation axes | 2 (if 1, 2 DoF), 0 otherwise |
| HF ILD flag | 1 |
| Reference Pose P’ (yaw, pitch and roll angle) | 27 |
| Coding strategy | 1 |
| Quantization strategy | 2 |
| Metadata bits | Remaining bits |

### 7.6.8	Interface for Split rendering

Split renderer and its interface provide support to ISAR codec design constraints. The details of the split rendering library API are provided in [7] for the fixed-point code and [12] for the floating-point code. The details of the split rendering library API are provided in [9].

* * * End of Changes * * * *