3GPP TSG-SA WG4 Meeting #127bis-e	S4-240703

# Online, 8-12 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.255 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding ISAR track-a split rendering feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Fraunhofer IIS, Ericsson LM, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ISAR, IVAS_codec |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The split rendering feature enabled with this CR was selected by SA4 according to the ISAR selection procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The feature is introduced in section 4 and described in more detail in a new section 8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IVAS codec will not offer a split rendering feature and potentially not be available on lightweight end devices. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, (new) 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.250 (pseudo-CR) TS 26.253 CR 00xx TS 26.254 CR 00xx TS 26.258 CR 00xx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[3]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error Concealment of Lost Packets".

[4]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services - Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions".

[5] 	3GPP TS 26.249: " Immersive Audio for Split Rendering Scenarios; Detailed Algorithmic Description of Split Rendering Functions".

* * * Next Change * * * *

# 4	General

Packet loss concealment serves to ensure the availability of useful audio output when valid packets are unavailable to the decoder. These losses are typically a result of impaired channel conditions like transmission errors or network congestion. The aim is to synthesize a substitution of the decoded audio represented by the lost packet, to prepare for a potential future packet loss, and to handle the transition from the concealment operation back to the decoded audio. The latter is also referred to as recovery operation. An overview of the IVAS codec’s decoder operation is given in clause 6.1 of [4], where Figure 6.1-1 shows the functional structure of the decoder. To complement the picture with the packet loss concealment functionality, Figure 1 below shows the packet loss concealment (PLC) operation of the decoder. A major part of the PLC resides in the core decoding tools, where the audio decoding is mainly handled by the core decoder based on EVS [2]. The Single Channel Elements (SCE) decoder comprises one core-decoder, the Channel Pair Elements (CPE) comprises one or two core-decoders and the Multichannel Coding Tool (MCT) comprises joint decoding using multiple core-decoders, all including associated PLC methods. For the Low Frequency Effect (LFE) channel of multichannel audio, an LFE decoder with associated PLC method is available. Spatial metadata including spatial coding parameters are reconstructed in the spatial parameter decoders or by the associated PLC methods of the respective spatial audio formats. Spatial audio output is finally generated by a scene decoder, upmixer and renderer based on the reconstructed transport channels and the reconstructed spatial metadata. In case of a missing or corrupted packet, a bad frame indicator (BFI) is input to the decoding tools, activating the PLC operation. Notably, scene decoding, upmixing and rendering processing are independent of a bad frame indicator.

The IVAS split rendering feature exposes a further interface, the interface between the entity carrying out pre-rendering and encoding into the intermediate audio representation and the end-device doing post-rendering. To cope with potential transmission errors on that interface, the decoder of the intermediate audio representation features packet loss concealment techniques besides the actual decoding scheme. Note that the IVAS specific split rendering functionality including packet-loss concealment is mostly described in TS 26.253 [4] whereas more generic split rendering functionality is specified in TS 26.249 [5].

Figure 1: Overview of error concealment operation

* * * Next Change * * * *

# 8	Error concealment for IVAS split rendering

The intermediate audio format of the IVAS split rendering feature comprises coded pose correction metadata and coded binaural audio. The binaural audio may be encoded using the LCLD coding format or the LC3plus coding format. The respective PLC schemes are described in [4] as follows:

- PLC for pose correction metadata is described in [4], clause 7.6.3.5.

- PLC for LCLD binaural audio coding is described in [4], clause 7.6.4.4.

- PLC for LC3plus binaural audio coding is described in [4], clause 7.6.4.6.

Note: In case binaural audio is transmitted using the PCM interface, no PLC method is provided. An implementor needs to make sure that suitable corresponding methods are available.

* * * End of Changes * * * *