3GPP TSG-SA WG6 Meeting #60	draft2 S6-241435

Changsha, China 15th – 19th April 2024	(revision of S6-241053)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0205 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registering the specific role in Application Server Registration procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, the group messaging procedure in TS23.554 involves the Application Server before the group message is sent to other participants of the group. It is proposed to classify this kind of Application as Controlling AS which has specific role in the group messaging. This CR is proposed to add the procedure of registering the specific role of Application Server in the Application Server Registration procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to add the procedure of registering the specific role of Application Server in the Application Server Registration procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Controlling AS related service logic of group messaging is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5, 8.2.6, 9.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: removed the changes on clause 9.1.2.4. New NOTEs are added to clause 8.2.5 and Table 9.1.2.3-1. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.5	Application Server Registration

The signalling flow for Application Server registration is illustrated in figure 8.2.5-1. Application Server may use the procedure in this clause to do registration.

NOTE:	If the Application Server does not use the Registration procedure, applicable Information Elements as listed in Table 9.1.2.3-1 need to be configured on the MSGin5G Server.

Pre-conditions:

1.	The Application Server has connected to the serving network successfully.

2.	An AS Service ID has been provisioned on the Application Server.

3.	The MSGin5G Server address has been provisioned on the Application Server.

4.	Both the Application Server and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.5-1: Application Server registration

1.	The Application Server sends an Application Server registration request to the MSGin5G Server. The request may include authorization information for the Application Server to register to the MSGin5G Server. The request includes the AS Service ID and may include Application Profile information as detailed in Table 9.1.2.3-1.

2.	Upon receiving the request, the MSGin5G Server validates the Application Server registration request and may verify the security credentials.

NOTE 1:	The authentication procedures in step 2 are built on top of the transport layer mechanism specified in Annex Y.4 of 3GPP TS 33.501 [16].

3.	The MSGin5G Server sends an Application Server registration response to the Application Server. The response includes the information elements as specified in Table 9.1.2.4-1. If the registration is successful, the MSGin5G Server stores the AS Profile information and Expected MSGin5G group list as detailed in Table 9.1.2.3-1.

NOTE 2:	The Application Server needs to register to all the MSGin5G Servers hosting the MSGin5G group that the Application Server is expected to control. The Application Server finds the corresponding MSGin5G Server based on the Group Service ID.

8.2.6	Application Server De-registration

By de-registering, the Application Server informs the MSGin5G Server that it wishes to terminate its association with the MSGin5G Server.

The procedure assumes that the Application Server is responsible for triggering the de-registration from the MSGin5G Server. The signalling flow for Application Server de-registration is illustrated in figure 8.2.6-1.

Pre-conditions:

1.	The Application Server is registered to the MSGin5G Server.

Figure 8.2.6-1: Application Server de-registration

1.	The Application Server determines to de-register from the MSGin5G Server.

2.	The Application Server sends an Application Server de-registration request to the MSGin5G Server that includes the AS Service ID, as detailed in Table 9.1.2.5-1.

3.	The MSGin5G Server validates the Application Server de-registration request. If the MSGin5G Server has authorized the Application Server successfully, it deletes any AS Profile information and Expected MSGin5G group list that it has stored.

NOTE:	The authentication procedures in step 3 are built on top of the transport layer mechanism specified in Annex Y.4 of 3GPP TS 33.501 [16].

4.	The MSGin5G Server replies with an Application Server de-registration response as detailed in Table 9.1.2.6-1.

* * * Next Change * * * *

9.1.2.3	M5S Application Server registration request

The information flows from the Application Server to the MSGin5G Server for registration request includes the information elements in Table 9.1.2.3-1.

Table 9.1.2.3-1: Application Server Registration request

| Information element | Status | Description |
| --- | --- | --- |
| AS service ID | M | The MSGin5G identifier of the Application Server. This ID is configured before registration. |
| Application ID | O | The identifier of the application specified by the application provider. |
| Authorization Information | O | The authorization information used to determine whether the Application Server is allowed to send the message |
| Notification target URI | O | The URL for receiving message, message delivery status report. The MSGin5G Server uses this URL to interact to Application Server. |
| Application Profile (NOTE 1) | O | The elements in Application Profile include the information of the Application Server, e.g. application name, application provider, application scenario description, application category. This IE is used by MSGin5G Server to compare with application client information. |
| Expected MSGin5G group list (NOTE 2) | O | A list of MSGin5G group which the Application Server will control, i.e. acts as a Controlling AS in these MSGin5G groups. |
| NOTE 1: The detailed definition of Application Profile is out of scope of this document. NOTE 2: The Application Server acts as VAL Server and gets the MSGin5G Group Service ID from the SEAL Group Management Server. |  |  |