3GPP TSG-SA WG6 Meeting #60	S6-241270

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0626 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS information update in ECS-ER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a common EAS is relocated due to maintenance reason (e.g. graceful shutdown) or overload reason, which can be detected by S-EAS or S-EES, a T-EAS discovery procedure is triggered by either S-EAS or S-EES. With ECS-ER deployed, the ECS-ER stored common EAS information needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add update procedure for common EAS storage in ECS-ER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Outdated info remains in ECS-ER which leads to incorrect common EAS being returned to consumer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Existing clause(s): 8.20.4.1 New clause(s): 8.20.2.x, 8.20.2.y, 8.20.3.y, 8.20.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.20.2.x	Common EAS information update

Figure 8.20.2.x-1: Common EAS information update

1.	The EES sends Common EAS information update request message to the ECS-ER. The request message includes EDN information, EES ID, EAS ID, EAS endpoint and Application Group ID.

2.	Upon receiving the request, the ECS-ER checks if there is any stored EAS serving the application group for the EAS ID within the same EDN. If existing common EAS is found, the ECS-ER updates the EAS endpoint information for subsequent binding; otherwise, the ECS-ER rejects the EES request in step 3.

3.	The ECS-ER responds the EES with Common EAS information update response message.

* * * Next Change * * * *

#### 8.20.3.x	Common EAS information update request

Table 8.20.3.x-1 describes the information elements for common EAS information update request from the EES to the ECS-ER.

Table 8.20.3.4-1: Common EAS information update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS ID | M | The identifier of the EAS. |
| EAS endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. |
| Application Group ID | M | Application group identifier as defined in clause 7.2.11. |
| EDN information | M | Information of EDN where the EAS resides. |
| > DNN | M | Data network name to identify the EDN. |
| > DNAI(s) | O | DNAI(s) associated with the EDN. |

#### 8.20.3.y	Common EAS information update response

Table 8.20.3.y-1 describes the information elements for common EAS information update response from the ECS-ER to the EES.

Table 8.20.3.y-1: Common EAS information update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. Only included when Failure response is included. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

#### 8.20.4.1	General

Table 8.20.4.1-1 illustrates the API for EAS information management.

Table 8.20.4.1-1: EAS information management APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_EASInfoManagement | Store | Request/Response | EES |
|  | Update | Request/Response | EES |
|  | Remove | Request/Response | EES |
|  | Get | Request/Response | EES |

* * * Next Change * * * *

#### 8.20.4.x	Eecs_EASInfoManagement_Update operation

API operation name: Eecs_EASInfoManagement_Update

Description: The consumer requests EAS information update service from the ECS.

Inputs: See clause 8.20.3.x.

Outputs: See clause 8.20.3.y.

See clause 8.20.2.x for details of usage of this operation.

* * * END of Change * * * *