3GPP TSG-SA WG6 Meeting #60	S6-241440

Changsha, China 15th – 19th April 2024	(revision of S6-241434)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0054 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assist selecting UAV flight path based on QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG, InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel19 3GPP SA1 work (UAV_ph3) contains the following requirement: [R-6.8-002] Based on a 3rd party request, the 5G system shall be able to assist the UTM with mechanism to provide to the 3rd party alternative UAV flight paths, e.g., based on required waypoints, QoS, and exclusion zones. The UAS layer can utilize various network layer statistics to find the optimal path based on VAL service requirements (e.g., QoS) and assist the UTM/UAV-C in proposing a flight route.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to define new UAS functionality on chosing and proposing a flight route to UTM/UAV-C with utilizing network statistics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SA1 requirement would not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-241434, S6-241037 |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 7.x	UAS provided flight routes

### 7.x.1	General

This feature enables the UAE server to provide possible fight path based on specific QoS requested from UTM or UAV. In particular, the UAE layer provides support for the flight route selection.

### 7.x.2	Procedures

#### 7.x.2.1	Flight route support

Figure 7.x.2.1-1 describes the flight route support procedure.

Pre-conditions:

-	UAV has performed the UAS UE registration procedure.

Figure 7.x.2.1-1: Flight route calculation procedure

1.	The UAS application specific server or UAE client (UAV or UAV-C) sends flight route request towards the UAE server. This request contains network parameters needed for the specific needs of the specific use case (start point, destination point, specific times of start point and end point, needed QoS, link reliability, shortest route needed, etc).

2.	The UAE server has the role of a route planning unit and based on the received end points and provisioned information (e.g., restricted areas, existing flight paths, available base stations coverage, shortest distance, shortest time duration) calculates possible waypoints as geographical coordinates and specific times.

3.	The UAE server checks with NWDAF (via NEF) the QoS sustainability analytics (section 6.9 of TS 23.288) in these areas of interest/waypoints identified in step 2. Location information towards the NWDAF is presented as area of interest or path of interest and can reflect a list of waypoints. Based on the received information from NWDAF the UAE server selects waypoints (geographical coordinates and time) which KPIs map the requested criteria in step1.

4.	The UAE server sends flight route reply with the selected waypoints in step 3 to the entity which has send the request (UTM, UAV or UAV-C).

### 7.x.3	Information flows

#### 7.x.3.1	Flight route request

Table 7.x.3.1-1 describes the information flow for the UAS application specific server or UAE client to request flight route from the UAE server.

Table 7.x.3.1-1: Flight route request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the flight route. This ID can be the USS identifier, when the UAS application specific server is the USS. |
| UAV ID | M | The identifier of the UAV/UAV-C (e.g. 3GPP UE ID or CAA level UAV ID) which requests the flight route. |
| Start point | M | Geographical coordinates of starting point |
| Start time | M | Time on which the UAV is at the starting point |
| Destination point | M | Geographical coordinates of destination point |
| Destination time | M | Time on which the UAV is at the destination point |
| Required minimum QoS | M | The needed QoS to support the flight mission. |
| Service availability | O | Percentage of needed service availability |
| Shortest route indicator | O | Indicator for a need of shortest route |

#### 7.x.3.2	Flight route reply

Table 7.x.3.2-1 describes the information flow for the UAS application specific server to reply with flight route the UAS application specific server or UAE client.

Table 7.x.3.2-1: Flight route response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the UAE server in response to flight route request indicating success or failure |
| List of waypoints (NOTE) | O | List of waypoints containing geographical coordinates and time for each waypoint |
| NOTE: If Result IE indicates success, then List of waypoints IE provided by UAE server is present |  |  |

* * * End of Changes * * *