3GPP TSG-SA5 Meeting #155 	S5-242520

Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0064 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.313 correction to the inconsistance statments of handover parameters for MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 28.541, handover trigger parameters including maximumDeviationHoTriggerLow and maximumDeviationHoTriggerHigh , the referenced attribute of “maximumDeviationHoTrigger” is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the referenced attribute align with TS 28.541. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would be unclear for the consumer to control the hanover parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.3, 7.1.5.2.2, 7.2.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

7.1.2	MRO (Mobility Robustness Optimisation)

7.1.2.1	MnS component type A

Table 7.1.2.1-1: MRO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

7.1.2.2	MnS Component Type B definition

7.1.2.2.1	Targets information

The targets of MRO are shown in the Table 7.1.2.2.1-1.

Table 7.1.2.2.1-1: MRO targets

| Target Name | Definition | Legal Values |
| --- | --- | --- |
| Total handover failure rate | (the number of failure events related to handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total intra-RAT handover failure rate | (the number of failure events related to intra-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total inter-RAT handover failure rate | (the number of failure events related to inter-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |

7.1.2.2.2	Control information

The parameter is used to control the MRO function.

Table 7.1.2.2.2-1: MRO control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| MRO function control | This attribute allows the operator to enable/disable the MRO functionality. See attribute dmroControl in TS 28.541 [13]. | Boolean On, off |

7.1.2.2.3	Parameters to be updated

Table 7.1.2.2.3-1: Ranges of handover parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger Low | This parameter defines the maximum allowed lower deviation of the Handover Trigger from the default point of operation (see clause 15.5.2.5 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerLow in TS 28.541 [13]. The upper and lower bounds of this parameter are configured separately. | [-20..20] in unit 0.5 dB |
| Maximum deviation of Handover Trigger High | This parameter defines the maximum allowed upper deviation of the Handover Trigger from the default point of operation (see clause 15.5.2.5 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerHigh in TS 28.541 [13]. The upper and lower bounds of this parameter are configured separately. | [-20..20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.2.5 in TS 38.300 [7]). See attribute minimumTimeBetweenHoTriggerChange in TS 28.541 [13]. | [0.. 604800] in unit Seconds |
| Tstore_UE_cntxt | The timer used for detection of too early HO, too late HO and HO to wrong cell. Corresponds to Tstore_UE_cntxt timer described in clause 15.5.2.5 in TS 38.300 [7]. See attribute tstoreUEcntxt in TS 28.541 [13]. | [0..1023] in unit 100 milliseconds |

Next change

7.1.5	LBO (Load Balancing Optimisation)

7.1.5.1	MnS component type A

Table 7.1.5.1-1: D-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 5 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

7.1.5.2	MnS Component Type B definition

7.1.5.2.1	Control information

The parameter is used to control the LBO function.

Table 7.1.5.2.1-1: D-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| D-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. See attribute dlboControl in TS 28.541 [13]. | Boolean On, off |

7.1.5.2.2	Parameters to be updated

Table 7.1.5.2.2-1: Ranges of HO and cell selection parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger Low | This parameter defines the maximum allowed lower deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerLow in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Maximum deviation of Handover Trigger High | This parameter defines the maximum allowed upper deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerHigh in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.1.4 in TS 38.300 [7]). See attribute minimumTimeBetweenHoTriggerChange in TS 28.541 [13]. | [0 .. 604800] in unit Seconds |

Next change

7.2.4	LBO (Load Balancing Optimisation)

7.2.4.1	MnS component type A

Table 7.2.4.1-1: C-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation --- modifyMOIAttributes operation - - deleteMOI operation - - notifyMOIAttributeValueChanges - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

7.2.4.2	MnS Component Type B definition

7.2.4.2.1	Parameters to be updated

Table 7.2.4.2.1-1: Ranges of HO and cell selection parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger Low | This parameter defines the maximum allowed lower deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerLow in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Maximum deviation of Handover Trigger High | This parameter defines the maximum allowed upper deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerHigh in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.2.5 in TS 38.300 [7]). See attribute minimumTimeBetweenHoTriggerChange in TS 28.541 [13]. | [0 .. 604800] in unit Seconds |

End of change