3GPP TSG-SA5 Meeting #155 	S5-242523

Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0067 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.313 Add missing attributes to the LBOFunction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to TS 28.541, the attribute isMLBallowed has been defined to indicates if mobility load balancing is allowed or prohibited from source cell to target cell. To ensure consistency, this attibute should be used as the updated parameter in the LBO case. - The description of D-LBO procedure is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Table 7.1.5.2.2-X in clause 7.1.5.2.2 - Remove Underline of the sentence “collects real-time load information to determine and” in clause 8.2.4 - Add clarification description in step1 and step3 in clause 8.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MLB Function would not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.5.2.2, 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

7.1.5	LBO (Load Balancing Optimisation)

7.1.5.1	MnS component type A

Table 7.1.5.1-1: D-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 5 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

7.1.5.2	MnS Component Type B definition

7.1.5.2.1	Control information

The parameter is used to control the LBO function.

Table 7.1.5.2.1-1: D-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| D-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. See attribute dlboControl in TS 28.541 [13]. | Boolean On, off |

7.1.5.2.2	Parameters to be updated

Table 7.1.5.2.2-1: Ranges of HO and cell selection parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger | This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTrigger in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.1.4 in TS 38.300 [7]). See attribute minimumTimeBetweenHoTriggerChange in TS 28.541 [13]. | [0 .. 604800] in unit Seconds |

Table 7.1.5.2.2-X: D-LBO parameter of neighbour cells

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Load Balancing Allowed | This indicates if mobility load balancing is allowed or prohibited from source cell to target cell. See attribute isMLBAllowed in TS 28.541 [13]. | Boolean TRUE,FALSE |

Next change

8.2.4	LBO (Load Balancing Optimisation)

Figure 8.2.4-1 depicts a procedure that describes how D-SON management function can manage the LBO function. It is assumed that the D-SON management function has consumed the performance assurance MnS to create PM jobs to collect handover related measurements.

Figure 8.2.4-1: D-LBO procedure

1. The D-SON management function consumes the management service for NF provisioning with modifyMOIAttributes operation (see clause 5.1.3 in TS 28.532 [3]) to configure the ranges of HO and reselection parameters and LBO parameter of neighbour cells for the LBO function.

1.a The MnS of provisioning sets the ranges for MRO function (NOTE).

2. The D-SON management function consumes the NF provisioning MnS with modifyMOIAttributes operation to enable the LBO function for a given NR cell if it is not enabled.

2.a The provisioning MnS enables the LBO function (NOTE).

3. The LBO function collects real-time load information to determine and perform actions to balance the traffic loads among NR cells and the LBO function may indicates its recommendation of LBO parameters of neighbour cells to D-SON management function based on its real-time load information.

4. D-SON management function collects LBO related performance measurements.

5. The D-SON management function analyses the measurements to evaluate the LBO performance,

6. The D-SON management function consume the MnS of provisioning with modifyMOIAttributes operation to update the ranges of handover parameters if the LBO failed to meet expection,

6.a. The MnS of provisioning updates the ranges of HO and/or reselection parameters (NOTE).

NOTE: The interface between provisioning MnS and D-LBO function is not subject to standardization.

End of change