3GPP TSG-SA5 Meeting #154	S5-241533

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0539 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.552 Add measurement Distribution of CCE Usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc., AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN WG2 has introduced a new gNB Layer 2 measurement [L2_PDCCH_Usage] to allow observability of PDCCH CCE Usage usage. A measurement to provide the utilization average is useful for monitoring. A more detailed measurement of the distribution would however be useful in cases where the actual values would assist to better understand the utilitization. This measurement is a useful indicator of whether a cell is under high load in scenarios where a cell on the PDCCH may experience high load in certain short times (e.g. in a millisecond) and recovers to normal very quickly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new indicator "Distrbution of PDCCH CCE Usage" to measure the distribution of the usage of resources of PDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of indicator to provide more detailed observability of the usage of PDCCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 5.1.1.2.x               Distribution of PDCCH CCE PRB Usage

a) This measurement provides the distribution of samples with PDCCH usage (in percentage) of physical resource blocks (PRBs) in different ranges. This measurement is a useful indicator of whether a cell is under high load in scenarios where a cell on the PDCCH may experience high load in certain short times (e.g. in a millisecond) and recovers to normal very quickly.

b) CC

c)   Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is PRB usage at sample n for PDCCH CCE, which is a percentage of PRBs used, averaged during time period tn (e.g. a millisecond) with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for PDCCH transmission shall be included; Pn is the total number of PRBs available for PDCCH transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

d)  Distribution of PDCCH CCE PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PDCCH CCE PRB usage provides the histogram result of the samples collected during time period tn.

e)   Depending on the value of the sample, the proper bin of the counter is increased.  The number of samples during one measurement period is defined by the vendor.

f)   A set of integers. Each representing the (integer) number of samples with a PDCCH CCE PRB percentage usage in the range represented by that bin.

g)  RRU.PrbPdcchDist.BinX, which indicates the distribution of PDCCH CCE PRB Usage for a cell.

h)  NRCellDU

i)   Valid for packet switched traffic

j)   5GS