3GPP TSG-SA5 Meeting #155 	S5-243049

# Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0542 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.255 Update scenario of clause 5.2.2.22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MATRIXX Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE_CH_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.2.2.22 describes a scenario of UE PDU session charging based on Business converged charging. In this scenario, charging information are exchanged between the C-CHF and the B-CHF for the purpose of UE PDU session charging. However using Charging Data Request/Response between the C-CHF and the B-CHF implies additional charging for the UE PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove Charging Data Request/Response between C-CHF and B-CHF in flow of Figure 5.2.2.22.2-1, and leave the exchange unspecified for this release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging UE PDU session twice. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.22.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-242852 |  |  |  |  |  |  |  |  |

First change

##### 5.2.2.22.2	PDU session establishment

The following figure 5.2.2.22.2-1 describes a UE PDU session converged influenced by business converged charging:

Figure 5.2.2.22.2-1: UE PDU session converged Charging influenced by business charging

0ch. Business charging information obtained by B-CHF from NWDAF, MnS Producer, NSACF, which could be e.g:

-	S-NSSAI service profile: e.g. "Max nb of PDU sessions", "Max nb of UEs”, "Duration of the slice"…

-	S-NSSAI KPIs;

- 	load level.

1 to 9ch-b. PDU session establishment for individual UE per Figure 5.2.2.2.2-1.

9ch-aa.	UE PDU session charging needs input from business charging

9ch-ab.	C-CHF sends a request to B-CHF, and receives charging information from B-CHF.

The messages exchange between C-CHF and B-CHF for this scenario is not specified in this release of the specification.

9ch-ac.	Account, Rating and reservation control for individual UE based on charging information received from B-CHF.

9ch-c.	C-CHF provides appropriate answer for the individual UE.

End of change