3GPP TSG-SA5 Meeting #155 	S5-242997

# Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0020 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add converged charging architecture for Ranging and Sidelink Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no converged charging architecture for Ranging and Sidelink Positioning |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add converged charging architecture for Ranging and Sidelink Positioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Converged charging for Ranging and Sidelink Positioning will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

4.1	High level LCS architecture

4.1.1	LCS architecture in GSM/UMTS networks

Figure 4.1.1 depicts the logical LCS architecture, as described inTS 23.271 [201].

Figure 4.1.1: LCS logical architecture with inter-GMLC [Lr] interface

As can be seen in figure 4.1.1, the following LCS elements are relevant for charging:

- V-GMLC,

- H-GMLC,

- R-GMLC.

Editor's note: Add a statement stating that the SGSN and the MSC have also a role in the LCS Charging and that the associated LCS Charging functionality is described in TS 32.250 and TS 32.251

4.1.x	Ranging and Sidelink Positioning architecture in 5G network

Figure 4.1.x.1 depicts the Ranging based services and Sidelink positioning architecture, as described in TS 23.586 [xx].

Figure 4.1.x.1: Reference architecture for Ranging based services and Sidelink positioning for non-roaming and same PLMN operation in SBI representation

Next change

4.y	LCS converged charging architecture

The LCS converged charging architecture, which is only applicable to Ranging and Sidelink Positioning in this release, is depicted in figure 4.y.1 in service-based representation.

Figure 4.y.1: LCS Converged charging architecture

Figure 4.y.2 depicts the LCS converged charging architecture in reference point representation for non-roaming, which is only applicable to Ranging and Sidelink Positioning in this release:

Figure 4.y.2: LCS converged charging architecture non-roaming reference point representation

End of changes