3GPP TSG-SA Meeting #104 	SP-240910

18 - 21 June, 2024, Shanghai, China 	(revision of SP-24xxxx)

Source:	Nokia

# Title:	New WID on Energy Efficiency and Energy Saving

# Document for:	Approval

# Agenda Item:	6.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Energy Efficiency and Energy Saving

Acronym:	EnergySys

Unique identifier:	TBD

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| EnergyServ | SA1 | 1000033 | Energy Efficiency as service criteria |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940037 | Enhancements of EE for 5G Phase 2 | The Rel-18 WI in SA5 may be potentially related to the proposed SA2 study. |
| 870022 | Enhancements on EE for 5G networks | The Rel-17 WI in SA5 may be potentially related to the proposed SA2 study. |
| 810023 | Energy efficiency of 5G | The Rel-16 WI in SA5 may be potentially related to the proposed SA2 study. |
| 981137 | Network energy savings for NR | The Rel-18 work in RAN1 and RAN2 may be potentially related to the proposed SA2 study, depending on the agreed Rel-19 scope. |

# 3	Justification

Energy consumption is a significant source of operations costs for Mobile Network Operators (MNOs) and depending on the energy generation mix that is used to power networks, it can also have impact on the environment. There has been increasing work in 3GPP on improving energy efficiency and energy saving. Some solutions studied how to optimize energy consumption by adapting the network itself, e.g. activating and deactivating parts of the network. Such change to the topology and components of the network could be done transparent to the network architecture in some cases, or it had to have implications with the architecture in other cases, e.g. reselection of proper network functions.

Stage 1 requirements for energy as a service criteria have been identified in Rel-19 EnergyServ by SA1. While the goal of energy efficiency is to provide the same services more efficiently, the goal of energy use control as service criteria will be to supervise services in an energy-aware manner, with ensuring the services offered as intended by service providers, network operators or subscribers, with determined constraints and consequences. A number of functional requirements have been identified that promise increased control over energy use to achieve service objectives for mobile network operators, service providers and their customers in the SA1 Rel-19 EnergyServ. Examples of these functional requirements include the ability to control energy use based on operator policies such as 'energy credit limits' and 'maximum energy usage rate' applying to services provided to a UE or group of UEs.

Also, 3GPP has already undertaken work in different working groups to provide recommendations on energy saving and actual OAM or Radio Network enhancements to save energy. SA plenary has issued also a 3GPP-wide recommendation on considering Energy efficiency as an important design criterion for the technical solutions 3GPP defines in their specifications (see SP-211621). SA2 will also be required to investigates options for improved system behaviour aimed at energy saving.

This work item is to complete the design of stage 2 solutions and document the normative specifications, based on the conclusions of the FS_EnergySys study.

# 4	Objective

This work aims to specify required 5GS enhancements needed  to improve energy efficiency and to support energy saving in the network, for the following aspects, based on the conclusions in the TR 23.700-66:

Editor’s note: The WID description needs to be updated based on decision of SA#104 on whether new functionality to collect and calculate energy consumption at expected granularity will be defined by SA2 or SA5, including any new requirement for energy information collection (e.g. per cell) and decisions by TSG RAN on what information per UE the RAN can report to CN within Rel-19 scope (e.g. data volume for a UE per energy state). Some of the granularities for information collection/exposure need to be confirmed at SA#104.

- WT #1. The following enhancements for the exposure of energy related information will be specified:

-  	Collection of the energy related information in the network user plane and exposure to the authorized consumer subject to operator’s policy energy related information at the following granularities:

-	per UE

-	Per UE/network slice

-	Per PDU session

-	Per UE/ QoS flow

NOTE: the collection of per NF(including gNB) and Network slice is already defined by SA5.

-	The support of the collection, and exposure of energy-related information for the granularity defined above will be specified in cooperation with SA5. This information is considered:

- Energy consumption information

- Renewable energy consumption information

- 	SA2, subject to agreement with SA5 charging, will specify any enhancement of procedures needed by SA5 Charging to collect information necessary to derive the above information.

-	The energy-related information exposure for the consumer NFs, including 5GC NFs and /or AF, may include an indication of requirements on update of the information, e.g. Periodic reporting or event-based reporting.

-	It is assumed that t

he sources of the information for the calculation of the User Plane energy consumption information considered are:

a) 	OAM, which provides  energy consumption information at the gNB(s) and UPF(s) serving the UE, or Network Slice serving the UE, and also the overall data volume for the gNB and UPF.

b) 	Charging subsystem, which provides the data volume information for a UE.

- WT #2. The following enhancements to  will be specified:

- 	The definition of Energy saving subscription information per UE that is stored as part of the subscription data in the UDM/UDR, to assist the network to perform energy saving strategies for the UE (this can be the subscribed network slices and, if so, no normative impact is expected for this aspect as the network slice configuration determines the related energy behaviour)

- 	Procedures for the PCF to receive UE energy-related policies from UDR taking into account energy information per UE from KI#1 and interactions with CHF to enforce spending limit based on energy criteria

- WT #3.  The following 5GS procedures enhancements will be specified to achieve energy saving:

- 	New energy related information and/or existing NF profile parameters in the NF Profile to allow an operator to influence NF discovery and selection based on their energy strategy.

-	Only parameters captured in existing solutions in the TR 23.700-66 will be considered. Coordination with SA WG5 may be needed to enable certain new parameters.

-	The information within NF profile is expected not to change frequently, e.g., constantly reflecting changes in energy consumption.

-	Enhancement on NF discovery and (re-)selection to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

-	UP path adjustment for a PDU session(s) will be specified.

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | System architecture for support of Energy Efficiency and Energy Saving | TSG SA #106 (Dec., 2024) |  |
| 23.502 | Procedures for support of Energy Efficiency and Energy Saving | TSG SA #106 (Dec., 2024) |  |
| 23.503 | Policy control for support of Energy Efficiency and Energy Saving | TSG SA #106 (Dec., 2024) |  |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Jungshin Park (Samsung), shin02.park@samsung.com

Secondary Rapporteur: Konstantinos Samdanis (Lenovo), ksamdanis@lenovo.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN issues are covered by RAN WGs (e. g. RAN2, RAN3)

Potential charging and OAM issues are covered by SA5.

Potential security issues are to be covered by SA3.

# 9	Supporting Individual Members

Editor’s note:The WID supporters need to be confirmed at SA#104

Supporting IM name

AT&T

BT

CATT

China Mobile

China Telecom

Cybercore

Deutsche Telekom

DISH Network

Ericsson

ETRI

Futurewei

Huawei

Intel

InterDigital

KDDI

KPN

Lenovo

LG Electronics

LG Uplus

MATRIXX Software

MediaTek

NEC

Nokia

NTT DOCOMO

OPPO

ORANGE

Rakuten Mobile

Samsung

SK Telecom

Toyota

Verizon

Vivo

ZTE