TSG SA Meeting #SP-104																		            SP-240940

18 - 21 June 2024, Shanghai, China 												Revision of SP-240717

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0203 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support cleartext HPLMN ID in PC5 U2N relay discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Xiaomi, Huawei, HiSilicon, China Telecom, InterDigital, Qualcomm, Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2024-06-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to clause 6.1.3.2.2 of the present TS, PROSE PC5 DISCOVERY message for UE-to-network relay discovery includes HPLMN ID in cleartext to identify the discovery security materials. The ProSe U2N remote UE can obtain the proper security material in order to unscramble and decrypt the message based on the HPLMN ID. However, based on existing encryption and scrambling operations defined in 33.503, the HPLMN ID may be scrambled and probably encrypted. If so, ProSe remote UE cannot decide which security material/key should be used to un-scramble or decrypt the message, and has to retry the unscrambling and decryption multiple times i.e. with different security materials till it reaches a match for the HPLMN ID, that is time and energy consuming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update scrambling and encryption operations to support clear text HPLMN ID in the U2N relay discovery message based on position of PLMN ID IE decided by CT1 in LS C1-243949. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ProSe U2N relay discovery over PC5 may fail or take long time |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.554 CR 0558 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Adding link to related CT#104 approved CR in CP-241262. |  |  |  |  |  |  |  |  |

################### Start the 1st change ##################

##### 6.1.3.2.3	Protection of discovery messages over PC5 interface

There are three types of security that are used to protect the restricted 5G ProSe Direct Discovery messages over the PC5 interface: integrity protection, scrambling protection, and message-specific confidentiality which are defined in clause 6.1.3.4.3 in TS 33.303 [4].

For the discovery messages that do not include HPLMN ID, the protection mechanisms specified in TS 33.303 [4] are reused with the following changes:

-	Input parameters to integrity protection algorithm as specified in clause A.6 in the present document.

-	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document.

-	In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message - 16, 256).

-	Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

XOR (0xFFFF || time-hash-bitsequence) with the most significant (L + 16) bits of discovery message.

NOTE 1:	16 is the size of Message Type and UTC-based counter LSB in bit length.

NOTE 2:	The maximum length of the discovery message to be scrambled is limited to 256 bits.

-	Step 2 of clause 6.1.3.4.3.2 of TS 33.303 [4] becomes:

Calculate MIC if a DUIK was provided, otherwise set MIC to a 32-bit random string. Then, set the MIC IE to the MIC.

-	Step 4 of clause 6.1.3.4.3.2 of TS 33.303 [4] is not processed.

NOTE 3: Protection for the discovery messages between the ProSe UEs is provided at the ProSe layer.

The discovery messages that include HPLMN ID are protected using the protection mechanism described above with the following changes:

- 	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document with the following changes:

-	The input parameter LENGTH is set to LEN(discovery message) - (LEN(Message Type) + LEN(UTC-based counter LSB) + LEN(HPLMN ID) + LEN(MIC)), where LEN(x) is the length of x in number of bits.

- In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the discovery message to be scrambled and set to Min (the length of discovery message – 16 – the length of HPLMN ID, 256).

- Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

XOR (0xFF..FF || time-hash-bitsequence) with the most significant (L + 16 + the length of HPLMN ID) bits of discovery message, where 0xFF..FF is (16 + the length of HPLMN ID) bits of length.

In 5G ProSe UE-to-UE Relay discovery, the End UE discovery infos to be included in the direct discovery set are protected using the protection mechanism described above with the following changes:

- 	Message-specific confidentiality mechanisms as specified in clause A.7 in the present document with the following changes:

-	discovery message is replaced by End UE discovery info

-	The length of Message Type is set to zero

-	In A.5 of TS 33.303 [4], the time-hash-bitsequence keystream is set to L least significant bits of the output of the KDF, where L is the bit length of the End UE discovery info to be scrambled and set to Min (the length of End UE discovery info - 8, 256).

-	Step 3 of clause 6.1.3.4.3.5 of TS 33.303 [4] becomes:

If L is set to 256:

XOR the time-hash-bitsequence with the most significant L bits of the End UE discovery info.

Otherwise:

XOR (time-hash-bitsequence || 0xFF) with the most significant (L + 8) bits of the End UE discovery info.

NOTE 4: 8 is the size of the UTC-based counter LSB field in bit length.

################### End the change ##################