3GPP TSG SA Meeting #105	SP-241124

# September 10 – 13, 2024, Melbourne, AU	(revision of SP-24xxxx)

Source: 	MediaTek Inc., Nokia, T-Mobile USA Inc.

Title: 	On AI/ML Cross- domain Aspects

Document for: 	Discussion

Agenda Item: 	3.3

Work Item / Release:	- / Rel-19

Abstract of the contribution: This paper discusses the current status of different AI/ML cross-domain aspects based on the progress of RAN WGs and proposes a way forward for SA WG-level studies.

# 1	Introduction

The study on Rel-19 CN-enhanced support for AI/ML [1] has been ongoing during past quarters at SA2 level and it has already concluded on several aspects, except part of AI/ML cross-domain coordination aspects, particularly below WTs:

- WT1.1 –Study whether and how to support UE data collection to meet requirements for RAN AI support for air interface operation (for RAN) for UE-side model training documented in 3GPP TR 38.843 clause 7.2.1.3.2. This includes identifying what benefit can be achieved from enhanced UE data collection in 5G System, and the potential impacts on the 5G framework, including potential enhancements to policy control and OAM. The WT will also discuss the possible data leakage from the operator’s domain which should be avoided and the network control over data collection. - WT1.2 –Study whether (and how) to support model transfer/delivery to the UE according to RAN1/RAN2 considerations documented in 3GPP TR 38.843 clause 7.1.2.4, including potential enhancements to policy control and OAM. Whether and what entities or functions transfer the AI/ML model or information to the UE will be studied as part of the work. This WT will also discuss the possible data leakage from the operator’s domain which should be avoided. - WT1.3: Study whether and how to support the alignment of model identification and model management between SA2 and RAN. Work will be based on the possible requirements defined by RAN1 and RAN2 considering the conclusions in 3GPP TR 38.843.

As described in the SID (see [1]), whether SA2 can start work on WT 1.1, 1.2 and 1.3 will be discussed at SA#105 (Sep. 2024) based on the outcome of the related work in the involved RAN WGs(s). Further change in description of WT 1.1, 1.2, 1.3 can be discussed at SA#105.

This paper discusses the current status of different AI/ML cross-domain aspects based on the progress of different RAN WGs and proposes a way forward for SA WG-level studies.

# 2	Discussion and Proposals

## 2.1 UE Data Collection

Following options were discussed in RAN WG2 for data collection for UE-side model training [2][3]. A comprehensive analysis of these options has been conducted, focusing on various aspects such as data transfer paths, protocol layers, MNO visibility of data content, and MNO control over the data collection process. Despite the detailed analysis, RAN WG2 has not yet reached a consensus on a preferred option. Key areas of debate remain, notably concerning the data visibility level and the control extent associated to Option 1b.

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Option 1a

Option 1a involves UEs collecting and directly transferring training data to Over-The-Top (OTT) servers, bypassing the MNO's infrastructure. This approach has no expected impact on 3GPP specifications, as it operates independently of the MNO's control. It represents a non-standardized approach, operating independently of 3GPP specifications, thus, it falls outside the purview of 3GPP standardization efforts. Given that 3GPP lacks insight into the implementation details of such a solution, it should not be adopted as a baseline for standardization. Instead, we should focus on options that align with 3GPP's standardization efforts and provide MNOs with the necessary control and visibility.

Proposal 1: Given that Option 1a (of UE Data Collection) operates outside the purview of 3GPP and lacks standardization specifics, it should not be adopted as a baseline solution within the 3GPP framework.

Option 1b, 2/3

According to current RAN WG2 discussion [1][2], both option 2 and option 3 provide full controllability for MNOs, allowing them to manage data transfer without the need for a SLA. They offer the possibilities for full, partial, or no standardized visibility of data content. The situation for Option 1b is less clear-cut. The level of MNO controllability and visibility is marked as "FFS", indicating that RAN WG2 cannot reach a consensus without input from SA groups.

Options 2 and 3 are favored from a RAN perspective for several reasons:

- While option 1b, exemplified by EVEX, offers a approach with a coarse level of controllability through API endpoints for session management and event exposure, option 2 and 3 allows for more precise, dynamic, and adaptive network control, directly influenced by the RAN node's real-time assessment of the radio environment. These options ensure that MNOs have full and granular control over the data.

- Option 2 and option 3 provides the possibility of full visibility of the data content at the RAN node, which can guarantee the integrity and security of the data collection process at RAN node when necessary.

- Option 2 and option 3 mitigate privacy concerns more effectively than Option 1b, as the data is stored within the MNO's domain, reducing the risk of unauthorized access or misuse.

- Data transfer is intrinsically linked to the configuration of data collection, which is to be done via control plane.

- Options 2 and 3 align with existing procedures of data collection for network-side model training and transfer, promoting consistency and efficiency.

- The RRC signaling impact is similar for both options, with the gNB receiving IEs containing the data and executing the forwarding process.

Proposal 2: Options 2 and 3 should be endorsed for data collection for UE-side model training as they offer the possibilities of full controllability and full visibility for MNOs and pose fewer privacy concerns.

Given that Options 2 and 3 would have SA level impacts in SA2, SA3 and SA5 to address different aspects of UE Data collection via CN or OAM, the associated SA WGs can be tasked to further study the architectural aspects of these two options as part of Rel-19 CN-enhanced support for AI/ML.

Proposal 3: SA Plenary tasks SA2, SA3, SA5 to further study UE data collection aspects based on Options 2 and 3 (of UE Data Collection) considering RAN WG2 analyses. For SA2, this can be achieved as part of study on CN-enhanced support for AI/ML (WT#1.1).

## 2.2 Model Transfer / Delivery

RAN WG1 Agreement (see [4]):

Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

Proposal 4: SA Plenary to descope WT1.2 of Rel-19 CN-enhanced support for AI/ML from Rel-19 given that RAN has not concluded yet.

## 2.3 Model Identification and Model Management

RAN WG1 Agreement (see [4]):

Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

Proposal 5: SA Plenary to descope WT1.3 of Rel-19 CN-enhanced support for AI/ML from Rel-19 given that RAN has not concluded yet.

# Conclusion

This paper discussed the current status of different AIML cross-domain aspects based on the progress of RAN WGs and proposes a way forward for SA WG-level studies based on following proposals:

Proposal 1: Given that Option 1a (of UE Data Collection) operates outside the purview of 3GPP and lacks standardization specifics, it should not be adopted as a baseline solution within the 3GPP framework.

Proposal 2: Options 2 and 3 should be endorsed for data collection for UE-side model training as they offer the possibilities of full controllability and full visibility for MNOs and pose fewer privacy concerns.

Proposal 3: SA Plenary tasks SA2, SA3, SA5 to further study UE data collection aspects based on Options 2 and 3 (of UE Data Collection) considering RAN WG2 analyses. For SA2, this can be achieved as part of study on CN-enhanced support for AI/ML (WT#1.1).

Proposal 4: SA Plenary to descope WT1.2 of Rel-19 CN-enhanced support for AI/ML from Rel-19 given that RAN has not concluded yet.

Proposal 5: SA Plenary to descope WT1.3 of Rel-19 CN-enhanced support for AI/ML from Rel-19 given that RAN has not concluded yet.

# References

[1] SP-240134, New SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)

[2] R2-2407785, Data collection table modified online, RAN2 Chair (InterDigital)

[3] R2-2407807, CR on RAN2 inputs to TR 38.843, Ericsson

[4] RAN WG1#118 Report, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_118/Report