3GPP TSG-SA WG4 Meeting #88 	S4-160498

# Memphis, TN, USA, 18th – 22nd  Apr 2016										 revision of S4-160478

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | 0537 | rev | 2 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Announcement Signaling of MooD Header Attachment to Unicast Requests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MI-MooD |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | An important feature of MBMS that was specified in TS 26.346 is MooD (MBMS operation on Demand). Unicast service demand can be measured by the network to determine whether or not to offload the service to the MBMS bearer. As indicated in the specification, for DASH-formatted service/content, such unicast demand measurement can be enabled by the UE sending a 3GPP-defined Mood header to a network proxy server in requests for the service over HTTP. However, the current specifcation does not precisely define how the MooD header should be sent – for example, in each Media Segment request, or only in each request for the MPD. The former method may incur excessively high processing overhead in both the UE and the proxy server, especially for live programs whereby the Segments are very short, for example 1-2 seconds in duration. The latter method could provide a means to avoid unnecessary processing load on the UE and proxy server while still enabling adequate dynamic measurement of service demand on unicast, assuming that nominal MPD updates are relatively frequent, e.g., occurs every few tens of seconds). It is also understood that sending of MooD header only in MPD requests might not be appropriate to track unicast demand. Therefore, it is desirable that the MBMS service provider can define content-specific rules for use by the UE in sending MooD header in unicast requests to the proxy server. Such signaling would inform the UE for which type of DASH content request should the MooD header be included, thus allowing the operator to control the UE and network load. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed change to the USBD fragment, under the mooDConfiguration element, to carry rules on how the MooD header should be attached to unicast requests from the UE to the proxy server. The solution mechanism is not biased to any specific content format or service type, in support of measurement of unicast demand for different types of MooD services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potentially unnecessarily high processing load imposed on the UE and/or the network proxy server in support of MooD services. MBMS operators cannot obtain finer-grain control on the reporting of unicast load for MooD services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST  CHANGE: Modifications to clause 11.2.1.2

#### 11.2.1.2	Extensions to the User Service Bundle Description

The MBMS User Service Bundle Description schema defined in this clause extends the MBMS Release 6 schema of clause 11.2.1.1. An MBMS User Service Bundle Description schema of the current release shall comply with MBMS User Service Bundle Description schema definition of Release 6 and the subsequent releases up to the current release.

An initiationRandomization element and terminationRandomization element carries the parameters to be used by the MBMS UE to randomize their initiation and/or termination operations over time. If the initiationRandomization element is present, all MBMS UEs shall randomize the initiation time as defined by the attributes of the elements. If the terminationRandomization element is present, all MBMS UEs shall randomize the termination time as defined by the attributes of the elements.

The initiationRandomization and/or terminationRandomization element may be part of:

- a bundleDescription, where it applies to all services in the service bundle

- a userServiceDescription, where it applies to all MBMS bearer services of a single service. If present, this overrides the element in bundleDescription

If the initiationRandomization element is not present, the MBMS UE does not randomize the User Service Initiation procedure over time. The MBMS UE should then perform the operation immediately when it is triggered.

If the terminationRandomization element is not present, the MBMS UE does not randomize the User Service Termination procedure over time. The MBMS UE should then perform the operation immediately when it is triggered.

An initiationRandomization element may contain the initiationStartTime attribute, which defines the start time for the initiation procedure randomization period. The value of the data field represents the 32 most significant bits of a 64 bit Network Time Protocol (NTP) [78] time value. If the initiationStartTime attribute is not present, the MBMS UE shall use the reception time of the User Service Discovery / Announcement information as initiationStartTime.

The initiationRandomization element shall contain the protectionPeriod attribute. The protectionPeriod attribute expresses the length of the protection period in seconds. The initiation procedure shall be randomly deferred during protection period.

The initiationRandomization element shall contain the randomTimePeriod attribute. The randomTimePeriod attribute expresses the length of a time interval (in seconds) over which requests are deferred. The MBMS UE shall calculate a random time for the execution of the initiation procedure. The method provides for statistically uniform distribution over a relevant period of time.

The terminationRandomization element shall contain the protectionPeriod attribute. The protectionPeriod attribute expresses the length of the protection period in seconds. The termination procedure execution shall be randomly deferred during protection period.

The terminationRandomization element shall contain the randomTimePeriod attribute. The randomTimePeriod attribute expresses the length of a time interval (in seconds) over which the operations are deferred. The MBMS UE shall calculate a random time for the execution of the termination procedure. The method provides for statistically uniform distribution over a relevant period of time.

If the MBMS UE is switched off during the termination randomization, the MBMS UE shall cancel the termination randomization.

The r8:alternativeAccessDelivery element shall extend the list of elements of the MBMS deliveryMethod element. Whenever present, it shall contain at least one unicastAccessURI element. The unicastAccessURI element provides unicast server information for OMA push for MBMS download service when the UE is outside of home network and file download delivery method is used. The unicastAccessURI element refers to a URI to be used for unicast access to the streaming service. If the r8:alternativeAccessDelivery element is available then the UE shall select one of the unicastAccessURI elements included. The timeShiftingBuffer attribute of r8:alternativeAccessDelivery may be used to indicate the minimal size of the time shifting buffer that will be provided for the current service by the PSS servers that are referenced in the list. The actual size of the timeshifting buffer of the selected server is returned in the SETUP response from the PSS Server.

The r12:inbandMetadata attribute of the deliveryMethod element, if present and set to "true" or "1", indicates that the MBMS download session associated with this deliveryMethod instance shall be the one exclusively used to carry all USD metadata fragments eligible for in-band delivery (namely, the Associated Delivery Procedure Description, Schedule Description, Filter Description and Media Presentation Description metadata fragments) along with MBMS User Service content(s).

The r12:appComponent child element of the deliveryMethod element, if present, identifies the application content component(s) carried on the MBMS delivery session associated with that deliveryMethod instance.  One or more instances of r12:appComponent may be present under a given deliveryMethod instance, to indicate the same number and types of application content component(s) carried on the corresponding delivery session.  The collection of r12:appComponent attribute names across all deliveryMethod instances of an MBMS User Service represents the complete set of application content components available to the MBMS application affiliated with that MBMS User Service.

The r12:serviceArea child element of the deliveryMethod element, if present, specifies the service area(s) for which non-DASH streaming content carried on the MBMS delivery session associated with that instance of deliveryMethod, are available for UE reception.  The semantics of r12:serviceArea shall comply to the MBMS Service Area Identity as defined in TS 23.003 [4] and TS 36.443 [104].

If the mimeType attribute within the r12:appService element , itself a child of the userServiceDescription element, indicates a UE supported value, then

- The presence of one or more r12:broadcastAppService element(s) under a given instance of the deliveryMethod element shall declare the one or more application service content items delivered over the FLUTE session associated with that deliveryMethod instance.

- The presence of the r12:unicastAppService element under a given instance of the deliveryMethod element declares the one or more application service content items delivered over unicast bearer(s) in support of unicast fallback delivery.

- Each instance of r12:broadcastAppService.basePattern or r12:unicastAppService.basePattern denotes a content item of a single application service delivered over broadcast or unicast, respectively.  Each broadcast content item may be designated as being available in specific MBMS service area(s) as given by the corresponding r12:broadcastAppService.serviceArea element(s).  If present, the serviceArea elements shall be a subset of the MBMS service area(s) present for the current serviceId under userServiceDescription.availabilityInfo.infoBinding.serviceArea elements. When one or more serviceArea element(s) are included under one or more r12:broadcastAppService, the union of all the MBMS Service Area Identities identified by the serviceArea elements under all the r12:broadcastAppService shall match the list of MBMS Service Area Identities included in the userServiceDescription.availabilityInfo.infoBinding.serviceArea elements for the same serviceId.  Each broadcast content item under its containing Period is identified by its basePattern value whose format and usage are described in clauses 7.6.2.1 and 7.6.2.3.  Each unicast content item under its containing Period is identified by its basePattern value whose format and usage are described in clauses 7.6.2.2 and  7.6.2.3.

If the mimeType attribute within the r12:appService element, itself a child of the userServiceDescription element, indicates a UE unsupported value, then a UE compliant with this specification shall ignore r12:broadcastAppService and the r12:unicastAppService elements in the deliveryMethod element.

A USD may include a r7:unicastAccessURI element for support of Release 7 UEs. UEs of release 8 onwards shall use alternativeAccessDelivery element for both OMA Push file download and unicast streaming.

The serviceClass attribute, if present, shall extend the list of attributes of the MBMS Release 6 userServiceDescription element. The serviceClass attribute is optional and contains the service class identifier for the delivered service according to the syntax defined in clause E.1.2 of [90]. Note that Annex E of [90] also foresees the registration of service class identifiers with the Open Mobile Naming Authority. The service class identifier is similar to MIME types and provides an unique identity to services. A MBMS UE may determine the receiving application instance out of the service class identifier.

A user service description may belong to a group of user service descriptions, which represent alternative configurations of the same user service. An example is an MBMS user service that is delivered over non-MBSFN bearer with a low bitrate and over MBSFN bearer with a high bitrate. In such a case, the UE is only expected to consume one variant of the service. The UE recognizes that a set of user service descriptions apply to one user service based on the serviceGroup element.

The userServiceDescription element may include a Registration element. If present, then the UE shall send the registration and deregistration to the given URI. In such a case, the User Service Bundle Description fragment may not be complete in the service announcement. Instead, it may contain references to metadata fragments (e.g. the session description) that are not embedded in the service announcement. When registration is requested, the userServiceDescription element shall contain a Registration element that describes the requested registration procedure. The registrationURI indicates the URI element to the server with whom the registration procedure shall be performed. In case more than one registrationURI is indicated, the UE shall select one randomly. The registrationThreshold is a number that indicates the percentage of UEs that are requested to register.  The UE shall randomly select a number between 0 and 100 and compare it against the threshold. In case the selected number is lower than the indicated threshold, the UE should perform registration. The threshold value "100" indicates that the UE shall perform registration, which is e.g. necessary when the USD is not complete.

The userServiceDescription element may include an r9:mediaPresentationDescription element when the associated MBMS User Service carries 3GP-DASH-formatted content using the download delivery method. The userServiceDescription element may also, or instead,  include an r12:appService element which contains a reference to an Application Service Description fragment which contains descriptions for contents of an application service delivered over either unicast, or broadcast, or both unicast and broadcast modes. If r12:appService element is present,and its mimeType attribute indicates a UE supported value, then it shall be used by UEs complying with this specification. If r12:appService is absent or its mimeType attribute indicates a UE unsupported value, and if r9:mediaPresentationDescription is present, then the r9:mediaPresentationDescription shall be used by UE complying with this release of the specification. The UE should expect that the received files correspond to a DASH Media Presentation as described by the MPD in [98]. The referenced MPD provides a reference to a corresponding Media Presentation Description metadata fragment whose contents are identical to the MPD as defined in [98].  The Media Presentation Description fragment may refer, via the InitializationSegmentURL, to one or more Initialization Segment Description (ISD)  metadata fragments, whose contents are identical to the Initialization Segment as defined in [98].  If the r12:appService element is present, the associated application service shall be a DASH Media Presentation delivered as an MBMS User Service.  The r12:appService element may contain the child elements identicalContent and/or alternativeContent which designate the sets of identical and/or alternative versions, respectively, of content items that may be substituted for one another.   Additional description of the contents and semantics of the r12:appService element are contained  in clause 7.6.3. If the r12:appService element is included, and its mimeType attribute indicates a UE unsupported value, then a UE compliant with this specification shall ignore all of the r12:appService child elements, and all of its child attributes.

The MBMS User Service Description may include a schedule element. If present, the schedule element includes a URI reference to the MBMS User Service schedule information, the latter corresponding to a Schedule Description metadata fragment as defined in sub-clause 11.2A.

The Schedule Description fragment may include a URI reference to the Filter Description metadata fragment to signify the intended use of content filtering.  Each filter data instance contains associated criteria or rules for use by UEs to selectively receive contents at the session level or the file level.  At the session level, the filter rules enable a UE to decide whether it should receive the entire contents of the associated delivery session(s) of the User Service.  At the file level, the filter rules enable a UE to decide whether it should receive individual files of the User Service, during the corresponding file delivery schedule(s).  Additional description of the association between Schedule Description and Filter Description fragments and intended usage are given in sub-clauses 11.2A and 11.2B.

The MBMS User Service Description may include an availabilityInfo element.  If present, it shall extend the list of child elements of the MBMS userServiceDescription element by indicating the presence of additional data pertaining to the availability of the service.

The availabilityInfo element shall include one or more infoBinding elements. The infoBinding element shall contain the child elements serviceArea and radiofrequency. A UE shall be capable of processing an infoBinding element that does not contain the child element serviceArea. Note that for backwards compatibility reasons, serviceArea needs to be indicated as optional in the USD schema (i.e. ‘minOccurs="0"’).  The serviceArea element declares the one or more service areas over which this MBMS User Service is provided.  This element is designated by the MBMS Service Area Identity (SAI) as defined in 3GPP TS 36.443 [104] and 3GPP TS 23.003 [77].  According to 3GPP TS 36.443 [104], MBMS Service Area Identity is frequency agnostic and can be mapped onto one or more cells.  The specific usage of the MBMS Service Area Identity, or its correlation to other network identification information, is not defined in this specification.  The radioFrequency element indicates the one or more RF frequencies in the E-UTRAN downlink which transmit this MBMS User Service over the service area(s) identified by the serviceArea element. The frequency parameter is coded as EARFCN in 3GPP TS 36.101 [105].  The MBMS client shall forward the service area and radio frequency information received in the USD to the lower layers, and the UE is expected to make use of such information in accordance with TS 36.300 [96] clause 15.4 as well as TS 36.304 [108] and TS 36.331 [97].

PLMN, p-serviceArea, and group may be present as attributes of the deliveryMethod element. The attribute PLMN or p-serviceArea identifies the area(s) by PLMN-ID or one or more MBMS SAI’s in which this instance of deliveryMethod is applicable. The UE is expected to use the Session Description fragment referenced by the deliveryMethod whose PLMN or p-serviceArea matches the UE’s location by the corresponding area type. Presence of either PLMN or p-serviceArea under deliveryMethod, but not both, is permitted. Multiple deliveryMethod elements sharing the same group attribute shall be considered as alternatives by the UE. For each group that the deliveryMethod’s affiliated PLMN or p-serviceArea matches the UE’s location, the UE shall acquire the content delivered on the corresponding FLUTE session. In the event of deliveryMethod instances associated with multiple groups, the UE shall acquire the content delivered on the FLUTE sessions from each of the groups with matching PLMN or p-serviceArea to the UE’s location. The PLMN attribute, when present, indicates the PLMN-ID in which the contents carried by the current deliveryMethod is accessible, and is defined as a concatenation of MCC and MNC, each represented as a 3-digit hexadecimal number. The p-serviceArea attribute, when present, indicates the one or more MBMS SAIs, each represented by a decimal number between 0 and 65,535 (inclusive), in which the contents carried by the current deliveryMethod is accessible.

The r12:KeepUpdatedService element indicates if the referenced userServiceDescription describes a keep-updated service. The URL to one or more registration servers is provided in the registrationServer element. If more than one registrationServer URL element is present, the MBMS UE shall choose one randomly.

The presence of the r12:mooDConfiguration element indicates whether the UE shall include the MooD Header (as defined in Clause 12) in its request for the service consumed over unicast and disable unicast Consumption Reporting.

The r12:mooDConfiguration element should be used to specify the MooD configuration information and shall take precedence over the OMA DM MO as defined in clause 12.2.2, if such a DM configuration object exists on the UE. The r12:mooDConfiguration element is used to configure offloading for any type of eligible content accessed over the unicast network via HTTP or RTP. If present, this element contains the rules to which the UE shall comply for attaching the MooD Header (as defined in Clause 12) in its requests for the service consumed over unicast and disable unicast Consumption Reporting. If the r12:mooDConfiguration element is present, then:

- The UE shall include the MooD Header in its request for the service delivery over unicast, in accordance to the criteria expressed in the mooDHeaderAttachment element;

  - In the case of DASH-formatted content, the dASHContent@rule attribute under mooDHeaderAttachment specifies the rule on how the MooD header should be included in unicast requests sent to the proxy server: with every Media Segment request, or with every MPD request. Specifically:

    - rule = “1” means that the MooD header shall be attached to each Media Segment request by the DASH client;

    - rule = “2” means that the MooD header shall be attached to each MPD request by the DASH client;

    - rule = “0” shall not be used;

    - rule values in the range from 3 to 255 are reserved for future use.

  - If the element dASHContent is not present, the default is rule = “1” (i.e. the MooD header shall be attached to each Media Segment request by the DASH client).

- The UE shall include the serviceId in the MooD Header from the userServiceDescription@serviceId attribute;

- If at least one proxyServer child element is present, then the UE shall select one proxy server address from the list of proxy servers, which is provided by the proxyServer child elements. The UE shall keep that proxy server URI for its subsequent requests for the same service delivery over unicast. If the proxyServer element is not present, and the MooD MO does not exist in the UE, then the MooD header shall be attached in the unicast content request to the HTTP server whose location is given by the URL of the associated content request.

- The UE shall include its location in the MooD Header as CGI, ECGI or MBMS SAI, according to the locationType attribute. Possible values of the locationType attribute are “CGI”, “ECGI” or “MBMS SAI”.

Otherwise, if r12:mooDConfiguration element is absent, the UE shall report unicast and broadcast consumption reporting as specified in clause 9.4A.2.

The following schema defines the Release 7 extensions to the User Service Bundle Description schema:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns="urn:3GPP:metadata:2007:MBMS:userServiceDescription"   xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3GPP:metadata:2007:MBMS:userServiceDescription"

elementFormDefault="qualified">

<xs:element name="initiationRandomization">

<xs:complexType>

<xs:attribute name="initiationStartTime" type="xs:unsignedInt" use="optional"/>

<xs:attribute name="protectionPeriod" type="xs:unsignedInt" use="required"/>

<xs:attribute name="randomTimePeriod" type="xs:unsignedInt" use="required"/>

</xs:complexType>

</xs:element>

<xs:element name="terminationRandomization">

<xs:complexType>

<xs:attribute name="protectionPeriod" type="xs:unsignedInt" use="required"/>

<xs:attribute name="randomTimePeriod" type="xs:unsignedInt" use="required"/>

</xs:complexType>

</xs:element>

<xs:element name="serviceGroup" type="serviceGroupType"/>

<xs:complexType name="serviceGroupType">

<xs:attribute name="groupID" type="xs:anyURI" use="required"/>

</xs:complexType>

<xs:element name="unicastAccessURI" type="xs:anyURI"/>

<xs:attribute name="serviceClass" type="xs:string"/>

</xs:schema>

The following schema defines the Release 8 extensions to the User Service Bundle Description schema:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns="urn:3GPP:metadata:2008:MBMS:userServiceDescription"   	xmlns:xs="http://www.w3.org/2001/XMLSchema" 	targetNamespace="urn:3GPP:metadata:2008:MBMS:userServiceDescription"

elementFormDefault="qualified">

<xs:element name="alternativeAccessDelivery">

<xs:complexType>

<xs:sequence>

<xs:element name="unicastAccessURI" type="xs:anyURI" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="timeShiftingBuffer" type="xs:unsignedInt" use="optional" default="0"/>

</xs:complexType>

</xs:element>

<xs:element name="Registration">

<xs:complexType>

<xs:sequence>

<xs:element name="registrationURI" type="xs:anyURI" minOccurs="1" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="registrationThreshold" type="xs:unsignedInt" use="optional" default="100"/>

</xs:complexType>

</xs:element>

</xs:schema>

The following schema defines the Release 9 extensions to the User Service Bundle Description schema:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns="urn:3GPP:metadata:2009:MBMS:userServiceDescription"   	xmlns:xs="http://www.w3.org/2001/XMLSchema" 	targetNamespace="urn:3GPP:metadata:2009:MBMS:userServiceDescription"

elementFormDefault="qualified">

<xs:element name="mediaPresentationDescription">

<xs:complexType>

<xs:sequence>

<xs:element name="mpdURI" type="xs:anyURI"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="schedule">

<xs:complexType>

<xs:sequence>

<xs:element name="scheduleDescriptionURI" type="xs:anyURI"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="availabilityInfo">

<xs:complexType>

<xs:sequence>

<xs:element name="infoBinding" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence>

<xs:element name="serviceArea" type="xs:unsignedShort" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="radioFrequency" type="xs:unsignedInt"   maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:schema>

The following schema defines the Release 12 extensions to the User Service Bundle Description schema.

The schema version attribute (part of the schema instruction) shall be included in the UE schema and the network schema.

NOTE 1: 	The value of the version attribute is intended to be increased by 1 in every future releases where new element(s) or attribute(s) are added to this extension. The value in the version attribute is used in the selection of the version of the current extension which may be used in a given version of the main USD schema, see clause J.1 for details.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns="urn:3GPP:metadata:2013:MBMS:userServiceDescription" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3GPP:metadata:2013:MBMS:userServiceDescription" elementFormDefault="qualified" version="1">

<xs:element name="broadcastAppService">

<xs:complexType>

<xs:sequence>

<xs:element name="basePattern" type="xs:anyURI" maxOccurs="unbounded"/>

<xs:element name="serviceArea" type="xs:unsignedShort" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:element name="unicastAppService">

<xs:complexType>

<xs:sequence>

<xs:element name="basePattern" type="xs:anyURI" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:element name="appService" type="appServiceType"/>

<xs:complexType name="appServiceType">

<xs:sequence>

<xs:element name="identicalContent" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence>

<xs:element name="basePattern" type="xs:anyURI" minOccurs="2" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:element name="alternativeContent" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence>

<xs:element name="basePattern" type="xs:anyURI" minOccurs="2" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="appServiceDescriptionURI" type="xs:anyURI" use="required"/>

<xs:attribute name="mimeType" type="xs:string" use="required"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:attribute name="inbandMetadata" type="xs:boolean"/>

<xs:element name="appComponent" type="xs:string"/>

<xs:element name="serviceArea" type="xs:unsignedShort"/>

<xs:element name="KeepUpdatedService">

<xs:complexType>

<xs:sequence>

<xs:element name="registrationServer" type="xs:anyURI" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="mooDConfiguration">

<xs:complexType>

<xs:sequence>

<xs:element name="proxyServer" type="xs:anyURI" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="mooDHeaderAttachment">

<xs:complexType>

<xs:sequence>

<xs:element name="dASHContent" minOccurs="0">

<xs:complexType>

<xs:attribute name="rule" type="xs:unsignedByte" use="required">

<xs:annotation>

<xs:documentation>0: not used

1: Media Segment requests

2: MPD requests

3-255: reserved for future use	</xs:documentation>

</xs:annotation>

</xs:attribute>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="locationType" type="xs:string" use="required"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

</xs:schema>

Note that Annex J contains a main User Service Description schema referencing the extensions schema.

END OF FIRST CHANGE