|  | Radiocommunication Study Groups |  |  |
| --- | --- | --- | --- |
| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| Source: Document 1A/TEMP/37 (edited) |  | Annex 12 to Document 1A/144-E |  |
|  |  | 12 December 2016 |  |
|  |  | English only |  |
| Annex 12 to Working Party 1A Chairman’s Report |  |  |  |
| Preliminary Draft Modification of Report ITU-R SM.2351-1 |  |  |  |
| Smart grid utility management systems |  |  |  |

The Attachment provides a preliminary draft modification of Report ITU-R SM.2351-1 based on submissions and other information presented to the November 2016 meeting of Working Party 1A, as contained in Documents 1A/80 (3GPP).

[Editorial note: in the modifications proposed, a number of subjective terms have been employed when discussing the deployment and use of smart grid devices; in order to improve understanding, further information is requested on quantifying these expectations.]

Attachment:	1

ATTACHMENT

Preliminary Draft Modification of REPORT ITU-R SM.2351-1

Smart grid utility management systems

(2015)

Table of Contents

[Editorial note: table of contents not reproduced]

# 1	Introduction

[Editorial note: No changes until Section 7]

# 7	Wireless standards for smart grid telecommunications

[Editorial note: No changes until Section 7.2]

## 7.2	WAN/NAN/FAN

The WAN/NAN/FAN communication networks share the need to carry data over relatively long distances (neighborhoods, cities) to operation centers. These networks can directly service the end node or serve as a backhaul. The type of solution that is selected depends on many considerations, some of which are:

–	Link distance

–	Availability of right of way (for cabled solutions)

–	Link capacity

–	Non-mains powered devices

–	Availability

–	Reliability

–	Regulated (e.g. licensable) versus unregulated (e.g. license exempt) spectrum.

The IEEE 802 LAN/MAN standards committee has developed several wireless standards that are being used to support Smart Grid applications.

These solutions include:

–	Wireless standards that support point-to-multipoint wireless

•	IEEE 802.11

•	IEEE 802.16

•	IEEE 802.20

•	IEEE 802.22

–	Wireless standards that support wireless mesh

•	IEEE 802.15.4

•	IEEE 802.11

Other wireless communication technologies that can contribute to smart grid requirements include cellular technologies and sound broadcasting. Cellular networks under 3GPP responsibility (i.e. GSM/EDGE, WCDMA/HSPA and LTE) have evolved from providing telephony services to support a wide range of data applications, with in-built security and quality of service support. In recent 3GPP releases standardization enhancements for machine-type communication (MTC) have also been introduced, including support for congestion control, improved device battery lifetime, ultra-low complexity devices and increasing numbers of devices (which could be of a very large magnitude by some projections), as well as addressing improved indoor coverage. Smart meters are available with individual monitoring and control functions provided using 3GPP technologies. Also, inaudible subcarriers have been used for decades for simple wide area switching between metering tariffs using FM broadcasting networks in the USA and the LF 198 kHz national coverage broadcasting service in the United Kingdom. The IEEE 802 LAN/MAN standards committee has developed several standards that are being used to support smart grid applications.

# 8	Interference considerations associated with the implementation of wired and wireless data transmission technologies used in power grid management systems

[Editorial note: No changes until Section 9]

# 9	Impact of widespread deployment of wired and wireless networks used for power grid management systems on spectrum availability

One of the objectives of the 3GPP cellular wireless technologies and the IEEE 802 family of standards is that the spectrum availability will not be affected by interference associated with wide-spread deployment of such technologies and devices.

This is vital consideration given that:

–	There are currently millions of installed wireless smart grid devices in a variety of countries and regions, e.g. Europe, Australia, North America, that are operating in shared spectrum. These deployments are growing and more are planned in these geographic regions because they have been successful and effective.

–	Mobile consumer wireless devices are in wide use globally. Each device may transfer gigabytes of data per month. The data usage of wireless smart grid devices is orders of magnitude smaller. The licensed spectrum, which is managed by wireless carriers, can easily handle the incremental traffic.

–	Existing regulations by regulators such as the Federal Communications Commission and UK Ofcom have successfully allowed for millions of wireless Smart grid devices to operate without harm to each other.

–	IEEE 802 wireless standards use a variety of technologies, e.g. frequency hopping, mesh routing, fragmentation, coding, and high burst rate, which enable reliable wireless Smart Grid Networks. In addition, wireless Smart Grid networks are resilient to link breaks and power outages.

–	Cellular wireless 3GPP technologies use a variety of techniques such as high level modulation and coding, resource block allocation, interference cancellation and mitigation, and MIMO to utilize the allocated spectrum efficiently. Additionally, Coordinated Multipoint provides additional robustness.

–	New cognitive radio sharing technologies developed within the IEEE 802 Standards can make efficient use of spectrum while doing no harm to other primary users operating in these bands or the adjacent bands.

–	Features embedded within IEEE 802 standards such as spectrum sensing, spectrum etiquette, channel set management and co-existence will ensure minimal interference to themselves and others.

–	Cellular wireless 3GPP technologies are continuously evolving and new features relevant to smart grids have been introduced in 3GPP Release 13 in order to:

- improve the radio coverage;

- increase the system capacity;

- extend device battery life time up to at least 10 years; and

- support devices of ultra-low complexity.

–	Wired Ethernet links do not use wireless spectrum, and are generally mandated to comply with applicable local and national codes for the limitation of electromagnetic interference for non-transmitting systems. As such, there should be no additional interference considerations to radiocommunication associated with the use of Ethernet in the implementation of wireless and wired technologies and devices used in support of power grid management systems.

One of the objectives of the 3GPP family of standards is that the spectrum availability will not be affected by interference associated with wide-spread deployment of such technologies and devices considering.

–	widespread, global deployment of systems providing global roaming of millions of user equipment,

–	reliable coverage of cellular network almost everywhere globally.

# 10	Conclusion

High-capacity, two-way communication networks employing wireless, PLT, or other telecommunications technologies that couple sensors and smart meters can transform existing distribution networks for utilities into smart grids.

Smart metering and communications via smart grid networks will in principle allow consumers to monitor and change their patterns of consumption to their best advantage. Utilities will also be able to introduce real time pricing measures in which charges can be adjusted continually to take account of considerations of total demand and the integrity of distribution grids. It will also be possible, in principle, to regulate the demand from particular classes of high usage domestic appliances and industrial equipment.

The overall objective is that these interactive smart grid networks can be monitored and controlled to enhance the efficiency, reliability, and security of the distribution networks for electricity, gas and water supplies, while assuring consumers of the continuity of supply.

Annex 1

Examples of existing standards related to power grid management systems

[Editorial note: No changes until Section A1.3]

### A1.3	3GPP Standards

3GPP has a variety of wireless standards that are applicable to first mile applications for power grid management systems. Recent releases of the 3GPP standards have introduced enhancements for Machine Type Communications (MTC), e.g.

Release 10:

•	Delay tolerant access establishment (UMTS, HSPA+, LTE)

- Extended access barring (GSM/EDGE)

Release 11:

•	Extended access barring (UMTS, HSPA+, LTE)

Release 12:

•	UE power saving mode (GSM/EDGE, UMTS, HSPA+, LTE)

•	Low complexity UE category (LTE)

Release 13:

•	Extended DRX (GSM/EDGE, UMTS, HSPA+, LTE)

•	Extended Coverage GSM Internet of Things (EC-GSM-IoT) (GSM/EDGE)

•	LTE Physical Layer Enhancements for MTC (eMTC) (LTE)

•	Narrow band Internet of Things (NB-IoT)

A summary of the technical and operating features, including above listed enhancements for MTC, of the relevant 3GPP wireless standards are given in the table below.

TABLE A1.7

Technical and operating features of 3GPP Technologies

| Functionality Characteristic |  | Measurement Unit | GSM/EDGE |  | EC-GSM-IoT |  | UMTS |  | HSPA+ |  | LTE Advanced Pro |  | eMTC | NB-IoT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ability to reliably establish an appropriate device link |  | % of time | Depends on deployment (typical > 99%) |  | Depends on deployment (typical > 99%) |  | Depends on deployment (typical > 99%) |  | Depends on deployment (typical > 99%) |  | Depends on deployment (typical > 99%) |  | Depends on deployment (typical > 99%) | Depends on deployment (typical > 99%) |
| Ability to maintain an appropriate connection |  | failure rate per 1000 sessions | Depends on deployment (typical < 1%) |  | Depends on deployment (typical < 1%) |  | Depends on deployment (typical < 1%) |  | Depends on deployment (typical < 1%) |  | Depends on deployment (typical < 1%) |  | Depends on deployment (typical < 1%) | Depends on deployment (typical < 1%) |
| Voice |  |  | Yes |  | Voice messaging supported |  | Yes |  | Yes |  | Yes |  | Yes (Possibly with reduced coverage.) | Voice messaging supported |
| Data |  | Max sustainable user data rate per user in Gbps/Mbps/kbps | GPRS: 172 kbps UL/DL EGPRS: 491 kbps UL/DL EGPRS2-A: 811 kbps DL 638 kbps UL |  | 98 kbps UL/DL (Taking protocol limitations into account.) |  | 1.92 Mbps DL 0.96 Mbps UL (Assuming only data connection.) |  | 294 Mbps DL 58.65 Mbps UL (Assuming a 15% reduction in throughput compared to Peak over the air data rates) |  | DL: Ranging between 0.85 Mbps and 21.2 Gbps depending on UE category. UL: Ranging between 0.85 Mbps and 11.6 Gbps depending on UE category. (Assuming a 15% reduction in throughput compared to Peak over the air data rates) |  | FD-FDD: 800 kbps DL 1 Mbps UL HD-FDD: 300 kbps DL 375 kbps UL (Taking protocol limitations into account.) | 21.3 kbps DL 62.5 kbps UL # (Taking protocol limitations into account.) |
| Video |  |  | Yes |  | No |  | Yes |  | Yes |  | Yes |  | Yes (Possibly with reduced coverage.) | No |
| Geographic coverage area |  | km2 | 35 km radius with normal timing advance; 120 km radius with extended timing advance |  | 35 km radius with normal timing advance |  | 120 km radius for extended range cells |  | 120 km radius for extended range cells |  | 100 km radius |  | 100 km radius | 40 km radius |
| Link budget |  | dB | EGPRS (Veh A50): 146.36/ 133.39 dB GPRS/EGPRS/EGPRS2-A: 144 dB |  | 164 dB (Assumes 33 dBm MS power class. In addition see 3GPP TR 45.820 for further assumptions) |  | Up to 147 dB |  | Up to 147 dB |  | Up to 143 dB DL; Up to 133 dB UL - |  | 155.7 dB (Assumes 20 dBm UE power class. In addition see 3GPP TR 36.888 for further assumptions) | 164 dB (Assumes 23 dBm UE power class. See 3GPP TR 45.820 for further assumptions) |
| Maximum relative movement rate |  | km/s | 350 km/h |  | ~100 km/h (No support for handover) |  | 350 km/h |  | 350 km/h |  | 350 km/h |  | ~100 km/h | ~100 km/h (No support for handover) |
| Maximum Doppler |  | Hz | 1000 with channel tracking equalizer |  |  |  | 648 |  | 648 |  | 648 |  | 70 Hz |  |
| Peak over the air uplink data rate |  | Instantaneous peak data rate in Gbps/Mbps/kbps | GPRS: 172 kbps EGPRS: 491 kbps EGPRS2-A: 638 kbps (Based on the number of information bits per radio block, see 3GPP TS 45.003.) |  | 491 kbps (Based on the number of information bits per radio block, see 3GPP TS 45.003.) |  | 1.024 Mbps UL (Assuming simultaneous speech (64 kbps) and data (0.96 Mbps) connections.) |  | 69 Mbps UL (Assuming dual carriers, 64QAM and 2 MIMO layers.) |  | Ranging between 1 Mbps and 13.6 Gbps depending on UE category. (See 3GPP TS 36.306 for UE categories up to Cat .) |  | FD-FDD: 1 Mbps HD-FDD: 1 Mbps (Assuming UE Category M1 (see 3GPP 36.306) | 250 kbps (Assuming UE Category NB1 (see 3GPP 36.306) |
| Peak over the air downlink data rate |  | Instantaneous peak data rate in Gbps/Mbps/kbps | GPRS: 172 kbps EGPRS: 491 kbps EGPRS2-A: 811 kbps (Based on the number of information bits per radio block, see 3GPP TS 45.003.) |  | 491 kbps (Based on the number of information bits per radio block, see 3GPP TS 45.003.) |  | 2.048 Mbps DL (Assuming simultaneous speech (128 kbps) and data (1.92 Mbps) connections.) |  | 346 Mbps DL (Assuming 15 HS-PDSCH codes, four carriers, 64QAM and 4 MIMO layers.) |  | Ranging between 1 Mbps and 25 Gbps depending on UE category (See 3GPP TS 36.306 for UE categories.) |  | FD-FDD: 1 Mbps HD-FDD: 1 Mbps (Assuming UE Category M1 (see 3GPP 36.306) | LTE in-band operation: 170 kbps Standalone operation: 226.7 kbps (Assuming UE Category NB1 (see 3GPP 36.306) |
| Peak goodput uplink data rate |  | Max sustainable user data rate in Gbps/Mbps/kbps | See Data row. |  | See Data row. |  | See Data row. |  | See Data row. |  | See Data row. |  | See Data row. | See Data row. |
| Peak goodput downlink data rate |  | Max sustainable user data rate in Gbps/Mbps/kbps | See Data row. |  | See Data row. |  | See Data row. |  | See Data row. |  | See Data row. |  | See Data row. | See Data row. |
| Public radio standard operating in unlicensed bands |  | GHz L/UL | Can be operated, but not currently specified. |  | Can be operated, but not currently specified. |  | Can be operated, but not currently specified. |  | Can be operated, but not currently specified. |  | Yes (License Assisted Access) - |  | Can be operated, but not currently specified. | Can be operated, but not currently specified. |
| Public radio standard operating in licensed bands |  | GHz L/UL | Multiple bands per 3GPP 45.005 |  | Multiple bands per 3GPP 45.005 |  | Multiple bands as per 3GPP 25.101 |  | Multiple bands as per 3GPP 25.101 |  | Multiple bands as per 3GPP 36.101 and 36.104 |  | Multiple bands as per 3GPP 36.101 and 36.104 | Multiple bands as per 3GPP 36.101 and 36.104 |
| Private radio standard operating in licensed bands |  | GHz L/UL | Can be operated, but not currently specified. |  | Can be operated, but not currently specified. |  | Can be operated, but not currently specified. |  | Can be operated, but not currently specified. |  | Yes, incl. push-to-talk and direct device-to-device technology |  | Can be operated, but not currently specified. | Can be operated, but not currently specified. |
| Duplex method |  | TDD/FDD | Half-duplex FDD |  | Half-duplex FDD |  | FDD and TDD |  | FDD and TDD |  | FDD and TDD, incl. full- and half-duplex FDD |  | FDD and TDD, incl. full- and half-duplex FDD | Half-duplex FDD |
| Carrier bandwidth |  | kHz | 200 kHz |  | 200 kHz |  | 5 MHz for FDD |  | 5 MHz for FDD |  | 1.4, 3, 5, 10, 15, 20 MHz Up to 640 MHz of aggregated bandwidth using Carrier Aggregation |  | 1.4 MHz | 180 kHz |
| Channel separation |  | kHz | 200 kHz |  | 200 kHz |  | 5 MHz for FDD |  | 5 MHz for FDD |  | Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2, where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective carriers |  | LTE in-band operation: 1.08 MHz Standalone operation: 1.4 MHz | LTE in-band operation: 180 kHz Standalone operation: 200 kHz |
| Number of non-overlapping channels in band of operation |  |  | See 3GPP 45.005 |  | See 3GPP 45.005 |  | See 3GPP 25.101 |  | See 3GPP 25.101 |  | See 3GPP 36.101 and 36.104 |  |  | See 3GPP 36.101 and 36.104 |
| Peak Spectral Efficiency |  | bits/sec/Hz | GPRS: 0.86 bit/s/Hz EGPRS: 2.46 bit/s/Hz EGPRS2-A: 4.05 bit/s/Hz DL 3.19 bit/s/Hz UL |  | 2.46 bit/s/Hz |  | 0.2048 bit/s/Hz UL; 0.4096 bit/s/Hz DL |  | 2.2 bit/s/Hz UL; 5.6 bit/s/Hz DL |  | 15 bit/s/Hz UL; 40 bit/s/Hz DL |  | LTE in-band operation: 0.93 bit/s/Hz Standalone operation: 0.71 bit/s/Hz | LTE in-band operation: 1.39 bit/s/Hz UL 0.94 bit/s/Hz DL Standalone operation: 1.25 bit/s/Hz UL 1.13 bit/s/Hz DL |
| Average Cell Spectral Efficiency |  | bits/sec/Hz/cell | 1.1760 Mbit/s/MHz/cell (Veh A50) (EGPRS) |  | Depending on deployment scenario |  | 0.67 DL (with Diversity); 0.47 UL (Pedestrian A) |  | Depending on deployment scenario, example value ranges are 1.1-1.6 DL; 0.7 – 2.3 UL |  | Depending on deployment scenario, example value ranges for Rel-8 are 1.8 - 3.2 DL; 0.7 - 1.05 UL |  | Depending on deployment scenario | Depending on deployment scenario |
| Frame duration |  | ms | 120/26 ms TDMA frame GPRS: 20 ms TTI EGPRS/EGPRS2-A: 10, 20 ms TTI |  | 20-80 ms TTI |  | 10 ms (2 ms TTI) |  | 10 ms (2 ms TTI) |  | 10 ms (1 ms TTI) |  | 10 ms (1 ms TTI) | 10 ms (1 ms minimum TTI) |
| Maximum packet size |  | Bytes | 1560 octets at RLC interface |  | 1560 octets at RLC interface |  | No fixed size for FDD (depends on modulation level and number of channelization codes); TDD (3.84 Mbps) = 12750 bytes (see 3GPP 25.321) |  | 42192 bits per stream on DL; 22996 bits for UL |  | 8188 bytes for DL/UL |  | 8188 bytes for DL/UL | 1600 bytes for DL/UL |
| Segmentation support |  | Yes/No | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |
| Diversity technique |  | antenna, polarization, space, time | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |
| Beam steering |  | Yes/No | No |  | No |  | No |  | Yes |  | Yes |  | Yes | No |
| Retransmission |  | ARQ/HARQ/- | Yes, e.g., ARQ, HARQ -Incremental Redundancy |  | Yes, e.g., ARQ, HARQ -Incremental Redundancy |  | Yes, e.g., ARQ/HARQ |  | Yes, e.g., ARQ/HARQ |  | Yes, e.g., ARQ/HARQ |  | Yes, e.g., ARQ/HARQ | Yes, e.g., ARQ/HARQ |
| Error correction technique |  |  | Punctured convolutional coding Turbo added in EGPRS2-A |  | Punctured convolutional coding |  | Convolutional and Turbo |  | Convolutional and Turbo |  | Turbo; Tail Biting Convolution on BCH |  | Turbo; Tail Biting Convolution on BCH | Tail Biting Convolutional in DL; Turbo in UL |
| Interference cancellation |  |  | Yes |  | Yes |  | No |  | Yes |  | Yes |  | Yes | Yes |
| RF frequency of operation |  |  | Multiple bands per 3GPP 45.005 |  | Multiple bands per 3GPP 45.005 |  | Specified in 3GPP 25.101 |  | Specified in 3GPP 25.101 |  | Specified in 3GPP 36.101 |  | Specified in 3GPP 36.101 | Specified in 3GPP 36.101 |
| Retries |  |  | Configurable |  | Configurable |  | Configurable |  | Configurable |  | Configurable |  | Configurable | Configurable |
| RSSI |  |  | Yes; 64 levels between -110 dBm+scale and -48 dBm+scale |  | EC-GSM-IoT reports received useful signal in 75 levels between -122 dBm and -48 dBm. |  | Yes; 77 levels between -100 dBm and -25 dBm |  | Yes; 77 levels between -100 dBm and -25 dBm |  | LTE reports Reference Signal Received Power (RSRP) for LTE neighbor cells and RSSI (77 levels between -100 dBm and -25 dBm) for HSPA and EDGE neighbor cells. See 3GPP TS 36.133. |  | LTE reports Reference Signal Received Power (RSRP) for LTE neighbor cells. See 3GPP TS 36.133. | NB-IoT measures Reference Signal Received Power (RSRP). |
| Lost packets |  |  | Depends on operating point but typically 1% residual BLER after HARQ |  | Depends on operating point but typically 1% residual BLER after HARQ |  | Residual BLER = 1% after HARQ |  | Depends on operating point but typically 1% residual BLER after HARQ |  | Depends on operating point but typically 1% residual BLER after HARQ |  | Depends on operating point but typically 1% residual BLER after HARQ | Depends on operating point but typically 1% residual BLER after HARQ |
| Mechanisms to reduce power consumption |  |  | Yes, e.g., DTX, DRX, extended DRX, Power Save Mode and power control |  | Yes, e.g., extended DRX, and Power Save Mode |  | Yes, e.g., DTX, DRX, , extended DRX and Power Save Mode |  | Yes, e.g., DTX, DRX, extended DRX and Power Save Mode |  | Yes, e.g., DTX, DRX, extended DRX and Power Save Mode |  | Yes, e.g. extended DRX and Power Save Mode | Yes, e.g. extended DRX and Power Save Mode |
| Low power state support |  |  | Yes, e.g. extended DRX, Power Save Mode. |  | Yes, e.g., extended DRX, and Power Save Mode. |  | Yes |  | Yes, e.g., Longer DTX/DRX cycles in all states |  | Yes, e.g. extended DRX, Power Save Mode. |  | Yes, e.g. extended DRX and Power Save Mode | Yes, e.g. extended DRX and Power Save Mode |
| Point to point |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |
| Point to Multipoint |  |  | Yes |  | No |  | Yes |  | Yes |  | Yes |  | No | No |
| Broadcast |  |  | Yes |  | No |  | Yes |  | Yes |  | Yes |  | ETWS, CMAS, SIB16 time info | SIB16 time info |
| Handover |  |  | Yes |  | No |  | Yes |  | Yes |  | Yes |  | Yes | No |
| Media Access Method |  |  | circuit-switched TDMA/FDMA Scheduled packet based TDMA/FDMA |  | Scheduled packet based TDMA/FDMA |  | circuit-switched CDMA |  | Scheduled packet based CDMA |  | Scheduled packet based OFDMA |  | Scheduled packet based OFDMA | Scheduled packet based OFDMA |
| Discovery |  |  | Sync and Broadcast channel |  | Sync and Broadcast channel |  | Sync and Broadcast channel |  | Sync and Broadcast channel |  | Sync and Broadcast channel |  | Sync and Broadcast channel | Sync and Broadcast channel |
| Association |  |  | Temporary Block Flow (TBF) |  | Temporary Block Flow (TBF) |  | Through various RNTIs |  | Through HRNTI and ERNTI assigned to UEs |  | Through CRNTI |  | Through CRNTI | Through CRNTI |
| Traffic priority |  | diffserv, resserv | 3GPP-defined priorities |  | 3GPP-defined priorities |  | 3GPP-defined priorities |  | 3GPP-defined priorities |  | 3GPP-defined priorities |  | 3GPP-defined priorities | 3GPP-defined priorities |
| Radio queue priority |  |  | Scheduler in base station |  | Scheduler in base station |  | Yes at the Node B scheduler |  | Yes at the Node B scheduler |  | Yes at the eNode B scheduler |  | Yes at the eNode B scheduler | Yes at the eNode B scheduler |
| Location awareness (x,y,z coordinates) |  |  | aGPS and UTDOA methods as per 3GPP spec |  | Timing Advanced based method as per 3GPP spec |  | aGPS and OTDOA methods as per 3GPP spec |  | aGPS and OTDOA methods as per 3GPP spec |  | A-GNSS, OTDOA, E-CID, UTDOA methods as per 3GPP spec |  | A-GNSS, E-CID methods as per 3GPP spec | E-CID method as per 3GPP spec |
| Ranging (distance reporting) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Encryption |  | Algorithms supported | A5/3, A5/4, GEA3 |  | KASUMI and SNOW 3G |  | KASUMI |  | KASUMI and SNOW 3G |  | SNOW 3G, AES, ZUC |  | SNOW 3G, AES, ZUC | SNOW 3G, AES, ZUC |
| Authentication |  |  | UE-to-NW (2G AKA) and mutual (3G AKA) |  | Mutual |  | UE-to-NW (2G AKA) and mutual (3G AKA) |  | UE-to-NW (2G AKA) and mutual (3G AKA) |  | Mutual |  | Mutual | Mutual |
| Replay protection in key exchange protocol |  |  | No (2G AKA) and yes (3G AKA) |  | Yes |  | No (2G AKA) and yes (3G AKA) |  | No (2G AKA) and yes (3G AKA) |  | Yes |  | Yes | Yes |
| Key exchange |  | Protocols and algorithms supported | Proprietary, 2G MILENAGE (2G AKA) and proprietary, MILENAGE, TUAK (3G AKA) |  | Proprietary, MILENAGE, TUAK |  | Proprietary, 2G MILENAGE (2G AKA) and proprietary, MILENAGE, TUAK (3G AKA) |  | Proprietary, 2G MILENAGE (2G AKA) and proprietary, MILENAGE, TUAK (3G AKA) |  | Proprietary, MILENAGE, TUAK |  | Proprietary, MILENAGE, TUAK | Proprietary, MILENAGE, TUAK |
| Interference sources |  |  | Other users, cells and networks |  | Other users, cells and networks |  | Other users, cells and networks |  | Other users, cells and networks |  | Other users, cells and networks |  | Other users, cells and networks | Other users, cells and networks |
| - Co-channel interference - Adjacent channel interference - Alternate channel interference - Collision avoidance - Protection mechanisms - Sensitivity to other interfering radio technologies - Degree of interference caused to other radio technologies - Sensitivity to power line RF emissions |  | Managed as per 3GPP specs and implementation | Managed as per 3GPP specs and implementation | Managed as per 3GPP specs and implementation | Managed as per 3GPP specs and implementation | Managed as per 3GPP specs and implementation | Managed as per 3GPP specs and implementation | Managed as per 3GPP specs and implementation |
| MAC address |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes | Yes |
| SIM card |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |
| Other identity |  |  | IMEI |  | IMEI |  | IMEI |  | IMEI |  | IMEI |  | IMEI | IMEI |
| Rogue detection |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |
| Base Standard SDO |  | SDO name | ATIS (3GPP Organizational Partner) |  | ATIS (3GPP Organizational Partner) |  | ATIS (3GPP Organizational Partner) |  | ATIS (3GPP Organizational Partner) |  | ATIS (3GPP Organizational Partner) |  | ATIS (3GPP Organizational Partner) | ATIS (3GPP Organizational Partner) |
| Profiling and Application Organizations |  | Association/Forum Name |  |  |  |  |  |  |  |  |  |  |  |  |
| Temperature range |  |  | As per 3GPP 45.005 |  | As per 3GPP 45.005 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| RF Noise sources - other radios |  |  | As per 3GPP 45.005 & 45.050 |  | As per 3GPP 45.005 & 45.050 |  | As per 3GPP 25.942 |  | As per 3GPP 25.942 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| RF Noise sources - other electrical equipment |  |  | As per 3GPP 45.005 & 45.050 |  | As per 3GPP 45.005 & 45.050 |  | As per 3GPP 25.943 |  | As per 3GPP 25.943 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| Rx sensitivity |  | dBm | As per 3GPP 45.005 |  | As per 3GPP 45.005 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| Tx Power peak |  | dBm | As per 3GPP 45.005 |  | As per 3GPP 45.005 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| Tx Power steps |  | dB | As per 3GPP 45.005 |  | As per 3GPP 45.005 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| Antenna gain |  | dBi | As per 3GPP 45.005 |  | As per 3GPP 45.005 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| Noise floor |  | dBm | As per 3GPP 45.050 |  | As per 3GPP 45.050 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 25.101 & 25.102 |  | As per 3GPP 36.101 & 36.104 |  | As per 3GPP 36.101 & 36.104 | As per 3GPP 36.101 & 36.104 |
| Modulation |  | GFSK, OFDM, BPSK, GMSK | GMSK, 8-PSK 16QAM/32QAM added in EGPRS2-A |  | GMSK, 8PSK |  | BPSK/QPSK |  | QPSK, 16QAM/64QAM |  | QPSK, 16QAM/64QAM/256QAM |  | QPSK, 16QAM | pi/2-BPSK, pi/4QPSK, QPSK |
| Forward error Coding |  |  | Punctured convolutional code |  | Punctured convolutional code |  | Convolutional and Turbo |  | Convolutional and Turbo |  | Turbo; Tail Biting Convolution on BCH |  | Turbo; Tail Biting Convolution on BCH | Turbo in UL; Tail Biting Convolution in DL |

### A1.4	3GPP2 Standards

[Editorial note: No changes in the remaining part of the Report]

____________________