3GPP TSG-SA5 Meeting #123	S5-191406

# Montreal, Canada, 21st Jan 2019 - 25th Jan 2019 							Revision of S5-191233

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0044 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct definition of Individual Partial record mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description of "Individual Partial record" mechanism is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a description of the "Individual Partial record" mechanism to the introduction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unclear definition of "Individual Partial record" mechanism will lead to different interpretations and inconsistent behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.1, 5.2.3.2.1, 5.2.3.2.2, 5.2.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.3.1	Introduction

The CHF CDRs for PDU session charging and roaming QBC are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in detail the conditions for opening, adding information to, and closing the CHF CDR.

Second change

##### 5.2.3.2.1	General

A PDU session charging CHF CDR is used to collect charging information related to the PDU session data information for a UE in Flow Based Charging. Multiple service data flow containers per PDU session can be added in the CHF CDR by the CHF, upon receiving a Charging Data Request including new counts as defined in subclause 5.2.1.1.

A CHF CDR shall be opened when the CHF receives Charging Data Request[Initial].

As an alternative to the default CHF behaviour, the "Individual Partial record" mechanism can be used based on Operator's policy configured in the CHF. In this case a new CDR shall be opened for each Charging Data Request[Initial, Update, Termination], charging information shall be added and the CDR shall then be closed. The Sequence Number will be incremented for each Charging Data Request[Initial, Update, Termination] received by the CHF.

Third change

##### 5.2.3.2.2	Triggers for CHF CDR charging information addition

When the CHF receives Charging Data Request[Update], with the change conditions identified in Table 5.2.3.2.2.1 the charging information shall be added in the PDU session charging CHF CDR, and the CDR shall remain open, as the default supported mechanism.

Table 5.2.3.2.2.1: Triggers for CHF CDR charging information addition

Trigger Conditions

Change of Charging conditions

QoS change

User Location change

AMF change

Change of UE presence in Presence Reporting Area(s)

Change of 3GPP PS Data off Status

Limit per Rating Group

Expiry of data time limit per RG

Expiry of data volume limit per RG

Expiry of data event limit per RG

Quota management triggers

Time threshold reached

Volume threshold reached

Unit threshold reached

Time quota exhausted

Volume quota exhausted

Unit quota exhausted

Expiry of quota validity time

Re-authorization request by CHF

In case the "Individual Partial record" mechanism is enabled, the Table 5.2.3.2.2.1 is not applicable. The charging information consists of a set of containers, which are added as "List of Multiple Unit Usage" parameter of the CHF CDR. Each container identifies the configured counts (volume separated for uplink and downlink, elapsed time or number of events) per rating group or combination of the rating group and service id, associated with the changed charging conditions.

Fourth change

##### 5.2.3.2.3	Triggers for CHF CDR partial record closure

When the CHF receives Charging Data Request[Update], with the change conditions identified in Table 5.2.3.2.3.1, the charging information shall be added in the PDU session charging CHF CDR, before the CDR is closed and a subsequent CHF CDR shall be opened with an incremented Sequence Number, as the default supported mechanism.

.

Table 5.2.3.2.3.1: Triggers for CHF CDR partial record closure

Trigger Conditions

Change of Charging conditions

UE time zone change

PLMN change

RAT type change

DNN-AMBR change

Removal of UPF

Management intervention

Limit per PDU session

Expiry of data time limit per PDU session

Expiry of data volume limit per PDU session

Expiry of data event limit per PDU session

Expiry of limit of number of charging condition changes

In case the "Individual Partial record" mechanism is enabled, the Table 5.2.3.2.3.1 is not applicable.

End of changes