3GPP TSG-SA5 Meeting #124 	S5-192192

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0062 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 5G JSON Solution Set to align with generic NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align 5G JSON solution set with JSON schema defined in generic NRM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated JSON defintion for NR RAN, 5GC and Network Slice to align with generic NRM Fixed syntax errors |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The JSON solution is incomplet and can’t be used by the developer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.3.2, D.4.3, G.3.2, G.4.3, J.3.2, J.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

## D.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| cellState | cellState | CellState |
| txDirection | txDirection | TxDirection |
| bwpContext | BwpContext | bwpContext |
| isInitialBwp | isInitialBwp | IsInitialBwp |
| configuredMaxTxPower | configuredMaxTxPower | ConfiguredMaxTxPower |
| fDDCenterFrequencyUL | fddCenterFrequencyUL | FddCenterFrequencyUL |
| fDDCenterFrequencyDL | fddCenterFrequencyDL | FddCenterFrequencyDL |
| fDDBandwidthUL | fddBandwidthUL | FddBandwidthUL |
| fDDBandwidthDL | fddBandwidthDL | FddBandwidthDL |
| localAddress | localAddress | IpEndPoint |
| remoteAddress | remoteAddress | IpEndPoint |
| gNBId | gnbId | GnbId |
| gNBDUId | gnbDuId | GnbDUId |
| gNBCUName | gnbCuName | gNBName |
| gNBDUName | gnbDuName | gNBName |
| nCI | nCi | NCi |
| nRPCI | nrPci | NrPCI |
| nRTac | nrTac | NrTac |
| sNSSAIList | snssaiList | SnssaiList |
| pointAArfcnULFDD | pointAArfcnULFDD | PointAArfcnULFDD |
| pointAArfcnDLFDD | pointAArfcnDLFDD | PointAArfcnDLFDD |
| pointAArfcnTDD | pointAArfcnTDD | PointAArfcnTDD |
| rRMPolicy | rrmPolicy | RrmPolicy |
| RRMPolicyRation2 | rrmPolicyRation2 | RrmPolicyRation2 |
| plmnId | plmnId | PlmnId |
| plmnIdList | plmnIdList |  | PlmnIdList |  |

End of 1st modification

Start of 2nd modification

## D.4.3	JSON schema "nrNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#",

"id": "http://3gpp.org/28541/nrNrm.json",

"description": "JSON based solution set definitions for NR NRM",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"gnbDuFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbDuId": {

"$ref": "#/definitions/GnbDuId"

},

"gnbDuName": {

"$ref": "#/definitions/GnbName"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

}

},

"required": [

"userLabel",

"gnbDuId",

"gnbId",

"gnbIdLength"

]

},

"gnbCuCpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

},

"gnbCuName": {

"$ref": "#/definitions/GnbName"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

"gnbIdLength",

"plmnIdList"

]

},

"gnbCuUpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

"gnbIdLength",

"plmnIdList"

]

},

"nrCellCu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"nCi": {

"$ref": "#/definitions/NCi"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

},

"snssaiList": {

"$ref": "#/definitions/SnssaiList"

},

"rrmPolicyType": {

"type": "integer"

},

"rrmPolicyNSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"rrmPolicyRatio": {

"type": "integer"

},

"rrmPolicy": {

"$ref": "#/definitions/RrmPolicy"

},

"rrmPolicyRatio2": {

"$ref": "#/definitions/RrmPolicyRatio2"

}

},

"required": [

"userLabel",

"nCI",

"plmnIdList"

]

},

"nrCellDu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AdministrativeState"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/OperationalState"

},

"availabilityStatus": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AvailabilityStatus"

},

"cellState": {

"$ref": "#/definitions/CellState"

},

"nCi": {

"$ref": "#/definitions/NCi"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

},

"snssaiList": {

"$ref": "#/definitions/SnssaiList"

},

"nrPci": {

"$ref": "#/definitions/NrPci"

},

"nrTac": {

"$ref": "#/definitions/NrTac"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"arfcnSUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"bSChannelBwSUL": {

"type": "integer"

}

},

"required": [

"userLabel",

"ngi",

"plmnIdList",

"nrPci",

"nrTac",

"arfcnDL",

"bSChannelBwDL"

]

},

"nrSectorCarrier": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"txDirection": {

"$ref": "#/definitions/TxDirection"

},

"configuredMaxTxPower": {

"type": "integer"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

},

"required": [

"userLabel",

"configuredMaxTxPower",

"txDirection"

]

},

"bwp": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"bwpContext": {

"$ref": "#/definitions/BwpContext"

},

"isInitialBwp": {

"$ref": "#/definitions/IsInitialBwp"

},

"subCarrierSpacing": {

"type": "integer"

},

"cyclicPrefix": {

"$ref": "#/definitions/CyclicPrefix"

},

"startRB": {

"type": "integer"

},

"numberOfRBs": {

"type": "integer"

},

},

"required": [

"userLabel",

"bwpContext",

"isInitialBwp",

"subCarrierSpacing",

"cyclicPrefix",

"startRB",

"numberOfRBs"

]

},

"eP_E1": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_XnC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_XnU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_NgC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_NgU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_F1C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_F1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_S1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_X2C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_X2U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"GnbId": {

"type": "string",

"description": "gNBId identifies a gNB within a PLMN."

},

"GnbIdLength": {

"type": "integer",

"description":  "This indicates the number of bits for encoding the gNB ID.",

"minimum": 22,

"maximum": 32

},

"GnbName": {

"type": "string",

"maxLength": 150

},

"GnbDuId": {

"type": "number",

"description": "GnbDuId represents the NB-DU ID defined in TS 38.473.",

"minimum": 0,

"maximum": 68719476735

},

"NCi": {

"type": "object",

"description": "NCgi uniquely identifies a NR cell  within a PLMN.",

"properties": {

"plmnId": {

"$ref": "#/definitions/PlmnId"

},

"nCI": {

"$ref": "#/definitions/NrCellId"

}

}

},

"SnssaiList": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"RrmPolicy": {

"type": "string",

"description": "RrmPolicy represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent.",

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"type": "string"

},

"ipv6Address": {

"type": "string"

},

"ipv6Prefix": {

"type": "string"

},

"transport": {

"type": "string"

},

"vlanId": {

"type": "integer"

}

}

},

"NrPci": {

"type": "integer",

"maximum": 503

},

"NrTac": {

"type": "integer",

"maximum": 16777215

},

"NrCellId": {

"type": "integer",

"maximum": 68719476735

},

"Sst": {

"type": "integer",

"maximum": 255

},

"Snssai": {

"type": "object",

"properties": {

"sst": {

"$ref": "#/definitions/Sst"

},

"sd": {

"type": "string"

}

}

},

"CellState": {

"enum": ["IDLE", "INACTIVE", "ACTIVE"]

},

"CyclicPrefix": {

"enum": ["15", "30", "60", "120"]

},

"TxDirection": {

"enum": ["DL", "UL", "DL and UL"]

},

"BwpContext": {

"enum": ["DL", "UL", "SUL"]

},

"IsInitialBwp": {

"enum": ["INITIAL", "OTHER", "SUL"]

},

"QuotaType": {

"enum": ["STRICT", "FLOAT"]

},

"RrmPolicyRatio2": {

"type": "object",

"properties": {

"groupId": {

"type": "integer"

},

"sNSSAIList": {

"$ref": "#/definitions/SnssaiList"

},

"quotaType": {

"$ref": "#/definitions/QuotaType"

},

"rRMPolicyMaxRation": {

"type": "integer"

},

"rRMPolicyMarginMaxRation": {

"type": "integer"

},

"rRMPolicyMinRation": {

"type": "integer"

},

"rRMPolicyMarginMinRation": {

"type": "integer"

}

}

},

"Mcc": {

"type": "string",

"pattern": "[02-79][0-9][0-9]"

},

"Mnc": {

"type": "string",

"pattern": "[0-9]{3}|[0-9]{2}"

},

"PlmnId": {

"type": "object",

"properties": {

"mcc": {

"$ref": "#/definitions/Mcc"

},

"mnc": {

"$ref": "#/definitions/Mnc"

}

}

},

"PlmnIdList": {

"type": "array",

"item": {

"$ref": "#/definitions/PlmnId"

}

}

}

}

End of 2nd modification

Start of 3rd modification

## G.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| aMFIdentifier | amfIdentifier | AmfIdentifier |
| aMFSetId | amfSetId | AmfSetId |
| aMFRegionId | amfRegionId | AmfRegionId |
| localAddress | localAddress | nrNrm:IPEndPoint |
| remoteAddress | remoteAddress | nrNrm:IPEndPoint |
| nfProfileList | nfProfileList | NfProfileList |
| nSIIdList | nSIIdList | NSIIdList |
| sNSSAIList | snssaiList | nrNrm:SnssaiList |
| plmnIdList | plmnIdList | nrNrm:PlmnIdList |
| sBIFQDN | sBIFQDN | SBIFqdn |
| sBIServiceList | sBIServiceList | SBIServiceList |
| nRTACList | tACList | TACList |
| weightFactor | weightFactor | WeightFactor |

End of 3rd modification

Start of 4th modification

## G.4.3	JSON schema "ngcNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#",

"id": "http://3gpp.org/28541/ngcNrm.json",

"description": "JSON based solution set definitions for 5GC",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"definitions": {

"amfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/definitions/AmfIdentifier"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"weightFactor": {

"$ref": "#/definitions/WeightFactor"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"amfSet": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

}

},

"required": [

"userLabel",

"plmnIdList",

"amfIdentifier",

"sBIFqdn",

"sBIServiceList",

"weightFactor"

]

},

"smfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"nRTACList",

"sBIFqdn",

"sBIServiceList"

]

},

"upfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"nRTACList"

]

},

"n3iwfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList"

]

},

"pcfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"ausfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udmFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udrFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udsfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"nrfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/definitions/NSIIdList"

},

"nFProfileList": {

"$ref": "#/definitions/NFProfileList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn"

]

},

"nssfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/definitions/NSIIdList"

},

"nFProfileList": {

"$ref": "#/definitions/NFProfileList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn"

]

},

"smsfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"lmfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList"

]

},

"ngeirFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"seppFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList",

]

},

"nwdafFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"eP_N2": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N3": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N4": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N5": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N6": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N7": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N8": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N9": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N10": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N11": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N12": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N13": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N14": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N15": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N16": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N17": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N20": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N21": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N22": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N26": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_SBI_X": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_SBI_IPX": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_S5C": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_S5U": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_Rx": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_MAP_SMSC": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_NLS": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_NLG": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"amfSet": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"plmnIdList",

"nRTACList",

"amfSetId"

]

},

"amfRegion": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"amfRegionId": {

"$ref": "#/definitions/AmfRegionId"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"plmnIdList",

"nRTACList",

"amfRegionId"

]

},

"AmfIdentifier": {

"type": "object",

"description": "AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer",

"properties": {

"amfRegionId": {

"$ref": "#/definitions/AmfRegionId"

},

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

},

"amfPointer": {

"$ref": "#/definitions/AmfPointer"

}

}

},

"AmfRegionId": {

"type": "integer",

"description": "AmfRegionId is defined in TS 23.003",

"maximum": 255

},

"AmfSetId": {

"type": "string",

"description": "AmfSetId is defined in TS 23.003",

"maximum": 1023

},

"AmfPointer": {

"type": "integer",

"description": "AmfPointer is defined in TS 23.003",

"maximum": 63

},

"nFProfileList": {

"type": "array",

"description": "List of NF profile",

"items": {

"$ref": "#/definitions/NFProfile"

}

},

"NFProfile": {

"type": "object",

"description": "NF profile stored in NRF, defined in TS 29.510",

"properties": {

"nFInstanceId": {

"type": "string",

"description": "uuid of NF instance"

},

"nFType": {

"$ref": "#/definitions/NFType"

},

"nFStatus": {

"$ref": "#/definitions/NFStatus"

},

"plmn": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnId"

},

"sNssais": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/Snssai"

},

"fqdn": {

"$ref": "#/definitions/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/definitions/Fqdn"

},

"ipv4Addresses": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv4Addr"

}

},

"ipv6Addresses": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv6Addr"

}

},

"ipv6Prefixes": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv6Prefix"

}

},

"capacity": {

"$ref": "#/definitions/Capacity"

},

"udrInfo": {

"$ref": "#/definitions/UdrInfo"

},

"amfInfo": {

"$ref": "#/definitions/amfInfo"

},

"smfInfo": {

"$ref": "#/definitions/SmfInfo"

},

"upfInfo": {

"$ref": "#/definitions/UpfInfo"

},

"nfServices": {

"type": "array",

"item": {

"$ref": "#/definitions/NFService"

}

}

}

},

"NFService": {

"type": "object",

"description": "NF Service is defined in TS 29.510",

"properties": {

"serviceInstanceId": {

"type": "string",

},

"serviceName": {

"type": "string",

},

"version": {

"type": "string",

},

"schema": {

"type": "string",

},

"fqdn": {

"$ref": "#/definitions/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/definitions/Fqdn"

},

"ipEndPoints": {

"type": "array",

"item": {

"$ref": "#/definitions/IpEndPoint"

}

},

"apiPrfix": {

"type": "string",

},

"defaultNotificationSubscriptions": {

"type": "array",

"item": {

"$ref": "#/definitions/DefaultNotificationSubscriptions"

}

},

"allowedPlmns": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnId"

},

"allowedNfTypes": {

"type": "array",

"item": {

"$ref": "#/definitions/NFType"

}

},

"allowedNssais": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"capacity": {

"$ref": "#/definitions/Capacity"

},

"supportedFeatures": {

"$ref": "#/definitions/SupportedFeatures"

}

}

},

"NFType": {

"type": "string",

"description": " NF name defined in TS 23.501",

"enum": [

"NRF",

"UDM",

"AMF",

"SMF",

"AUSF",

"NEF",

"PCF",

"SMSF",

"NSSF",

"UDR",

"LMF",

"GMLC",

"5G_EIR",

"SEPP",

"UPF",

"N3IWF",

"AF",

"UDSF",

"DN"

]

},

"Fqdn": {

"type": "string"

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "#/definitions/Ipv4Addr"

},

"ipv6Address": {

"$ref": "#/definitions/Ipv6Addr"

},

"ipv6Prefix": {

"$ref": "#/definitions/Ipv6Prefix"

},

"transport": {

"$ref": "#/definitions/TransportProtocol"

},

"port": {

"type": "integer"

}

}

},

"UdrInfo": {

"type": "object",

"properties": {

"supiRanges": {

"type": "array",

"items": {

"$ref": "#/definitions/SupiRange"

}

}

}

},

"SupiRange": {

"type": "object",

"properties": {

"start": {

"type": "string"

},

"end": {

"type": "string"

},

"pattern": {

"type": "string"

}

}

},

"AmfInfo": {

"type": "object",

"properties": {

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

}

}

},

"SmfInfo": {

"type": "object",

"properties": {

"dnnList": {

"type": "array",

"items": {

"$ref": "#/definitions/Dnn"

}

}

}

},

"UpfInfo": {

"type": "object",

"properties": {

"sNssaiUpfInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/SnssaiUpfInfoItem"

}

}

}

},

"SnssaiUpfInfoItem": {

"type": "object",

"properties": {

"sNssai": {

"$ref": "#/definitions/Snssai"

},

"dnnUpfInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/DnnUpfInfoListInfo"

}

}

}

},

"DnnUpfInfoItem": {

"type": "object",

"properties": {

"dnn": {

"$ref": "#/definitions/Dnn"

}

}

},

"DefaultNotificationSubscription": {

"type": "object",

"properties": {

"notificationType": {

"$ref": "#/definitions/NotificationType"

},

"callbackUri": {

"$ref": "#/definitions/Uri"

},

"n1MessageClass": {

"$ref": "#/definitions/N1MessageClass"

},

"n2InformationClass": {

"$ref": "#/definitions/n2InformationClass"

}

}

},

"NotificationType": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"N1_MESSAGES",

"N2_INFORMATION",

"LOCATION_NOTIFICATION"

]

},

"TranportProtocol": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"TCP"

]

},

"NFStatus": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"REGISTERED",

"SUSPENDED"

]

},

"NFRegistrationData": {

"type": "object",

"properties": {

"heartBeatTimer": {

"type": "integer"

},

"nfProfile": {

"$ref": "#/definitions/NFProfile"

}

}

},

"NSIIdList": {

"type": "array",

"items": {

"nSIId": {

"$ref": "#/definitions/NSIId"

}

}

},

"NSIId": {

"type": "string",

"description": "NSI Id is defined in TS 29.531"

},

"NrCellId": {

"type": "string",

},

"SBIFqdn": {

"$ref": "#/definitions/Fqdn"

},

"SBIServiceList": {

"type": "array",

"item": {

"$ref": "#/definitions/NFService"

}

},

"TACList": {

"type": "array",

"item": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/NrTac"

}

},

"weightFactor": {

"type": "integer"

}

}

}

End of 4th modification

Start of 5th modification

## J.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| nSSIId | nSSIId | genericNrm:Dn |
| serviceProfileId | serviceProfileId | string |
| sliceProfileId | sliceProfileId | string |
| constituentNSSIIdList | constituentNSSIIdList | genericNrm:DnList |
| mFList | mFList | genericNrm:DnList |
| operationalState | operationalState | stateManagementNrm: operationalStateType |
| administrativeState | administrativeState | stateManagementNrm: administrativeStateType |
| nsInfo | nsInfo | NsInfo |
| plmnIdList | plmnIdList | nrNrm:PlmnIdList |
| perfReq | perfReq | PerfReq |
| sNSSAIList | snssaiList | nrNrm:SnssaiList |
| maxNumberofUEs | maxNumberofUEs | number |
| coverageAreaTAList | coverageAreaTAList | ngcNrm:TACList |
| latency | latency | number |
| uEMobilityLevel | uEMobilityLevel | number |
| resourceSharingLevel | resourceSharingLevel | number |
| serviceProfileList | serviceProfileList | object |
| sliceProfileList | sliceProfileList | object |

End of 5th modification

Start of 6th modification

## J.4.3	JSON schema "sliceNrm.json"

{

"$schema": "http://json-schema.org/draft-04/schema#",

"id": "http://3gpp.org/28541/sliceNrm.json",

"description": "JSON based solution set definitions for network slice and network slice subnet",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"networkSlice": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"nSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/operationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/administrativeState"

},

"serviceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

}

},

"required": [

"userLabel",

"userDefinedNetworkType",

"nSSIId",

"operationalState",

"administrativeState",

"serviceProfileList"

]

},

"networkSliceSubnet": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"mFIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/DnList"

},

"constituentNSSIIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/DnList"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/OperationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AdministrativeState"

},

"nsInfo": {

"$ref": "#/definitions/NsInfo"

},

"sliceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

}

},

"required": [

"userLabel",

"mFIdList",

"constituentNSSIIdList",

"operationalState",

"administrativeState",

"sliceProfileList"

]

},

"MobilityLevel": {

"type": "string",

"enum": [

"STATIONARY",

"NOMADIC",

"RESTRICTED MOBILITY",

"FULLY MOBILITY",

]

},

"SharingLevel": {

"type": "string",

"enum": [

"SHARED",

"NON-SHARED"

]

},

"PrefReq": {

"type": "object",

"properties": {

"prefReqEmbb": {

"$ref": "#/definitions/PrefReqEmbb"

},

"prefReqUrllc": {

"$ref": "#/definitions/PrefReqUrllc"

}

}

},

"PerfReqEmbb":{

"type": "object",

"properties": {

"expDataRateDL": {

"type": "number"

},

"expDataRateUL": {

"type": "number"

},

"areaTrafficCapDL": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"userDensity": {

"type": "number"

},

"activityFactor": {

"type": "number"

},

"uESpeed": {

"type": "number"

},

"coverage": {

"type": "string"

}

}

},

"PerfReqUrrlc":{

"type": "object",

"properties": {

"e2eLatency": {

"type": "number"

},

"jitter": {

"type": "number"

},

"survivalTime": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"cSAvailability": {

"type": "number"

},

"reliability": {

"type": "number"

},

"expDataRate": {

"type": "number"

},

"payloadSize": {

"type": "number"

},

"trafficDensity": {

"type": "string"

},

"connDensity": {

"type": "number"

},

"serviceDimension": {

"type": "string"

}

}

},

"ServiceProfile": {

"type": "object",

"properties": {

"serviceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#/definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number"

},

"resourceSharingLevel": {

"type": "integer"

}

}

},

"ServiceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/ServiceProfile"

}

},

"SliceProfile": {

"type": "object",

"properties": {

"sliceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#/definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number"

},

"resourceSharingLevel": {

"type": "integer"

}

}

},

"SliceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/SliceProfile"

}

},

"NsInfo": {

"type": "object",

"properties": {

"nsInstanceId": {

"type": "string"

},

"nsName": {

"type": "string"

},

"description ": {

"type": "string"

}

}

}

}

}

End of 6th modification