3GPP TSG-SA5 Meeting #124 	S5-192326

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192193

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0063 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update YANG Solution Set to align with Stage 2 definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align YANG solution set with Stage 2 definition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated YANG definition for NR RAN, 5GC and Network Slice for alignment and syntax correction, as well as reflect the right revision of the TS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned stage 3 with IS definition potentially leading to incorrect implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7, E.4.3.1, E.4.3.2, E.4.3.4, E.4.3.5, E.4.3.6, E.4.3.7, E.4.3.8, E.4.3.9, E.4.3.10, H.4.3.1, H.4.3.2, H.4.3.3, H.4.3.4, H.4.3.5, H.4.3.6, H.4.3.7, H.4.3.8, H.4.3.9, H.4.3.10, H.4.3.12, H.4.3.14, H.4.3.15, H.4.3.16, H.4.3.19, K.4.3.1, K.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

# 7	Solution Set (SS)

The present document defines the following NRM Solution Set definitions for NR and NG-RAN:

-	XML based 3GPP NR and NG-RAN NRM Solution Set (Annex C).

-	JSON based 3GPP NR and NG-RAN NRM Solution Set (Annex D).

-	YANG based 3GPP NR and NG-RAN NRM Solution Set (Annex E).

The present document defines the following NRM Solution Set definitions for 5GC:

-	XML based 3GPP 5GC NRM Solution Set (Annex F).

-	JSON based 3GPP 5GC NRM Solution Set (Annex G).

-	YANG based 3GPP 5GC NRM Solution Set (Annex H).

The present document defines the following NRM Solution Set definitions for network slice and network slice subnet:

-	XML based 3GPP Network Slice NRM Solution Set (Annex I).

-	JSON based 3GPP Network Slice NRM Solution Set (Annex J).

-	YANG based 3GPP Network Slice NRM Solution Set (Annex K).

End of 1st modification

Start of 2nd modification

## E.4.3	YANG schema

### E.4.3.1	General type definition "nrm-types-3gpp.yang"

module nrm-types-3gpp {

namespace "urn:3gpp:tsg:sa5:nrm:types";

prefix "nrm-type";

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

import ietf-yang-types { prefix yang; revision-date "2010-09-24"; }

revision 2018-07-31 {

description

"Initial revision.";

}

typedef t_dn {

type string;

description

"The dn type represents a Distinguish Name of the object.";

}

typedef t_mcc {

description "The mobile country code consists of three decimal digits, The first digit of the mobile country code identifies the geographic region (the digits 1 and 8 are not used):";

type string {

pattern '[02-79][0-9][0-9]';

}

}

typedef t_mnc {

description "the mobile network code consists of two or three decimal digits (for example: MNC of 001 is not the same as MNC of 01)";

type string {

pattern '[0-9][0-9][0-9]|[0-9][0-9]';

}

}

grouping pLMNId {

leaf MCC {

type t_mcc;

}

leaf MNC {

type t_mnc;

}

}

typedef t_NCI {

description "The NCI shall be of fixed length of 36 bits and shall be coded using full hexadecimal representation. The exact coding of the NCI is the responsibility of each PLMN operator";

reference "23.003";

type union {

type string {

length 36;

pattern '[01]*';

}

type string {

length 9;

pattern '[a-fA-F0-9]*';

}

}

}

typedef t_operationalState {

type enumeration {

enum "Enabled";

enum "Disabled";

}

}

typedef t_administrativeState {

type enumeration {

enum "Locked";

enum "Shutdown";

enum "Unlocked";

}

}

typedef t_availabilityStatus {

type enumeration {

enum "IN TEST";

enum "FAILED";

enum "POWER OFF";

enum "OFF LINE";

enum "OFF DUTY";

enum "DEPENDENCY";

enum "DEGRADED";

enum "NOT INSTALLED";

enum "LOG FULL";

}

}

typedef t_cellState {

type enumeration {

enum "Idle";

enum "Inactive";

enum "Active";

}

}

typedef t_s-NSSAI {

type union {

type uint8;

type uint32;

}

}

typedef t_SST {

type uint8;

}

typedef t_nRPCI {

type uint32;

}

typedef t_tAC {

type union {

type string {

length 4;

pattern '[a-fA-F0-9]*';

}

type string {

length 6;

pattern '[a-fA-F0-9]*';

}

}

}

typedef t_subcarrierSpacing {

type enumeration {

enum 15;

enum 30;

enum 60;

enum 120;

}

}

typedef t_cyclicPrefix {

type enumeration {

enum "Normal";

enum "Extended";

}

}

typedef t_aMF-Region-id {

type string {

length 8;

pattern '[01]*';

}

}

typedef t_aMF-Set-id {

type string {

length 10;

pattern '[01]*';

}

}

typedef t_aMF-Pointer {

type string {

length 6;

pattern '[01]*';

}

}

grouping aMFI {

leaf AMF-Region-id {

type t_aMF-Region-id;

}

leaf AMF-Set-id {

type t_aMF-Set-id;

}

leaf AMF-Pointer {

type t_aMF-Pointer;

}

}

typedef t_TransportProtocol {

type enumeration {

enum "TCP";

enum "STCP";

enum "UDP";

}

}

// type definition especially for core NF

typedef NFType {

type enumeration {

enum "NRF";

enum "UDM";

enum "AMF";

enum "SMF";

enum "AUSF";

enum "NEF";

enum "PCF";

enum "SMSF";

enum "NSSF";

enum "UDR";

enum "LMF";

enum "GMLC";

enum "5G_EIR";

enum "SEPP";

enum "UPF";

enum "N3IWF";

enum "AF";

enum "UDSF";

enum "BSF";

enum "CHF";

}

}

typedef NFStatus {

type enumeration {

enum "REGISTERED";

enum "SUSPENDED";

}

}

typedef NotificationType {

type enumeration {

enum "N1_MESSAGES";

enum "N2_INFORMATION";

enum "LOCATION_NOTIFICATION";

}

}

typedef NotificationEventType {

type enumeration {

enum "NF_REGISTERED";

enum "NF_DEREGISTERED";

enum "NF_PROFILE_CHANGED";

}

}

typedef DataSetId {

type enumeration {

enum "SUBSCRIPTION";

enum "POLICY";

enum "EXPOSURE";

enum "APPLICATION";

}

}

typedef UPInterfaceType {

type enumeration {

enum "N3";

enum "N6";

enum "N9";

}

}

typedef N1MessageClass {

type enumeration {

enum "5GMM";

enum "SM";

enum "LPP";

enum "SMS";

}

}

typedef N2InformationClass {

type enumeration {

enum "SM";

enum "NRPPa";

enum "PWS";

enum "PWS-BCAL";

enum "PWS-RF";

}

}

typedef t_Load {

description "Latest known load information of the NF ranged from 0 to 100 in percentage ";

type uint8 {

range 0..100;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping SupiRange {

leaf start {

type string;

}

leaf end {

type string;

}

leaf pattern {

type string;

}

}

grouping IdentityRange {

leaf start {

type string;

}

leaf end {

type string;

}

leaf pattern {

type string;

}

}

grouping Guami {

container plmnId { uses pLMNId; }

container amfId { uses aMFI; }

}

grouping Tai {

container plmnId { uses pLMNId; }

leaf tac { type t_tAC; }

}

typedef t_Dnn {

type string;

}

grouping DnnUpfInfoItem {

leaf dnn {

type t_Dnn;

}

}

grouping SnssaiUpfInfoItem {

leaf sNssai {

type t_s-NSSAI;

}

list dnnUpfInfo {

key dnn;

uses DnnUpfInfoItem;

}

}

grouping NFServiceVersion {

description "";

leaf apiVersionInUri {

description "";

type string;

}

leaf apiFullVersion {

type string;

description "";

}

leaf expiry {

description "";

type yang:date-and-time;

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type t_TransportProtocol;

}

leaf port {

type uint16;

}

}

grouping defaultNotificationSubscription {

leaf notificationType {

type NotificationType;

}

leaf callbackUri {

type inet:uri;

}

leaf n1MessageClass {

type N1MessageClass;

}

leaf n2InformationClass {

type N2InformationClass;

}

}

grouping InterfaceUpfInfoItem {

leaf interfaceType {

type UPInterfaceType;

}

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

leaf endpointFqdn {

type inet:domain-name;

}

}

leaf networkInstance {

type string;

}

}

grouping Ipv4AddressRange {

leaf start {

type inet:ipv4-address;

}

leaf end {

type inet:ipv4-address;

}

}

grouping Ipv6PrefixRange {

leaf start {

type inet:ipv6-prefix;

}

leaf end {

type inet:ipv6-prefix;

}

}

typedef t_NsiId {

type string;

}

typedef t_uEMobilityLevel {

type enumeration {

enum "stationary";

enum "nomadic";

enum "restricted mobility";

enum "fully mobility";

}

}

typedef t_resourceSharingLevel {

type enumeration {

enum "shared";

enum "not-shared";

}

}

typedef TxDirection {

type enumeration {

enum "DL";

enum "UL";

enum "DL and UL";

}

}

typedef BwpContext {

type enumeration {

enum "DL";

enum "UL";

enum "SUL";

}

}

typedef IsInitialBwp {

type enumeration {

enum "INITIAL";

enum "OTHER";

}

}

typedef t_quotaType {

type enumeration {

enum "strict";

enum "float";

}

}

grouping RRMPolicyRatio2{

leaf groupId {

type uint16;

}

leaf-list sNSSAI {

type t_s-NSSAI;

}

leaf quotaType {

type t_quotaType;

}

leaf rRMPolicyMaxRation {

type uint8;

}

leaf rRMPolicyMarginMaxRation {

type uint8;

}

leaf rRMPolicyMinRation {

type uint8;

}

leaf rRMPolicyMarginMinRation {

type uint8;

}

}

}

### E.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "Subnetwork.yang", “vsDataContainer.yang”, “MeContext.yang”, “ManagementNode.yang”

module Top {

namespace "urn:3gpp:tsg:sa5:nrm:Top";

prefix "top";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

organization "3gpp SA5";

description "root class to be inherited/reused by other child class including ME, MF, EP, etc.";

reference "28.622";

revision 2018-07-31 {

description "Initial revision";

}

grouping Top {

leaf objectClass {

mandatory "true";

config "false";

description "";

type string;

}

leaf objectInstance {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

description "";

}

}

module SubNetwork {

namespace "urn:3gpp:tsg:sa5:nrm:SubNetwork";

prefix "subnet";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

organization "3gpp SA5";

description "define basic SubNetwork which will be augmented by other IOCs";

revision 2018-07-31 {

description "Initial revision";

}

grouping SubNetwork {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "derived from top_";

type nrm-type:t_dn;

}

leaf dnPrefix {

mandatory "true";

config "false";

description "derived from domain_";

type string;

}

leaf userLabel {

description "derived from domain_";

mandatory "true";

config "true";

type string;

}

leaf userDefinedNetworkType {

description "derived from domain_";

mandatory "true";

config "true";

type string;

}

leaf-list setOfMcc {

min-elements "0";

description "";

config "false";

type nrm-type:t_mcc;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

container SubNetwork {

uses SubNetwork;

}

}

module ManagedElement {

namespace "urn:3gpp:tsg:sa5:nrm:ManagedElement";

prefix "me";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

organization "3gpp SA5";

description "define basic ManagedElement which will be augmented by other IOCs";

revision 2018-07-31 {

description "Initial revision";

}

grouping ManagedElement {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf vendorName {

mandatory "true";

config "false";

description "";

type string;

}

leaf userDefinedState {

mandatory "true";

config "true";

description "";

type string;

}

leaf swVersion {

mandatory "true";

config "false";

description "";

type string;

}

leaf dnPrefix {

mandatory "true";

config "false";

description "";

type string;

}

leaf userLabel {

description "";

mandatory "true";

config "true";

type string;

}

leaf locationName {

mandatory "true";

config "false";

type string;

}

leaf-list managedBy {

description "";

config "false";

type nrm-type:t_dn;

}

leaf-list managedElementTypeList {

min-elements "1";

config "false";

description "";

type string;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

}

module ManagedFunction {

namespace "urn:3gpp:tsg:sa5:nrm:ManagedFunction";

prefix "mf";

import Top { prefix top; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

description "common/basic MF class to be inherited/reused by other child MF classes ";

revision 2018-07-31 {

description "Initial revision";

}

grouping  ManagedFunction {

description "";

uses top:Top;

leaf id {

description "";

mandatory "true";

config "false";

type nrm-type:t_dn;

}

leaf userLabel {

mandatory "false";

config "true";

type string;

}

container vnfParametersList {

description "It shall be supported if the MF instance is realized by one or more VNF instance(s). Otherwise this attribute shall be absent ";

leaf vnfInstanceId {

mandatory "true";

type string;

}

leaf vnfdId  {

type string;

}

leaf flavourId  {

description "Identifier of the VNF Deployment Flavour applied to this VNF instance";

reference "clause 9.4.3 of ETSI GS NFV-IFA 008 ";

type string;

}

leaf autoScalable {

description "Indicator of whether the auto-scaling of this VNF instance is enabled or disabled. The type is Boolean";

type boolean;

}

}

container peeParametersList {

description "It shall be supported if the control and monitoring of PEE parameters is supported by the ManagedFunction or sub-class instance.";

leaf siteIdentification {

mandatory "true";

description "The identification of the site where the ManagedFunction resides.";

type string;

}

leaf siteLatitude  {

description "The latitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global referenceframe (WGS 84). Positive values correspond to the northern hemisphere. This attribute is optional in case of BTSFunction and RNCFunction  instance(s). allowedValues: -90.0000 to +90.0000 ";

when "contains(../../objectClass,'BTSFunction') or contains(../../objectClass,'RNCFunction')";

type decimal64 {

fraction-digits 4;

range "-90.0000 .. 90.0000";

}

}

leaf siteLongitude {

description "The longitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. This attribute is optional in case of BTSFunction and RNCFunction instance(s). allowedValues: -180.0000to +180.0000 ";

when "contains(../../objectClass,'BTSFunction') or contains(../../objectClass,'RNCFunction')";

type decimal64 {

fraction-digits 4;

range "-180.0000 .. 180.0000";

}

}

leaf siteDescription {

mandatory "true";

description "An operator defined description of the site where the ManagedFunction instance resides.";

type string;

}

leaf equipmentType {

mandatory "true";

description "The type of equipment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. ";

type enumeration {

enum RRU;

enum BBU;

enum Wide-area-BS-cabinet;

enum Medium-range-BS;

}

}

leaf environmentType {

mandatory "true";

description "The type of environment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. ";

type enumeration {

enum Indoor;

enum Outdoor;

}

}

leaf powerInterface {

mandatory "true";

description "The type of power. allowedValues: see clause 4.4.1 of ETSI ES202 336-12 [18]. ";

type enumeration {

enum AC;

enum DC;

}

}

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

}

module EP_RP {

namespace "urn:3gpp:tsg:sa5:nrm:EP_RP";

prefix "ep-rp";

import Top { prefix top; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

description "common/basic MF class to be inherited/reused by other child MF classes ";

revision 2018-07-31 {

description "Initial revision";

}

grouping local-address {

leaf ip_address {

type inet:ip-address;

}

leaf vlan_id {

type uint16;

}

}

grouping EP_RP {

description "";

uses top:Top;

leaf id {

description "";

mandatory "true";

type nrm-type:t_dn;

}

leaf userLabel {

description "";

mandatory "false";

type string;

}

leaf farEndEntity {

mandatory "false";

type nrm-type:t_dn;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

grouping EP_XX {

description "as all EPs defined in NGRAN and 5GC have same attributes, define a common EP for all endpoints for reusing";

uses EP_RP;

container localAddress  {

presence "true";

description "";

uses local-address;

}

leaf remoteAddress  {

description "";

mandatory "false";

type inet:ip-address;

}

}

grouping EP_E1 {

description "";

uses EP_XX;

}

grouping EP_F1C {

description "";

uses EP_XX;

}

grouping EP_F1U {

description "";

uses EP_XX;

}

grouping EP_XnC {

description "";

uses EP_XX;

}

grouping EP_XnU {

description "";

uses EP_XX;

}

grouping EP_NgC {

description "";

uses EP_XX;

}

grouping EP_NgU {

description "";

uses EP_XX;

}

grouping EP_X2C {

description "";

uses EP_XX;

}

grouping EP_X2U {

description "";

uses EP_XX;

}

grouping EP_S1U {

description "";

uses EP_XX;

}

grouping EP_N2 {

description "";

uses EP_XX;

}

grouping EP_N3 {

description "";

uses EP_XX;

}

grouping EP_N4 {

description "";

uses EP_XX;

}

grouping EP_N5 {

description "";

uses EP_XX;

}

grouping EP_N6 {

description "";

uses EP_XX;

}

grouping EP_N7 {

description "";

uses EP_XX;

}

grouping EP_N8 {

description "";

uses EP_XX;

}

grouping EP_N9 {

description "";

uses EP_XX;

}

grouping EP_N10 {

description "";

uses EP_XX;

}

grouping EP_N11 {

description "";

uses EP_XX;

}

grouping EP_N12 {

description "";

uses EP_XX;

}

grouping EP_N13 {

description "";

uses EP_XX;

}

grouping EP_N14 {

description "";

uses EP_XX;

}

grouping EP_N15 {

description "";

uses EP_XX;

}

grouping EP_N16 {

description "";

uses EP_XX;

}

grouping EP_N17 {

description "";

uses EP_XX;

}

grouping EP_N20 {

description "";

uses EP_XX;

}

grouping EP_N21 {

description "";

uses EP_XX;

}

grouping EP_N22 {

description "";

uses EP_XX;

}

grouping EP_N26 {

description "";

uses EP_XX;

}

grouping EP_S5C {

description "";

uses EP_XX;

}

grouping EP_S5U {

description "";

uses EP_XX;

}

grouping EP_Rx {

description "";

uses EP_XX;

}

grouping EP_MAP_SMSC {

description "";

uses EP_XX;

}

grouping EP_NLS {

description "";

uses EP_XX;

}

grouping EP_NLG {

description "";

uses EP_XX;

}

grouping EP_N27 {

description "";

uses EP_XX;

}

grouping EP_N31 {

description "";

uses EP_XX;

}

grouping EP_N32 {

description "";

uses EP_XX;

}

grouping EP_SBI_X {

description "It's used for the services consumed by the contol plane function. Far entity and remoteAdress are requied for this endpoint";

uses EP_XX;

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

}

grouping EP_SBI_IPX {

description "It's used for the SEPP services consumed by the contol plane function. Far entity and remoteAdress are requied for this endpoint";

uses EP_XX;

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

}

grouping sAP {

leaf dn {

type inet:domain-name;

}

leaf port {

type uint16;

}

}

grouping EP_NBI_X {

description "The service access point of the control plane function. It's used for the services (including SEPP service) provided by the contol plane function. Far entity and remoteAdress are uncessary for this endpoint ";

uses EP_RP;

container localSAP  {

presence "true";

description "";

uses sAP;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

}

}

module ManagementNode {

namespace "urn:3gpp:tsg:sa5:nrm:ManagedElement";

prefix "managementNode";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

organization "3gpp SA5";

revision 2018-12-18 {

description "15.1.0";

}

container ManagementNode {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf userLabel {

description "";

mandatory "true";

config "true";

type string;

}

leaf-list managedElements {

description "";

config "false";

type nrm-type:t_dn;

}

leaf vendorName {

mandatory "true";

config "false";

description "";

type string;

}

leaf locationName {

mandatory "true";

config "false";

type string;

}

leaf userDefinedState {

mandatory "true";

config "true";

description "";

type string;

}

leaf swVersion {

mandatory "true";

config "false";

description "";

type string;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

}

module MeContext {

namespace "urn:3gpp:tsg:sa5:nrm:MeContext";

prefix "meContext";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

organization "3gpp SA5";

revision 2018-12-18 {

description "Initial Version";

}

container MeContext {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "derived from top_";

type nrm-type:t_dn;

}

leaf dnPrefix {

mandatory "true";

config "false";

description "derived from domain_";

type string;

}

}

}

module VsDataContainer {

namespace "urn:3gpp:tsg:sa5:nrm::VsDataContainer";

prefix "vsData";

import Top { prefix top; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

revision 2018-12-18 {

description "Initial revision";

}

grouping VsDataContainer {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "derived from top_";

type nrm-type:t_dn;

}

leaf vsDataType {

mandatory "true";

config "true";

description "";

type string;

}

leaf vsDataFormatVersion {

mandatory "true";

config "false";

description "";

type string;

}

anydata vsData;

list vsDataContainer {

min-elements "0";

uses VsDataContainer;

}

}

}

### E.4.3.4	NGRAN submodule, "ngran-gNBCUCPFunction.yang"

submodule ngran-gNBCUCPFunction {

belongs-to ngran { prefix ngran; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include ngran-nRCellCU;

description "GNBCUCPFunction derived from basic ManagedFunction, for 3 split deployment mode only";

revision 2018-08-02 {

description "15.0.2";

}

grouping GNBCUCPFunction {

uses mf:ManagedFunction;

leaf gNBCUName {

description "PrintableString(SIZE(1..150,))";

mandatory "false";

config "true";

type string {

length "1..150";

}

}

leaf gNBId {

description "It identifies a gNB within a PLMN. See gNB Identifier (gNB ID) of subclause 8.2 of 3GPP TS 38.300 [3]). See Global gNB ID in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. ";

mandatory "true";

config "false";

type uint32;

}

leaf gNBIdLength {

description "It indicates the number of bits for encoding the gNB ID.";

mandatory "true";

config "false";

type uint32 {

range "22..32";

}

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

choice gNB-type {

case gNB {

list EP-NgC { uses ep-rp:EP_NgC; }

list EP-XnC { uses ep-rp:EP_XnC; }

}

case en-gNB {

container EP-X2C { uses ep-rp:EP_X2C; }

}

}

list EP_F1C { uses ep-rp:EP_F1C; }

list EP_E1 { uses ep-rp:EP_E1; }

list NRCellCU {

uses NRCellCU;

}

}

}

### E.4.3.5	NGRAN submodule, "ngran-gNBCUUPFunction.yang"

submodule ngran-gNBCUUPFunction {

belongs-to ngran { prefix ngran; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "GNBCUUPFunction derived from basic ManagedFunction, for 3 split deployment mode only";

revision 2018-08-02 {

description "15.0.2";

}

grouping GNBCUUPFunction {

uses mf:ManagedFunction;

leaf gNBId {

description "It identifies a gNB within a PLMN. See gNB Identifier (gNB ID) of subclause 8.2 of 3GPP TS 38.300 [3]). See Global gNB ID in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. ";

mandatory "true";

config "false";

type uint32;

}

leaf gNBIdLength {

description "It indicates the number of bits for encoding the gNB ID.";

mandatory "true";

config "false";

type uint32 {

range "22..32";

}

}

list pLMNId {

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_F1U { uses ep-rp:EP_F1U; }

container EP_E1 { uses ep-rp:EP_E1; }

choice gNB-type {

case gNB {

list EP_NgU { uses ep-rp:EP_NgU; }

list EP_XnU { uses ep-rp:EP_XnU; }

}

case en-gNB {

list EP_X2U { uses ep-rp:EP_X2U; }

list EP_S1U { uses ep-rp:EP_S1U; }

}

}

}

}

### E.4.3.6	NGRAN submodule, "ngran-gNBDUFunction.yang"

submodule ngran-gNBDUFunction {

belongs-to ngran { prefix ngran; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

include ngran-nRCellDU;

include ngran-nRSectorCarrier;

description "GNBDUFunction derived from basic ManagedFunction, for 3 split deployment mode only";

revision 2018-08-02 {

description "15.0.2";

}

grouping GNBDUFunction {

uses mf:ManagedFunction;

leaf gNBDUId {

description "It uniquely identifies the DU at least within a gNB. See gNB-DU ID in subclause 9.3.1.9 of 3GPP TS 38.473 ";

mandatory "true";

config "true";

type uint32;

}

leaf gNBDUName {

description "PrintableString(SIZE(1..150,))";

mandatory "false";

config "true";

type string {

length "1..150";

}

}

leaf gNBId {

description "It identifies a gNB within a PLMN. See gNB Identifier (gNB ID) of subclause 8.2 of 3GPP TS 38.300 [3]). See Global gNB ID in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. ";

mandatory "true";

config "false";

type uint32;

}

leaf gNBIdLength {

description "It indicates the number of bits for encoding the gNB ID.";

mandatory "true";

config "false";

type uint32 {

range "22..32";

}

}

container EP_F1C { uses ep-rp:EP_F1C; }

list EP_F1U { uses ep-rp:EP_F1U; }

list NRCellDU {

uses NRCellDU;

}

list NRSectorCarrier {

uses NRSectorCarrier;

}

list BWP {

uses BWP;

}

}

}

### E.4.3.7	NGRAN submodule, "ngran-nRCellCU.yang"

submodule ngran-nRCellCU {

belongs-to ngran { prefix ngran; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "NRCellCU derived from basic ManagedFunction";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRCellCU {

uses mf:ManagedFunction;

leaf nCI {

config "true";

description "It uniquely identifies a NR cell within a PLMN.";

reference "See NR Cell identifier (NCI) subclause 8.2 of 3GPP

TS 38.300 [3].";

type nrm-type:t_NCI;

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key MCC;

uses nrm-type:pLMNId;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPPTS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf rRMPolicyType {

config "true";

type uint16;

}

leaf rRMPolicyNSSIId {

config "true";

type nrm-type:t_dn;

}

leaf rRMPolicyRatio {

config "true";

type uint8 {	range 0..100; }

}

leaf rRMPolicy {

description "It represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent.";

config "true";

type string;

}

container rRMPolicyRatio2 {

presence "true";

config "true";

uses nrm-type:RRMPolicyRatio2;

}

}

}

### E.4.3.8	NGRAN submodule, "ngran- nRCellDU.yang"

submodule ngran-nRCellDU {

belongs-to ngran { prefix ngran; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "NRCellDU derived from basic ManagedFunction";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRCellDU {

uses mf:ManagedFunction;

leaf nCI {

config "true";

description "It uniquely identifies a NR cell within a PLMN.";

reference "See NR Cell identifier (NCI) subclause 8.2 of 3GPP TS 38.300 [3].";

type nrm-type:t_NCI;

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key MCC;

uses nrm-type:pLMNId;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

type nrm-type:t_s-NSSAI;

}

leaf operationalState  {

config "false";

description "CM, It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled).";

type nrm-type:t_operationalState;

}

leaf administrativeState  {

description "CM, It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services.";

config "true";

type nrm-type:t_administrativeState;

}

leaf availabilityStatus  {

config "false";

description "CM";

type nrm-type:t_availabilityStatus;

}

leaf cellState  {

description "It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active)";

mandatory "true";

config "false";

type nrm-type:t_cellState;

}

leaf nRPCI {

description "";

mandatory "true";

config "true";

type nrm-type:t_nRPCI;

}

leaf nRTAC {

mandatory "true";

config "true";

type nrm-type:t_tAC;

}

leaf arfcnDL {

mandatory "true";

config "true";

type uint32;

}

leaf arfcnUL {

config "true";

type uint32;

}

leaf arfcnSUL {

config "true";

type uint32;

}

leaf bSChannelBwDL {

mandatory "true";

config "true";

type uint32;

}

leaf bSChannelBwUL {

config "true";

type uint32;

}

leaf bSChannelBwSUL {

config "true";

type uint32;

}

//refer to nRSectorCarrier

leaf-list NRSectorCarrier {

min-elements "0";

description "";

config "true";

type nrm-type:t_dn;

}

//refer to bwp

leaf-list bWP {

min-elements "0";

description "";

config "true";

type nrm-type:t_dn;

}

}

}

### E.4.3.9	NGRAN submodule, "ngran-nRSectorCarrier.yang"

submodule ngran-nRSectorCarrier {

belongs-to ngran { prefix ngran; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

description "NRSectorCarrier";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRSectorCarrier {

description "";

uses mf:ManagedFunction;

leaf txDirection {

mandatory "true";

config "true";

type nrm-type:TxDirection;

}

leaf configuredMaxTxPower {

mandatory "true";

config "false";

description "";

type uint16;

}

leaf arfcnDL {

config "true";

type uint32;

}

leaf arfcnUL {

config "true";

type uint32;

}

leaf bSChannelBwDL {

config "true";

type uint32;

}

leaf bSChannelBwUL {

config "true";

type uint32;

}

leaf sectorEquipmentFunction {

mandatory "true";

config "true";

description "fqBand for NR should be defined";

reference "23.622";

type nrm-type:t_dn;

}

}

}

#### E.4.3.10	NGRAN submodule, "ngran-bWP.yang"

submodule ngran-bWP {

belongs-to ngran { prefix ngran; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

description "BWP";

revision 2018-09-28 {

description "Initial revision";

}

grouping BWP {

description "";

uses mf:ManagedFunction;

leaf bwpContext {

mandatory "true";

config "true";

type nrm-type:BwpContext;

}

leaf isInitialBwp {

mandatory "true";

config "false";

description "";

type nrm-type:IsInitialBwp;

}

leaf subCarrierSpacing {

mandatory "true";

config "true";

type uint32;

}

leaf cyclicPrefix {

mandatory "true";

config "true";

type nrm-type:t_cyclicPrefix;

}

leaf startRB {

mandatory "true";

config "true";

type uint32;

}

leaf numberOfRBs {

mandatory "true";

config "true";

type uint32;

}

}

}

End of 2nd modification

Start of 3rd modification

## H.4.3	YANG schema

### H.4.3.1	General type definition "nrm-types-3gpp.yang", refer to Annex E.4.3.1

### H.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork", refer to Annex E.4.3.2

### H.4.3.3	NGC module, composition of all 5GC ManagedFunctions in "ngc.yang"

module ngc {

namespace "urn:3gpp:tsg:sa5:nrm:ngc";

prefix "ngc";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import ManagedElement { prefix me; revision-date "2018-07-31"; }

include ngc-AMFFunction;

include ngc-SMFFunction;

include ngc-UPFFunction;

include ngc-N3IWFFunction;

include ngc-PCFFunction;

include ngc-AUSFFunction;

include ngc-UDMFunction;

include ngc-UDRFunction;

include ngc-UDSFFunction;

include ngc-NRFFunction;

include ngc-NSSFFunction;

include ngc-SMSFFunction;

include ngc-LMFFunction;

include ngc-NGEIRFunction;

include ngc-SEPPFunction;

include ngc-NWDAFunction;

include NFProfile;

revision 2018-08-07 {

description "Initial revision";

}

container SubNetwork {

uses subnet:SubNetwork;

list ManagedElement {

uses me:ManagedElement;

list AMFFunction { uses AMFFunction; }

list SMFFunction { uses SMFFunction; }

list UPFFunction { uses UPFFunction; }

list N3IWFFunction { uses N3IWFFunction; }

list PCFFunction { uses PCFFunction; }

list AUSFFunction { uses AUSFFunction; }

list UDMFunction { uses UDMFunction; }

list UDRFunction { uses UDRFunction; }

list UDSFFunction { uses UDSFFunction; }

list NRFFunction { uses NRFFunction; }

list NSSFFunction { uses NSSFFunction; }

list SMSFFunction { uses SMSFFunction; }

list LMFFunction { uses LMFFunction; }

list NGEIRFunction { uses NGEIRFunction; }

list SEPPFunction { uses SEPPFunction; }

list NWDAFunction { uses NWDAFunction; }

}

list AMFSet { uses AMFSet; }

list AMFRegion { uses AMFRegion; }

}

}

### H.4.3.4	NGC submodule, "ngc-AMFFunction.yang", “ngc-AMFRegion.yang”, “ngc-AMFSet.yang”

submodule ngc-AMFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "AMFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping AMFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

container aMFIdentifier {

presence "true";

config "true";

description "";

uses nrm-type:aMFI;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description " ";

type inet:domain-name;

}

leaf-list sBIService {

description "";

min-elements "1";

config "false";

type string;

}

leaf weightFactor {

mandatory "true";

config "true";

description "";

type uint8;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N2 { uses ep-rp:EP_N2; }

list EP_N8 { uses ep-rp:EP_N8; }

list EP_N11 { uses ep-rp:EP_N11; }

list EP_N12 { uses ep-rp:EP_N12; }

list EP_N14 { uses ep-rp:EP_N14; }

list EP_N15 { uses ep-rp:EP_N15; }

list EP_N17 { uses ep-rp:EP_N17; }

list EP_N20 { uses ep-rp:EP_N20; }

list EP_N22 { uses ep-rp:EP_N22; }

list EP_N26 { uses ep-rp:EP_N26; }

list EP_NLS { uses ep-rp:EP_NLS; }

list EP_NLG { uses ep-rp:EP_NLS; }

list EP_SBI_X {

uses ep-rp:EP_SBI_X;

}

}

}

submodule ngc-AMFRegion {

belongs-to ngc { prefix ngc; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

revision 2018-12-18 {

description "initial verison";

}

grouping AMFRegion {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf aMFRegionId {

config "true";

description "";

type nrm-type:t_aMF-Region-id;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf-list aMFSet {

description "AMF Set list in the aMFRegion";

min-elements "0";

config "true";

type nrm-type:t_dn;

}

}

}

submodule ngc-AMFSet {

belongs-to ngc { prefix ngc; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

revision 2018-12-18 {

description "initial verison";

}

grouping AMFSet {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf aMFSetId {

config "true";

description "";

type nrm-type:t_aMF-Set-id;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf-list aMFSetMember {

description "AMF funciton list in the aMFSet";

min-elements "0";

config "true";

type nrm-type:t_dn;

}

leaf aMFRegion {

description "associated aMFRegion the aMFSet belong to";

type nrm-type:t_dn;

}

}

}

### H.4.3.5	NGC submodule, "ngc-AUSFFunction.yang"

submodule ngc-AUSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "AUSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping AUSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N12 { uses ep-rp:EP_N12; }

list EP_N13 { uses ep-rp:EP_N13; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.6	NGC submodule, "ngc-LMFFunction.yang"

submodule ngc-LMFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "LMFFunction derived from basic ManagedFunction";

revision 2018-12-18 {

description "15.1.0";

}

grouping LMFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_NLS { uses ep-rp:EP_NLS; }

}

}

### H.4.3.7	NGC submodule, "ngc-N3IWFFunction.yang"

submodule ngc-N3IWFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "N3IWFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping N3IWFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_N2 { uses ep-rp:EP_N2; }

list EP_N3 { uses ep-rp:EP_N3; }

}

}

### H.4.3.8	NGC submodule, "ngc-NGEIRFunction.yang"

submodule ngc-NGEIRFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "NGEIRFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NGEIRFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N17 { uses ep-rp:EP_N17; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.9	NGC submodule, "ngc-NRFFunction.yang"

submodule ngc-NRFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

include NFProfile;

description "NRFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NRFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description " ";

type inet:domain-name;

}

leaf-list sBIService {

description "";

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list nFProfile {

description "CM only required when NF profile is registered and deregistered by management system.";

min-elements "1";

config "true";

key nfInstanceId;

uses NFProfile;

}

leaf-list nSIId {

description "Is it really needed if network slicing feature enabled Seems it's only required for NSSF";

min-elements "1";

config "false";

type nrm-type:t_NsiId;

}

list EP_N27 { uses ep-rp:EP_N26; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

submodule NFProfile {

belongs-to ngc { prefix ngc; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

revision 2018-08-08 {

description "Initial revision";

}

grouping UdrInfo {

leaf groupId {

type string;

}

list supiRange {

key "start end pattern";

uses nrm-type:SupiRange;

}

list gpsiRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

list externalGroupIdentifiersRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

leaf-list supportedDataSet {

type nrm-type:DataSetId;

}

}

grouping UdmInfo {

leaf groupId {

type string;

}

list supiRange {

key "start end pattern";

uses nrm-type:SupiRange;

}

list gpsiRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

list externalGroupIdentifiersRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

leaf routingIndicator {

type string;

}

}

grouping AusfInfo {

leaf groupId {

type string;

}

list supiRange {

key "start end pattern";

uses nrm-type:SupiRange;

}

leaf routingIndicator {

type string;

}

}

grouping AmfInfo {

leaf amfRegionId {

type string;

}

leaf amfSetId {

type string;

}

list guami {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Guami;

}

list tai {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Tai;

}

list backupInfoAmfFailure {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Guami;

}

list backupInfoAmfRemoval {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Guami;

}

}

grouping SmfInfo {

leaf-list dnn {

type nrm-type:t_Dnn;

}

list tai {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Tai;

}

leaf pgwFqdn {

type inet:domain-name;

}

}

grouping UpfInfo {

list sNssaiUpfInfo {

key sNssai;

uses nrm-type:SnssaiUpfInfoItem;

}

leaf smfServingArea {

type string;

}

list interfaceUpfInfo {

//FFS to use "choiced" identifier

key k_id;

leaf k_id {

type string;

}

//key "interfaceType address";

uses nrm-type:InterfaceUpfInfoItem;

}

}

grouping PcfInfo {

leaf-list dnn {

type nrm-type:t_Dnn;

}

}

grouping BsfInfo {

list ipv4AddressRange {

key "start end";

uses nrm-type:Ipv4AddressRange;

}

list ipv6PrefixRange {

key "start end";

uses nrm-type:Ipv6PrefixRange;

}

}

grouping NFService {

leaf serviceInstanceID {

description "";

mandatory "true";

type string;

}

leaf serviceName {

mandatory "true";

type string;

}

container version { uses nrm-type:NFServiceVersion; }

leaf schema {

mandatory "true";

type string;

}

leaf fqdn {

type inet:domain-name;

}

leaf interPlmnFqdn {

type inet:domain-name;

}

list ipEndPoint {

//FFS to use "choiced" identifier

key k_id;

leaf k_id {

type string;

}

//     		key "address port";

uses nrm-type:ipEndPoint;

}

leaf apiPrefix {

type string;

}

list defaultNotificationSubscription {

key notificationType;

uses nrm-type:defaultNotificationSubscription;

}

list allowedPlmn {

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf-list allowedNfType {

type nrm-type:NFType;

}

leaf-list allowedDomain {

type string;

}

leaf-list allowedNssais {

type nrm-type:t_s-NSSAI;

}

leaf capacity {

type uint8;

}

leaf load {

type nrm-type:t_Load;

}

leaf supportedFeatures {

type nrm-type:SupportedFeatures;

}

}

grouping NFProfile {

leaf nfInstanceId {

mandatory "true";

type string;

}

leaf nfType {

mandatory "true";

type nrm-type:NFType;

}

leaf nfStatus {

mandatory "true";

type nrm-type:NFStatus;

}

container plmn {

uses nrm-type:pLMNId;

}

leaf-list sNssais {

type nrm-type:t_s-NSSAI;

}

leaf-list nsi {

type string;

}

leaf fqdn {

type inet:domain-name;

}

leaf interPlmnFqdn {

type inet:domain-name;

}

leaf-list ipv4Address {

type inet:ipv4-address;

}

leaf-list ipv6Address {

type inet:ipv6-address;

}

leaf-list ipv6Prefix {

type inet:ipv6-prefix;

}

leaf capacity {

type uint8;

}

leaf load {

type nrm-type:t_Load;

}

leaf locality {

type string;

}

leaf priority {

type uint8;

}

container udrInfo { uses UdrInfo; }

container udmInfo { uses UdmInfo; }

container ausfInfo { uses AusfInfo; }

container amfInfo { uses AmfInfo; }

container smfInfo { uses SmfInfo; }

container upfInfo { uses UpfInfo; }

container pcfInfo { uses PcfInfo; }

container bsfInfo { uses BsfInfo; }

list nfService {

min-elements "0";

description "";

key serviceInstanceID;

uses NFService;

}

}

}

### H.4.3.10	NGC submodule, "ngc-NSSFFunction.yang"

submodule ngc-NSSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "NSSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NSSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description " ";

type inet:domain-name;

}

leaf-list sBIService {

description " ";

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf-list nSIId {

description "Is it really needed if network slicing feature enabled Seems it's only required for NSSF";

min-elements "1";

config "false";

type nrm-type:t_NsiId;

}

list EP_N22 { uses ep-rp:EP_N22; }

list EP_N31 { uses ep-rp:EP_N31; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.12	NGC submodule, "ngc-PCFFunction.yang"

submodule ngc-PCFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "PCFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping PCFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N5 { uses ep-rp:EP_N5; }

list EP_N7 { uses ep-rp:EP_N7; }

list EP_N15 { uses ep-rp:EP_N15; }

list EP_N16 { uses ep-rp:EP_N16; }

list EP_Rx { uses ep-rp:EP_Rx; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.14	NGC submodule, "ngc-SMFFunction.yang"

submodule ngc-SMFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "SMFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping SMFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N4 { uses ep-rp:EP_N4; }

list EP_N7 { uses ep-rp:EP_N7; }

list EP_N10 { uses ep-rp:EP_N10; }

list EP_N11 { uses ep-rp:EP_N11; }

list EP_N16 { uses ep-rp:EP_N16; }

list EP_S5C { uses ep-rp:EP_S5C; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.15	NGC submodule, "ngc-SMSFFunction.yang"

submodule ngc-SMSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "SMSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping SMSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_N20 { uses ep-rp:EP_N20; }

list EP_N21 { uses ep-rp:EP_N21; }

list EP_MAP_SMSC { uses ep-rp:EP_MAP_SMSC; }

}

}

### H.4.3.16	NGC submodule, "ngc-UDMFunction.yang"

submodule ngc-UDMFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "UDMFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping UDMFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N8 { uses ep-rp:EP_N8; }

list EP_N10 { uses ep-rp:EP_N10; }

list EP_N13 { uses ep-rp:EP_N13; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.19	NGC submodule, "ngc-UPFFunction.yang"

submodule ngc-UPFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "UPFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping UPFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N3 { uses ep-rp:EP_N4; }

list EP_N4 { uses ep-rp:EP_N7; }

list EP_N9 { uses ep-rp:EP_N10; }

list EP_N6 { uses ep-rp:EP_N11; }

list EP_S5U { uses ep-rp:EP_S5C; }

}

}

End of 3rd modification

Start of 4th modification

## K.4.3	YANG schema

### K.4.3.1	General type definition "nrm-types-3gpp.yang", refer to Annex E.4.3.1

### K.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork", refer to Annex E.4.3.2

End of 4th modification