3GPP TSG-SA WG2 Meeting #133 	S2-1905691

13th – 17th May, 2019, Reno, Nevada, USA

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1440 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on event ID of NEF event exposure service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-05-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The text for event ID in the Nnef_EventExposure_Subscribe service operation has reference statement for PCF related service only since Feb. 2018, as agreed CR0001, S2-183014. However, Nnef_EventExposure is not only for PCF related event exposure but also for Monitoring Events supported by the AMF or UDM as specified in 4.15.3.1. Current text description for Event ID in the input parameter of the Nnef_EventExposure_Subscribe does not cover the monitoring event related with the AMF or UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional reference to 4.15.3.1 (Monitoring Event) of TS 23.502 for the description of the Event ID in the Nnef_EventExposure_Subscribe. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Event ID of Nnef_EventExposure cannot be used for monitoring event. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Release-16 CR (Mirror) is also submitted. |  |  |  |  |  |  |  |  |

***** Start of changes *****

#### 5.2.6.2	Nnef_EventExposure service

##### 5.2.6.2.1	General

See clause 4.15.3.1.

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs (required): (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify service operation, target of event reporting (GPSI or External Group Identifier), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID).

Inputs (optional): Event Filter, Subscription Correlation ID (in case of modification of the event subscription), Expiry time.

Outputs (required): When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs (optional): First corresponding event report is included, if available (see clause 4.15.1).

***** End of Changes *****