SA WG2 Meeting #133 	S2-1906529

# May 13 – 17, 2019 ; Reno, Nevada, USA		                              (revision of S2-1905097)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1316 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify which parameters are (not) applicable for I-SMF selection. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-04-24 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The list of parameters for (I-)SMF selection needs tidying |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following parameters are not applicable for I-SMF selection - per (S-NSSAI, subscribed DNN): whether EPC interworking is supported (all SMF(s) are assumed to be able to control the UPF mapping between EPC bearers and 5GC QoS flows) 6.3.2: remove “Access technology being used by the UE” + typo (MA-PDU) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.3 ; 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.34.3	I-SMF selection

The AMF is responsible of detecting when to add or to remove an I-SMF for a PDU Session. For this purpose, the AMF gets from NRF information about the Service Area of SMF(s). During mobility events such as Hand-Over or AMF change, if the service area of the SMF does not include the new UE location, then the AMF selects and inserts an I-SMF which can serve the UE location and the S-NSSAI. Conversely if the AMF detects that an I-SMF is no more needed it removes the I-SMF and interfaces directly with the SMF of the PDU Session. If the AMF detects that the SMF cannot serve the UE location (e.g. due to mobility), then the AMF selects a new I-SMF serving the UE location. In case the existing I-SMF cannot serve the UE location, then the AMF initiates a I-SMF change.

At PDU Session Establishment in non-roaming and roaming with LBO scenarios, if the AMF cannot select an SMF with a Service Area supporting the current UE location for the selected DNN, S-NSSAI etc, the AMF selects an SMF for the selected DNN, S-NSSAI and in addition selects an I-SMF serving the UE location. During the lifetime of a PDU Session, the AMF selects a new I-SMF if it determines that neither the current SMF nor the current I-SMF (if any) can serve the UE location.

Comparing to the SMF selection function defined in clause 6.3.2, following parameters are not applicable for I-SMF selection:

-	Data Network Name (DNN).

-	Subscription information from UDM.

NOTE : all SMF(s) are assumed to be able to control the UPF mapping between EPC bearers and 5GC QoS flows

NEXT CHANGE

### 6.3.2	SMF discovery and selection

The SMF selection functionality is supported by the AMF and SCP and is used to allocate an SMF that shall manage the PDU Session. The SMF selection procedures are described in clause 4.3.2.2.3 of TS 23.502 [3].

The SMF discovery and selection functionality follows the principles stated in clause 6.3.1.

If the AMF does discovery, the AMF shall utilize the NRF to discover SMF instance(s) unless SMF information is available by other means, e.g. locally configured on AMF. The AMF provides UE location information to the NRF when trying to discover SMF instance(s). The NRF provides NF profile(s) of SMF instance(s) to the AMF. In addition, the NRF also provides the SMF service area of SMF instance(s) to the AMF. The SMF selection functionality in the AMF selects an SMF instance and an SMF service instance based on the available SMF instances obtained from NRF or on the configured SMF information in the AMF.

NOTE 1:	Protocol aspects of the access to NRF are specified in TS 29.510 [58].

The SMF selection functionality is applicable to both 3GPP access and non-3GPP access.

The SMF selection for Emergency services is described in clause 5.16.4.5.

The following factors may be considered during the SMF selection:

a)	Selected Data Network Name (DNN).

b)	S-NSSAI.

c)	NSI-ID.

NOTE 2:	The use of NSI -ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI.

d)	Access technology being used by the UE

e)	Support for Control Plane CIoT 5GS Optimisation.

f)	Subscription information from UDM, e.g.

-	per DNN: whether LBO roaming is allowed.

-	per S-NSSAI: the subscribed DNN(s).

-	per (S-NSSAI, subscribed DNN): whether LBO roaming is allowed.

-	per (S-NSSAI, subscribed DNN): whether EPC interworking is supported.

g)	Void

h)	Local operator policies

NOTE 3: these policies can take into account whether the SMF to be selected is an I-SMF or a V-SMF or a SMF.

i)	Load conditions of the candidate SMFs.

j)	Analytics (i.e. statistics or predictions) for candidate SMFs' load as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

k)	UE location (i.e. TA).

l)	Service Area of the candidate SMFs.

m)	Capability of the SMF to support a MA PDU Session.

In the case of delegated discovery, the AMF, shall send all the available factors a)-d) to the SCP and shall also indicate if interworking with EPS is required and if the PDU session shall be home routed.

In addition, the AMF may indicate to the SCP which NRF to use (in case of NRF dedicated to the target slice).

Editor's note:	It is FFS how to capture that required discovery and selection information should be conveyed to the SCP with as minimal impact on the end-to-end signalling performance latency between AMF and SMF.

If there is an existing PDU Session and the UE requests to establish another PDU Session to the same DNN and S-NSSAI and the UE subscription data indicates the support for interworking with EPS for this DNN and S-NSSAI, the same SMF shall be selected. In any such a case where the AMF can determine which SMF should be selected, if delegated discovery is used, the AMF shall indicate a desired NF Instance ID so that the SCP is able to route the message to the relevant SMF. Otherwise, if UE subscription data does not indicate the support for interworking with EPS for this DNN and S-NSSAI, a different SMF may be selected. For example, to support a SMF load balancing or to support a graceful SMF shutdown (e.g., a SMF starts to no more take new PDU Sessions).

In the home-routed roaming case, the SMF selection functionality selects an SMF in VPLMN as well as an SMF in HPLMN. In the context of Network Slicing this is specified in clause 4.3.2.2.3.3 of TS 23.502 [3].

When the UE requests to establish a PDU Session to a DNN and an S-NSSAI, if the UE MM Core Network Capability indicates the UE supports EPC NAS and optionally, if the UE subscription indicates the support for interworking with EPS for this DNN and S-NSSAI, the selection functionality (in AMF or SCP) selects a combined SMF+PGW-C. Otherwise, a standalone SMF may be selected.

If the UDM provides a subscription context that allows for handling the PDU Session in the visited PLMN (i.e. using LBO) for this DNN and S-NSSAI and, optionally, the AMF is configured to know that the visited VPLMN has a suitable roaming agreement with the HPLMN of the UE, the following applies:

-	If the AMF does discovery, the SMF selection functionality in AMF selects an SMF from the visited PLMN.

-	If delegated discovery is used, the AMF indicate to the SCP that home routing is not needed, the delegated discovery and selection functionality in SCP shall discover and select an SMF from the visited PLMN.

If an SMF in VPLMN cannot be derived for the DNN and S-NSSAI, or if the subscription does not allow for handling the PDU Session in visited PLMN using LBO, then the following applies:

-	If the AMF does discovery, both an SMF in VPLMN and an SMF in HPLMN are selected, and the DNN and S-NSSAI is used to derive an SMF identifier from the HPLMN.

Editor's note:	It is FFS how the SCP handles the selection of the vSMF and hSMF in the HR roaming case and it is also FFS how the vSMF is informed of the selected hSMF in the case of delegated discovery and HR case.

If the initially selected SMF in VPLMN (for roaming with LBO) detects it does not understand information in the UE request, it may reject the N11 message (related with a PDU Session Establishment Request message) with a proper N11 cause triggering the AMF to select both a new SMF in the VPLMN and a SMF in the HPLMN (for home routed roaming).

The AMF selects SMF(s) considering support for CIoT 5GS optimisations (e.g. Control Plane CIoT 5GS Optimisation).

Additional details of AMF selection of an I-SMF are described in the clause 5.34.

In the case of home routed scenario, the AMF selects a new V-SMF if it determines that the current V-SMF cannot serve the UE location. The selection/relocation is same as an I-SMF selection/relocation as described in the clause 5.34.

END OF CHANGES