3GPP TSG-SA5 Meeting #126	S5-195785

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019							revision of S5-195306

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0077 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the selection mode in PDU session information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per the description in TS 23.502, the DNN Selection Mode is determined by AMF and indicates whether an explicitly subscribed DNN has been provided by the UE in its PDU Session Establishment Request. The DNN Selection Mode should be added into PDU Session Information for 5G charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the "DNN Selection Mode" in "PDU session information" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The index indicating how the DNN was selected is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2,6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

6.2.1.2	Definition of PDU session charging information

PDU session specific charging information used for 5G data connectivity charging is provided within the PDU session charging Information.

The detailed structure of the PDU Session Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of PDU Session Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Charging Id | OC | This field holds the Charging Id for PDU session. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| Roamer In Out | OC | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |
| User Location Info | OC | This field indicates details of where the UE is currently located (access-specific user location information). |
| User Location Time | OC | The NTP time at which the UE was last known to be in the location. |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Presence Reporting Area Information | OC | This field contains part of the Presence Reporting Area Information of UE as defined in TS 23.501[200], comprising the Presence Reporting Area identifier(s) and an indication on whether the UE is inside or outside the Presence Reporting Area, if available. |
| PDU Session Information | M | Group of PDU session information. |
| PDU Session ID | M | This field holds identifier of PDU session. |
| Network Slice Instance Identifier | OM | This field holds network slice information the PDU session belongs to. |
| PDU Type | OM | This field holds the type of PDU session. |
| PDU Address | Oc | Group of UE IP address. It may have multiple occurrences. |
| PDU IP Address | OC | This field holds the IP Address of the served SUPI allocated for PDU session, i.e. IPv4 address or IPv6 prefix. |
| PDU Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDU Address. The field needs not available for prefix length of 64 bits. |
| Dynamic Address Flag | OC | This field indicates whether served PDP/PDN address is dynamically allocated. This field is missing if address is static. |
| SSC Mode | OC | This field holds SSC mode of PDU session. |
| SUPI PLMN ID | OC | This field holds PLMN ID of the SUPI. |
| Serving Network Function ID | Oc | Group of serving Network Function identifier |
| Serving Network Function Functionality | M | This field holds the functionality of the Serving Network Function: i.e. AMF, SMF. When this field holds "AMF" then it is related to AMF in the same PLMN as the SMF consuming the charging service. When this field holds "SMF" then it is related to V-SMF for home routed roaming. |
| Serving Network Function Name | OC | This field holds the name of the serving Network Function (i.e. AMF). |
| Serving Network Function Addresses | OC | This field holds the IP Addresses of the Serving Network Function. |
| Serving Network Function FQDN | OC | This field holds the FQDN the Serving Network Function. When the the Serving Network Function is an AMF, this FQDN is the AMF name as defined in subclause 5.9.5 of 3GPP TS 23.501 [200]. |
| Serving Network Function PLMN ID | OC | This field holds the PLMN ID of the network the Serving Network Function belongs to. |
| AMF Identifier | OC | This field holds the AMF identifier. |
| Serving CN PLMN ID | Oc | This field holds the serving Core Network Operator PLMN ID selected by the UE if different from SMF PLMN ID. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT) currently serving the UE. |
| Data Network Name Identifier | M | This field contains the identifier of the DNN the user is connected to. |
| DNN Selection Mode | OC | This field indicates whether the requested DNN corresponds to an explicitly subscribed DNN or to the usage of a wildcard subscription. |
| Authorized QoS Information | OC | This field holds the authorized QoS applied to PDU session. |
| Subscribed QoS Information | OC | This field holds the subscribed default QoS for the PDU session. |
| Authorized Session-AMBR | OC | This field holds the authorized Session-AMBR for the PDU session. |
| Subscribed Session-AMBR | OC | This field holds the subscribed Session-AMBR for the PDU session. |
| PDU session start Time | OC | This field holds the timestamp when PDU session starts. |
| PDU session stop Time | OC | This field holds the timestamp when PDU session terminates. |
| Diagnostics | OC | This field holds a more detailed reason for the release of the PDU session and complements the "Change Condition" information. |
| Charging Characteristics | OC | This field holds the Charging Characteristics for this PDU session. |
| Charging Characteristics Selection Mode | OC | This field holds information about how the "Charging Characteristics" was selected. |
| 3GPP PS Data Off Status | OC | This field holds the 3GPP Data off Status when UE's 3GPP Data Off status is Activated or Deactivated. |
| Session Stop Indicator | OC | This field indicates to the CHF that the PDU session has been terminated. |
| Unit Count Inactivity Timer | OC | This field holds the threshold for the time period when no units has been counted by the SMF. It holds either the value configured in SMF, if it is supported, or the value to be used as received from the CHF. A value of zero indicates that this mechanism shall not be used. This field is not applicable to QBC. |
| RAN Secondary RAT Usage Report | OC | This field holds the secondary RAT usage reported from NG-RAN. |
| NG RAN Secondary RAT Type | OC | This field holds the value of Secondary RAT Type, as provided by the NG-RAN. |
| Qos Flows Usage Reports | OC | This field holds a list of containers per QFI with volumes reported, each container is time stamped. |
| QoS Flow Id | OM | This field holds the QoS flow Identifier (QFI) |
| Start Timestamp | OC | This field holds the start timestamp of the collected usage. |
| End Timestamp | OC | This field holds the end timestamp of the collected usage. |
| Downlink Volume | OC | This field holds the amount of used volume in downlink direction. |
| Uplink Volume | OC | This field holds the amount of used volume in uplink direction. |

Next change

6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMF for 5G data connectivity converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element |  | Node Type | SMF (FBC) |  | SMF (QBC) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | -UT- |  | -UT- |  |
| Subscriber Identifier |  |  | IUT- |  | IUT- |  |
| NF Consumer Identification |  |  | IUT- |  | IUT- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  |
| Notify URI |  |  | IU- |  | IU- |  |
| Triggers |  |  | -UT- |  | -UT- |  |
| Multiple Unit Usage |  |  | IUT- |  | - |  |
| Rating Group |  |  | IUT- |  | - |  |
| Requested Unit |  |  | IU-- |  | - |  |
| Used Unit Container |  |  | -UT- |  | - |  |
| Triggers |  |  | -UT- |  | - |  |
| PDU Container Information |  |  | -UT- |  | - |  |
| UPF ID |  |  | IUT- |  | IUT- |  |
| PDU Session Charging Information |  |  | IUT- |  | IUT- |  |
| Charging Id |  |  | IUT- |  | IUT- |  |
| User Information |  |  | IUT- |  | IUT- |  |
| User Location Info |  |  | IUT- |  | IUT- |  |
| UE Time Zone |  |  | IUT- |  | IUT- |  |
| Presence Reporting Area Information |  |  | -UT- |  | -UT- |  |
| PDU Session Information |  |  | IUT- |  | IUT- |  |
| PDU Session ID |  |  | IUT- |  | IUT- |  |
| Network Slice Instance Identifier |  |  | IUT- |  | IUT- |  |
| PDU Type |  |  | IUT- |  | IUT- |  |
| PDU Address |  |  | IUT- |  | IUT- |  |
| SSC Mode |  |  | IUT- |  | IUT- |  |
| SUPI PLMN ID |  |  | IUT- |  | IUT- |  |
| Serving Network Function ID |  |  | IUT- |  | IUT- |  |
| Serving CN PLMN ID |  |  | IUT- |  | IUT- |  |
| RAT Type |  |  | IUT- |  | IUT- |  |
| Data Network Name Identifier |  |  | IUT- |  | IUT- |  |
| DNN Selection Mode |  |  | IUT- |  | IUT- |  |
| Authorized QoS Information |  |  | IUT- |  | IUT- |  |
| Subscribed QoS Information |  |  | IUT- |  | IUT- |  |
| Authorized Session-AMBR |  |  | IUT- |  | IUT- |  |
| Subscribed Session-AMBR |  |  | IUT- |  | IUT- |  |
| PDU session start Time |  |  | I--- |  | I--- |  |
| PDU session stop Time |  |  | --T- |  | --T- |  |
| Diagnostics |  |  | --T- |  | --T- |  |
| Charging Characteristics |  |  | IUT- |  | IUT- |  |
| Charging Characteristics Selection Mode |  |  | IUT- |  | IUT- |  |
| Charging Rule Base Name |  |  | IUT- |  | - |  |
| 3GPP PS Data Off Status |  |  | IUT- |  | IUT- |  |
| Session Stop Indicator |  |  | --T- |  | --T- |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  |
| RAN Secondary RAT Usage Report |  |  | -UT- |  | -UT- |  |
| Roaming QBC information |  |  | - |  | IUT- |  |
| Multiple QFI container |  |  | - |  | IUT- |  |
| UPF ID |  |  | - |  | IUT- |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging.

Table 6.2.2.1: Supported fields in Charging Data Response message

| Information Element |  | Node Type | SMF (FBC) |  | SMF (QBC) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/U/T/E |  | I/U/T/E |  |
| Session Identifier |  |  | I--- |  | I--- |  |
| Invocation Timestamp |  |  | IUT- |  | IUT- |  |
| Invocation Result |  |  | IUT- |  | IUT- |  |
| Invocation Sequence Number |  |  | IUT- |  | IUT- |  |
| Session Failover |  |  | IU-- |  | IUT- |  |
| Triggers |  |  | IU-- |  | IU-- |  |
| Multiple Unit information |  |  | IU-- |  | - |  |
| Result Code |  |  | IU-- |  | - |  |
| Rating Group |  |  | IU-- |  | - |  |
| UPF ID |  |  | IU-- |  | - |  |
| Granted Unit |  |  | IU-- |  | - |  |
| Validity Time |  |  | IU-- |  | - |  |
| Final Unit Indication |  |  | IU-- |  | - |  |
| Time Quota Threshold |  |  | IU-- |  | - |  |
| Volume Quota Threshold |  |  | IU-- |  | - |  |
| Unit Quota Threshold |  |  | IU-- |  | - |  |
| Quota Holding Time |  |  | IU-- |  | - |  |
| Triggers |  |  | IU-- |  | - |  |
| PDU Session Charging Information |  |  | IU-- |  | IU-- |  |
| Charging Id |  |  | - |  | - |  |
| User Information |  |  | - |  | - |  |
| User Location Info |  |  | - |  | - |  |
| UE Time Zone |  |  | - |  | - |  |
| Presence Reporting Area Information |  |  | IU-- |  | IU-- |  |
| PDU Session Information |  |  | IU-- |  | IU-- |  |
| PDU Session ID |  |  | - |  | - |  |
| Network Slice Instance Identifier |  |  | - |  | - |  |
| PDU Type |  |  | - |  | - |  |
| PDU Address |  |  | - |  | - |  |
| SSC Mode |  |  | - |  | - |  |
| SUPI PLMN ID |  |  | - |  | - |  |
| Serving Network Function ID |  |  | - |  | - |  |
| Serving CN PLMN ID |  |  | - |  | - |  |
| RAT Type |  |  | - |  | - |  |
| Data Network Name Identifier |  |  | - |  | - |  |
| DNN Selection Mode |  |  | - |  | - |  |
| Authorized QoS Information |  |  | - |  | - |  |
| Subscribed QoS Information |  |  | - |  | - |  |
| Authorized Session-AMBR |  |  | - |  | - |  |
| Subscribed Session-AMBR |  |  | - |  | - |  |
| PDU session start Time |  |  | - |  | - |  |
| PDU session stop Time |  |  | - |  | - |  |
| Diagnostics |  |  | - |  | - |  |
| Charging Characteristics |  |  | - |  | - |  |
| Charging Characteristics Selection Mode |  |  | - |  | - |  |
| Charging Rule Base Name |  |  | - |  | - |  |
| 3GPP PS Data Off Status |  |  | - |  | - |  |
| Session Stop Indicator |  |  | - |  | - |  |
| Unit Count Inactivity Timer |  |  | IU-- |  | - |  |
| RAN Secondary RAT Usage Report |  |  | - |  | - |  |
| Roaming QBC information |  |  | - |  | IU-- |  |
| Multiple QFI container |  |  | - |  | - |  |
| UPF ID |  |  | - |  | - |  |
| Roaming Charging Profile |  |  | - |  | IU-- |  |

End of change