3GPP TSG-WG SA2 Meeting #136 	S2-1911661

# Reno, USA, November 18 – 22, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1931 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Availability after DDN Failure event with UPF buffering procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 has agreed S2-1910636 about how to subscribe and determines Availability after DDN Failure even if buffering in the SMF. But in case of buffering in UPF is activated, the procedure is not defined. For the case of the UPF buffering, the following scenarios are considered: - When the SMF receives the subscription from UDM, for a UE’s specific PDU session, the SMF has be informed the UE is unreachable and extended buffer is activated. In this case, the SMF can further request the UPF to report the traffic information of the discard downlink data, comparing traffic information with the traffic descriptor correspond to the event, the SMF can determines whether event is happened, and notify the related AMF. - When the SMF receives the subscription from UDM, for a UE’s specific PDU session, the SMF has not be information the UE is unreachable. When DN is received by the SMF and the SMF is informed by the AMF the UE is unreachable, the SMF determines to activate the extended buffer in the UPF, the SMF further request the UPF to report the traffic information of the discard downlink data, then the SMF can determine whether event is happened. - For the above two cases, if the extended buffer is not activated, the SMF can request UPF discard the buffered data and report the information to the SMF, then the SMF can determine whether event is happened. Based on the above analysis, in any case, the SMF can request the UPF to report the information of discard DL packets for event detection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify how to subcribe and determine the Availability after DDN Failure event if buffering in the UPF is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If extended buffer in SMF is not supported, Availability after DDN Failure event can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.3.2b	Information flow for availability after DDN Failure event with UPF buffering

The procedure is used if the SMF requests the UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to notifications about availability after DDN failure.

Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID.

Figure 4.15.3.2b-1: Information flow for availability after DDN Failure event with UPF buffering

0.	The SMF (in the non-roaming case the H-SMF, in the roaming case the V-SMF, in case of PDU session with I-SMF the I-SMF) configures the relevant UPF to buffer packets as described in clause 5.8.3 in 23.501 [2].

1.	AF interacts with NEF to subscribe availability after DDN failure subscription procedure as described in clause 4.15.3.2a.

2-3.	If the SMF is informed the UE is unreachable via a Namf_Communication_N1N2MessageTransfer service operation the SMF requests the UPF to report the traffic information of the discarded downlink data when the SMF received the availability after DDN failure subscription from the UDM. When the UPF determines to discard a DL data, it reported the traffic information to the SMF. By comparing the traffic information of downlink packets received from the UPF with the Traffic Descriptor(s) received in the event subscription(s), the SMF determines whether any traffic from an AF having an event subscription occured and if so, the SMF sends the event report, by means of Nsmf_EventExposure_Notify message, to the AMF indicated as notification endpoint. If the UE is not reachable after the AMF received the notification from the SMF, the AMF shall set a Notify-on-available-after-DDN-failure flag corresponding to the Notification Correlation Id and the identifier of the UE if available.

4-5.	[Conditional] The AMF detects the UE is reachable and sends the event report(s) based on the Notify-on-available-after-DDN-failure flag, by means of Namf_EventExposure_Notify message(s), only to the NEF(s) indicated as notification endpoint(s) identified via the corresponding subscription in step 3. In this way, only the AF(s) for which DL traffic transmission failed are notified.

6.	The NEF sends Nnef_EventExposure_Notify message with the "Availability after DDN Failure" event to AF.

# * * * * End of changes * * * *