SA WG2 Meeting #136	S2-1912101

# November 18 – 22, 2019, Reno, NV, USA 	(revision of S2-1911694)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0061 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Non-5G Capable (N5GC) devices alignment with SA3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Charter Communications, CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA3 uses the term Non-5G Capable (N5GC) devices for EAP based authentication procedure in isolated network deployment. Terminology alignment between this specification and SA3 is important to avoid confusion. Also using the term “Stand-alone Non-Public Network” to describe isolated network in 23.316 is confusing as SNPN in TS 23.501 has a very specific meaning which requires the use of NID for network selection procedure. It is proposed to replace the term “Stand-alone Non-Public Network” with similar wording as introduced in TS 33.501 for N5GC devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replacing “non 5GC NAS capable devices” to Non-5G Capable (N5GC) devices. Clarify that device capabilities include an indication that this is a N5GC device. Replacing “Stand-alone Non-Public Network” in this wireline context – i.e., for isolated deployment with wireline access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing spec between SA2 and SA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

## 4.10a	Non-5G capable device behind 5G-CRG and FN-CRG.

For isolated 5G networks (i.e., roaming is not considered) with wireline access, non-5G capable (N5GC) devices connecting via W-5GAN can be authenticated by the 5GC using EAP based authentication method(s) as defined in TS 33.501 [11]. The following call flow describes the overall authentication procedure with FN-CRG.

Figure 4.10a-1: Non-5GC device authenticated by 5GC procedure

1.	W-AGF registers FN-RG to 5GC based on clause 7.2.1.3.

2.	FN-CRG is configured as L2 bridge mode and forwards any L2 frame forward to W-AGF. 802.1x authentication is triggered. This can be done either by N5GC device sending a EAPOL-start frame to W-AGF or W-AGF receives a frame from an unknown MAC address.

3.	W-AGF, on behalf of the N5GC device, formulate a N1: Registration Request message to AMF with device capability indicator that the device is non-5G capable.

4.	AMF selects a suitable AUSF based on registration type.

5.	EAP based authentication defined in TS 33.501 [11] is performed between the AUSF and N5GC device.

Once the N5GC device has been authenticated, the AUSF provides relevant security related information to the AMF. AUSF shall return the SUPI (e.g., username of the N5GC device) to AMF only after the authentication is successful.

NOTE:	Each device is registered to 5GC with its own unique SUPI.

AMF performs other registration procedures as required (see TS 23.502 [3] clause 4.2.2.2.2).

6.	AMF sends Registration Accept message to W-AGF.

Only one PDU session per N5GC device is supported.

Roaming is not supported.

For 5G-CRG, it behaves as FN-CRG (i.e., L2 bridge mode) when handling N5GC device.

End of changes