3GPP TSG-WG SA2 Meeting #136 	S2-1911185

# Reno, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0092 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of QoS requirments parameter used for QoS Sustainabiltiy Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - TS 23.288 clause 6.9 QoS Sustainabiltiy Analytics defines QoS requriements to be used in the Analytics Filter Information as following: “ - QoS requirements: - Standardized 5QI, and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR); - For non-standardized QoS Characteristics, the QoS Characteristics attributes PDB and PER and their values (optional).” It is not clear which QoS parameter (e..g, 5QI, PDB, PER, GFBR) is mandatory and which is optional.If all of the QoS parameters are optional whether the QoS requirents become optional as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adjust the description in clause 6.9.1 on QoS requirments in the Analytics Filter Information to make it clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear description for CT implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.9	QoS Sustainability Analytics

6.9.1	General

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an observation period in the past in a certain area or the likelihood of a QoS change for an observation period in the future in a certain area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e., a Request-Response model). The request includes the following parameters:

-	Analytics ID = "QoS Sustainability".

-	Analytics Filter Information:

-	QoS requirements:

-	5QI (standardized or pre-configured), and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR);

-	or the QoS Characteristics attributes PDB, PER and their values.

-	Location information: could be an area or a path of interest. The location information could reflect a list of waypoints.

NOTE:	In this Release, the consumer of the "QoS Sustainability" Analytics ID will provide location information in the area of interest format (TAIs or Cell IDs) which is understandable by NWDAF.

-	Observation Period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested.

-	Threshold(s): The threshold(s) indicate level(s) used for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification. The level(s) relate to the QoS KPIs, i.e. the RAN UE Throughput, the QoS flow Retainability, etc., for the relevant 5QI(s) defined in TS 28.554 [10]:

-	for a 5QI of GBR resource type, the Threshold(s) refer to the QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Threshold(s) refer to the RAN UE Throughput KPI.

-	S-NSSAI (optional).

The NWDAF collects the corresponding statistics information on the QoS KPI for the relevant 5QI of interests from the OAM, i.e. the QoS flow Retainability or the RAN UE Throughput as defined in TS 28.554 [10].

# * * * * End of changes * * * *