SA WG2 Meeting #S2-136	S2-1912522

18 – 22 November, 2019, Reno, NV, US	(revision of S2-1912510)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1592 | rev | 10 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of Editor's Note on UCMF-AMF interaction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Ericsson, Nokia, Nokia Shanghai bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is the following Editor’s Note in clause 5.4.4.1a: Editor's note: The interaction between UCMF and AMF in order to switch to operate based on manufacturer-assigned UE Radio Capability ID for a particular type of UE is FFS. SA2#133 agreed that the network can use the Registration Accept message or the UE Configuration Update command message to request the UE to delete any PLMN-assigned UE Radio Capability IDs for this PLMN. SA2#133 also agreed the Nucmf_UECapabilityManagement_Notify service operation (TS 23.502 clause 5.2.18.3.5) which can be used for deletion of dictionary entries in the consumer NF (e.g. in the AMF). ##### 5.2.18.3.5 Nucmf_UECapabilityManagement_Notify service operation Service Operation name: Nucmf_UECapabilityManagement_Notify Description: Producer NF provides notifications about creation or deletion of UCMF dictionary entries to subscribed consumer NF. Inputs, Required: Notification Type ("creation", "deletion"). Inputs, Optional: If Notification Type is set to "creation": One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID and the corresponding UE radio access capability. If Notification Type is set to "deletion": One or more UE Radio Capability IDs. As indicated in the referenced text, the dictionary entries that need to be deleted are indicated by the corresponding UE Radio Capability ID. What is missing is description of the AMF behaviour upon reception of a Notify message of “deletion” type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.4.4.1a: clarified the AMF behaviour upon reception of a Notify message of “deletion” type. Rev 10: Added the option of UE Radio Capability ID deletion based on TACs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the RACS feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********** Start change **************

#### 5.4.4.1a	UE radio capability signalling optimisation (RACS)

With the increase of the size of UE radio capabilities driven e.g. by additional frequency bands and combinations thereof for E-UTRA and NR, an efficient approach to signal UE Radio Access Capability information over the radio interface and other network interfaces is defined with RACS.

In this Release of the specification, RACS does not apply to NB-IOT.

RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called UE Radio Capability ID. A UE Radio Capability ID can be either UE manufacturer assigned or PLMN-assigned, as specified in clause 5.9.10. The UE Radio Capability ID is an alternative to the signalling of the radio capabilities container over the radio interface, within NG-RAN, from NG-RAN to E-UTRAN, from AMF to NG-RAN and between CN nodes supporting RACS.

PLMN-assigned UE Radio Capability ID is assigned to the UE using the UE Configuration Update procedure, or Registration Accept as defined in TS 23.502 [3].

The UCMF (UE radio Capability Management Function) stores all UE Radio Capability ID mappings in a PLMN and is responsible for assigning every PLMN-assigned UE Radio Capability ID in this PLMN, see clause 6.2.21.

In order to be able to interpret the UE Radio Capability ID a Network Function or node may store a local copy of the mapping between the UE Radio Capability ID and its corresponding UE Radio Access Capabilities information i.e. a dictionary entry. When no mapping is available between a UE Radio Capability ID and the corresponding UE Radio Capability information in a Network Function or node, this Network Function or node shall be able to retrieve this mapping and store it.

-	An AMF which supports RACS shall store such UE Radio Capability ID mapping at least for all the UEs that it serves that have a UE Radio Capability ID assigned.

-	The NG-RAN performs local caching of the UE Radio Access Capabilities for the UE Radio Capability IDs for the UEs it is serving, and potentially for other UE Radio Capability IDs according to suitable local policies.

-	When the NG-RAN needs to retrieve the mapping of a UE Radio Capability ID to the corresponding UE Radio Capability information, it queries the AMF using N2 signalling defined in TS 38.413 [34].

-	When the AMF needs to retrieve a PLMN-assigned UE Radio Capability ID for a UE from the UCMF, it provides the UE Radio Capabilities Information, IMEI/TAC and SV for the UE. The UE Radio Capabilities Information is associated with IMEI/TAC and SV. The UCMF stores the association of this IMEI/TAC and SV with this UE Radio Capability ID.

-	When the AMF retrieves the UE Radio Capability Information associated to a UE Radio Capability ID it provides the UE Radio Capability ID it to UCMF in order to obtain a mapping of a UE Radio Capability ID to the corresponding UE Radio Capabilities information.

A network may utilise the PLMN-assigned UE Radio Capability ID, without involving the UE, e.g. for use with legacy UEs.

Mutual detection of the support of the RACS feature happens between NG-RAN nodes at Xn setup and between NG-RAN and AMF at N2 setup time. To allow for a mix of RACS-supporting and non-RACS-supporting RAN nodes over the Xn interfaces, the UE Radio Capability ID should be included in the Path Switch signalling during Xn based handover and Handover Request during N2 based handover between AMF and NG-RAN. In addition, RACS-supporting RAN nodes can be discovered across inter-CN node boundaries e.g. using the Configuration Transfer procedure. The support of RACS by peer AMFs or MMEs is based on configuration in a PLMN or across PLMNs.

A UE that supports WB-EUTRA and/or NR indicates its support for RACS to AMF using UE MM Core Network Capability as defined in clause 5.4.4a.

A UE that supports RACS and stores an applicable UE Radio Capability ID for the current UE Radio Configuration in the PLMN, shall signal the UE Radio Capability ID in the Initial Registration procedure as defined in TS 23.502 [3]. If both PLMN-assigned for the current PLMN and UE manufacturer-assigned UE Radio Capability IDs are stored in the UE and applicable in the PLMN, the UE shall signal the PLMN-assigned UE Radio Capability ID in the Registration Request message.

When a PLMN decides to request a particular type of UE to use manufacturer-assigned UE Radio Capability ID(s):

-	The UCMF sends a Nucmf_UECapabilityManagement_Notify message to the AMF including either a list of UE Radio Capability IDs (if the UE was previously using any PLMN assigned IDs) or the TAC values corresponding to UE types that are requested to use manufacturer-assigned UE Radio Capability ID. These values are stored in a “Manufacturer Assigned operation requested list” in the AMF.

-	The AMF uses the Registration Accept message or the UE Configuration Update command message to request the UE to delete all the PLMN-assigned UE Radio Capability ID(s) for this PLMN if the UE is, respectively, registering or is registered with PLMN assigned ID or TAC values matching one value in the Manufacturer Assigned operation requested list.

NOTE w: It is expected that in a given PLMN the UCMF and AMFs will be configured to either use a Manufacturer Assigned operation requested list based on a list of PLMN assigned UE Radio Capability IDs or a list of TACs, but not both.

NOTE x: The strategy for triggering of the deletion of PLMN-assigned UE Radio Capability ID(s) in the UE by the AMF is implementation-specific (e.g. can be used only towards UEs in CM_Connected state).

-	a UE that receives indication to delete all the PLMN-assigned UE Radio Capability IDs in the Registration Accept message, or UE Configuration Update command message, deletes any PLMN-assigned UE Radio Capability IDs for this PLMN. The UE proceeds to register with a Manufacturer assigned UE Radio Capability ID that is applicable to the current UE Radio configuration.

-	When the “Manufacturer Assigned operation requested list” contains PLMN assigned UE Radio Capability IDs, the UCMF shall avoid re-assigning PLMN assigned UE Radio Capability IDs that were added to the “Manufacturer Assigned operation requested list” in the AMFs to any UE.

-	The AMF stores a PLMN assigned ID in the Manufacturer Assigned operation requested list for a time duration that is implementation specific, but TACs are stored until the UCMF require to remove certain TACs from the list (i.e. the list of TACs which are requested to use Manufacturer Assigned IDs in the AMF and UCMF is synchronised at all times).

- 	The UCMF can request at any time the AMF to remove PLMN assigned ID(s) or TAC(s) values form the Manufacturer Assigned operation requested list.

NOTE y: The AMF can decide to remove a UE Radio Capability ID from the “Manufacturer Assigned operation requested list” list e.g. because no UE with that UE Radio Capability ID has connected to the network for long time. If later a UE with such UE Radio Capability ID connects to the network, the AMF contacts the UCMF to resolve the UE Radio Capability ID, and at this point the UCMF can trigger again the deletion of the UE Radio Capability ID by including this in the Manufacturer Assigned operation requested list of the AMF.

The serving AMF stores the UE Radio Capability ID for a UE in the UE context and provides this UE Radio Capability ID to NG-RAN as part of the UE context information using N2 signalling. During inter PLMN mobility, the new AMF shall delete the UE Radio Capability ID received from the old AMF, unless the operator policy indicates that all UE Radio Capability IDs used in the old PLMN is also valid in the new PLMN.

The UE stores the PLMN-assigned UE Radio Capability ID in non-volatile memory when in RM-DEREGISTERED state and can use it again when it registers in the same PLMN.

NOTE 3:	It is assumed that UE does not need to store the access stratum information (i.e. UE-EUTRA-Capability and UE-NR-Capability specified in TS 36.331 [51] and TS 38.331 [28], respectively) that was indicated by the UE to the network when the PLMN-assigned UE Radio Capability ID was assigned by the network. However, it is assumed that the UE does store the related UE configuration (e.g. whether or not GERAN or UTRAN or MBMS is enabled/disabled).

At any given time at most one UE Radio Capability ID is stored in the UE context in CN and RAN.

The number of PLMN-assigned UE Radio Capability IDs that the UE stores in non-volatile memory is left up to UE implementation. However, to minimise the load (e.g. from radio signalling) on the Uu interface and to provide smoother inter-PLMN mobility (e.g. at land borders) the UE shall be able to store at least the latest 16 PLMN-assigned UE Radio Capability IDs (along with the PLMN that assigned them). This number is independent of the UE-manufacturer-assigned UE Radio Capability ID(s) the UE may store.

It shall be possible for a UE to change, e.g. upon change in its usage settings, the set of UE Radio Capabilities in time and signal the associated UE Radio Capability ID, if available. The UE stores the mapping between the UE Radio Capability ID and the corresponding UE Radio Capability information for every UE Radio Capability ID it stores.

The NG-RAN may apply RRC filtering of UE radio capabilities when it retrieves the UE Radio Capabilities information from the UE as defined in TS 38.331 [28].

NOTE 4:	In a RACS supporting PLMN, the filter of UE Radio Capabilities configured in NG-RAN is preferably as wide in scope as possible (e.g. PLMN-wide). In this case, it corresponds e.g. to the super-set of bands, band-combinations and RATs the PLMN deploys and not only to the specific NG-RAN node or region.

NOTE 5:	If the filter of UE Radio Capabilities configured in two NG-RAN nodes is different, during handover between these two nodes, it is possible that the target NG-RAN node might need to enquire the UE for its UE Radio Capability information again and trigger re-allocation of a PLMN-assigned UE Radio Capability ID leading to extra signalling. Additionally, a narrow filter might reduce the list of candidate target nodes.

If a UE supports both NB-IoT and possibly other RATs the UE handles the RACS procedures as follows:

-	Since there is no support for RACS in NB-IoT, if the UE supports RACS in non-NB-IoT RATs (i.e. for WB-EUTRA and/or NR).

-	NB-IoT specific UE Radio Capability information is handled in UE, RAN and AMF according to clause 5.4.4.1.

-	when the UE is not camping on NB-IoT, the UE provides UE radio capabilities for other RATs but not NB-IoT UE radio capabilities, as per TS 38.331 [28]. As a result the UE Radio Capability ID that is assigned by the network corresponds only to the UE Radio Capabilities of the non-NB-IoT RATs. The UE uses the UE Radio Capability IDs assigned only in Registration Update procedures performed over non-NB-IoT RATs.

Support for RACS in EPS is defined in TS 23.401 [26].

*********** End change **************