3GPP TSG-WG SA2 Meeting #136 	S2-1912589

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-1912374)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1821 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of SMF management of 5GLAN PDU sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF management of PDU sessions are specified TS 23.501.. In order to realize local switch, N19 tunnling, SMF needs to correlate all the PDU sessions esttablished to the same 5GLAN DNN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDU sessions established to the same 5G VN (e.g. identified by the DNN for 5G LAN-type service) shall be correlated by the SMF. This may be used by the SMF to configure the UPF with the group level N4-session including packet detection and forwarding rules to implement “local switch”, “N19 tunneling” and “N6 routing” forwarding options |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to generate group level N4 session is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.29.3	PDU Session management

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarification and enhancement:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G VN group

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G VN group and SMF selection is described in clause 6.3.2.

NOTE 1:	The network is configured so that the same SMF is always selected for a certain 5G VN group.

NOTE 2:	Having a dedicated SMF serving a 5G VN does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G VN group

-	The UE provides a DNN associated with the 5G VN group to access the 5G LAN-type services for that 5G VN, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G VN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G VN group. There is no 5G VN group specific authentication or authorization defined.

-	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G VN group.

-	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G VN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G VN group.

-	Session and service continuity SSC mode 1, SSC mode 2, and SSC mode 3 as described in clause 5.6.9 are applicable to N6-based traffic forwarding of 5G VN communication within the associated 5G VN group under the same SMF.

-	A PDU Session provides both unicast and multicast communication for a DNN associated to a 5G VN group. The PSA UPF determines whether the communication is for unicast or multicast based on the destination address of the received data, and performs unicast or multicast communication handling.

-	In order to realize N19 traffic routing, SMF correlates PDU sessions established to the same 5G VN group (identified by the DNN for 5G LAN-type service) and uses this to configure the UPF with the group level N4-session including packet detection and forwarding rules for N19 tunneling forwarding.

# * * * * End of changes * * * *