3GPP TSG-SA5 Meeting # 128	S5-197762

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0071 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct and update the RESTful HTTP-based solution set of provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16,NETSLICE |  |  |  |  |  | Date: |  |  | 08-11-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The solution set has some errors and inconsistencies with the latest version of TS 32.158 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The errors are corrected and the inconsistencies with TS 32.158 are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CRUD operations cannot be implemented properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.1.1, 12.1.1.1.2, 12.1.1.1.3, 12.1.1.1.4.x (new), 12.1.1.1.4.y (new), 12.1.1.1.5, 12.1.1.3.1, 12.1.1.3.2.1.2, 12.1.1.3.2.1.3.1, 12.1.1.3.2.1.3.2, 12.1.1.3.2.1.3.3, 12.1.1.3.2.1.3.4, 12.1.1.4.1, 12.1.1.4.2.1, 12.1.1.4.2.2, 12.1.1.4.2.4, 12.1.1.4.2.5, 12.1.1.4.2.x (new), 12.1.1.4.2.y (new), 12.1.1.4.2.z (new), 12.1.1.4.4.2, 12.1.1.4.4.x (new), A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-197429 |  |  |  |  |  |  |  |  |

First modification

#### 12.1.1.1	Mapping of operations

##### 12.1.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.1.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| createMOI | PUT | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-first-part}/{className}={id} | M |
| getMOIAttributes | GET | http://{URI-DN-PREFIX}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-FIRST-PAT}/{className}={id} | M |
| modifyMOIAttributes | PUT PATCH | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-first-part}/{className}={id} | M |
| deleteMOI | DELETE | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-first-part}/{className}={id} | M |
| subscribe | POST | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/subscriptions | M |
| unSubscribe | DELETE | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/subscriptions | M |
|  | DELETE | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/subscriptions/{subscriptionId} | M |

##### 12.1.1.1.2	Operation "createMOI"

This operation creates a single resource representing a managed object instance.

Table 12.1.1.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| managedObjectClass managedObjectInstance | path | /{className=id} | className: string id: string | M |
| referenceObjectInstance | n/a | n/a | n/a | n/a |
| attributeListIn | request body | data | resourcePut-RequestType | M |

The IS parameter "referenceObjectInstance" has no SS equivalent in the present document.

Table 12.1.1.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | data | resourceCreation-ResponseType | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow for creating a resource is as follows:

1.	The Service Consumer sends a HTTP PUT request to the Service Provider.

The target URI identifies the location of the new resource to be created.

The message body shall carry the complete representation of the resource to be created.

2.	The Service Provider sends a HTTP PUT response to the Service Consumer.

On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource and the message body the complete representation of the new resource.

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

##### 12.1.1.1.3	Operation "getMOIAttributes"

This operation retrieves one or multiple resources representing managed object instances.

Table 12.1.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | query | scope | scope-QueryType style: form explode: true | M |
| filter | query | filter | filter-QueryType | O |
| attributeListIn | query | attributes | attributes-QueryType style: form explode: false | O |
|  |  | fields | fields-QueryType style: form explode: false | O |

The SS parameters "scope", "filter", "attributes" and "fields" are defined in TS 32.158 [15].

Table 12.1.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | data | resourceRetrieval-ResponseType | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow for retrieval of one or multiple resources is as follows:

1.	The Service Consumer sends a HTTP GET request to the Service Provider.

The path component of the target URI identifies the base resource.

The "scope" query parameter identifies other resources besides the base resource.

The "filter" query parameter is applied to the set of scoped resources. Only resources passing the filter criteria are targeted.

The fields query parameter identifies the attributes to be returned.

2.	The Service Provider sends a HTTP GET response to the Service Consumer.

On success, "200 OK" shall be returned. The response message body is constructed according to the hierarchical response construction method (TS 32.158 [15]).

On failure, an appropriate error code shall be returned. The response message body should provide additional error information

##### 12.1.1.1.4	Operation "modifyMOIAttributes"

###### 12.1.1.1.4.x	Mapping to HTTP PUT

HTTP PUT is used for a full update of a single resource.

Table 12.1.1.1.4.x-1: Mapping of IS operation input parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | n/a | n/a | n/a | n/a |
| filter | n/a | n/a | n/a | n/a |
| modificationList | request body | data | resourcePut-RequestType | M |

The IS parameters "scope" and "filter" have no meaning when targeting a single resource with the target URI and are not mapped.

Table 12.1.1.1.4.x-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS operation parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type |  | SQ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| attributeListOut |  | response body |  | data |  | resourceUpdate-ResponseType |  | O |  |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow is as follows:

1.	The Service Consumer sends a HTTP PUT request to the Service Provider.

The target URI identifies the target resource.

The message body shall contain the representation the target resource shall be replaced with.

2.	The Service Provider sends a HTTP PUT response to the Service Consumer.

On success, "200 OK" or "204 No Content" shall be returned. In the former case the response carries the representation of the updated resource in the message body. In the latter case the response has no message body. A "200 OK" response including the representation of the updated resource shall be sent in case the updated representation of the resource is not identical to the representation received in the request.

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

###### 12.1.1.1.4.y	Mapping to HTTP PATCH

HTTP PATCH is used to create, update or delete one or multiple resources.

Table 12.1.1.1.4.y-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | n/a | n/a | n/a | n/a |
| filter | n/a | n/a | n/a | n/a |
| modificationList | request body | n/a | jsonMergePatch-RequestType jsonEnhancedMergePatch-RequestType jsonPatch-RequestType | M |

The IS parameters "scope" and "filter" have no SS equivalents in the present document.

The MnS producer shall support at least one media type out of

-	"application/merge-patch+json" (RFC 7396 [12])

-	"application/json-patch+json" (RFC 6902 [13])

-	"application/enhanced-merge-patch+json" (TS 32.158 [15])

Table 12.1.1.1.4.y-2: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | n/a | n/a | n/a | n/a |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow for modification of one or multiple resources is as follows:

1.	The Service Consumer sends a HTTP PATCH request to the Service Provider.

The path component of the target URI identifies the base resource.

The message body shall contain the patch document.

2.	The Service Provider sends a HTTP PATCH response to the Service Consumer.

On success, "200 OK" or "204 No Content" shall be returned. When "200 OK" is returned the message body shall include a representation of the updated resources constructed according to the hierarchical response construction method (TS 32.158 [15]).

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

##### 12.1.1.1.5	Operation "deleteMOI"

This operation deletes one or multiple resources representing managed object instances.

Table 12.1.1.1.5-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | query | scope | scope-QueryType style: form explode: true | M |
| filter | query | filter | filter-QueryType | O |

Table 12.1.1.1.5-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| deletionlist | response body | data | resourceDeletion-ResponseType | O |
| status | response status codes | n/a | n/a | M |
|  | response body | error | error-ResponseType | O |

The message flow for deletion of one or multiple resources is as follows:

1.	The Service Consumer sends a HTTP DELETE request to the Service Provider.

The path component of the target URI identifies the base resource.

If present, the "scope" query parameter identifies other resources besides the base resource.

The "filter" query parameter may be applied to the set of scoped resources. Only resources passing the filter criteria are targeted.

When no query parameters are present only the resource identified by the target URI shall be deleted.

2.	The Service Provider sends a HTTP DELETE response to the Service Consumer.

On success, when no query parameters are present in the request and only one resource is deleted, "204 No Content" shall be returned. Otherwise, when query parameters are present in the request, "200 OK" shall be returned and the response message body shall carry the URIs of the deleted resources.

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

##### 12.1.1.1.6	Operation "subscribe"

See clause 12.2.1.1.8.

##### 12.1.1.1.7	Operation "unsubscribe"

See clause 12.2.1.1.9.

Next modificatios

#### 12.1.1.3	Resources

##### 12.1.1.3.1	Resource structure

Figure 12.1.1.3.1-1 shows the resource structure of the Provisioning MnS. The "subscriptions" resource is a collection resource.

Figure 12.1.1.3.1-1: Resource URI structure of the Provisioning MnS

Table 12.1.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.1.1.3.1-1: Resources and methods overview

| Resource name |  | Resource URI |  | HTTP method |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOI |  | /{className}={id} |  | PUT |  | Creates a resource representing a managed object instance |  |
| MOI |  | /{className}={id} |  | GET |  | Retrieves one or multiple resources representing managed object instances |  |
| MOI |  | /{className}={id} |  | PATCH |  | Modifies one or multiple resources representing managed object instances |  |
| MOI |  | /{className}={id} |  | DELETE |  | Deletes one or multiple resources representing managed object instances |  |
| subscriptions | /subscriptions | POST | Creates a subscription |
| subscriptions | /subscriptions | DELETE | Deletes all subscriptions made with a consumerReferenceId |
| subscription | /subscriptions/{subscriptionId} | DELETE | Deletes a single subscription |
| notificationSink | /notificationSink | POST | Sends notifications |

##### 12.1.1.3.2	Resource definitions

###### 12.1.1.3.2.1	Resource "/{className}/{id}"

12.1.1.3.2.1.1	Description

This resource represents a managed object instance.

12.1.1.3.2.1.2	URI

Resource URI: http://{URI-DN-PREFIX}/{root}/ProvMnS/v1600/{LDN-first-part}

The resource URI variables a defined in the following table.

Table 12.1.1.3.2.1.2-1: URI variables

| Name |  | Definition |
| --- | --- | --- |
| URI-DN-prefix |  | See subclause 4.4 of TS 32.158 [15] |
| root |  | See subclause 4.4 of TS 32.158 [15] |
| LDN-first-part |  | See subclause 4.4 of TS 32.158 [15] |
| className |  | The class name of the resource to be targeted |
| id |  | The id of the resource to be targeted |
| Name | Definition |  |

12.1.1.3.2.1.3	HTTP methods

12.1.1.3.2.1.3.1	HTTP PUT

This method shall support the URI query parameters specified in the following table.

- Table 12.1.1.3.2.1.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

- Table 12.1.1.3.2.1.3.1-2: Data structures supported by the PUT request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| resourceCreation-RequestType | The resource representation of the resource to be created | M |

- Table 12.1.1.3.2.1.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| resourceCreation-ResponseType | 200 OK | The resource representation of the resource created | M |
| error-Type | 4xx/5xx | Returned in case of an error | O |

12.1.1.3.2.1.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

- Table 12.1.1.3.2.1.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| scope | scope-QueryType | This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. | M |
| filter | filter-QueryType | This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to "true" are targeted. | O |
| attributes | attributes-QueryType | This parameter specifies the attributes of the scoped resources that are returned. The value is a comma-separated list of attribute names. | O |
| fields | fields-QueryType | This parameter specifies the attribute fields of the scoped resources that are returned. The value is a comma-separated list of JSON pointers to the attribute fields. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

- Table 12.1.1.3.2.1.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |

- Table 12.1.1.3.2.1.3.2-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| resourceRetrieval-ResponseType | 200 OK | The resource representations of the resources retrieved. | M |
| error-Type | 4xx/5xx | Returned in case of an error | M |

12.1.1.3.2.1.3.3	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

- Table 12.1.1.3.2.1.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

- Table 12.1.1.3.2.1.3.3-2: Data structures supported by the PATCH request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| jsonMergePatch-RequestType jsonEnhancedMergePatch-RequestType jsonPatch-RequestType | Describes the set of modifications to be applied to the targeted resources. | M |

- Table 12.1.1.2.1.1.3.3-3: Data structures supported by the PATCH response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| error-Type | 4xx/5xx | Returned in case of an error | M |

12.1.1.3.2.1.3.4	HTTP DELETE

This method shall support the URI query parameters specified in the following table.

- Table 12.1.1.3.2.1.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| scope | scope-QueryType | This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. | M |
| filter | filter-QueryType | This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to "true" are targeted. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

- Table 12.1.1.3.2.1.3.4-2: Data structures supported by the DELETE request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

- Table 12.1.1.3.2.1.3.4-3: Data structures supported by the DELETE response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| resourceDeletion-ResponseType | 200 OK | The resources URI's deleted are returned. | M |
| error-Type | 4xx/5xx | Returned in case of an error | M |

Next modificatios

#### 12.1.1.4	Data type definitions

##### 12.1.1.4.1	General

Table 12.1.1.4.1-1: Data types defined in this specification

| Data type |  | Reference | Description |  |
| --- | --- | --- | --- | --- |
| General types |  |  |  |  |
| dateTime-Type |  |  | Date and time |  |
| long-Type |  | 12.1.1.4.4.2 | Long type |  |
| uri-Type |  | 12.1.1.4.4.2 | Type of an URI |  |
| Types used in paths |  |  |  |  |
| className-PathType |  | 12.1.1.4.4.2 | Used in the path to identify a resource object |  |
| id-PathType |  | 12.1.1.4.4.2 | Used in the path to identify a resource object |  |
| Types in query parts |  |  |  |  |
| attributes-QueryType | 12.1.1.4.2.x | Used in the query part of HTTP GET to select the attributes of the scoped resource objects to be returned. |
| fields-QueryType |  | 12.1.1.4.2.1 | Used in the query part of HTTP GET to select the resource object properties (attributes) to be returned |  |
| filter-QueryType |  | 12.1.1.4.2.2 | Used in the query part of HTTP GET, HTTP PATCH and HTTP DELETE to filter scoped resource objects |  |
| scope-QueryType |  | 12.1.1.4.2.3 | Used in the query part of HTTP GET, HTTP PATCH and HTTP DELETE to extend the set of targeted resources beyond the base resource identified with the path component of the URI |  |
| Types used in request bodies |  |  |  |  |
| resourcePut-RequestType |  | 12.1.1.4.2.4 | Used in the request body of HTTP PUT describing the resource to be created or updated |  |
| jsonMergePatch-RequestType | 12.1.1.4.2.y | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/merge-patch+json" (RFC 7396 [12]) |
| jsonEnhancedMergePatch-RequestType | 12.1.1.4.2.z | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/enhanced-merge-patch+json" (TS 32.158 [15]) |
| jsonPatch-RequestType |  | 12.1.1.4.2.5 | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources resources for the media type "application/json-patch+json" (RFC 6902 [13]) |  |
| subscription-RequestType |  | 12.1.1.4.2.12 | Used in the request body of HTTP POST on /subscriptions to create alarm notification subscriptions |  |
| Types used in response bodies |  |  |  |  |
| error-ResponseType |  | 12.1.1.4.2.6 |  |  |
| resourcePut-ResponseType |  | 12.1.1.4.2.7 | Used in the response body of HTTP PUT describing the resource created or updated |  |
| resourceDeletion-ResponseType |  | 12.1.1.4.2.8 | Used in the response body of HTTP DELETE identifying the URIs of the deleted resources |  |
| resourceModification-ResponseType |  | 12.1.1.4.2.9 | Used in the response body of HTTP PATCH describing the set of modified resources |  |
| resourceRetrieval-ResponseType |  | 12.1.1.4.2.10 | Used in the response body of HTTP GET to return the resources identified in the request for retrieval, or the selected attributes in case the fields query parameter is used |  |
| subscription-ResponseType |  | 12.1.1.4.2.13 | Used in the response body of HTTP POST on /subscriptions to create alarm notification subscriptions |  |
| Types used for resources |  |  |  |  |
| resourceRepresentationType |  | 12.1.1.4.2.11 | Used for resource representations |  |
| subscription-ResourceType |  |  | Representation of a subscription resource |  |
| Types used in notifications |  |  |  |  |
| notifyMOICreation-NotifType |  | 12.1.1.4.2.15 | Used in the request body of HTTP POST for the notification type notifyMOICreation |  |
| notifyMOIDeletion-NotifType |  | 12.1.1.4.2.16 | Used in the request body of HTTP POST for the notification type notifyMOIDeletion |  |
| notifyMOIAttributeValueChange-NotifType |  | 12.1.1.4.2.17 | Used in the request body of HTTP POST for the notification type notifyMOIAttributeValueChange |  |
| Types referenced by the definitions above |  |  |  |  |
| additionalText-Type |  | 12.1.1.4.4.2 | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |  |
| attributeNameValuePair-Type |  | 12.1.1.4.3.1 | Attribute name and attribute value pair |  |
| correlatedNotification-Type |  | 12.1.1.4.3.2 | Describes the correlated notifications of a single source |  |
| notificationId-Type |  | 12.1.1.4.4.2 | Notification identifier as defined in ITU-T Rec. X. 733 [4] |  |
| notificationType-Type |  | 12.1.1.4.4.3 | Notification type (notifyMOICreation, etc.) |  |
| sourceIndicator-Type |  | 12.1.1.4.4.4 | Indicates the source of the operation that led to the generation of the notification. |  |

##### 12.1.1.4.2	Query, message body and resource data types

###### 12.1.1.4.2.1	Type fields-QueryType

Table 12.1.1.4.2.1-1: Definition of type fields-QueryType

| Type | Definition | Description |
| --- | --- | --- |
| fields-QueryType | array(string) | Used in the query part of HTTP GET to select the resource object properties (attribute fields) to be returned |

###### 12.1.1.4.2.2	Type filter-QueryType

Table 12.1.1.4.2.2-1: Definition of type filter-QueryType

| Type | Definition | Description |
| --- | --- | --- |
| filter-QueryType | string | Used in the query part of HTTP GET, and HTTP DELETE to filter scoped resource objects |

###### 12.1.1.4.2.3	Type scope-QueryType

Table 12.1.1.4.2.3-1: Definition of type scope-QueryType

| Type | Definition | Description |
| --- | --- | --- |
| scope-QueryType | string | Used in the query part of HTTP GET, HTTP PATCH and HTTP DELETE to extend the set of targeted resources beyond the base resource identified with the path component of the URI |

###### 12.1.1.4.2.4	Type resourcePut-RequestType

Table 12.1.1.4.2.4-1: Definition of type resourcePut-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | resourceRepresentation-Type | Key indicating thev request body contains data. | M |

###### 12.1.1.4.2.5	Type jsonPatch-RequestType

Table 12.1.1.4.2.5-1: Definition of type jsonPatch-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | object | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/json-patch+json" (RFC 6902 [13]). | M |

###### 12.1.1.4.2.6	Type error-ResponseType

Table 12.1.1.4.2.6-1: Definition of type error-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| error | object | Key indicating the response body contains an error | M |
| > errorInfo | string | Attribute allowing to convey error information in string format | M |

###### 12.1.1.4.2.7	Type resourceCreation-ResponseType

Table 12.1.1.4.2.7-1: Definition of type resourceCreation-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | resourceRepresentation-Type | Used in the response body of HTTP PUT describing the resource created | M |

###### 12.1.1.4.2.8	Type resourceDeletion-ResponseType

Table 12.1.1.4.2.8-1: Definition of type resourceDeletion-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | array(uri-Type) | Used in the response body of HTTP DELETE identifying the URIs of the deleted resources | M |

###### 12.1.1.4.2.9	Type resourceModification-ResponseType

Table 12.1.1.4.2.9-1: Definition of type resourceModification-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | resourceRepresentation-Type | Used in the response body of HTTP PATCH describing the set of modified resources | M |

###### 12.1.1.4.2.10	Type resourceRetrieval-ResponseType

Table 12.1.1.4.2.10-1: Definition of type resourceRetrieval-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | resourceRepresentation-Type | Used in the response body of HTTP GET to return the resources identified in the request for retrieval, or the selected attributes in case the field query parameter is used | M |

###### 12.1.1.4.2.11	Type resourceRepresentation-Type

Table 12.1.1.4.2.11-1: Definition of type resourceRepresentation-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| href | uri-Type | The URI of the resource | M |
| class | string | The class name of the resource | M |
| id | string | The id of the resource object | M |
| attributes | object | The attributes object whose members are the class attributes and values. | M |

###### 12.1.1.4.2.12	Type subscription-RequestType

Table 12.1.1.4.2.12-1: Definition of type subscription-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | subscription-ResourceType | Used in the request body of HTTP POST on /subscriptions describing the representation of the subscription to be created | M |

###### 12.1.1.4.2.13	Type subscription-ResponseType

Table 12.1.1.4.2.13-1: Definition of type subscription-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | subscription-ResourceType | Used in the response body of HTTP POST on /subscriptions describing the representation of the created subscription | M |

###### 12.1.1.4.2.14	Type subscription-ResourceType

Table 12.1.1.4.2.14-1: Definition of type subscription-ResourceType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| consumerReference | uri-Type | The URI of the endpoint to send the notification to (/notificationSink). | M |
| timeTick | long-Type | Time window within which the subscriber intends to subscribe again to confirm its subscription, see clause 10.2.2.2.5.1 | O |
| filter | filter-Type | Filter settings for this subscription, to define the subset of all notifications this subscription relates to. A notification is sent to the subscriber if the filter matches, or if there is no filter. | O |

###### 12.1.1.4.2.15	Type notifyMOICreation-NotifType

Table 12.1.1.4.2.15-1: Definition of type notifyMOICreation-NotifType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource where the event (alarm) occurred | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyMOICreation) | M |
| > eventTime | dateTime-Type | Event (MOI creation) occurrence time | M |
| > systemDN | systemDN-Type | System DN | M |
| body |  |  |  |
| > correlatedNotifications | array(correlatedNotification-Type) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| > sourceIndicator | sourceIndicator-Type | Indicates the source of the operation that led to the generation of this notification. | O |
| > attributeList | array(attributeNameValuePair-Type) | The attributes (name/value pairs) of the created MOI. | O |

###### 12.1.1.4.2.16	Type notifyMOIDeletion-NotifType

Table 12.1.1.4.2.16-1: Definition of type notifyMOIDeletion-NotifType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource where the event (alarm) occurred | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyMOIDeletion) | M |
| > eventTime | dateTime-Type | Event (MOI creation) occurrence time | M |
| > systemDN | systemDN-Type | System DN | M |
| body |  |  |  |
| > correlatedNotifications | array(correlatedNotification-Type) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| > sourceIndicator | sourceIndicator-Type | Indicates the source of the operation that led to the generation of this notification. | O |
| > attributeList | array(attributeNameValuePair-Type) | The attributes (name/value pairs) of the deleted MOI. | O |

###### 12.1.1.4.2.17	Type notifyMOIAttributeValueChange-NotifType

Table 12.1.1.4.2.17-1: Definition of type notifyMOIAttributeValueChange-NotifType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource where the event (alarm) occurred | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyMOIAttributeValueChange) | M |
| > eventTime | dateTime-Type | Event (MOI creation) occurrence time | M |
| > systemDN | systemDN-Type | System DN | M |
| body |  |  |  |
| > correlatedNotifications | array(correlatedNotification-Type) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| > sourceIndicator | sourceIndicator-Type | Indicates the source of the operation that led to the generation of this notification. | O |
| > attributeList | array(attributeNameValuePair-Type) | The attributes (name/value pairs) of the modified MOI. | O |

#### 12.1.1.4.2.x	Type attributes-QueryType

Table 8.3.2.2-1: Definition of type filter-QueryType

| Type | Definition | Description |
| --- | --- | --- |
| attributes-QueryType | array(string) | Used in the query part of HTTP GET to select the attributes of the scoped resource objects to be returned. |

###### 12.1.1.4.2.y	Type jsonMergePatch-RequestType

Table 12.1.1.4.2.y-1: Definition of type jsonMergePatch-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | object | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/merge-patch+json" (RFC 7396 [12]) | M |

###### 12.1.1.4.2.z	Type jsonEnhancedMergePatch-RequestType

Table 12.1.1.4.2.z-1: Definition of type jsonMergePatch-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a | object | Used in the request body of HTTP PATCH describing the set of modifications to be applied to the targeted resources for the media type "application/enhanced-merge-patch+json" (TS 32.158 [15]) | M |

##### 12.1.1.4.3	Referenced structured data types

###### 12.1.1.4.3.1	Type attributeNameValuePair-Type

Table 12.1.1.4.3.1-1: Definition of type attributeNameValuePair-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| attributeName | string | Name of the attribute | M |
| attributeValue | anyType | Value of the attribute, can be any type | M |

###### 12.1.1.4.3.2	Type correlatedNotification-Type

Table 12.1.1.4.3.2-1: Definition of type correlatedNotification-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| source | uri-Type | Source of the correlated notifications | M |
| notificationIds | array(notificationId-Type) | Notification identifiers of correlated notifications of that source | M |

###### 12.1.1.4.4	Simple data types and enumerations

12.1.1.4.4.1	General

This clause defines simple data types and enumerations that are used by the data structures defined in the previous clauses.

12.1.1.4.4.2	Simple data types

Table 12.1.1.4.3.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| className-PathType | string | Used in the path component for the class name |
| subscriptionId-PathType | sring | Used in the path component to identify a subscription |
| id-PathType | string | Type used in the path component for the id. |
| consumerReferenceId-QueryType | uri-Type | Used in the query part of HTTP DELETE on /subscriptions to delate all subscriptions made with a specific consumerReferenceId |
| additionalText-Type | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| filter-Type | string | Filter of a subscription resource |
| notificationId-Type | long | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| scopeLevel-Type | integer | Scope level |
| systemDN-Type | string | Type of the System DN |

12.1.1.4.4.3	Enumeration notificationType-Type

Table 12.1.1.4.4.3-1: Enumeration notificationType-Type

| Enumeration value | Description |
| --- | --- |
| notifyMOICreation | Notification type is notifyMOICreation |
| notifyMOIDeletion | Notification type is notifyMOIDeletion |
| notifyMOIAttributeValueChange | Notification type is notifyMOIAttributeValueChange |

12.1.1.4.4.4	Enumeration sourceIndicator-Type

Table 12.1.1.4.4.4-1: Enumeration sourceIndicator-Type

| Enumeration value | Description |
| --- | --- |
| resourceOperation | The notification was generated in response to an internal operation of the resource. |
| mangementOperation | The notification was generated in response to a management operation applied across the managed object boundary external to the managed object |
| sONOperation | The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . |
| unknown | It is not possible to determine the source of the operation. |

#### 12.1.1.4.4.x	Enumeration scopeType-Type

Table 8.3.4.4-1: Enumeration scopeType-Type

| Enumeration value | Description |
| --- | --- |
| BASE_ONLY | Selects only the base resource. The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_ALL | Selects the base resource and all of its subordinate resources (incl. the leaf resources). The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_NTH_LEVEL | Selects all resources on the level, which is indicated by the "scopeLevel" parameter, below the base resource. The base resource is at "scopeLevel" zero. |
| BASE_SUBTREE | Selects the base resource and all of its subordinate resources down to and including the resources on the level indicated by the "scopeLevel" parameter. The base resource is at "scopeLevel" zero. |

Next modificatios

## A.1	Provisioning management service

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.532 Provisioning Management Service",

"version": "16.2.0",

"description": "OAS 3.0.1 specification of the Provisioning Management Service"

},

"servers": [

{

"url": "http://{URI-DN-prefix}/{root}/ProvMnS/v1620/{LDN-first-part}",

"variables": {

"URI-DN-prefix": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"root": {

"description": "See subclause 4.4 of TS 32.158",

"default": "3GPPManagement"

},

"LDN-first-part": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/{className}={id}": {

"parameters": [

{

"name": "className",

"in": "path",

"required": true,

"schema": {

"$ref": "#/components/schemas/className-PathType"

}

},

{

"name": "id",

"in": "path",

"required": true,

"schema": {

"$ref": "#/components/schemas/id-PathType"

}

}

],

"put": {

"summary": "Updates a single resource or creates it if it does not exist",

"description": "With HTTP PUT a complete resource is updated or created if it does not exist. The target resource is identified by the target URI.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourcePut-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\") for resource update. The representation of the updated resource is returned in the message body. This response shall be returned when the updated resource representation is not identical to the representation in the request.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceUpdate-ResponseType"

}

}

}

},

"201": {

"description": "Success case (\"201 Created\") for resource creation. The representation of the created resource is returned in the message body.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceCreation-ResponseType"

}

}

}

},

"204": {

"description": "Success case (\"204 No Content\") for resource update. The response has no message body. This response may be returned only when the updated resource representation is identical to the representation in the request."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"get": {

"summary": "Read resources",

"description": "With HTTP GET resources are read. The resources to be read are identified with the path component (base resource) and the query component (scope, filer) of the URI. The fields query component allows to select the resource properties to be returned.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI.",

"required": true,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

},

"style": "form",

"explode": true

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are targeted.",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

},

{

"name": "attributes",

"in": "query",

"description": "This parameter specifies the attributes of the scoped resources that are returned.",

"required": true,

"schema": {

"$ref": "#/components/schemas/attributes-QueryType"

},

"style": "form",

"explode": false

},

{

"name": "fields",

"in": "query",

"description": "This parameter specifies the attribute field of the scoped resources that are returned.",

"required": false,

"schema": {

"$ref": "#/components/schemas/fields-QueryType"

},

"style": "form",

"explode": false

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected attributes are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"patch": {

"summary": "Patch one or multiple resources",

"description": "With HTTP PATCH resources are created, updated or deleted. The resources to be modified are identified with the path component of the URI (base resource) and with the patch document.",

"requestBody": {

"description": "The request body describes changes to be made to the target resources as defined in RFC 7396 (JSON Merge Patch).",

"required": true,

"content": {

"application/merge-patch+json": {

"schema": {

"$ref": "#/components/schemas/jsonMergePatch-RequestType"

}

},

"application/enhanced-merge-patch+json": {

"schema": {

"$ref": "#/components/schemas/jsonEnhancedMergePatch-RequestType"

}

},

"application/json-patch+json": {

"schema": {

"$ref": "#/components/schemas/jsonPatch-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\")."

},

"204": {

"description": "Success case (\"204 No Content\")."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"delete": {

"summary": "Delete one or multiple resources",

"description": "With HTTP DELETE resources are deleted. The resources to be deleted are identified with the path component (base resource) and the query component (scope, filer) of the URI.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI.",

"required": true,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

}

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are returned.",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

}

],

"responses": {

"200": {

"description": "On success, when query parameters are present in the request, \"200 OK\" is returned. The message body carries the URIs of the deleted resources."

},

"204": {

"description": "On success, when no query parameters are present in the request and only one resource is deleted, \"204 No Content\" is returned. The message body is empty.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceDeletion-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"uri-Type": {

"type": "string"

},

"nrm-Type": {

"type": "object"

},

"scopeType-Type": {

"type": "string",

"enum": [

"BASE_ONLY",

"BASE_NTH_LEVEL",

"BASE_SUBTREE",

"BASE_ALL"

]

},

"scopeLevel-Type": {

"type": "integer"

},

"className-PathType": {

"type": "string"

},

"id-PathType": {

"type": "string"

},

"attributes-QueryType": {

"type": "array",

"items": {

"type": "string"

}

},

"fields-QueryType": {

"type": "array",

"items": {

"type": "string"

}

},

"filter-QueryType": {

"type": "string"

},

"scope-QueryType": {

"type": "object",

"properties": {

"scopeType": {

"$ref": "#/components/schemas/scopeType-Type"

},

"scopeLevel": {

"$ref": "#/components/schemas/scopeLevel-Type"

}

}

},

"resourcePut-RequestType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"jsonMergePatch-RequestType": {

"$ref": "#/components/schemas/nrm-Type"

},

"jsonEnhancedMergePatch-RequestType": {

"$ref": "#/components/schemas/nrm-Type"

},

"jsonPatch-RequestType": {

"type": "object"

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"resourceCreation-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"resourceUpdate-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"resourceDeletion-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/uri-Type"

}

}

}

},

"resourceRetrieval-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

}

},

"resourceRepresentation-Type": {

"type": "object",

"properties": {

"href": {

"$ref": "#/components/schemas/uri-Type"

},

"class": {

"type": "string"

},

"id": {

"type": "string"

},

"attributes": {

"type": "object"

}

}

}

}

}

}

End of modificatios