3GPP TSG-SA5 Meeting #127	S5-196684

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019                    Revision of S5-196258

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0070 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify CHF selection via NRF and roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2019-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CHF selection via NRF is unclear. In roaming a CHF in VPLMN needs to be selected by the SMSF in VPLMN, and the selection mechanism via NRF is not specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify CHF selection mechanism in non-roaming and roaming. Add an indicator for in-bound roamers in SMS Charging Information for converged charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging not possible for SMSs due to unappropriate CHF selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.1.x(New), 6.5.2, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.4.1.1	General

Converged charging may be performed by the SMSF interacting with CHF using Nchf specified in TS 32.290 [19] and TS 32.291 [20]. In order to provide the data required for the management activities outlined in TS 32.240 [2] (Credit-Control, accounting, billing, statistics etc.), the SMSF shall be able to perform converged charging for each of the SMS transactions.

The SMSF shall be able to perform convergent charging by interacting with CHF, for charging data related to SMS over NAS. The Charging Data Request and Charging Data Response are exchanged between the SMSF and the CHF, based on either IEC or ECUR scenarios specified in TS 32.290 [19]. The Charging Data Request is issued by the SMSF towards the CHF when certain conditions (chargeable events) are met.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [19].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following sub-clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [20].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [3].

The chargeable events or messages exchanged between the SMSF and the other nodes are described with generic names 
(i.e SMS submit, SMS deliver), to reflect SMS reception or sending by/from the SMSF, independently from the protocol conveying the SMS.

Next change

#### 5.4.1.x	CHF selection

The CHF to be used by the SMSF can be:

-	Discovered via NRF upon SMS activation from AMF to SMSF.

-	SMSF locally provisioned in charging characteristics.

The option depends on Operator's policies.

When CHF selection by SMSF is performed via NRF based discovery:

-	For UE determined by SMSF as served by the same PLMN as the SMSF (i.e. non-roaming scenario), the CHF of the PLMN can be discovered based on the UE identifier.

-	For UE determined by SMSF in V-PLMN as an in-bound roamer (i.e. roaming scenario), the CHF of the V-PLMN can be discovered based on the PLMN Id of the UE H-PLMN.

Next change

### 6.5.2	Definition of SMS charging information

SMS specific charging information used for SMS converged charging is provided within the SMS charging Information.

# Table 6.5.2.1: Structure of SMS Charging information

| Information Element | Category | Description |
| --- | --- | --- |
| Originator Info | OM | This field is a grouped field and holds information on originator of the SMS |
| Originator SUPI | OM | This field holds the SUPI of the originator of the SMS, if available. This field is present if different from subscriber identifier field. |
| Originator GPSI | OC | This field holds the GPSI of the originator of the SMS, if available. |
| Originator Other Address | OM | This field holds the address of the originator of the SMS, when different from SUPI and GPSI, if available: e.g. email, short code. This field may have multiple occurences. |
| Originator Received Address | OC | Described in table 6.3.1.2.1 |
| Originator SCCP Address | OC | Described in table 6.3.1.2.1 |
| SM Originator Interface | OM | Described in table 6.3.1.2.1 |
| SM Originator Protocol Id | OC | Described in table 6.3.1.2.1: SM Protocol Id information element |
| Recipient Info | OC | Described in table 6.3.1.2.1 |
| Recipient SUPI | OM | This field holds the SUPI of the recipient of the SMS, if available. This field is present if different from subscriber identifier field. |
| Recipient GPSI | OC | This field holds the GPSI of the recipient of the SMS, if available. |
| Recipient Other Address | OC | This field holds the address of the recipient of the SMS, when different from SUPI and GPSI, if available: e.g. email, short code. This field may have multiple occurences |
| Recipient Received Address | OC | Described in table 6.3.1.2.1 |
| Recipient SCCP Address | OC | Described in table 6.3.1.2.1 |
| SM Destination Interface | OM | Described in table 6.3.1.2.1 |
| SM Recipient Protocol Id | OC | Described in table 6.3.1.2.1: SM Protocol Id information element. |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) used by the UE the SMS transaction, if available. |
| Roamer In Out | OC | This field holds an indication of the UE is an in-bound roamer. This field is present only if UE is identified as a roamer. |
| User Location Info | OC | Described in table 6.3.1.2.1 |
| UE Time Zone | OC | Described in table 6.3.1.2.1 |
| RAT Type | OC | Described in table 6.3.1.2.1 |
| SMSC Address | OM | Described in table 6.3.1.2.1 |
| SM Data Coding Scheme | OM | Described in table 6.3.1.2.1 |
| SM Message Type | OM | Described in table 6.3.1.2.1 |
| SM Reply Path Requested | OC | Described in table 6.3.1.2.1 |
| SM User Data Header | OC | Described in table 6.3.1.2.1 |
| SM Status | OC | Described in table 6.3.1.2.1 |
| SM Discharge Time | OC | Described in table 6.3.1.2.1 |
| Number of Messages Sent | OC | Described in table 6.3.1.2.1 |
| SM Service Type | OC | Described in table 6.3.1.2.1 |
| SM Sequence Number | OC | Described in table 6.3.1.2.1 |
| SMS result | C | Described in table 6.3.1.2.1 |
| Submission Time | OC | Described in table 6.3.1.2.1 |
| SM Priority | OC | Described in table 6.3.1.2.1 |
| Message Reference | OM | Described in table 6.3.1.2.1 |
| Message Size | OM | Described in table 6.3.1.2.1 |
| Message Class | OM | Described in table 6.3.1.2.1 |
| Delivery Report Requested | OC | Described in table 6.3.1.2.1 |

Next change

### 6.5.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMS Node for 5G SMS converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.5.3.1 defines the basic structure of the supported fields in the Charging Data Request message for SMS converged charging.

# Table 6.5.3.1: Supported fields in Charging Data Request message

| Information Element | Node Type | SMSF |
| --- | --- | --- |
|  | Supported Operation Types | ITE |
| Session Identifier |  | ITE |
| Subscriber Identifier |  | ITE |
| NF Consumer Identification |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Sequence Number |  | ITE |
| Notify URI |  | - |
| Triggers |  | ITE |
| Multiple Unit Usage |  | ITE |
| SMS Charging Information |  |  |
| Originator Info |  | ITE |
| Recipient Info |  | ITE |
| User Equipment Info |  | ITE |
| Roamer In Out |  | ITE |
| User Location Info |  | ITE |
| UE Time Zone |  | ITE |
| RAT Type |  | ITE |
| SMSC Address |  | ITE |
| SM Data Coding Scheme |  | ITE |
| SM Message Type |  | ITE |
| SM Reply Path Requested |  | ITE |
| SM User Data Header |  | ITE |
| SM Status |  | ITE |
| SM Discharge Time |  | ITE |
| Number of Messages Sent |  | ITE |
| SM Service Type |  | ITE |
| SM Sequence Number |  | ITE |
| SMS result |  | ITE |
| Submission Time |  | ITE |
| SMPriority |  | ITE |
| Message Reference |  | ITE |
| Message Size |  | ITE |
| Message Class |  | ITE |
| Delivery Report Requested |  | ITE |

Table 6.5.3.2 defines the basic structure of the supported fields in the Charging Data Response message for SMS converged charging.

# Table 6.5.3.2: Supported fields in Charging Data Response message

| Information Element | Node Type | SMSF |
| --- | --- | --- |
|  | Supported Operation Types | ITE |
| Session Identifier |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Result |  | ITE |
| Invocation Sequence Number |  | ITE |
| Session Failover |  | I-- |
| Triggers |  | - |
| Multiple Unit information |  | - |

End of changes