3GPP TSG-<SA5> Meeting # 127	S5-196437

# Sophia Antipolis, France,  14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0007 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct resource representation formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 32.160 rules for mapping stage 2 NRM artefacts to stage 3 JSON schema has been specified. This requires some updates to the clause on resoure representation formats. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The clause on resoure representation formats is aligned with the mapping rules in TS 32.160. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The resource representation format described in TS 32.158 is not aligned with the mapping rules in TS 32.160 leading to interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6, 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

## 7.6	Resource objects

Resource objects (resources) are representations of managed object instances. They shall be compliant to the following JSON schema when one instance of a class is allowed.

{

"type": "object",

"properties": {

"ClassName": {

"type": "object",

"properties": {

"href": { "type": "string" },

"class": { "type": "string" },

"id": { "type": "string" },

"attributes": {

"type": "object",

"properties": {}

}

},

"required": ["id"]

}

}

}

or by the following schema when more than one instance of a class is allowed

{

"type": "object",

"properties": {

"ClassName": {

"type": "array",

"items": {

"type": "object",

"properties": {

"href": { "type": "string" },

"class": { "type": "string" },

"id": { "type": "string" },

"attributes": {

"type": "object",

"properties": {}

}

},

"required": ["id"]

}

}

}

}

An object, whose name is equal to the NRM class name, encapsulates the resource representation.

The "attributes" object contains NRM attributes as properties. In the generic schema above the "attributes" object has no properties. These properties are defined in other specifications.

Only the "id" is required to be always present. The "href" property with the URI of the resource and the "class" property with the name of the NRM class can be omitted, or not specified at all in concrete JSON schemas for resource representations.

TS 32.160 [16] specifies the complete mapping of stage 2 NRM definitions to stage 3 JSON schema definitions.

Next modificatio

## 7.7	Resource objects carried in data objects and arrays

When a resource object is carried in a data object the schema is given by

{

"type": "object",

"properties": {

"data": {

"type": "object",

"properties": {

"ClassName": {

"type": "object",

"properties": {

"href": { "type": "string" },

"class": { "type": "string" },

"id": { "type": "string" },

"attributes": {

"type": "object",

"properties": {}

}

},

"required": ["id"]

}

}

}

}

}

Multiple instance of the same NRM class are supported by a JSON array.

{

"type": "object",

"properties": {

"data": {

"type": "object",

"properties": {

"ClassName": {

"type": "array",

"items": {

"type": "object",

"properties": {

"href": { "type": "string" },

"class": { "type": "string" },

"id": { "type": "string" },

"attributes": {

"type": "object",

"properties": {}

}

},

"required": ["id"]

}

}

}

}

}

}

End of modificatios