SA WG2 Meeting #S2-135	S2-1909005

14 - 18 October, 2019, Split, Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0046 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to bulk operation MT-LR procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | -In clause 6.8.1, “UE registration status” is subscribed from UDM. This is not a defined event for the Nudm_EventExposure_Subscribe service. -Misspelling of “exposure” in fig 6.8.1 and step 5a and 8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Change to subscription of “UE reachability status” event from UDM. -Correct typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure in 6.8.1 would not be consistent with the Nudm services defined in 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

## 6.8	Bulk Operation of LCS Service Request Targeting to Multiple UEs

The procedure described in this clause applies to 5GC_MT_LR and Deferred 5GC-MT-LR request targeting to a group of UE identified by an internal group ID, if available.

Figure 6.8.1: Bulk operation of LCS service request targeting to multiple UEs

1.	This step is the same as step 1 of clause 6.1.2 and step 1 of clause 6.3, with the difference that the LCS request is targeting a group of UE identified by a group ID. NEF/GMLC may map the external group ID to the list of UE ID (i.e. SUPI) using Nudm_SDM_Get (Group Identifier Translation, External Group Identifier) service operation.

2.	If deferred location is requested, the GMLC responds to the LCS Client / AF with an acknowledgment.

Steps 3 to 5 are carried out once per UE.

3.	The GMLC invokes a Nudm_SDM_Get (LCS privacy, SUPI) service operation towards the UDM to get the privacy settings of the target UE. The GMLC checks the privacy settings. For the UE whose privacy setting does not allow it to be located, steps 4 and 5 are skipped. The GMLC may also subscribe to and receive notification of UE privacy profile updates according to steps 0 and 4 of clause 6.12.1.

4.	The GMLC invokes a Nudm_UECM_Get service operation towards the UDM of the target UE with SUPI of this UE. The UDM returns the current serving AMF ID to the GMLC.

5a.	If no AMF ID is returned at step 4:

-	if it is a deferred location request and the GMLC supports the storage of the LCS service request for a group of UE, the GMLC subscribes the UE reachability status to the UDM using Nudm_EventExposure_Subscribe service operation with the data key "SUPI";

-	otherwise, the GMLC returns an appropriate error message to the LCS client or AF.

5b. If the UDM returns the current serving AMF ID to the GMLC at step 4:

-	the GMLC initiates 5GC_MT_LR procedure (from step 4 onwards) as described in clause 6.1.3;

-	or the GMLC initiates Deferred 5GC-MT-LR Procedure (from step 4 onwards) as described in clause 6.3 with the difference that Step 8 is skipped.

6.	For the deferred location request, if any UE in the group didn't get its serving AMF ID at step 4, Tthe GMLC may store the LCS service request locally if the GMLC supports the storage of the LCS service request for a group of UE; otherwise, this step is skipped.

Further steps apply to the UEs of the group who was not registered to the network when the LCS service request is received at GMLC.

7.	UE performs the registration as described in clause 4.2.2 of TS 23.502, during which an AMF is selected to serve the UE, and the AMF ID is stored into UDM.

8. UDM notifies the GMLC who had subscribed the UE registration at step 4a using Nudm_ EventExposure_Notify service operation, which includes "SUPI" and UE registration status.

9.	GMLC initiates 5GC_MT_LR procedure (from step 2 onwards) or Deferred 5GC-MT-LR Procedure (from step 2 onwards) as described in clause 6.1.3 or clause 6.3.

***** END OF CHANGES *****