3GPP TSG-SA2 Meeting #136	S2-1912213

# 18 - 22 November, 2019, Reno, Nevada, USA                                       (was S2-1911805)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0060 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the procedure of obtaining Non-UE Associated Network Assistance Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The procedure for Obtaining Non-UE Associated Network Assistance Data is not associated with a UE location session, so the UE identifier should not be included in step 5. - The UE’s funcationality is not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding one UE funcationality. - In clause 6.11.3, the procedure for Obtaining Non-UE Associated Network Assistance Data is not associated with a UE location session, so the UE identifier should not be included in step 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - One parameter in step 5 in clause 6.11.3 is not correct. - The UE’s funcationality is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5, 6.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution is the resubmission of the CR0060 (S2-1909776), which was not open at SA2#135 meeting. |  |  |  |  |  |  |  |  |

**************** First Change ****************

### 4.3.5	UE

A target UE may support positioning according to four different modes:

-	UE assisted mode (the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location);

-	UE based mode (the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN);

-	standalone mode (the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN);

-	network based mode (a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate).

NOTE:	The transmission of UE signals for network based mode may or may not be transparent to the UE.

Positioning procedures used by a UE for NG-RAN access are described in TS 38.305 [9].

A limited set of UE positioning capabilities can be transferred to the 5GCN during registration of the UE as described in TS 24.501 [11]. Some of these positioning capabilities may be transferred subsequently to an LMF as described in TS 29.572 [12]. UE positioning capabilities may also be transferred directly to a location server (e.g. LMF).

Additional functions which may be supported by a UE to support location services include the following.

-	Support location requests received from a network for 5GC-MT-LR, 5GC-NI-LR or a deferred 5GC-MT-LR for periodic or triggered location.

-	Support location requests to a network for a 5GC-MO-LR.

-	Support privacy notification and verification for a 5GC-MT-LR or deferred 5GC-MT-LR for periodic or triggered location.

-	Send updated privacy requirements to a serving AMF (for transfer to a UDR via UDM).

-	Support periodic or triggered location reporting to an LMF.

-	Support change of a serving LMF for periodic or triggered location reporting.

-	Support cancelation of periodic or triggered location reporting.

-	Support multiple simultaneous location sessions.

-	Support the reception of unciphered and/or ciphered assistance data broadcast by NG-RAN.

-	Support the reception of ciphering keys for the assistance data from the AMF.

**************** Second Change ****************

### 6.11.3	Obtaining Non-UE Associated Network Assistance Data

Figure 6.11.3-1 shows a procedure which may be used by an LMF to support network assisted and network based positioning. This procedure is not associated with a UE location session. It is used to obtain network assistance data from a NG-RAN node (e.g. gNB or ng-eNB). The procedure may be based on an NRPPa protocol in TS 38.455 [15] between the LMF and NG-RAN.

Figure 6.11.3-1: Procedure for Obtaining Non-UE Associated Network Assistance Data

1.	The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Network Positioning message to a NG-RAN node (gNB or ng-eNB) in the NG-RAN. The service operation includes the Network Positioning message and the target NG-RAN node identity. The Network Positioning message may request position related information from the NG-RAN.

2.	The AMF forwards the Network Positioning message to the target NG-RAN node indicated in step 1 in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF.

3.	The target NG-RAN node obtains any position related information requested in step 2.

4.	The target NG-RAN node returns any position related information obtained in step 3 to the AMF in a Network Positioning message included in an N2 Transport message. The target NG-RAN node shall also include the Routing identifier in the N2 Transport message received in step 2.

5.	The AMF invokes the Namf_Communication_N2InfoNotify service operation towards the LMF indicated by the routing identifier received in step 4. The service operation includes the Network Positioning message received in step 4. Steps 1 to 5 may be repeated to request further position related information from the NG-RAN.

**************** End Changes ****************