# 3GPP TSG SA Meeting #87E	SP-200089

# 17-20 March 2020, Electronic meeting

Source:	SA WG2

Title:	New WID: Support of different slices over different Non 3GPP access (TEI17)

Document for:	Approval

Agenda Item:	6.4

3GPP SA WG2 Meeting #137E 	S2-2002602

Feb 24 - 27, 2020, Elbonia              	(revision of S2-2002043)

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

## Title:	Support of different slices over different Non 3GPP access

## Acronym:	TEI17_N3SLICE

## Unique identifier:	870008

## Potential target Release:	Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

## 3	Justification

Per Current 3GPP specifications,

- the AMF receives from the 5G AN (in N3GPP case: N3IWF / TNGF / W-AGF) information on the slices (S-NSSAI) supported by the 5G AN TA(s). This is done as part of NG set up request (38.413)

| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TA. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

- all the 5G AN (N3IWF / TNGF / W-AGF) correspond to the same unique TA

This mean that support of slices has to be homogeneous between (Un)trusted Non 3GPP access (N3IWF/TNGF)  and wireline access (W-AGF) as well as between different wireline AN of a PLMN;

Requiring the support of slices to be homogeneous between (Un)trusted Non 3GPP access (N3IWF/TNGF) and wireline access (W-AGF) looks like an un-necessary restriction.

## 4	Objective

The objective for this work item is to specify the support of the possibility for operators to allocate a TAI per non 3GPP access gateway (e.g. N3IWF / TNGF / W-AGF) : each non 3GPP access gateway is locally configured with its own TAI and the slices it supports. The TAI value is provided (as in case of 3GPP access) to AMF over N2 in NG SET UP message. There is one TAI value per non 3GPP access gateway.

Different  non 3GPP access gateways (e.g. different N3IWF / TNGF / W-AGF) can thus advertise different TAI values, and can support different slices.

As TAI values are not advertised over Non 3GPP access, UE(s) do not need to be aware of  the fact that different non 3GPP access gateways may correspond to a different TAI beyond the fact that they (UE) may receive different TAI values within the TAI list sent by the network over successive registration over non 3GPP access.

No protocol impact is expected.

Expected load is 0,5 TU.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Support of different slices over different Non 3GPP access | TSG#88 |  |
| 23.502 | Support of different slices over different Non 3GPP access | TSG#88 |  |

## 6	Work item Rapporteur(s)

Laurent.thiebaut@nokia.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Motorola Mobility

Lenovo

Orange

Deutsche Telecom

Telecom Italia

Vodafone