3GPP TSG-SA WG6 Meeting #37-e	S6-200717

# e-meeting, 14th – 26th May 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0218 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the one-to-one SDS and FD communication upgrade flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2020-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Step related to the continuation of communication after bearer upgrade is saying floor control which is not applicable for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the step. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.8.2, 7.5.2.11.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

##### 7.4.2.8.2	Procedure

The procedure in figure 7.4.2.8.2-1 describes the case where an authorized MCData user is upgrading an ongoing MCData one-to-one SDS communication to an MCData emergency one-to-one SDS communication. This procedure is applicable only when MCData one-to-one SDS communication is established as described in subclause 7.4.2.3 "One-to-one standalone short data service using media plane" or as described in subclause 7.4.2.4 "One-to-one short data service session".

Pre-conditions:

1.	Both members of the MCData one-to-one SDS communication belong to the same MCData system.

2.	MCdata one-to-one SDS communication is already in progress.

Figure 7.4.2.8.2-1 MCData one-to-one SDS communication upgraded to MCData emergency one-to-one SDS communication

1.	The MCData user at MCData client 1 initiates an emergency. MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled.

2.	MCData client 1 requests the MCData server to upgrade the one-to-one MCData SDS communication to in-progress emergency by sending a MCData one-to-one SDS communication upgrade request.

3.	The MCData server sends the MCData one-to-one SDS communication upgrade request towards the MCData client of the other participant.

4.	The MCData user is notified of the in-progress emergency of the MCData emergency one-to-one SDS communication.

5.	The receiving MCData client acknowledges the MCData one-to-one SDS communication upgrade request and sends MCData one-to-one SDS communication upgrade response to the MCData server.

6.	The MCData server adjusts the priority of the underlying bearer for both participants of the MCData one-to-one SDS communication. The priority is retained until the communication session ends.

7.	The MCData server sends MCData one-to-one SDS communication upgrade response to MCData client 1.

8.	MCData client 1 and MCData client 2 continue with the MCData one-to-one SDS communication, which has been transformed into an MCData emergency one-to-one SDS communication.

* * * * * * * SECOND CHANGE * * * * * * *

##### 7.5.2.11.2	Procedure

The procedure in figure 7.5.2.11.2-1 describes the case where an authorized MCData user is upgrading a MCData one-to-one FD communication to a MCData emergency one-to-one FD communication. This procedure is applicable only when MCData one-to-one file distribution communication is established as described in subclause 7.5.2.5 "One-to-one file distribution using media plane".

Pre-conditions:

1.	Both members of the one-to-one FD communication belong to the same MCData system.

2.	One-to-one FD communication is already in progress.

Figure 7.5.2.11.2-1 One-to-one FD communication upgrade to an emergency one-to-one FD communication

1.	The MCData user at MCData client 1 initiates an emergency. MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled.

2.	MCData client 1 requests the MCData server to upgrade the MCData one-to-one FD communication to in-progress emergency by sending a MCData one-to-one FD upgrade request.

3.	The MCData server sends the MCData one-to-one FD upgrade request towards the MCData client of the other participant.

4.	The MCData user is notified of the in-progress emergency of the MCData emergency one-to-one FD communication.

5.	The receiving MCData client acknowledges the MCData one-to-one FD upgrade request and sends MCData one-to-one FD upgrade response to the MCData server.

6.	The MCData server adjusts the priority of the underlying bearer for both participants of the MCData one-to-one FD communication. The priority is retained until the communication ends.

7.	The MCData server sends MCData one-to-one FD upgrade response to MCData client 1.

8.	MCData client 1 and MCData client 2 continue with the MCData one-to-one FD communication, which has been transformed into an MCData emergency one-to-one FD communication.

* * * * * * * END CHANGES * * * * * * *