3GPP TSG-SA3 Meeting #99e 	S3-201103

# e-meeting, 11 – 15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.515 | CR | 0005 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deletion of the test case on TEID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 28/04/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The test case on TEID uniquess shall be deleted from the present document, to align with the recent modification made to 23.501. The test case in clause 4.2.2.1.2 of the present document tests the funcitainality of SMF to generate a unique TEIDs for each session. The test was generated based on the requirement from clause 5.8.2.3.1, 23.501, v16.3.0 that “Allocation and release of CN Tunnel Info is performed when a new PDU Session is established or released. This functionality is supported either by SMF or UPF, based on operator's configuration on the SMF.” However, clause 5.8.2.3.1, 23.501, v16.4.0 has the following modification to allow only UPF allocate the CN Tunnel Info: “The CN Tunnel Info allocation and release is performed by the UPF. The SMF shall indicate to the UPF when the UPF is required to allocate/release CN Tunnel Info. Therefore, the test case of SMF generating TEID shall be removed from the present document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the test case on TEID uniqueness |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existance of the test case that no longer has the requirement Misalignment of specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** First Change ********************************

## 4.2 	SMF-specific security functional adaptations of requirements and related test cases

### 4.2.1	Introduction

Present clause contains SMF-specific security functional adaptations of requirements and related test cases.

### 4.2.2	Security functional requirements on the SMF deriving from 3GPP specifications and related test cases

#### 4.2.2.1	Security functional requirements on the SMF deriving from 3GPP specifications

##### 4.2.2.1.1	Priority of UP security policy

Requirement Name: Priority of UP security policy

Requirement Reference: TS 23.501 [1], clause 5.10.3

Requirement Description: "User Plane Security Policy from UDM takes precedence over locally configured User Plane Security Policy." as specified in TS 23.501 [1], clause 5.10.3

Threat References: TR 33.926 [4], clause J.2.2.1 Non-compliant UP security policy handling

Test Case:

Test Name: TC_UP_POLICY_PRECEDENCE_SMF

Purpose:

Verify that the user plane security policy from the UDM takes precedence at the SMF under test over locally configured user plane security policy.

Pre-Conditions:

Test environment with AMF and UDM may be simulated.

Both UDM and SMF under test are configured with UP security policy, and the UP security policies are different.

There is no Session Management Subscription data in SMF.

Execution Steps

1)	The tester triggers PDU session establishment procedure by sending Nsmf_PDUSession_CreateSMContext Request message to the SMF.

2)	The SMF under test retrieves the Session Management Subscription data using Nudm_SDM_Get service from UDM, where the Session Management Subscription data includes the user plane security policy stored in UDM.

3)	The tester captures the Namf_Communication_N1N2MessageTransfer message sent from the SMF under test to the AMF.

Expected Results:

There is a Security Indication IE in the N2 SM information contained in the Namf_Communication_N1N2MessageTransfer message, which is the same with the UP security policy configured in the UDM.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

void

***************************** First Change ********************************