3GPP TSG-SA4 Meeting #109-e 	S4-200982

# Electronic meeting, Telco, May 25-29 and June 2-3, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0164 | rev | 2 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Partial File Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Enensys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDASH, TEI15 |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Partial file handling operation in the context of an DASH-over-MBMS service is described in clauses 5.3.2 and 7.3.8 of TS 26.247, and are based on/aligned with the partial file delivery mechanisms for MBMS as defined in TS 26.346. A CR against Rel-15 TS 26.346 in Tdoc S4-200712 is submitted to the SA4#109e meeting to propose a correction to the existing partial file handling functionality – namely, by adding support for independent signaling of partial file handling capability by the HTTP server (e.g., the MBMS client) to the MBMS application. This contribution represents a companion and auxiliary CR to that document by providing corresponding corrective text on partial file handling functionality as described in TS 26.247, to maintain alignment with the mechanisms as defined in TS 26.346. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Spec text change to permit the MBMS application to initially request strictly the full file, and upon receiving a signal from the MBMS client that only a partial file is available, leads to a subsequent file request that contains indication of its acceptance of partial file content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to leverage partial file delivery to a compatible application to the fullest possible extent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 7.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.346 (Rel-15), CR S4-200980 |  |  |
|  |  | X |  | Other core specifications |  |  |  | TS 26.347 (Rel-15), CR S4-200984 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S4-200870 |  |  |  |  |  |  |  |  |

1st CHANGE: Modifications to clause 5.3.2

### 5.3.2	Partial File Handling

In certain cirumstances, for example in the case of MBMS download delivery of 3GP-DASH content, a file or segment may only be partially available on the HTTP server.

If the application, i.e. the 3GP file format client or the 3GP-DASH client supports the handling of partial files, it shall signal its capability using the partial-file-accept request defined in TS 26.346 [42], clause 7.9.2.1, i.e. using the Accept header application/3gpp-partial in combination with e.g. an HTTP GET request. If the application uses a partial-file-accept request, then the application shall also be able to handle the request response as defined in TS 26.346 [42], clause 7.9.2.2.

If the 3GP file format server or the 3GP-DASH server supports the handling of partial files, it should support the HTTP response format as defined in TS 26.346 [42], clause 7.9.2.2.

END OF 1st CHANGE

2nd CHANGE: Modifications to clause 7.3.8

### 7.3.8	Partial File Handling

3GP-DASH clients should support partial-file-accept requests and partial file responses as defined in clause 5.3.2. If 3GP-DASH clients support partial file handling they shall use partial-file-accept requests as defined in TS 26.346 [42], clause 7.9.2.1.

Without excluding other response options, as a response to a partial-file-accept request using a regular HTTP GET request a 3GP-DASH client may typically receive one of the following responses:

1)	200 OK with Content-Type set to the Media Type of the requested object

2)	200 OK with the Content-Type set to application/3gpp-partial and the message format according to the definition in clause 7.9.2.2 of TS26.346 [42].

3)	416 Requested Range Not Satisfiable with the additional information according to the definition in clause 7.9.2.2 of TS26.346 [42].

4)	404 Not Found

If the 3GP-DASH server supports the handling of partial files, then it should implement the HTTP response format as defined in TS 26.346 [42], clause 7.9.2.2. Consequently, a 3GP-DASH client may receive a response indicating 404 Not Found along with the Content-Type header set to ‘application/3gpp-partial’ as indication of partial file availability at the server.

Case 1 is the regular response.

Guidelines for handling request responses according to case 4 from above are provided in Annex A.7.

Guidelines for handling request responses 2 and 3 from above are provided in Annex A.9.

END OF 2nd CHANGE