3GPP TSG SA Meeting #88-e							SP-200403

Online, 30 June - 03 July 2020

3GPP TSG SA4 Meeting #108-e						S4-200567

Online, 02-09 April 2020

Agenda Item:		19.3

Source:			HEAD acoustics GmbH (Rapporteur)

Title:				Summary for WI ANTeM

WI code(s):			870010

Leading WG:		SA4

Release:			Rel-16

### 1	Introduction

The work item ANTeM [1] was subsequently initiated to the feasibility study FS_ANTeM [2] in order to follow the conclusion of TR 26.921 [3] regarding handset speech quality testing under ambient noise conditions. Results of the round robin test conducted in FS_ANTeM indicated that an alternative noise field simulation provides test results equivalent to the currently specified method, while having a more efficient equalization procedure and less variance across labs at the same time. To include this new method in TS 26.132 [4], a corresponding change request [5] was agreed.

### 2	Description

For speech quality evaluation of UEs under ambient noise conditions, the technical specification TS 26.132 [4] provides test methodologies. For measurements in handset mode, the background noise playback system according to ETSI ES 202 396-1 [6] is used. Within the study item FS_ANTeM, the applicability of the more recent ambient noise simulation according to ETSI TS 103 224 [7] was investigated, which provides the advantage of a fully automated calibration routine.

The so-called "flexible configurations" of ETSI TS 103 224 [7] were found to be useful, since they can be used in conjunction with the currently specified binaural noise types of TS 26.132 [4] to provide backward compatibility with the currently used simulation according to ETSI ES 202 396-1 [6].

The equivalency of the two noise field simulation systems was also shown by almost identical measurement results in the round robin test (comprehensive results of this study are provided in TR 26.921 [3]). The resulting CR [5] specified the flexible configurations of ETSI TS 103 224 [7] as an additional and preferred simulation method.

### 3	References

[1]	S4-200192: "New WID on Ambient noise test methodology for evaluation of acoustic UE performance".

[2]	S4-181230: "New SID on ambient noise test methodology for acoustic performance evaluation of UEs".

[3]	3GPP TR 26.921: "Investigations on ambient noise reproduction systems for acoustic testing of terminals".

[4]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[5]	S4-200305: "Alternative noise field simulation method for terminal testing" (CR 0102 to TS 26.132).

[6]	ETSI ES 202 396-1: "Speech and multimedia Transmission Quality (STQ); Speech quality performance in the presence of background noise; Part 1: Background noise simulation technique and background noise database".

[7]	ETSI TS 103 224: "Speech and multimedia Transmission Quality (STQ); A sound field reproduction method for terminal testing including a background noise database".