SA WG2 Meeting #S2-138E	S2-2002730

20 – 24 April, 2020, E-meeting	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0436 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS parameter mapping for TSN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current description of the TSN functionality defines how the 5QI is calculated in the PCF based on the TSN QoS parameters, but it is not described how tthe rest of the 5GS QoS parameters that are mandatory for a GBR QoS flow are calculated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A description on how the PCF calculates the GBR and ARP is included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description of the TSN QoS parameters to 5GS QoS profile mapping. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2246 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 6.1.3.23	Support of integration with Time Sensitive Networking

Time Sensitive Networking (TSN) support is defined in TS 23.501 [2], where the 5GS represents virtual TSN bridge(s) based on the defined granularity model. The TSN AF and PCF interact to perform QoS mapping as described in clause 5.28.4 of TS 23.501 [2].

The PCF provides the following parameters to the TSN AF:

-	5GS bridge information:

-	5GS Bridge address (unique MAC address that identifies the bridge used to derive the bridge ID);

-	UE-DS-TT Residence time;

-	Port Management Information Container and the related port number:

-	Ethernet port of DS-TT;

-	port number of the Ethernet port;

-	MAC address of the Ethernet port.

The TSN AF may use this information to construct IEEE managed objects, to interwork with IEEE TSN networks.

The TSN AF decides the TSN QoS information (i.e. priority, delay and maximum TSC Burst Size) based on the received the configuration information of 5GS Bridge from the CNC as defined in clause 5.28.2 of TS 23.501 [2], the bridge delay information at the TSN AF and the UE-DS-TT Residence time.

The PCF receives a request from the TSN AF that shall include:

-	UE MAC address (i.e. MAC address of the DS-TT port) for PDU session.

The PCF receives a request from the TSN AF that may include:

-	Service data flow filter containing the Flow Descriptions (e.g. Ethernet PCP, VLAN ID, destination MAC address of the TSN stream);

-	TSN QoS Parameters for the service data flow:

-	TSN QoS container: describes the TSC stream traffic characteristics (burst arrival time, periodicity, (both in reference to TSN GM), and Flow direction needed for TSCAI determination (as described in clauses 5.27 and 5.28 of TS 23.501 [2]);

-	TSN QoS information, i.e. priority, maximum TCS Burst Size and delay.

-	Port Management Information Container and target port number.

The PCF performs Session binding using the UE MAC address, and then the PCF derives the TSN QoS information into a 5QI. The PCF generates a PCC Rule with service data flow filter containing the Flow Descriptions provided by the TSN AF, the mapped 5QI, ARP, GBR and MBR and the associated TSC QoS container as received from the AF. The PCF derives the 5QI value as defined in TS 23.501 [2], clause 5.27.3. The PCF derives the GBR using the Maximum Bit Rate provided by the TSN AF and the ARP is assigned a value preconfigured for TSN services. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

NOTE:	TSC burst size can also represent the burst sizes of multiple TSN streams that have been aggregated.

* * * * End of Change * * * *