| 3GPP TS 28.536 V1.0.0 (2020-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4 	Communication service assurance service	7

4.1	Stage 2	7

4.1.1	Overview of interfaces	7

4.1.2	Model	8

4.1.2.1 	Imported and associated information entities	8

4.1.2.1.1 	Imported information entities and local labels	8

4.1.2.2	Class diagram	8

4.1.2.2.1	Relationships	8

4.1.2.2.2	Inheritance	8

4.1.2.3	Class definitions	9

4.1.2.3.1	AssuranceControlLoop	9

4.1.2.2.3	AssuranceGoalStatus	9

4.1.2.3.3	AssuranceControlLoopGoal <<ProxyClass>>	10

4.1.2.3.4	ObservationTimePeriod <<dataType>>	10

4.1.2.4	Attribute definitions	10

4.1.2.4.1	Attribute properties	10

4.1.2.4.2	Constraints	11

4.1.2.4.3	Notifications	11

4.1.2.5	Common notifications	12

4.1.2.5.1 	Alarm notifications	12

4.1.2.5.2	Configuration notifications	12

4.1.3	Procedures	12

4.1.3.1	SLS Assurance Procedure	12

4.2	Stage 3	13

4.2.1	Solution Set (SS) for JSON/YAML	13

Annex A (informative):	Control loop deployed in different layers	14

A.1 	Introduction	14

A.2 	Control loop in communication service layer	14

A.3 	Control loop in network slice layer	14

A.4 	Control loop in network slice subnet layer	15

A.5 Control loop in NF layer	15

Annex B (normative):	OpenAPI definition of the COSLA NRM	16

B.1	General	16

B.2	Solution Set (SS) definitions	16

B.2.1	OpenAPI document "coslaNrm.yml"	16

Annex C (informative):	Change history	19

.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document describes closed loop assurance solution enabling a service provider or an operator to continuously deliver the requested level of communication service quality to the customer and is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.535: Management Services for Communication Service Assurance; Requirements

TS 28.536: Management Services for Communication Service Assurance; Stage 2 and stage 3

The solution described builds upon the management services specifications as identified below:

TS 28.530: Management and orchestration; Concepts, use cases and requirements

TS 28.533: Management and orchestration; Architecture framework

TS 28.532: Management and orchestration; Generic management services.

TS 28.540: Management and orchestration; 5G Network Resource Model (NRM); Stage 1

TS 28.541: Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

TS 28.531: Management and orchestration; Provisioning

TS 28.545: Management and orchestration; Fault Supervision (FS)

TS 28.550: Management and orchestration; Performance assurance

TS 28.552: Management and orchestration; 5G performance measurements

TS 28.554: Management and orchestration; 5G End to end Key Performance Indicators (KPI)

# 1	Scope

The present document describes the management services for communication service assurance and specifies stage 2 and stage 3 for closed loop communication service assurance solution that adjusts and optimizes the services provided by NG-RAN and 5GC.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI GS ZSM 002 (V1.1.1) (2019-08): "Zero-touch network and Service Management (ZSM); Reference Architecture".

[3]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[4]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[9]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[10]	3GPP TS 32.160: "Management and orchestration; Management service template".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4 	Communication service assurance service

## 4.1	Stage 2

### 4.1.1	Overview of interfaces

Communication service assurance relies on a set of management services that together provide the CSP with the capability to assure the communication service as per agreement with a CSC (e.g. enterprise). The overall solution and information flows between management services and control steps [2] are shown in Figure 4.1.1.1.

Figure 4.1.1.1: Overview of information flows

In Figure 4.1.1.1 the controlled entity represents the resources used by a communication service and the assurance of this communication service is provided by the loop between the different management services provided by the management system. The input to the loop is the data concerning the resources used by the communication service which is monitored by the control step Monitor and the output of the step " Analysis and Decide" may be a possible action from the control step "Execute", when for example the service experience degrades, the resources used by a communication service have to be adjusted. The data associated with the communication service is monitored by the management services for data collection and threshold monitoring, the management service provides information to the assurance root cause analysis management service and based on that information the assurance root cause analysis takes place followed by propose mitigation or suggestion to solve the problem. The mitigation or problem-solving suggestion is executed to bring the behaviour of the communication service within the requested boundaries of the metrics (SLS goals) that are controlled by the loop.

NOTE: 	The interface for interaction between the capabilities in the Analyse and decide step is not addressed in this release.

The management services available between the control step for "Monito" and "Analyse" as well as "Decide" are based on file transfer described in TS 28.550 [3], or data streaming described in TS 28.550 [3] and notifications described in TS 28.545 [4].

Editor's Note: Further details including the relationship between the content of the figure "Overview of information flows", Trace as well as MDT are FFS

### 4.1.2	Model

#### 4.1.2.1 	Imported and associated information entities

##### 4.1.2.1.1 	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [5], IOC, Top | Top |
| TS 28.622 [5], IOC, SubNetwork | SubNetwork |
| TS 28.622 [6], ProxyClass, ManagedEntity | ManagedEntity |
| TS 28.541 [6], dataType, ServiceProfile | ServiceProfile |
| TS 28.541 [6], dataType, SliceProfile | SliceProfile |

#### 4.1.2.2	Class diagram

#### 4.1.2.2.1	Relationships

Figure 4.1.2.2.1.1: Assurance management NRM fragment

#### 4.1.2.2.2	Inheritance

Figure 4.1.2.2.2.1: Assurance management inheritance relationships

#### 4.1.2.3	Class definitions

##### 4.1.2.3.1	AssuranceControlLoop

4.1.2.3.1.1	Definition

This IOC represents the capabilities of a control loop, these include:

- 	to automatically adjust a ManagedEntity (for example a network slice) to meet the objective described in AssuranceControlLoopGoal

- 	to report the effectiveness of an AssuranceControlLoop

- 	state management of an AssuranceControlLoop

- 	to keep track of the lifecycle of an AssuranceControlLoop

4.1.2.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| controlLoopLifeCyclePhase | M | T | T | F | T |
| observationTimePeriod | M | T | T | F | T |
| assuranceGoalStatus | M | T | F | F | T |

4.1.2.3.1.3	Constraints

No constraints have been defined for this release.

4.1.2.3.1.4	Notifications

The common notifications defined in clause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.2.3	AssuranceGoalStatus

4.1.2.2.3.1	Definition

This class represents the status of the controlLoopGoal at the end of an observationPeriod. The status can be reported as actual status and predicted status. Data that is monitored by an assuranceControlLoop and includes measurements [x] and KPI's [y] and predictions that are applicable to the assuranceControlLoopGoals.

An assuranceGoalStatus holds the value of the observation and where applicable the value of a prediction. Depending on the AssuranceGoal the type of the AssuranceGoalStatusObserved and AssuranceGoalStatusPredicted can be different for different AssuranceGoalStatus MOIs.

4.1.2.2.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| AssuranceGoalStatusObserved | M | T | T | F | T |
| AssuranceGoalStatusPredicted | O | T | T | F | T |

.

4.1.2.3.2.3	Attribute constraints

No constraints have been defined for this release.

4.1.2.2.3.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.3	AssuranceControlLoopGoal <<ProxyClass>>

4.1.2.3.3.1	Definition

This IOC represents the <<dataType>> ServiceProfile and <<dataType>> SliceProfile, defined in network slice NRM in [6].

4.1.2.3.3.2	Attributes

The attributes are defined in network slice NRM in [6].

4.1.2.3.3.3	Attribute constraints

The attribute constraints are defined in network slice NRM in [6].

4.1.2.3.3.4	Notifications

The notifications of IOCs using the <<dataType>> ServiceProfile or <<dataType>> SliceProfile are defined in network slice NRM in [6].

##### 4.1.2.3.4	ObservationTimePeriod <<dataType>>

4.1.2.3.4.1	Definition

This datatype represents the time that a goal is observed which can be specified in seconds, minutes, hours or days depending on the goal that is being observed.

4.1.2.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| observationTime | M | T | T | F | T |
| timeUnit | M | T | T | F | T |

4.1.2.3.3.3	Attribute constraints

No constraints have been defined for this release.

4.1.2.3.3.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

#### 4.1.2.4	Attribute definitions

##### 4.1.2.4.1	Attribute properties

The following table defines the properties of attributes that are specified in the present document.

Table 4.1.2.4.1.1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| controlLoopLifeCyclePhase | It indicates the lifecycle phase of the ControlLoop. AllowedValues: Preparation, Commissioning, Operation and Decommissioning. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationTimePeriod | It indicates the time duration over which a controlLoopGoal is observed. During the observation period various observation data is collected to assess if the controlLoopGoal has been met The observation time is expressed in timeUnits. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeUnit | It indicates the unit of time used to express the observationTime AllowedValues: second, minute, hour, day | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationTime | It indicates the observation time expressed in number of timeUnit. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatus | It reports the status of the controlLoopGoal at the end of an observationPeriod. The status can be reported as actual status or predicted status. | type: <<dataType>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatusObserved | It indicates the actual value of the controlLoopGoal at the end of an observation period | type: AssuranceGoalStatusObserved multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatusPredicted | It indicates the predicted value of the controlLoopGoal at the end of an observation period see note 1, or of a future observation period, see note 2. | type: AssuranceGoalStatusPredicted multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: The predictive capability is provided by using a different population for assessment than the population for which measurements are available. NOTE 2: The predictive capability is provided by using a method for predicting the most likely status in the future. |  |  |

##### 4.1.2.4.2	Constraints

No constraints have been identified for this release

##### 4.1.2.4.3	Notifications

This subclause presents a list of notifications, defined in [7], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [10], would capture the DN of an instance of an IOC defined in the present document.

#### 4.1.2.5	Common notifications

##### 4.1.2.5.1 	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [7], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance, defined in TS 32.302 [8], shall capture the DN of an instance of a class defined in the present document.

##### 4.1.2.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [7], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance, defined in TS 32.302 [8], shall capture the DN of an instance of a class defined in the present document.

### 4.1.3	Procedures

#### 4.1.3.1	SLS Assurance Procedure

Figure 4.1.3.1.1

1. AssuranceControlLoop_consumer derives AssuranceControlLoopGoal from the ServiceProfile or SliceProfile.

2. AssuranceControlLoop_consumer provides the AssuranceControlLoopGoal to Entities_Participating_in_loop by utilizing the provision management services defined in as defined in clause 11.1.1.3 of TS 28.532 [7].

NOTE 1: 	In case the Entities_Participating_in_loop represents CrossDomain_Entities_Participating_in_loop, the AssuranceControlLoopGoal is the attribute(s) of the ServiceProfile. In case the Entities_Participating_in_loop represents Domain_Entities_Participating_in_loop, the AssuranceControlLoopGoal is the attribute(s) of the SliceProfile.

3. Entities_Participating_in_loop subscribes the related performance data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1 of TS 28.550 [3].

4. Entities_Participating_in_loop, optionally, subscribes the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 is used.

5. Entities_Participating_in_loop collects the related performance, fault, QoE and MDT data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1 of TS 28.550 [3].

6. Entities_Participating_in_loop, optionally, collects the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 of TS 29.520 [y] is used.

7. Entities_Participating_in_loop assesses if the AssuranceControlLoopGoal has been fulfilled.

8. Entities_Participating_in_loop assesses if and which action to take in case the AssuranceControlLoopGoal has not been fulfilled.

9. As per the mitigation action (e.g., scale out) resources are changed, the generic provisioning management service as defined in clause 11.1 of TS 28.532 [7] is utilized for the same.

10. Action completed.

NOTE 2: 	The Entities_Participating_in_loop continues to monitor and analyse the performance and perform the adjustment until the attribute(s) of SliceProfile is assured.

11. AssuranceControlLoop_consumer receives the confirmation of assurance fulfilment from Entities_Participating_in_loop by utilizing the provision management services defined in as defined in clause 11.1.1.3 of TS 28.532 [7].

## 4.2	Stage 3

### 4.2.1	Solution Set (SS) for JSON/YAML

The JSON/YAML solution set is documented in clause B.2.

Annex A (informative):
Control loop deployed in different layers

## A.1 	Introduction

This example gives a high-level view of control loops deployed in different layers, which consists of control loop in communication service layer, control loop in network slice layer, control loop in network slice subnet layer and control loop in NF layer, as described as Figure A.1.1, where the analytic could be leverage MDAS, and different control loops can provide input (interact with) to other control loops (in the same layer or different layers) and obtain the output from other control loops (in the same layer or different layers).

Figure A.1.1: Control loop in different layers

## A.2 	Control loop in communication service layer

SLA/SLS requirements provided from CSC are translated into serviceProfile, which represents the requirements for communication service assurance to the CSP. Coordination between control loop in communication service layer and control loop in network slice layer is needed to calculate the communication service resource requirements and to assure the communication service SLA/SLS requirements.

When the communication service is active, network slice performance is monitored and analysed for the communication service according to end user service experiences.

If service degradation occurs or it is predicted, the 3GPP management system could take actions, i.e. the allocated resources are scaled up or the SLS is adjusted based on pre-agreement/interaction between CSP and CSC.

## A.3	Control loop in network slice layer

After the network slice provisioning, the network slice performance (e.g. KPI, QoE) are monitored and reported. When network slice performance is not met, the 3GPP management system identifies the root cause and may reconfigure the resources according to analytical report from MDAS producer. The network slice resources are also modified accordantly in case the network slice performance requirement needs to be changed based on communication service requirement adjustment.

## A.4	Control loop in network slice subnet layer

The 3GPP management system could have the capability of service observation (e.g., the supervision to the NSSI) based on MDAS. Based on such observation, management actions on the NSSI might be performed if NSSI performance requirements fulfillment indicates a problem. Another possible scenario is that, when the NSSI performance requirement is changed because of the network slice modification management action, the NSSI resources might be also reconfigured.

## A.5	Control loop in NF layer

Editor's note: the control loop in NF layer is FFS.

Annex B (normative):
OpenAPI definition of the COSLA NRM

# B.1	General

This annex contains the OpenAPI definition of the COSLA NRM in YAML format.

The Information Service (IS) of the COSLA NRM is defined in clause 3.

Mapping rules to produce the OpenAPI definition based on the IS are defined in 3GPP TS 32.160 [10].

# B.2	Solution Set (SS) definitions

## B.2.1	OpenAPI document "coslaNrm.yml"

openapi: 3.0.3

info:

title: coslaNrm

version: 16.4.0

description:

OAS 3.0.1 specification of the Cosla NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.536 V16.4.0; 5G NRM, Slice NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.536/

paths: {}

components:

schemas:

#------------ Type definitions ---------------------------------------------------

ControlLoopLifeCyclePhase:

anyOf:

- type: string

enum:

- PREPARATION

- COMMISSIONING

- OPERATION

- DECOMMISSIONING

- type: string

TimeUnit:

anyOf:

- type: string

enum:

- SECOND

- MINUTE

- HOUR

- DAY

- type: string

OperationalState:

anyOf:

- type: string

enum:

- ENABLED

- DISABLED

- type: string

AdministrativeState:

anyOf:

- type: string

enum:

- LOCKED

- SHUTTING_DOWN

- UNLOCKED

- type: string

ObservationTime:

type: integer

ObservationTimePeriod:

type: object

AssuranceControlLoopGoal:

type: object

AssuranceGoalStatus:

type: object

AssuranceGoalStatusObserved:

type: object

AssuranceGoalStatusPredicted:

type: object

#-------- Definition of concrete IOCs --------------------------------------------

AssuranceControlLoop-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- type: object

properties:

operationalState:

$ref: '#/components/schemas/OperationalState'

administrativeState:

$ref: '#/components/schemas/AdministrativeState'

controlLoopLifeCyclePhase:

$ref: '#/components/schemas/ControlLoopLifeCyclePhase'

observationTimePeriod:

allOf:

- $ref: '#/components/schemas/ObservationTimePeriod'

- type: object

properties:

observationTime:

$ref: '#/components/schemas/ObservationTime'

timeUnit:

$ref: '#/components/schemas/TimeUnit'

AssuranceGoalStatus:

allOf:

- $ref: '#/components/schemas/AssuranceGoalStatus'

- type: object

properties:

assuranceGoalStatusObserved:

$ref: '#/components/schemas/AssuranceGoalStatusObserved'

assuranceGoalStatusPredicted:

$ref: '#/components/schemas/AssuranceGoalStatusPredicted'

managedEntity-Multiple:

$ref: '#/components/schemas/ManagedEntity-Multiple'

assuranceControlLoopGoal:

$ref: '#/components/schemas/AssuranceControlLoopGoal'

ManagedEntity-Single:

oneOf:

- $ref: 'sliceNrm.yaml#/components/schemas/NetworkSlice'

- $ref: 'sliceNrm.yaml#/components/schemas/NetworkSliceSubnet'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

AssuranceControlLoop-Multiple:

type: array

items:

$ref: '#/components/schemas/AssuranceControlLoop-Single'

ManagedEntity-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedEntity-Single'

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | SA5#126 |  |  |  |  | Skeleton | 0.0.0 |
| 2019-10 | SA5#127 | S5-196741 |  |  |  | pCR 28.536 Add scope | 0.1.0 |
| 2019-10 | SA5#127 | S5-196742 |  |  |  | pCR 28.536 Add introduction | 0.1.0 |
| 2019-10 | SA5#127 | S5-196744 |  |  |  | pCR 28.536 Introduce table of content | 0.1.0 |
| 2019-11 | SA5#128 | S5-197788 |  |  |  | pCR 28.536 Add Overview of interfaces | 0.2.0 |
| 2020-04 | SA5#130e | S5-202341 |  |  |  | pCR TS 28.536 Move and update Annex A Management control loop in different later from TS 28.535 | 0.3.0 |
| 2020-04 | SA5#130e | S5-202399 |  |  |  | pCR TS 28.536 Communication service assurance stage 2 solution | 0.3.0 |
| 2020-06 | SA5#131e | S5-203432 |  |  |  | pCR 28.536 Generic SLS Assurance Procedure | 0.4.0 |
| 2020-06 | SA5#131e | S5-203433 |  |  |  | TS 28.536 Add attributes to assurance control loop | 0.4.0 |
| 2020-06 | SA5#131e | S5-203428 |  |  |  | pCR TS 28.536 Remove CSI from clause 4.1.1 | 0.4.0 |
| 2020-06 | SA5#131e | S5-203434 |  |  |  | pCR TS 28.536 Add stage 3 solution | 0.4.0 |
| 2020-06 | SA#88-e | SP-200479 |  |  |  | EditHelp review Presented for information and approval | 1.0.0 |