# 3GPP TSG SA Meeting #88e	SP-200629

# e-meeting, 30 June - 3 July 2020	(revision of SP-200549_rev2)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0761 | rev | 3 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the use of SUPI as the Identity in EAP-AKA’ key derivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-Sec |  |  |  |  |  | Date: |  |  | 2019-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When EAP-AKA’ is used, it is unclear how SUPI is to be used as the input for the Identity in the MK key derivation using PRF’ KDF SUPI renamed to IMSI or NAI in Annex A.7.0 to avoid possible inconsistent defintion of SUPI compared to TS 23.003. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In alignment with how KAMF is derived in Annex A.7.0 of TS 33.501, it is clarified that SUPI Type is excluded from the Identity In Annex A.7.0, SUPI changed to IMSI or NAI in four places and clarification that characeter coding is applicable to both SUPI types (these changes have no technical impact but is just to clear up possible inconsistencies with the definition of SUPI in TS 23.003). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different interpretations by ME and the network implementations may lead to interoperability problems, leading to failure of EAP-AKA’ authentication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.0, F.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

## A.7.0	Parameters for the input S to the KDF

When deriving a KAMF from KSEAF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x6D

-	P0 = IMSI or NAI

-	L0 = P0 length - number of octets in P0

-	P1 = ABBA parameter

-	L1 = P1 length - number of octets in P1

The input key KEY shall be the 256-bit KSEAF.

For P0, when the SUPI type is IMSI, P0 shall be set to IMSI as defined in clause 2.2 of TS 23.003 [19]. For P0, when the SUPI type is network specific identifier, the P0 shall be set to Network Access Identifier (NAI) as defined in clause 28.7.2 of TS 23.003 [19]. P0 shall be represented as a character string as specified in B.2.1.2 of TS 33.220 [28], for both SUPI types.

For ABBA parameter values please refer to clause A.7.1.

**** NEXT CHANGE ****

# F.3	Subscriber identity and key derivation

EAP-AKA' uses the subscriber identity (Identity) as an input to the key derivation when the key derivation function has value 1 (i.e. MK = PRF'(IK'|CK',"EAP-AKA'"|Identity)). RFC 4187 [21] clause 7 describes that the Identity is taken from the EAP-Response/Identity or EAP-Response/AKA-Identity AT_IDENTITY attribute sent by the peer. This principle is not applied to the 5GS.

If the AT_KDF_INPUT parameter contains the prefix "5G:", the AT_KDF parameter has the value 1 and the authentication is not related to fast re-authentication, then the UE shall set the Identity for key derivation as follows. When the SUPI Type is IMSI, the Identity shall be set to IMSI as defined in clause 2.2 of TS 23.003 [19]. When the SUPI type is network specific identifier, the Identity shall be set to Network Access Identifier (NAI) as defined in clause 28.7.2 of TS 23.003 [19]. This principle applies to all full EAP-AKA' authentications, even if the UE sent SUCI in NAS protocol or if the UE sent SUCI in the response to the EAP identity requests as described in Table F.2-1 or if no identity was sent because the network performed re-authentication. The only exception is fast re-authentication when the UE follows the key derivation as described in RFC 5448 [12] for fast re-authentication.

NOTE 1:	The fast re-authentication is not supported in 5GS.

NOTE 2: 	The prefix "5G:" is part of serving network name as specified in clause 6.1.1.4.

**** END OF CHANGES ****