3GPP TSG-SA WG6 Meeting #44	S6-211665

e-meeting, 12nd July – 20th July 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0020 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the EAS status of EAS discovery filter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no value for the EEC to provide the EAS status in EAS discovery filter during EAS discovery request to EES. Based on the EAS status available at the EES, the EES determines the EAS(s) to be provided to the EEC as part of the EAS discovery response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EAS status of EAS discovery filter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the EAS status is set as disabled in the EAS discovery filter, and if the EES selects the list of EAS which are disabled and provide to the EEC, such EAS(s) cannot be used by the AC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Target DNAI (NOTE) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| NOTE: This IE shall not be included when the request originates from the EEC. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

* * * End of Change * * * *