3GPP TSG-SA WG6 Meeting #45	S6-211884

e-meeting, 25th August – 3rd September 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0283 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the supports of stored and forward functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MCData service supports stored and forward function when the UE is not available at the time of the service delivery to avoid potential data loss. It is part of the data delivery service that applies to all MCData services as stated in 5.2 Reception control “The MCData service may store data waiting for delivery in a temporary store and notify availability to the receiving MCData users”. The transmission and reception control procedurs are covered in clause 7.6.2 “Transmission and reception control for on-network”. There are 3 procedures to show how SDS and FD are covered and one procedure to show how to retrieve the deferred comunication. The stored and forward function is described in one of the FD procedure which gives the qrong impression that only the last procedure “send data without mandatory download” applies to the stored and forward. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the stored and forward description to the SDS procedure and “send data with mandatory download” prcedure to avoid possible mis-interpretation that the stored and forward only applies to the “send data without mandatory download”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will give wrong guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.2.2, 7.6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * First Change * * *

#### 7.6.2.2	Automatic transmission for SDS

##### 7.6.2.2.1	General

When SDS message is sent, the data is transferred from the sending MCData client to the receiving MCData client without any request/grant procedure or any explicit actions on the part of the receiving MCData client.

##### 7.6.2.2.2	Procedure

The procedure in figure 7.6.2.2.2-1 describes the case where MCData SDS is automatically transmitted to the selected recipient user or affiliated members of the selected MCData group.

Pre-conditions:

1.	MCData user is configured with permission to transmit data.

2.	Optionally, MCData client may have an activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.6.2.2.2-1: Automatic transmission for SDS

1.	MCData user selects the data to transmit and the recipient MCData user or MCData group. The MCData user at MCData client 1 may include a functional alias used within the SDS data transmission.

2.	MCData client checks for MCData user's permission to transmit data and checks if the data size is less than the maximum data size for SDS. MCData client calculates the total transmission request size and determines to select one of the appropriate procedures described in subclause 7.4.2.2, 7.4.2.3, 7.4.2.5 or 7.4.2.6.

3.	Transmission control on the MCData server verifies if the MCData user is authorized to transmit and the data size in the received transmission request does not exceed the maximum data size for SDS. MCData server verifies whether the provided functional alias, if present, can be used and has been activated for the MCData user.

4.	MCData server may send one of the following MCData control indications:

a)	If the MCData user does not have permission to transmit data to another MCData user or MCData group then the "No permission to transmit data" control indication is sent.

b)	If the selected transmission is for a MCData group and there are no affiliated group members, then the transmission control on MCData server sends the "No affiliated members found to the selected MCData group" control indication.

c)	MCData server may queue the data transmit request for later transmission with control indication "Request to transmit is queued".

Otherwise continue with step 5.

5.	MCData server automatically transmits the data and, if available, the functional alias of the originating MCData client 1 to the selected MCData user or the affiliated members or all group members (in the case of lossless communication) of the selected MCData group according to the procedures selected in step 2.

6.	The MCData server may store the data in temporary storage (e.g. if the recipient is not available at the time of data delivery or network congestion or data deferred by the user) and may send MCData indication to notify the recipient of available data for retrieval:

a)	If the timer expired for periodic announcement with the list of available recently invited data communications, the recipient MCData client waiting to receive the temporarily stored data receives MCData indication with the list of available temporarily stored data waiting to download in the Deferred data list .

b)	If the temporarily stored data is expired, the data may be purged from the temporary store and the recipient MCData user may be informed in MCData indication with "Data expired and not available to download anymore" in the Deferred data list.

* * Next Change * * *

#### 7.6.2.3	Send data with mandatory download

##### 7.6.2.3.1	General

The mandatory download procedure allows a sending MCData client to send data to a receiving MCData client, where the receiving MCData client is compelled to download the data.

##### 7.6.2.3.2	Procedure

The procedure in figure 7.6.2.3.2-1 describes the case where MCData user is using FD. The FD is subjected to transmission control prior to transmitting data to the selected recipient user or affiliated members of the selected MCData group for mandatory download.

Pre-conditions:

1.	MCData user is configured with permission to transmit data.

2.	MCData administrator has configured maximum data size for FD.

3.	Maximum amount of data that a MCData user can transmit in a single request is configured.

4.	Maximum amount of time that a MCData user can transmit in a single request is configured.

5.	Optionally, MCData client may have an activated functional alias to be used.

6.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.6.2.3.2-1: Send data with mandatory download

1.	MCData user selects the data to transmit, the recipient MCData user or MCData group and the indication for mandatory download. The MCData user at MCData client 1 may include a functional alias used within the FD data transfer.

2.	MCData client has checked for MCData user's permission to transmit data. MCData client calculates the total transmission request size and determines to select one of the appropriate procedures described in subclause 7.5.2.4, 7.5.2.5, 7.5.2.6 or 7.5.2.7 and including the mandatory download indication.

3.	Transmission control on the MCData server verifies if the MCData user is authorized to transmit, the data size in the received transmission request is within the maximum size allowed for FD transmission and is within the maximum amount of time that a MCData user can transmit in a single request. MCData server verifies whether the provided functional alias, if present, can be used and has been activated for the MCData user.

4.	MCData server may send one of the following MCData control indications:

a)	If the MCData user does not have permission to transmit data to another MCData user or MCData group then the "No permission to transmit data" control indication is sent.

b)	If the data size requested to be transmitted is more than the maximum amount of data that an MCData user can transmit in a single request, the transmission control on MCData server rejects the data transmission request and sends the rejection control indication "Rejected due to exceeding data transmit size".

c)	If the data transmission time exceeds the maximum amount of time that an MCData user can transmit in a single request, the transmission control on MCData server rejects the data transmission request and sends the rejection control indication "Rejected due to exceeding data transmit time".

d)	If the selected transmission is for a MCData group and there are no affiliated group members, then the transmission control on MCData server sends the "No affiliated members found to the selected MCData group" control indication.

e)	MCData server may queue the data transmit request for later transmission with control indication "Request to transmit is queued".

Otherwise continue with step 5.

5.	MCData server transmits the data and, if available, the functional alias of the originating MCData client 1 to the selected MCData user or the affiliated members or all group members (in the case of lossless communication) of the selected MCData group according to the procedures selected in step 2 (where the recipient MCData client receives the data automatically).

6.	The MCData server may store the data in temporary storage (e.g. if the recipient is not available at the time of data delivery or network congestion or data deferred by the user) and may send MCData indication to notify the recipient of available data for retrieval:

a)	If the timer expired for periodic announcement with the list of available recently invited data communications, the recipient MCData client waiting to receive the temporarily stored data receives MCData indication with the list of available temporarily stored data waiting to download in the Deferred data list .

b)	If the temporarily stored data is expired, the data may be purged from the temporary store and the recipient MCData user may be informed in MCData indication with "Data expired and not available to download anymore" in the Deferred data list.