3GPP SA Meeting #93-E 	SP-210997

Sep 14 - 20, 2021, Elbonia	(revision of SP-210xxx)

# Source:	Nokia, Nokia Shanghai Bell, Verizon, AT&T, Sennheiser, Huawei, HiSilicon, Matrixx, ZTE, China Unicom, Vivo, NTT Docomo, ETRI, Xiaomi, Orange, China Mobile, KDDI.

# Title:	New SID: Study on 5G Timing Resiliency and TSC enhancements

# Document for:	Information

Agenda Item:	6.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Study on 5G Timing Resiliency and TSC&URLLC enhancements

## Acronym: FS_5GTTUe

## Unique identifier: 	 ?

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 910039 | 5G Timing Resiliency System | Stage 1 work item |
| 800007 | Service requirements for cyber-physical control applications in vertical domains (cyberCAV) | Stage 1 work item |
|  | (Stage 1 work item for the above study to be added) |  |

## 3	Justification

Following are the justifications for the study objectives:

1) SA1 is also specifying requirements for 5G System to remain resilient if there is GNSS failure and for 5G System to act as a backup and offer wireless and indoor-capable time synchronization service for other applications (e.g. financial, power grid systems).

2) Generic TSC and exposure enhancements to 5GS for IP and ETH applications are needed for the following reasons:

- Current Exposure framework enables AF to request QoS parameters, provide traffic characteristics but not redundancy aspect which is important for some IP and ETH applications

3) Redundancy is most efficient when applied end to end.

4) It may be possible to apply TSN networks in the transport network. In such deployments, it may be beneficial to be able to interwork so that the transport network can also provide delay guarantees.

It has been raised whether RAN feedback on when the burst should be sent may be helpful. The performance benefit of such proposals can be studied, and the gain can be compared with the complexity of such solutions.

## 4	Objective

Following are the objectives for this study:

- Support for 5G Timing Resiliency requirements defined by SA1. Monitoring, detection and reporting of Time Source failure and consistent mitigation action within 5G System.

  - Study holdover capabilities within 5GS entities (i.e., system entities involved in time distribution or timestamping like UPF/NW-TT, gNB, UE/DS-TT)

  - Study exposure of 5GS support for timing resiliency and negotiation with 3rd parties and the ability for 3rd party to request to request resilient timing with specific KPIs (e.g., accuracy, coverage area, time interval).

- Study if any exposure enhancements to 5GS are needed for AF to request redundancy within the 3GPP network:

  - Ability for AF to influence the use of redundancy mechanisms within the 3GPP network, such as PDCP duplication or redundant transmission on N3/N9 (i.e. in addition to QoS) for a given application.

- Study the procedures needed to establish redundant end to end paths via the 3GPP network with endhosts that may be outside the 3GPP network:

  - The redundant paths via the 3GPP network may use for example Dual Connectivity based end to end Redundant User Plane Paths. Identify if any gaps exist in the 3GPP mechanisms which need to be addressed.

- Support for low latency and low jitter.

  - Study mechanisms for interworking with TSN transport networks. Study interworking mechanisms with TSN networks deployed in the transport network in order to support of E2E determinism and low latency communication and efficient N3 transmission

  - Study if it is beneficial to set the applications burst sending time based on feedback from RAN. Evaluate the expected performance improvements that may arise in comparison with the costs.

It is expected that the study will need 8 TUs:

1) Objective 1 (5G timing resiliency) – 2 TU(s)

2) Objective 2 (Exposure enh) – 2 TUs.

3) Objective 3 - 2 TUs.

4) Objective 4 - 2 TUs.

It is expected that the normative phase will need 7 TUs:

## 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.abc | Study on timing resiliency and TSC & URLLC enhancements | SA#96 June 2022(TBD) | SA#97 Sep 2022(TBD) | Devaki Chandramouli, Devaki.chandramouli@nokia.com |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

Devaki Chandramouli, Devaki.chandramouli@nokia.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Potential RAN impact to be covered by RAN WGs.

Potential security impact to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Verizon

AT&T

Sennheiser

Huawei

HiSilicon

Matrixx

ZTE

China Unicom

Vivo

NTT Docomo

ETRI

Xiaomi

Orange

China Mobile

Tencent

T-Mobile USA

Interdigital

Samsung

CATT

Deutsche Telekom

KDDI

Ericsson

Panasonic

Denso

BMWi

SK Telecom

Oracle

CBN

Broadcom

LG Electronics