TSG SA Meeting #SP-93E	SP-211100

14 - 20 September 2021, Electronic meeting                                revision of SP-210943

Source:	SA WG2

Title:	New WID: 'Architecture Enhancement for NR Reduced Capability Devices' [ARCH_NR_REDCAP]

Document for:	Approval

Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Architecture Enhancement for NR Reduced Capability Devices

Acronym:	ARCH_NR_REDCAP

Unique identifier:	930018

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-17

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900062 | Support of Reduced Capability NR Devices (NR_REDCAP) | RAN work item to support of Reduced Capability NR Devices |

Dependency on non-3GPP (draft) specification: none

3	Justification

The RAN WG has studied a UE feature and parameter list with lower end capabilities, relative to Release 16 eMBB and URLLC NR to serve the three use cases, including Industrial wireless sensors, Video Surveillance and Wearables. To support the Reduced Capability (RedCap) NR Devices, NR_REDCAP WID (RP-210918) is approved.

According to the current standard progress in RAN WG, the following potential system impacts on Extended DRX enhancements have been identified

- Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle. It is different from LTE solution for eDRX cycle = 10.24 s in RRC_IDLE, and will impact 5GC to handle the UE support both RedCap and LTE, e.g. on paging handling and registration area management.

- Extended DRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24 s for RRC_IDLE in NR/5GC.

- Extended DRX for Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for Idle need to be checked by CT1.

Beyond the system impacts on Extended DRX enhancements, the following system impacts also need to be considered:

- RedCap UE type identification and potential optimization in 5GC, e.g. the similar concept of NB-IoT/LTE-M traffic identification defined in EPC/5GC.

- The end-to-end functionality definition of RedCap UE in SA2, e.g. define which functionality needs to be supported or not for UE complexity reduction, similar to Annex M (informative): Functions and procedures over NB-IoT RAT in TS 23.401.

4	Objective

This Work Item has the following objectives to support Reduced Capability NR Devices:

- Support eDRX enhancements for RedCap UE, including:

  - Specify how to support Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH., e.g. potential enhancements on paging handling and/or registration area management.

  - Specify how to support extended DRX up to 10485.76 s for RRC_IDLE.

- Support RedCap UE type identification and potential optimization, e.g. RedCap traffic identification in 5GC.

- End-to-end functionality definition of RedCap UE, e.g. the functionality definition to support UE complexity reduction for RedCap UE, similar to Annex M (informative): Functions and procedures over NB-IoT RAT in TS 23.401.

NOTE 1: the SA2 work need to be coordinated with RAN WGs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A | N/A | N/A | N/A | N/A | N/A |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Specify system enhancements to support of Reduced Capability NR Devices | SA#94, Dec, 2021 | 23.501 |
| 23.502 | Specify system enhancements to support of Reduced Capability NR Devices | SA#94, Dec, 2021 | 23.502 |

6	Work item Rapporteur(s)

Aihua Li, China Mobile

liaihua@chinamobile.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

RAN2, RAN3, CT1 and CT4

9	Supporting Individual Members

Supporting IM name

China Mobile

Qualcomm

Huawei

Xiaomi

Nokia

China Unicom

CATT

ZTE

MediaTek Inc.

vivo

HiSilicon