TSG SA Meeting #SP-94E	SP-211308_rev1

14 - 20 December 2021, Electronic meeting

TSG SA Rel-18 Prioritization Workshop	SP-211145r02

9-10 December 2021, Electronic meeting

SA WG2 Meeting #S2-147E	S2-2108154

18 - 22 October, 2021, Electronic meeting	(revision of S2-21yyxxxx)

# Source:	Qualcomm Incorporated

# Title:	New SID on Study of Further Architecture Enhancement for UAV and UAM

# Document for:	Approval

Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study of Further Architecture Enhancement for UAV and UAM

## Acronym: FS_AEUA

## Unique identifier:	940051

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| ID_UAS | SA WG1 | 810049 | Remote Identification of Uncrewed Aerial Systems |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | The study analyzed and concluded on UAV connectivity, identification, and tracking. |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | SA2 WID created the basic supporting mechanisms for the UAS ecosystem |

## 3	Justification

SA1 TS 22.125 Rel. 17 has defined new requirements not covered by 3GPP SA2 work in ID_UAS in release 17. A gap analysis is needed to identify which requirements need to be supported, including:

-	providing means to USS to request and obtain real-time status information of a UAV regarding the communication link status and information regarding the service status (information of whether the communication service to the UAV can be provided with a certain QoS by the network) for UAVs in a certain geographical area and/or at a certain time (see R-6.2-001, R-6.2-002, R-6.3-002).

-	Monitoring by USS/UTM of the C2 communication with required QoS for pre-defined C2 communication models (see R-6.5.003).

NOTE: Aerial Joint Connectivity (ACJA), a joint GSMA-GUTMA activity, has defined in deliverable “ACJA – WT2: Interface for data exchange between MNOs and UTM ecosystem - NetworkCoverage Service Definition” a set of interfaces/services required for this purpose, which should be considered as input for the work.

SA1 requirements and the work being done in the aviation industry is applicable to traditional UAVs, but also to Urban Air Mobility (UAM) scenarios being standardized and regulated.

Rel. 17 FS_ID_UAS did not address the support of UAV-to-UAV communications using cellular technologies (e.g. PC5). The original Rel. 17 SID for UAS included the use of PC5 in “Study how the 3GPP system can provide support for UAV to UAV and UAV to ground identification and tracking of UAVs”, but this was deprioritized from rel.17 in TSG SA#86. It is necessary to study how the adoption of PC5 solutions for Broadcast Remote Identification and C2 communication would enable re-using 3GPP technologies for enabling this important building block in the deployment of UAVs and UAM for BVLOS and LOS.

Moreover, the aviation industry is developing advanced requirements for Detect And Avoid (DAA) solutions in a variety of aviation groups, to enable UAV-to-UAV communications to support DAA solutions to ensure safe co-existence of UAV and UAM in congested airspaces, considering both in-coverage and out-of-coverage scenarios and inter-PLMN scenarios. TS 22.125 supports these scenarios with requirements R-5.2.2-001, and R-5.2.2-004 to R-5.2.2-0011, which refer to UAV-to-UAV direct communication using 3GPP technologies (e.g. PC5).

## 4	Objective

The aim of this study work is to investigate and identify potential architecture and system level enhancements for 5GS and EPS to support additional scenarios and requirements for UAV (Uncrewed Aerial Vehicles) and UAM (Urban Air Mobility).

Specifically, the objectives include:

WT#1. Study the gaps and system architecture enhancements required to:

-	WT#1.1. Study how to transport Broadcast Remote Identification and C2 communications via the 3GPP system  by identifying how and whether existing mechanisms can be re-used, and identifying architectural and functional modifications required;

-	WT#1.2. Identify how and whether existing mechanisms can be re-used, and identify architectural and functional modifications required, in order to support aviation applications such as Detect And Avoid (DAA), etc.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 |  |  |  |  |
| WT#1.1 | 2 | 1.5 | Maybe | WT#2.1 is self-contained, but has synergies/overlaps with WT#2.2 |
| WT#1.2 | 1.25 | 0.75 | Maybe | WT#2.2 is self-contained, but has synergies/overlaps with WT#2.1 |

Total TU estimates for the study phase: 3.25

Total TU estimates for the normative phase: 2.25

Total TU estimates:  3.25 + 2.25 = 5.5

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 23.xyz | Study of further architecture enhancements for uncrewed aerial systems and urban air mobility | TSG#95 Mar. 2022 | TSG#96 Jun. 2022 |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.256 |  | TSG#97 Sept. 2022 |  |

## 6	Work item Rapporteur(s)

Stefano Faccin

sfaccin@qti.qualcomm.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA3 for the Security aspects, RAN for the RAN related issues.

## 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated

Verizon

AT&T

InterDigital

FirstNet

Futurewei

CMCC

Oppo

Nokia

Nokia Shanghai Bell

T-Mobile

Panasonic

SK Telecom

Samsung

Ericsson