TSG SA Meeting #SP-94E	SP-211318_rev2

14 - 20 December 2021, Electronic meeting

TSG SA Rel-18 Prioritization Workshop	SP-211164_rev1

9-10 December 2021, Electronic meeting

3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2109355

# Elbonia, November 15 – 22, 2021	(revision of S2-2108574)

Source:	Huawei, HiSilicon, Samsung, CATT, China Mobile, China Telecom, China Unicom, ZTE, Juniper, SK Telecom, KT Corp, LG Uplus, KPN, Siemens, Robert Bosch GmbH, LGE, Spreadtrum Communications, ETRI, CAICT, China Southern Power Grid, CEPRI, CBN, SIA, DISH Network, Volkswagen AG, Quectel

Title:	New SID on generic group management, exposure and communication enhancements

Document for:	Approval

Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on generic group management, exposure and communication enhancements

Acronym:	FS_GMEC

Unique identifier:	940061

Potential target Release: Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X |  |  |  |  |
| Don't know |  | X | X |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a Study Item

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_5GLAN_enh | SA2 | 840025 | Study on enhancement of support for 5G LAN-type service |
| Vertical_LAN | SA2 | 830042 | 5GS Enhanced support of Vertical and LAN Services |
| SEI | SA1 | 920039 | Smart Energy and Infrastructure |
| QoS_MON | SA1 | 790004 | QoS Monitoring |
| cyberCAV | SA1 | 800007 | Service requirements for cyber-physical control applications in vertical domains |
| eCAV | SA1 | 840050 | Enhancements for cyber-physical control applications in vertical domains |
| FS_Resident | SA1 | 880040 | Study of Enhancements for Residential 5G |
| PIRates | SA1 | 930029 | Personal IoT and Residential networks Service Requirements |
| EXPOSE | SA1 | 930023 | Service exposure interfaces for industry |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

3	Justification

3GPP has specified many enablers for vertical use, for example the features defined by IIOT, NPN and URLLC. Among them, R16 5G LAN-type service offers private communication using IP and/or non-IP type communications for UE and devices behind UE, using an optimized communication path controlled by a single SMF. However, the cardinality of SMF per 5G VN is limited to one in Rel-16, even if the 5G VN is very large. Multiple SMFs should be supported, for instance for administrative domains or a large multi-site company or a large scale industrial setting that spans multiple countries. And at the same time, the SMFs need to support single/common 5G VN wide area.

In Rel-18 SA1 study on 5G Smart Energy and Infrastructure “5SEI” has concluded new requirement (clause 6.13.2, 6.28 of TS 22.261 and clause 5.2, 5.6 and 9 of TS 22.104) including that the 5G system shall allow a UE to request a communication service to send data to different groups of UEs at the same time and the 5G system shall allow different QoS policy for each group the UE communicates with.

Currently, BUM (Broadcast, Unknown Unicast, Multicast) traffic for 5G VN are delivered via individual delivery (e.g. individual copies are delivered via individual PDU sessions) to all UEs in a VN (even if a UE is not interested in the traffic), and there are some open issues or restrictions on BUM traffic forwarding that needs to be addressed or removed, for example, "No support of forwarding packets with destination MAC address not known by SMF/UPF", " No support for forwarding a broadcast/multicast packet with source address not known to SMF/UPF", " Multicast group formation of selected members of a 5G VN for Ethernet type data communication is not described ", " Signalling scalability issues for large VN groups with lots of devices (MAC addresses) served by PDU sessions related with this VN group", "No support of loop-free due to topology changes", "restriction for maximum of 16 VLAN tags or maximum of allowed MAC addresses" etc.

Recently, the 5G Alliance for Connected Industries and Automation (5G-ACIA) has provided to 3GPP a whitepaper (S2-2102128) including a set of functional requirements that the 5GS has to satisfy in terms of supporting certain information exchange between 5GC and industrial application domain, and exposure of 5G capabilities. The main goal is to enable the management, operation, monitoring and use of such networks and network services from an enterprise perspective easily without having to rely on sophisticated, heavy-weight tools and in-depth knowledge on the underlying 5G technology. Some requirements regarding device management, e.g. connectivity management, connectivity monitoring, group management etc. have not yet been fulfilled and need further studies.

4	Objective

Work Task 1: Study 5G capabilities exposure for industrial and automation applications:

-	Work Task 1.1: Enhance group attribute management and group status event reporting:

-	 set/modify the group attributes: provisioning of service area or QoS applicable to each UE of a given group

-	 subscribe to group status event reporting for the event "newly registered or (de)-registered group member"

NOTE:	The above sub-WT#1.1 assumes that existing QoS mechanism and service area mechanism are re-used for enforcement of service area or QoS applicable to each UE of a given group, thus neither new QoS nor service area enforcement mechanism will be specified.

-	Work Task 1.2: Study whether and how to enhance NEF exposure framework to enable capability exposure for  provisioning of traffic characteristics and monitoring of performance characteristics applicable to each UE of a given group

NOTE:	It is assumed that the above sub-WT#1.2 focuses on the exposure enhancements and no new enforcement mechanisms will be specified for this sub-WT. The traffic characteristics include e.g., transfer interval, data volume per cycle time, average and peak date rates, silence time interval, and PDU Session Type. The performance characteristics include communication service availability, communication service reliability, end-to-end latency, service bit rate and packet error rate. Which traffic characteristics are relevant for 5GS and which performance characteristics need to be monitored will also be studied as part of this WT.

Work Task 2: enhancements of 5G VN group communication:

-	Work Task 2.1: Support group communication for a 5G VN which supports multiple SMFs, including support of SMF redundancy for reliability of the 5G VN group communication

Work Task 3: Whether additional mechanism or enhancement is needed and how to support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy (requirement regarding 5SEI as indicated in clause 6.13.2 of TS 22.261)

Work Task 4: Whether and how to support loop-free Ethernet topology in the case of Ethernet PDU Sessions

NOTE:	It is assumed that the above WT#4 will consider how to adapt 5GS to existing protocols to support loop-free.

Work Task 5: Whether and how to optimize BUM (Broadcast, Unknown Unicast, Multicast) traffic forwarding considering the open issues and restrictions in Rel-16 (clause 5.8.2.13.3 of TS 23.501)

Work Task 8: Support of dynamic management of groups where the list of members is not pre-defined based on their subscription data in UDM as described in WT#6 of FS_eEDGE_5GC_ph2

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 | 2.5 | 1.25 |  |  |
| WT#1.1 | 1 | 0.5 | No | self-contained |
| WT#1.2 | 1.5 | 0.75 | Maybe (SA lead) | self-contained |
| WT#2 | 3.25->2.75->0.75 | 1.25->1->0.25 |  |  |
| WT#2.1 | 0.75 | 0.25 | No | self-contained |
| WT#3 | 0.75 | 0.25 | No | self-contained |
| WT#4 | 1 | 0.5 | No | self-contained |
| WT#5 | 0.5 | 0.25 | No | self-contained |
| WT#8 | 1.5->0.5 | 0.75->0.25 | No | Depends on WT6 in Edge computing ph2 SID (S2-2108166) NOTE: The dependency might be removed based on discussion on further uses cases besides Edge computing ph2 SID. |

Total TU estimates for the study phase: 11->6

Total TU estimates for the normative phase: 5->2.75

Total TU estimates: 6 + 2.75 = 16->8.75

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on generic group management, exposure and communication enhancements | SA#96 (June. 2022) | SA#97 (Sept. 2022) | Zhu Qianghua, Huawei, zhuqianghua@huawei.com Sang-Jun Moon, Samsung, moonst@samsung.com |

6	Work item Rapporteur(s)

Qianghua Zhu, Huawei, zhuqianghua@huawei.com

Primary Rapporteur, Responsible for Work Task 1, 8

Sang-Jun Moon, Samsung, moonst@samsung.com

Secondary Rapporteur, Responsible for Work Task 2, 3, 4, 5

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA3 for security aspects. SA5 for management and charging aspects. SA6 for application layer aspects.

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Samsung

CATT

China Mobile

China Telecom

China Unicom

ZTE

Juniper

SK Telecom

KT Corp

LG Uplus

KPN

Siemens

Robert Bosch GmbH

LGE

Spreadtrum Communications

ETRI

CAICT

China Southern Power Grid

CEPRI

CBN

SIA

DISH Network

Volkswagen AG

Quectel